3GPP TSG RAN WG1#122	R1-2506577

Bengaluru, India, Aug 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR | 0029 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to incorporate new agreements on ISAC CM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Sensing_NR |  |  |  |  |  | Date: |  |  | 2025-08-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN1 #122, multiple agreements were made for the corrections on TR 38.901 v19.0.0. The reasons for all the changes are. - Missing abbreviations - Confusion on the definitions on RCS model 1 and RCS model 2 - Two sets of names of RCS components exist, i.e., A, B1, B2 and σM, σD, σS - Missing definition of the first aerial UE and the second aerial UE, and the relation of heights of the two aerial UEs - In Table 7.9.3-1, confusion in the determination of reference TRs for UMi/UMi-AV, UMa/UMa-AV, RMa/RMa-AV - In Table 7.9.3-4 and Table 7.9.3-5, it is confusing whether they refers to sensing scenarios or existing communication scenarios - Missing square in the parameter cd in Section 7.9.4.2 step 4, i.e., maxd3D-cd2+(ch-hTX)2c,0 - In 7.9.4.2, d3D is distance, cannot be used as delay - Error in polarization matrix generation in Clause 7.9.5.2 - In Table 7.9.6.3-1, the number of selected STX/SRX pairs can only be 2 or 1. - On UT distribution in Table 7.9.6.1-3, the reference TR 36.885 doesn’t have defined minimum inter-vehicle distance - The procedure on low power cluster generation is not correctly implemented according to the agreement made in RAN1 #121 - Confusion on the definition of Gamma distribution - In background channel generation for monostatic sensing in Clause 7.9.4.2, step 8 in section 7.5 is performed which breaks channel reciprocity - In target channel generation in Clause 7.9.4.1, the polarization matrix of target cannot fulfil channel reciprocity for monostatic sensing. - In background channel generation for monostatic sensing in Clause 7.9.4.2, confusion on indoor/outdoor property of RP compared to the BS. - In Table 7.9.1-3 and Table 7.9.6.1-2, the parameter “Minimum 3D distances between STX/SRX and sensing target” is not defined in TR 38.901 for certain cases. - For wording “between pairs of STX/SRX and sensing target”, it is confusing if it covers the pair of STX and SRX - The tdoc number of calibration results R1-2504950 is outdated - In Table 7.9.6.1-3, the minimum distance between STX/SRX and sensing target is not suitable - Multiple typos need correction |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes to TR 38.901 v19.0.0 are: - Add missing abbreviations - Add definitions on RCS model 1 and RCS model 2 - Align with the names of RCS components, i.e., replacing RCS components A, B1, B2 with σM, σD, σS - Add the missing definition of the first aerial UE and the second aerial UE, and the relation of heights of the two aerial UEs - In Table 7.9.3-1, group UMi/UMi-AV, UMa/UMa-AV, RMa/RMa-AV to avoid confusion on the determination of reference TRs - In Table 7.9.3-4 and Table 7.9.3-5, adding prefix “sensing scenario” to avoid confusion with existing communication scenarios in TR 38.901 - Add the missing square in the parameter cd in Section 7.9.4.2 step 4, i.e., maxd3D-cd2+(ch-hTX)2c,0 - In 7.9.4.2, correct d3D to d3Dc to represent delay - Correct the error in polarization matrix generation in Clause 7.9.5.2 - In Table 7.9.6.3-1, add statement that the number of selected STX/SRX pairs is 2 or 1. - On UT distribution in Table 7.9.6.1-3, correct the reference TR to 37.885 since it has defined minimum inter-vehicle distance - Correct the procedure on low power cluster generation based on the agreement made in RAN1 #121 - Adding definition of Gamma distribution to avoid confusion - In background channel generation for monostatic sensing in Clause 7.9.4.2, add statement that step 8 in section 7.5 should be skipped for channel reciprocity and add the correct behaviour - In target channel generation in Clause 7.9.4.1, add the limitation of the polarization matrix of target for reciprocity for monostatic sensing. - In background channel generation for monostatic sensing in Clause 7.9.4.2, clarify indoor/outdoor property of RP is same as the BS. - In Table 7.9.1-3 and Table 7.9.6.1-2, refer the parameter “Minimum 3D distances between STX/SRX and sensing target” to TR 38.858 for certain cases. - For wording “between pairs of STX/SRX and sensing target”, delete “pairs of” - The tdoc number for calibration results R1-2504950 is replaced by R1-2506616 - In Table 7.9.6.1-3, the minimum distance between STX/SRX and sensing target refers to 37.885 - Multiple typo corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion on the implementation of the channel model for ISAC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.9.1, 7.9.2.1, 7.9.4.0, 7.9.4.1, 7.9.4.2, 7.9.5.2, 7.9.5.5, 7.9.6.1, 7.9.6.2, 7.9.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged text omitted >

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

< Unchanged text omitted >

ZSD	Zenith angle Spread of Departure

ISAC	Integrated Sensing and Communication

RP	Reference Point

< Unchanged text omitted >

### 7.9.0	Introduction

The channel model for ISAC in Clause 7.9 is designed based on the channel model defined in the previous clauses within Clause 7 taking into account the known properties, e.g., location, Radar Cross-Section (RCS), polarization and etc. of one or more physical objects. A physical object is categorized as a sensing target (ST) or an environment object (EO). A ST is an object of interest for sensing. An EO is a non-target object with known location. Two types of EO are supported in the ISAC channel model. A first type of EO (type-1 EO) has similar characteristic as a ST and is modelled in the same way as a ST. In the following descriptions in Clause 7.9, only the related details on ST are described, which are also applicable to type-1 EO. A second type of EO (type-2 EO, 7.9.5.2) is of large size and is modelled differently from a ST.

The large scale and small scale parameters of the channel between any two of sensing transmitter (STX), ST and sensing receiver (SRX) in a sensing scenario are obtained from the Technical Reports for the same communication scenario unless updates on the parameter values are specially described. The ST is considered as a receiver or transmitter, respectively, in the determination of a proper channel model for a STX-ST link or a ST-SRX link.

The common framework for the ISAC channel between a pair of STX and SRX is composed of a component for one or multiple target channel(s) and a component for a background channel (described in Clause 7.9.4.3 and 7.9.5.3). One or multiple STs can be modelled in the channel between one pair of STX and SRX. The target channel of ST k includes all multipath components impacted by ST k, 1≤k≤K, where K is the number of STs. The background channel includes other multipath components not belonging to any of the K target channel(s).

### 7.9.1	Scenarios

Sensing scenario X is defined as a scenario for sensing where STX/SRX are selected among the TRPs and UEs in the corresponding communication scenario X. X can be UMi/UMa/RMa/InH/InF/UMi-AV/UMa-AV/RMa-AV/Urban grid/Highway/High Speed Train (HST). Based on the interested type of ST, the sensing scenarios can be grouped to UAV sensing scenarios (ISAC-UAV), Automotive sensing scenarios (ISAC-Automotive), Human sensing scenarios (ISAC-Human), Automated Guided Vehicles (AGV) sensing scenarios (ISAC-AGV), and Objects creating hazards on roads/railways sensing scenarios (ISAC-Objects creating hazards on roads/railways).

The detailed sensing scenario description in this clause can be used for channel model calibration.

ISAC-UAV

In the ISAC-UAV sensing scenarios, the sensing targets are outdoor UAVs below or above the buildings in urban or rural areas. Monostatic or bistatic sensing can be performed using TRPs and/or UEs, including UEs on other UAVs. Details on ISAC-UAV scenarios are listed in Table 7.9.1-1.

Table 7.9.1-1: Evaluation parameters for UAV sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | UMi, UMa, RMa, SMa UMi-AV, UMa-AV, RMa-AV [36.777] |
| Sensing transmitters and receivers properties | STX/SRX Locations | STX/SRX locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. see note 1 |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h see note 2 and 3 |
|  | 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. N = {1, 2, 3, 4, 5} see note 4 Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | Size: Option 1: 1.6m x 1.5m x 0.7m Option 2: 0.3m x 0.4m x 0.2m |
| Minimum 3D distances between STX/SRX and sensing target |  | Min distances based on min TRP/UE distances defined in TR36.777. |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a target Option 2: 10 meters |
| NOTE 1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other STX/SRX properties (e.g. mobility) are also taken from the corresponding communication scenario. NOTE 2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target NOTE 3: time-varying velocity may be considered for future evaluations. NOTE 4: N=0 may be considered for the evaluation of false alarm |  |  |

ISAC-Automotive

In the ISAC-Automotive sensing scenarios, the sensing targets are passenger vehicles or trucks and buses traveling on roads and streets in urban and rural areas. Monostatic or bistatic sensing can be performed using TRPs and/or UEs, including UEs on other vehicles and roadside UEs (RSU-type UEs). Details on ISAC-Automotive scenarios are listed in Table 7.9.1-2.

Table 7.9.1-2: Evaluation parameters for Automotive sensing scenarios

| Parameters |  | Values |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban Grid. UMi, UMa, RMa, SMa |
| Sensing transmitters and receivers properties |  | STX/SRX locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. Additional option: ISD between TRPs of Urban Grid is 250m |
| Sensing target | LOS/NLOS | LOS and NLOS (including NLOSv) |
|  | Outdoor/indoor | Outdoor |
|  | Mobility (horizontal plane only) | Based on TR37.885 mobility for urban grid or highway scenario |
|  | Distribution (horizontal) | Based on dropping in TR37.885 per urban grid or highway communication scenario |
|  | Orientation | Lane direction in horizontal plane |
|  | Physical characteristics (e.g., size) | Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 |
| Minimum 3D distances between STX/SRX and sensing target |  | For Highway, Urban Grid - Min distances based on min TRP/UE distances defined in TR37.885 For UMi, UMa, RMa, SMa - Min distances based on min TRP/UE distances defined in TR38.901 |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 10 m. |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | Type-2 EO for Urban Grid - up to 4 walls modelled as Type-2 EO, per building of size 413m x 230m x 20m. |

ISAC-Human

In the ISAC-Human sensing scenarios, the sensing targets are children and adult persons in indoor (room, office, factory) and outdoor (urban, rural) locations. Monostatic or bistatic sensing can be performed using TRPs and/or UEs in the corresponding communication scenarios. Details on ISAC-Human scenarios are listed in Table 7.9.1-3.

Table 7.9.1-3: Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Indoor office, indoor factory Indoor room [TR38.808] | UMi, UMa, RMa, SMa |
| Sensing transmitters and receivers properties | STX/SRX Locations | STX/SRX locations are selected among the TRPs and UE locations in the corresponding communication scenario | STX/SRX locations are selected among the TRPs and UE locations in the corresponding communication scenario |
|  | STX/SRX Mobility for UEs | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/h | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/h |
| Sensing target | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/h (horizontal plane with random direction straight-line trajectory) | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/h (horizontal plane with random direction straight-line trajectory) |
|  | 3D distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment see note 1 | Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. see note 1 |
|  | Orientation | Random over the horizontal area | Random over the horizontal area |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): Child: 0.2m x 0.3m x 1m Adult Pedestrian: 0.5m x 0.5m x 1.75m | Size (Length x Width x Height): Child: 0.2m x 0.3m x 1m Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Minimum 3D distances between STX/SRX and sensing target |  | For TRP monostatic and TRP-TRP bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 0 m For UE monostatic and UE-UE bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 1 m (as specified in TR 38.858) | For TRP monostatic and TRP-TRP bistatic sensing modes, the minimum 2D distances between STX/SRX and the sensing target are 10 m for UMi scenarios and 35 m for UMa scenarios For UE monostatic and UE-UE bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 1 m (as specified in TR 38.858) |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 1m | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 1m |
| NOTE 1: N=0 may be considered for the evaluation of false alarm |  |  |  |

ISAC-AGV

In the ISAC-AGV sensing scenarios, the sensing targets are automated guided vehicles (AGVs) inside a factory. Monostatic or bistatic sensing can be performed using TRPs and/or UEs in the corresponding communication scenario. Details on ISAC-AGV sensing scenarios are listed in Table 7.9.1-4.

Table 7.9.1-4: Evaluation parameters for AGV sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | InF (Table 7.8-7) |
| Sensing transmitters and receivers properties |  | STX/SRX location are selected among the TRPs and UEs location in the corresponding communication scenario STX/SRX Mobility for UEs Option 1: 0 km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/h |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | 3D mobility | Horizontal velocity with random straight-line trajectory Option 1: Uniform distribution in the range of up to 30 km/h Option 2: Fixed velocities {3, 10} km/h |
|  | 3D distribution | Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane |
|  | Orientation | Horizontal plane only |
|  | Physical characteristics (e.g., size) | Size Option 1: 0.5m x 1.0m x 0.5m Option 2: 1.5 m x 3.0m x 1.5 m |
| Minimum 3D distances between STX/SRX and sensing target |  | Min distances based on min TRP/UE distances defined in TR38.901 |
| Minimum 3D distance between sensing targets |  | Option A: At least larger than the physical size of a target Option B: Fixed value, 5m |

ISAC-Objects creating hazards on roads/railways

In the ISAC-Objects creating hazards on roads/railways sensing scenarios, the sensing targets are adult humans and children and animals in communication scenarios involving vehicles or high-speed trains. Monostatic or bistatic sensing can be performed using TRPs and/or UEs, including UEs on other vehicles and roadside UEs (RSU-type UEs). Details on ISAC- Objects creating hazards on roads/railways sensing scenarios are listed in Table 7.9.1-5.

Table 7.9.1-5: Evaluation parameters for objects creating hazards on roads/railways sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban grid, HST (High Speed Train, TR 38.802) UMi, UMa, RMa, SMa |
| Sensing transmitters and receivers properties | STX/SRX Locations | STX/SRX locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenarios. Additional option: ISD between TRPs of Urban Grid is 250 m |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: up to 10 km/h for humans and animals |
|  | 3D distribution | Uniformly distributed in horizontal plane |
|  | Orientation | Random distribution in horizontal plane |
|  | Physical characteristics (e.g., size) | For human/pedestrians: Child: 0.2m x 0.3m x 1m Adult: 0.5m x 0.5m x 1.75m For animals: Size: 1.5m x 0.5m x 1 m |
| Minimum 3D distances between STX/SRX and sensing target |  | For Highway, Urban Grid, HST - Min. distance is based on min TRP/UE distances defined in TR37.885 and TR38.802 For UMi, UMa, RMa, SMa - Min distances based on min TRP/UE distances defined in TR38.901 |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 10 m. |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | Type-2 EO for Urban Grid - up to 4 walls modelled as Type-2 EO, per building of size 413m x 230m x 20m. |

< Unchanged text omitted >

### 7.9.2	Physical object model

#### 7.9.2.0	Introduction

A ST is modelled with one or multiple scattering points. Each scattering point of a ST (SPST) is used to model the total scattering effects of some adjacent scattering centres at the ST. The impact of a SPST to the channel includes at least two aspects, i.e., the RCS (Radar Cross Section, 7.9.2.1) and the polarization matrix (7.9.2.2).

#### 7.9.2.1	RCS of a sensing target

< Unchanged text omitted >

A first RCS model, called RCS model 1, is to characterize a ST as single SPST with component σD of the monostatic RCS values fixed to 1. σM is the mean of linear monostatic RCS values at the SPST. For UAV of small size and human with RCS model 1, the values/pattern 10lgσMσD, denoted as σMD_dBθi,ϕi,θs,ϕs, of the RCS for a SPST is deterministic based on the incident angle (θi,ϕi,) and the scattered angle (θs,ϕs).

< Unchanged text omitted >

A second RCS model, called RCS model 2, adopts an angular dependent component σD of the monostatic RCS values for each SPST, and is to split a ST into single or multiple SPSTs. For UAV with large size and human, single SPST is modelled. While for vehicle and AGV, both models with single and multiple SPSTs are provided. For vehicle and AGV modelled with multiple scattering points, the recommended five scattering points are located at the front, left, back, right and roof side of the vehicle respectively. The orientation of a ST in LCS is provided as follows.

< Unchanged text omitted >

#### 7.9.2.2	Cross-polarization matrix of a sensing target

The cross-polarization matrix CPMsp,i of a SPST for a pair i of incident/scattered angles is defined in LCS of the ST and is generally modelled by amplitude factors αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e.,

< Unchanged text omitted >

### 7.9.3	Reference channel models and required updates

A transmitter or receiver in the sensing operation can be TRP, terrestrial UE, vehicle UE, aerial UE, AGV UE or RSU-type UE. The reference TR(s) to generate the channel for each combination of transmitter and receiver for each sensing scenario are provided in Table 7.9.3-1, where the terrestrial UE, AGV are referred as terrestrial UE.

Table 7.9.3-1: Reference TRs to generate channel for ISAC

| Case | Tx/Rx | Rx/Tx | Reference TR to define the channel model |
| --- | --- | --- | --- |
| 1 | TRP | TRP | For sensing scenario UMi/UMi-AV, UMa/UMa-AV, RMa/RMa-AV, InH, InF, : - TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - For InF, hUE is changed to the same height as the BS - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hUE=35m (see note 1) For sensing scenario Highway - TRP-UE link of scenario RMa in Clause 7 of TR 38.901 by setting hUE=35m for FR1(see note 1) - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 For sensing scenario Urban grid - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 For sensing scenario HST - TRP-UE link of scenario RMa in Clause 7 of TR 38.901 by setting hUE=35m for FR1 (see note 1) - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 for FR2 |
| 2 | TRP | terrestrial UE | For sensing scenario UMi/UMi-AV, UMa/UMa-AV, RMa/RMa-AV, InH, InF:, - TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in Clause 7 of TR 38.901 For sensing scenario Highway and Urban grid - P2B link of scenario Highway and Urban grid in Clause 6 of TR 37.885 For sensing scenario HST - TRP-UE link of scenario RMa in Clause 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in Clause 7 of TR 38.901 for FR2 |
| 3 | TRP | vehicle UE | For sensing scenario Highway and Urban grid - V2B link of scenario Highway and Urban grid in Clause 6 of TR 37.885 For sensing scenario UMi, UMa, and RMa - TRP-UE link of scenario UMi, UMa, and RMa in Clause 7 of TR 38.901 |
| 4 | TRP | aerial UE | For sensing scenario UMa-AV, UMi-AV, and RMa-AV - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in Clause Annex A and B of TR 36.777 for FR1 - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |
| 5 | terrestrial UE | terrestrial UE | For sensing scenario UMi/UMi-AV, UMa/UMa-AV, RMa/RMa-AV, InH, InF : - UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hBS =1.5m (see note 2) For sensing scenario Highway and Urban grid - P2P link in Clause 6 of TR 37.885 For sensing scenario HST - TRP-UE link of scenario RMa in Clause 7 of TR 38.901 for FR1, e.g., hBS=1.5m, - UE-UE link of scenario UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 for FR2 |
| 6 | terrestrial UE | vehicle UE | For sensing scenario UMi, UMa, RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hBS =1.5m For sensing scenario Highway and Urban grid - V2P link in Clause 6 of TR 37.885 |
| 7 | terrestrial UE | aerial UE | For sensing scenario UMi-AV, UMa-AV, and RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1 - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |
| 8 | vehicle UE | vehicle UE | For sensing scenario Highway and Urban grid - V2V link of scenario Highway and Urban grid in Clause 6 of TR 37.885 For sensing scenario UMi, UMa, and RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hBS =1.5m |
| 9 | aerial UE | aerial UE | For sensing scenario UMi-AV, UMa-AV, RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE for FR1 (see note 4) - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |
| 10 | TRP | RSU-type UE | Highway and Urban grid - B2R link in Clause 6 of TR 37.885 |
| 11 | RSU-type UE | normal UE | Highway and Urban grid - V2V link in Clause 6 of TR 37.885, with antenna height at RSU is 5m |
| 12 | RSU-type UE | RSU-type UE | Highway and Urban grid - V2V link in Clause 6 of TR 37.885, with antenna height at RSU is 5m |
| 13 | RSU-type UE | vehicle UE | Highway and Urban grid - V2V link in Clause 6 of TR 37.885, with antenna height at RSU is 5m |
| NOTE 1: ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 NOTE 2: ASD and ZSD statistics updated to be the same as ASA and ZSA NOTE 3: Indoor office scenario can be categorized into 5 sub-indoor scenarios defined in TR38.808. NOTE 4: First aerial UE height is hUT1, second aerial UE height is hUT2, where abshUT1-hBS≤abshUT2-hBS. |  |  |  |

< Unchanged text omitted >

Table 7.9.3-4: LOS condition determination for Case 7

| Reference scenario to determined LOS probability | Applicability range in terms of aerial UE height |
| --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for sensing scenario UMi-AV/UMa-AV/RMa-AV | Sensing scenario UMi-AV, UMa-AV: 1.5m≤hUT≤22.5m Sensing scenario RMa-AV: 1.5m≤hUT≤10m |
| UMi-AV in Table B-1 in TR 36.777 for sensing scenario UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for RMa-AV | Sensing scenario UMi-AV, UMa-AV: 22.5m<hUT≤300m Sensing scenario RMa-AV: 10m<hUT≤300m |

Table 7.9.3-5: LOS condition determination for Case 9

| Reference scenario to determined LOS probability | Applicability range in terms of aerial UE height |
| --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for sensing scenario UMi-AV/UMa-AV/RMa-AV | Sensing scenario UMi-AV, UMa-AV: 1.5m≤hUT1,hUT2≤22.5m Sensing scenario RMa-AV: 1.5m≤hUT1,hUT2≤10m |
| UMi-AV in Table B-1 in TR 36.777 for sensing scenario UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for sensing scenario RMa-AV | Sensing scenario UMi-AV, UMa-AV: 22.5m<maxhUT1,hUT2≤300m, 1.5m≤minhUT1,hUT2≤22.5m Sensing scenario RMa-AV: 10m<maxhUT1,hUT2≤300m, 1.5m≤minhUT1,hUT2≤10m |
| UMa-AV in Table B-1 in TR 36.777 for aerial UE height in range (22.5m, 100m] for sensing scenario UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for aerial UE height in range (10m, 40m] for sensing scenario RMa-AV | Sensing scenario UMi-AV, UMa-AV: 22.5m<hUT1,hUT2≤100m Sensing scenario RMa-AV: 10m<hUT1,hUT2≤40m |
| LOS probability is 100% | Sensing scenario UMi-AV, UMa-AV: 100m<maxhUT1,hUT2≤300m, 22.5m<minhUT1,hUT2≤100m Sensing scenario RMa-AV: 40m<maxhUT1,hUT2≤300m, 10m<minhUT1,hUT2≤40m |

< Unchanged text omitted >

### 7.9.4	Fast fading model

#### 7.9.4.0	Introduction

< Unchanged text omitted >

Step 1: Set environment, network layout, and antenna array parameters

a)	Choose one of the sensing scenarios (ISAC-UAV, ISAC-Automotive, ISAC-Human, ISAC-AGV, ISAC-Objects creating hazards on roads/railways) and related communication scenarios (e.g. UMa, UMi-Street Canyon, RMa, InH-Office, Urban grid, Highway, or InF). Choose a global coordinate system and define zenith angle θ, azimuth angle ϕ, and spherical basis vectors θ, ϕ as shown in Figure 7.5-2.

b)	Give number of STX, ST and SRX. Give the number of the SPST(s) of each ST.

< Unchanged text omitted >

#### 7.9.4.1	Target channel

Following Step 1 in Clause 7.9.4.0, the target channels for the one or multiple STs between a pair of STX and SRX is generated using the following procedure with parameters derived by Table 7.9.3-2. It assumes a ST k consists of P scattering points indexed p=0,1,..P-1, P≥1. The propagation conditions for different STX-SPST links and SPST-SRX links can be modelled by spatial consistency procedure in Clause 7.9.5.1.

< Unchanged text omitted >

Step 6: Generate cluster powers.

The cluster power of cluster n in a STX-SPST link are generated using Step 6 of Clause 7.5, i.e., Ptx,nk,p=Pn.

The cluster power of cluster n' in the corresponding SPST-SRX link of the same SPST are generated using Step 6 of Clause 7.5 by replacing subscript n with n’, i.e., Prx,n'k,p=Pn'.

< Unchanged text omitted >

Coefficient generation:

Step 13: Draw initial random phases for paths in set R.

The random initial phases for each ray m of a cluster n in a STX-ST link is generated using Step 9 of Clause 7.5, i.e., Φtx,n,mk,p,θθ,Φtx,n,mk,p,θϕ,Φtx,n,mk,p,ϕθ,Φtx,n,mk,p,ϕϕ=Φn,mθθ,Φn,mθϕ,Φn,mϕθ,Φn,mϕϕ

The random initial phases for each ray m’ of a cluster n’ in a ST-SRX link is generated using Step 9 of Clause 7.5 by replacing subscript n, m with n’, m’, i.e., Φrx,n',m'k,p,θθ,Φrx,n',m'k,p,θϕ,Φrx,n',m'k,p,ϕθ,Φrx,n',m'k,p,ϕϕ=Φn',m'θθ,Φn',m'θϕ,Φn',m'ϕθ,Φn',m'ϕϕ.

For monostatic sensing mode, Φtx,n,mk,p,θθ,Φtx,n,mk,p,θϕ,Φtx,n,mk,p,ϕθ,Φtx,n,mk,p,ϕϕ are respectively equal to Φrx,n',m'k,p,θθ,Φrx,n',m'k,p,θϕ,Φrx,n',m'k,p,ϕθ, Φrx,n',m'k,p,θϕ, Φrx,n',m'k,p,ϕϕ if n=n' and m=m'.

Draw random initial phases Φn',m',m,nk,p,θθ,Φn',m',m,nk,p,θϕ,Φn',m',m,nk,p,ϕθ,Φn',m',m,nk,p,ϕϕ for each path  (k,p,n',m',n,m) in set R at SPST p and for four different polarisation combinations (θθ, θϕ, ϕθ, ϕϕ). The distribution for initial phases is uniform within (-π, π). Φn',m',m,nk,p,θθ,Φn',m',m,nk,p,θϕ,Φn',m',m,nk,p,ϕθ,Φn',m',m,nk,p,ϕϕ are independently determined for each path of a SPST of the same ST and of the different STs. Φn',m',m,nk,p,θθ,Φn',m',m,nk,p,θϕ,Φn',m',m,nk,p,ϕθ,Φn',m',m,nk,p,ϕϕ of a path of a SPST keeps unchanged until it is updated.

Step 14: Generate channel coefficients for paths in set R for each receiver and transmitter element pair u, s.

The channel coefficient for a path (k,p,n',m',n,m) in set R is generated by

Hu,s,n',m',n,mk,pt=Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pdθθ2+dϕϕ22

∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pexpj2πrrx,k,p,n',m'Ttt.drx,u+rtx,k,p,n,mTtt.dtx,sλ0expj2πt0tfD,n',m',n,mk,ptdt	(7.9.4-4)

with the Doppler frequency fD,n',m',n,mk,pt defined as

fD,n',m',n,mk,pt=rrx,k,p,n',m'Ttvrxt+rk,p,n',m'Ttvk,ptλ0+rtx,k,p,n,mTtvtxt+rk,p,n,mTtvk,ptλ0	(7.9.4-5)

where,

-	CPMn',m',n,mk,p is the polarization matrix of the SPST p.

CPMn',m',n,mk,p=expjΦn',m',m,nk,p,θθκn',m',n,mk,p-1expjΦn',m',m,nk,p,θϕκn',m',n,mk,p-1expjΦn',m',m,nk,p,ϕθexpjΦn',m',m,nk,p,ϕϕ	(7.9.4-6)

CPMn,m, n',m'=k,pCPMn',m',n,mk,pTif paths (k,p,n,m,n',m') and (k,p,n',m',n,m) are in set R for mono-static sensing mod.

< Unchanged text omitted >

-	vk,pt is the velocity of SPST p of ST k, vk,pt=vma,kt+vmi,k,pt, where vma,kt is the velocity of the ST k, vmi,k,pt is velocity due to micro motion of SPST p of ST k.

-	t0 denotes a reference point in time that defines the initial phase, e.g., t0=0.

< Unchanged text omitted >

#### 7.9.4.2	Background channel

For TRP-TRP, TRP-UE, UE-TRP and UE-UE bistatic sensing modes, following Step 1 in Clause 7.9.4.0, the background channel between a pair of STX and SRX is generated using Step 2 to Step12 of Clause 7.5 with parameters derived by Table 7.9.3-3. The absolute time of arrival in clause 7.6.9 is applied. The resulting channel is denoted as Hu,sbkτ,t.

For TRP monostatic and UE monostatic sensing modes, the background channel between a pair of STX and SRX is generated using the following steps after Step 1 in Clause 7.9.4.0.

Step 2: Generate 3 reference points (RPs) for the STX/SRX.

Draw the 2D distance between the STX/SRX and each RP and the height of each RP respectively GΓαd,βd+cd and GΓαh,βh+ch with parameters defined in Table 7.9.4.2-1/2. Note that Gαd,βd and Gαh,βh are random values generated by the shape-rate parameterized Gamma distribution fx|α,β=βαΓ(α)xα-1e-βx. With uniform distribution within range -π,π, draw the LOS AOD (ϕLOS,AOD) between the STX/SRX and the first RP. The LOS AOD is further rotated by 23π and 43π to respectively derive the LOS AOD from the STX/SRX to the second and third RPs. Consequently, the 3D location of each RP can be calculated.

< Unchanged text omitted >

Step 3: Assign NLOS propagation condition to the channel between the STX/SRX and each of the 3 RPs. Note: O2I is not applicable.

Step 4: Generate the channel between the STX/SRX and each of the 3 RPs using Step 3 to Step12 of Clause 7.5 and Clause 7.6.10 with parameters derived by Table 7.9.3-3, with updates as follows.

-	In Step 7 of Clause 7.5, the arrival angles ϕn,m,AOA,θn,m,ZOA are respectively equal to the departure angles ϕn,m,AOD, θn,m,ZOD. The rays in a cluster with θn,m,ZOA less than 50, 80 and 90 degrees respectively for scenario UMi, UMa and RMa are dropped. The dropping is not applicable to other sensing scenarios.

- Step 8 of Clause 7.5 is not applicable, except that the random coupling of AOD angles ϕn,m,AOD with ZOD angles θn,m,ZOD within a cluster n or within a sub-cluster in the case of two strongest clusters. Apply the same coupling in the corresponding arrival angles ϕn,m,AOA,θn,m,ZOA.

-	The absolute time of arrival in clause 7.6.9 is applied with d3Dc replaced by maxd3Dd3D-cd2cd+(ch-hTX)2c,0, where hTX is the height of the STX/SRX. Three ∆τ are independently generated and respectively applied to the 3 channels between the STX/SRX and the 3 RPs.

NOTE:	In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset should be set as 0.

< Unchanged text omitted >

### 7.9.5	Additional modelling components

< Unchanged text omitted >

#### 7.9.5.2	Type-2 environment object

< Unchanged text omitted >

The effective polarization matrix of the type-2 EO reflection ray is given by

CPMEO=cosγ2-sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1-sinγ1cosγ1	(7.9.5-10)

where,

-	γ1=atan2-eθEO, ZODnplane,eϕEO, AODnplane. nplane represents the normal vector of the incident plane. nplane=vtx→w×vw→rx, in which vtx→w=xw-xtx,yw-ytx,zw-ztx and vw→rx=xrx-xw,yrx-yw,zrx-zw. eθEO, ZOD represents the spherical basis vector of incident ray in vertical direction. eθEO,ZOD=cosθEO, ZODcosϕEO, AOD,cosθEO, ZODsinϕEO, AOD,-sinθEO, ZODT. eϕEO,AOD represents the spherical basis vector of incident ray in horizontal direction. eϕEO,AOD=-sinφEO, AOD,cosφEO, AOD,0T.

-	γ2=atan2eθEO, ZOAnplane,-eϕEO, AOAnplane. eθEO, ZOA represents the polar basis vector of scattering ray in vertical direction. eϕEO, AOA represents the spherical basis vector of scattering ray in horizontal direction. eθEO, ZOA=cosθEO, ZOAcosϕEO, AOA,cosθEO, ZOAsinϕEO, AOA,-sinθEO, ZOAT. eϕEO, AOA=-sinϕEO, AOA,cosϕEO, AOA,0T.

< Unchanged text omitted >

#### 7.9.5.5	Lower power clusters

In addition to the clusters/rays in background channel generated in Clause 7.9.4.2, more clusters/rays using NLOS propagation condition can be generated between a sensing Tx and a sensing Rx for bi-static sensing or between a sensing Tx and a reference point r, where r=0, 1, 2 for mono-static sensing to simulate the weak rays in the background channel. The following procedure is applicable for bistatic sensing or for monostatic sensing.

Step 1: Generate a first background channel Hu,sbk1τ,t for bi-static sensing and Hu,sbk1,rτ,t for reference point r for mono-static sensing according to Clause 7.9.4.2.

Step 2: Generate a second background channel Hu,sbk2τ,t for bi-static sensing and Hu,sbk2,rτ,t for reference point r for mono-static sensing according to Clause 7.9.4.2 including utilizing the same DS, ASA, ASD, ZSA, ZSD, Cθ, Cϕas used for the first step, if applicable, with the following revised parameters: 1) The number of clusters (N in Clause 7.5) is increased to 360; 2) The number of ray per cluster (M in Clause 7.5) is reduced to 1; 3) If LOS condition is assumed in Step 1, use NLOS condition to generate DS, ASA, ASD, ZSA, and ZSD values.

Step 3: The power of the clusters/rays in the second background channel is scaled down by PmaxS110G10, and is added to the first background channel, where, G=-25 dB, PmaxS1 is the maximum power of the NLOS clusters in the first background channel for bi-static sensing and PmaxS1,r is the maximum power of the NLOS clusters for reference point r in the first background channel. The final background channel Hu,sbkτ,t is 7.9.5-15a for bi-static sensing mode and 7.9.5-15b for mono-static sensing.

Hu,sbkτ,t=Hu,sbk1τ,t+PmaxS1 10G10 P1S110G10∙Hu,sbk2τ,t	(7.9.5-15a)

Hu,sbkτ,t=Hu,sbk1τ,t+r=02PmaxS1,r 10G10∙Hu,sbk2,rτ,t 	 (7.9.5-15b)

< Unchanged text omitted >

### 7.9.6	Channel model calibration

#### 7.9.6.1	Large scale calibration

For the purposes of large scale calibration without fast fading modelling for sensing targets UAV, human, automotive and AGV, the following calibration parameters are respectively provided in Table 7.9.6.1-1/2/3/4. Unspecified parameters in Table 7.9.6.1-2/3/4 are the same as those in Table 7.9.6.1-1. The calibration results based on TR 38.901 V19.0.0 can be found in R1-2506616.

Table 7.9.6.1-1: Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| UT height | 1.5m for terrestrial UTs, |
| UT Tx power | 23dBm |
| UT Distribution | - The overall number of UTs is 30 uniformly distributed in the center cell. - All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. - Vertical distribution of aerial UE: Fixed height value of 200 m. - FR1 is assumed for aerial UE. |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component σM of the RCS for each scattering point | -12.81 dBsm |
| Minimum 3D distances between STX/SRX and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Coupling loss for target channel | power scaling factor (pathloss, shadow fading, and RCS component σM included): LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| STX/SRX selection | Best N = 4 STX-SRX pairs to be selected for the target. see note 1 |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) see note 2 |
| NOTE 1: Based on the STX-SRX pairs with the smallest power scaling factor of the target channel. NOTE 2: CDFs can be separately generated for target channel, background channel. |  |

Table 7.9.6.1-2. Simulation assumptions for large scale calibration for Human sensing targets

| Parameters | Indoor Values |  | Outdoor Values |
| --- | --- | --- | --- |
| Scenario | Indoor office 12 sectors per 120m * 50m * 3m ISD = 20m InF-SH Hall size: L=300 x W=150 m Room height: 10m 18 BSs on a square lattice with spacing D, located D/2 from the walls. - D=50m |  | UMa (ISD = 500m), UMi (ISD= 200m) |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |  |  |
| Target type | Adult Pedestrian: 0.5m x 0.5m x 1.75m. See note. |  |  |
| BS Tx power | FR1: 24dBm FR2: 23dBm |  | FR1: 56dBm FR2: 41dBm |
| UT height | 1m |  | 1.5m |
| UT Distribution | Per Table 7.8-1 Indoor-Office Number of UTs: 20 |  | Per Table 7.8-1. Number of UTs/cell: 10 |
| Sensing target distribution | 100% indoor, 1 target uniformly distributed (across multiple drops) over the convex hull of the TRP deployment |  | 100% outdoor. 1 target uniformly distributed (across multiple drops) within the center cell. |
| Component σM of the RCS for each scattering point | -1.37 dBsm |  |  |
| Minimum 3D distances between STX/SRX and sensing target |  | For TRP monostatic and TRP-TRP bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 0 m For UE monostatic and UE-UE bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 1 m (as specified in TR 38.858) |  | For TRP monostatic and TRP-TRP bistatic sensing modes, the minimum 2D distances between STX/SRX and the sensing target are 10 m for UMi scenarios and 35 m for UMa scenarios For UE monostatic and UE-UE bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 1 m (as specified in TR 38.858) |  |  |
| Wrapping Method | N/A |  | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| NOTE: Height of scattering point 1.5m |  |  |  |

Table 7.9.6.1-3. Simulation assumptions for large scale calibration for Automotive sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | For FR1: - Urban Grid (ISD=500m, BS height=25m) - Highway (ISD=1732m, BS height=35m) For FR2: - Urban Grid (ISD=250m, BS height=25m) - Highway (ISD=500m, BS height=25m) For Urban Grid ISD =250m - The layout is configured as follows: - Red triangles: BS with 250m ISD, 18 BSs are located. |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |
| Target type | Vehicle type 2 [TR37.885] |
| UT height | 1.5m for pedestrian type UE 5m for RSU type UE 1.6m for vehicle type UE |
| UT Distribution | For Highway: - Vehicle Type UT distribution follows vehicle UE dropping as in Clause 6.1.2from TR37.885. - RSU-type UTs are uniformly allocated with 100m spacing in the middle of the freeway as per TR37.885. For Urban grid: - Vehicle Type UT distribution follows vehicle UE dropping as in Clause 6.1.2 from TR37.885. - Pedestrian type UT, the dropping using equally spaced along the sidewalk with a fixed inter-pedestrian X m dropped per TR37.885 - Total number of pedestrian UEs is 16 in the centre grid. - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/(16*N)’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). - N=1; - RSU-type UT: the dropping is at the center of intersection per TR37.885. see note 1 |
| Sensing target distribution | Per TR37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Highway: one target uniformly distributed (across multiple drops) within the simulation region. Vehicle speed is 140 km/h in all the lanes as baseline. - Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline. see note 2 |
| Component σM of the RCS for each scattering point | 11.25 dBsm |
| Minimum 3D distances between STX/SRX and sensing target | Min distances based on min TRP/UE distances defined in TR37.885 |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping.As defined in urban grid/highway scenario |

| STX/SRX selection | Best N = 4 STX-SRX pairs to be selected for the target, with the following exceptions for Highway for FR1: - For TRP-TRP monostatic sensing, N=2 - For TRP-TRP bistatic sensing, N=1 |  |
| --- | --- | --- |

NOTE 1: A single UT type is used per calibration, e.g., pedestrian type UT, RSU type UT, or vehicle type UTPer TR37.885 NOTE 2: A vehicle with size L*W*H can be dropped with 5 scattering points (front/left/right/back/roof), respectively with locations (L/2,0,H/2), (0,W/2,H/2), (0,-W/2,H/2), (-L/2,0,H/2) and (0,0,H) in LCS. A vehicle with size L*W*H can be dropped with 1 scattering point with location (0,0,H/2) in LCS. In the case of vehicle with 5 scattering points, spatial consistency is enabled with the following assumptions: - The correlation for LOS/NLOS condition of the 5 points is assumed equal to 1. LOS/NLOS condition can be calculated based on the distance of the STX/SRX to the centroid of the ST, then apply the LOS/NLOS condition to each of the 5 points. - The correlation for stochastic cluster paths of the 5 points is assumed equal to 1. The stochastic cluster paths can be calculated between the STX/SRX and the centroid of the ST, then the stochastic cluster paths are added to each of the 5 points.

Table 7.9.6.1-4. Simulation assumptions for large scale calibration for AGV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | InF: 18 TRPs per the big hall: L = 300 m x W = 150 m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS - 8 m for high BS scenario |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |
| Target type | Option 1: 0.5m x 1.0m x 0.5m |
| BS Tx power | FR1: 24dBm FR2: 23dBm |
| UT height | 1.5m |
| UT Distribution | Per Table 7.8-7 Indoor Factory. Number of UTs: 30 |
| Sensing target distribution | 100% indoor, 1 target uniformly distributed (across multiple drops) over the convex hull of the TRP deployment |
| Component σM of the RCS for each scattering point | -4.25 dBsm see note |
| NOTE: For calibration purposes, other value(s) are not precluded, |  |

#### 7.9.6.2	Full calibration

For the purpose of full calibration including the fast fading modelling for sensing scenarios UAV, human, automotive and AGV, the calibration parameters are respectively provided in Table 7.9.6.2-1/2/3/4. Unspecified parameters in the tables are the same as those in Table 7.9.6.1-1/2/3/4 for the same sensing target. If still not specified, the parameters in Table 7.9.6.2-1 are used. The calibration results based on TR 38.900 V19.0.0 can be found in R1-2506616.

Table 7.9.6.2-1: Simulation assumptions for full calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| RCS for each scattering point | Component σM: -12.81 dBsm Component σD: 0 dB Component σS: 3.74 dB for standard deviation The same values are used for monostatic RCS and bistatic RCS |
| Fast fading model | TR 36.777 Annex B.1.3 |
| (u, std) for XPR of target | Mean 13.75 dB, deviation 7.07 dB |
| The power threshold for path dropping after concatenation for target channel | -40 dB |
| Coupling loss for target channel | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component σM included) - for small scale RCS σD/σS and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), STX/SRX antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pd112+d2222 ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Absolute delay | The model of UMa scenario defined in Table 7.6.9-1 is reused for UMa-AV for all sensing modes. |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of coupling losses between STX/SRX and all reference points) See note. CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). For monostatic sensing mode: delay spread and angle spread of the background channel is calculated separately for each reference point. Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996. |
| NOTE: CDFs can be separately generated for target channel, background channel |  |

Table 7.9.6.2-2: Simulation assumptions for full calibration for Human sensing targets

| Parameters | Indoor Values | Outdoor Values |
| --- | --- | --- |
| Scenario | As specified in Table 7.9.6.1-2 | As specified in Table 7.9.6.1-2 |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |  |
| Target type | Adult Pedestrian: 0.5m x 0.5m x 1.75m see note |  |
| UT Distribution | Per Table 7.8-2 Indoor-Office Number of UTs: 20 | Per Table 7.8-2 Number of UTs/cell: 10 |
| RCS for each scattering point | Component σM: -1.37 dBsm Component σD: 0 dB Component σS: 3.94 dB The same values are used for monostatic RCS and bistatic RCS | Component σM: -1.37 dBsm Component σD: 0 dB Component σS: 3.94 dB The same values are used for monostatic RCS and bistatic RCS |
| Fast fading model | For BS to UE link: Follow the procedure defined in TR38.901 For BS to target link: Follow the procedure defined in TR38.901 for target to UE link: Follow the procedure defined in 38.858 | For BS to UE link: Follow the procedure defined in TR38.901 For BS to target link: Follow the procedure defined in TR38.901 for target to UE link: Follow the procedure defined in 38.858 |
| (u, std) for XPR of target | (19.81, 4.25) dB | (19.81, 4.25) dB |
| Absolute delay | The model of InH, InF-SH scenarios defined in Table 7.6.9-1. | The model of UMa/UMi scenario defined in Table 7.6.9-1. |
| NOTE: Height of scattering point 1.5m |  |  |

Table 7.9.6.2-3: Simulation assumptions for full calibration for Automotive sensing targets

< Unchanged text omitted >

Table 7.9.6.2-4: Simulation assumptions for full calibration for AGV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | As specified in Table 7.9.6.1-4 |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |
| Target type | Option 1: 0.5m x 1.0m x 0.5m |
| RCS for each scattering point | The values/pattern of component σMσD are generated by Table 7.9.2.1-6 The same values are used for monostatic RCS and bistatic RCS |
| Fast fading model (Full calibration) | For BS to UE link: Follow the procedure defined in TR38.901 For BS to target link: Follow the procedure defined in TR38.901 for target to UE link: Follow the procedure defined in TR 38.858 |
| (u, std) for XPR of target | (9.60, 6.85) dB |
| Absolute delay | The model of InF scenario defined in Table 7.6.9-1 is reused for all sensing modes. |

#### 7.9.6.3	Calibration of additional features

The calibration parameters for the calibration of spatial consistency and type-2 EO can be respectively found in Table 7.9.6.3-1/2. Unspecified parameters in these tables are the same as those in Tables 7.9.6.1-2/3 and Tables 7.9.6.2-2/3. The calibration results based on TR 38.900 V19.0.0 can be found in R1-2506616.

< Unchanged text omitted >

Table 7.9.6.3-2: Simulation assumptions for calibration of type-2 EO

| Parameter | Values |
| --- | --- |
| Scenario | Urban grid |
| Cell layout | For FR1: ISD = 500m, the layout is defined as below: The BSs are placed at the top of buildings at one corner. Specifically, the road grids shall be shifted by (ISD3-10,32*ISD3-10) m in horizontal plane, or the BSs are shifted by (-ISD3+10,-32*ISD3+10)m in horizontal plane equivalently. For FR2 ISD=250m, the cell layout is as that specified in Table 7.9.6.1-3 |
| Sensing mode | TRP- pedestrian UE bistatic TRP- RSU type UE bistatic TRP monostatic TRP-TRP bistatic RSU type UE - RSU type UE bistatic (optional) |
| EO deployment | 9 buildings with size 413m x 230m x 20m illustrated by the green blocks as in the figure shown in the row of cell layout. |
| ST distribution | one target is uniformly distributed (across multiple drops) within the center of the outside lane of the grid. |
| UT distribution | - For pedestrian UT - Pedestrian type UE, the dropping using equally spaced along the sidewalk with a fixed inter-pedestrian X m dropped per TR37.885. - Total number of pedestrian UEs is 16 in the centre grid. - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/(16*N)’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). - N=1; - For RSU type UT - The dropping is at the center of intersection per TR37.885. |
| EM-parameters | Refer to the row for concrete in Table 7.6.8-1 |
| Metrics | CDF curves: - Full coupling loss: calculate the coupling loss for each Tx-EO-ST-LOS-Rx and Tx-LOS-ST-EO-Rx rays. - CDF of the Delay. - CDF of the AoA, AoD, ZoA, ZoD. Additional CDF curves: - Coupling loss for target channel - CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread [TR 25.996, Annex A]. |
| NOTE: Other necessary but unspecified parameters in this table are the same as those in the table for urban grid scenario calibration. |  |

< Unchanged text omitted >