3GPP TSG-RAN WG1 Meeting #122	R1-2506643

# Bengaluru, India, August 25 – 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0580 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to LTE_terr_bcast_Ph2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In some cases, the value of RK-K+1 in the frequency interleaver may be negative or zero, in which case the row permutation is not implementable - In some cases, it is observed that an alternative value of the cyclic shift may provide performance improvement |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that row permutation is only applied if RK-K+1>0 ; - Add one new methods for cyclic shift |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Specification is not implementable for cases of RK-K+1≤0. - New values of cyclic shift are not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1, 6.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.5.1	Cyclic shift for PMCH

The input to the cyclic shift is the block of bits b0, …, bMbit-1 defined in clause 6.3.1 and the ouput is a block of bits b0, …, bMbit-1.

The block of bits b0, …, bMbit-1 shall be cyclically shifted by Xi bits to form the block b0, …, bMbit-1 according to

bn=b(n-Xi mod Mbit)

where

-	Xi is given by

-	if the higher-layer parameter pmch-CyclicShiftAlpha is set to alpha1 or alpha2

Xi=r=C-SiC-1Er

Si=iα mod C

-	if the higher-layer parameter pmch-CyclicShiftAlpha is set to alpha3

Xi=Ainf,nsfNscQmN

Ainf,nsf=m=07c810nf mod 128+nsf+m2m mod N

where the pseudo-random sequence c(i) is defined by clause 7.2 and shall be initialized with cinit=NIDMBSFN at the beginning of each radio frame for which nf mod 128=0, the number of subcarriers available in one OFDM symbol for PMCH is given by Nsc=MbitQmL, and Qm is the modulation order.

-	i∈0, 1, …, N-1 denotes the ith subframe to which the transport block is mapped

-	N is the number of subframes to which the transport block is mapped

-	Er is the number of bits in the rth codeblock within a subframe as defined in TS 36.212 [3]

-	C is the number of codeblocks in the transport block

-	α is given by

-	if the higher-layer parameter XXX is set to alphaOne

α=1

-	if the higher-layer parameter XXX is set to alphaOther

α=CNL

-	L is the number of OFDM symbols used for the PMCH transmission.

### 6.5.2	Frequency-domain interleaving

The input to the interleaver is the block of complex-valued symbols yp0, …, yp(Msymbap-1)  defined in clause 6.3.4.1 and the output from the interleaver is a block of complex-valued symbols yp0, …, yp(Msymbap-1).

Interleaving shall be performed as follows:

-	Divide the block of complex-valued symbols yp0, …, yp(Msymbap-1) into L sets such that set l∈0,1,…,L-1 contains the complex-valued symbols to be mapped to OFDM symbol l

-	For each of the sets l=0, 1, …, L-1

-	Set the number of columns K of the block interleaver to

K=Cgcd⁡(L,C)

where

-	L is the number of OFDM symbols used for the PMCH transmission

-	C is the number of codeblocks as defined in clause 5.1.2 of [36.212]

-	gcd⁡(u,v) denotes the greatest common divisor of u and v

-	Set the number of rows of the block interleaver R to the smallest integer fulfilling

Ml≤RK

where

-	Ml is the number of resources elements available for mapping of complex-valued modulation symbols in OFDM symbol l

-	Write the modulation symbols ypi in set l column-wise into the block interleaver in increasing order of i, starting with column 0 and row 0 and in increasing order of first the row number and then the column number. If Ml<RK, append <NULL> elements to fill the size-RK block interleaver.

-	If RK-K+1>0, permute the rows such that row ri is moved to row i where

ri=ri-1+dri-1≤R-d-1idRotherwise
d=RRRK-K+1
r0=0
1≤i≤R-1

-	Cyclically shift the columns of row r with an offset sr given by

sr=(-1)rr+12mod K
r=0,1,…,R-1

-	Read the interleaved modulation symbols  ypi in set l row-wise from the block interlaver in increasing order of i, starting with column 0 and row 0 and in increasing order of first the column number and then the row number, discarding any <NULL> elements.