3GPP TSG-RAN WG1 Meeting #122	R1-2506646

# Bengaluru, India, August 25-29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0379 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Rel-19 LTE-based 5G Broadcast Phase 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FUTUREWEI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-09-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing and unaligned RRC parameters |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include missing RRC parameters and align other RRC parameters |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Two RRC parameters are missing and other RRC parameters are unaligned |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.1.4.1.2	Bit collection, selection and transmission

The circular buffer of length  for the r-th coded block is generated as follows:

for k = 0,…,

for k = 0,…,

for k = 0,…,

Denote the soft buffer size for the transport block by NIR bits and the soft buffer size for the r-th code block by Ncb bits. The size Ncb is obtained as follows, where C is the number of code blocks computed in clause 5.1.2:

- 	for DL-SCH, PCH transport channels, and MCH configured with pmch-TimeInterleavingN

- 	for UL-SCH, MCH not configured with pmch-TimeInterleavingN, SL-SCH and SL-DCH transport channels

For UE category 0, for DL-SCH associated with SI-RNTI and RA-RNTI and PCH transport channel, Ncb is always equal to Kw.

Where NIR for MCH configured with pmch-TimeInterleavingN is equal to:

,

Where the coefficient β is configured by scalingFactorBeta, M is configured by pmch-TimeInterleavingM, and  is the total number of soft channel bits according to a reference UE category configured by refUE-CategoryDL.

Where NIR for DL-SCH and PCH transport channels is equal to:

Where the coefficient  is 1 for subframe duration, 0.5 for slot duration and for subslot duration of initial PDSCH transmission. For retransmission of a transport block with slot or subslot duration, is 1 if the transport block has initially been transmitted with subframe duration. For retransmission of a transport block with subframe duration,  is 0.5 if the transport block has initially been transmitted with slot duration, and if the transport block has initially been transmitted with subslot duration.

If the UE signals ue-CategoryDL-v14xy indicating UE category M2, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-CategoryDL-v14xy. Otherwise, if the UE signals ue-CategoryDL-v1310 indicating UE category M1, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-CategoryDL-v1310. Otherwise, if the UE signals ue-CategoryDL-v13xy, and is configured with transmission mode 9 or transmission mode 10 and the higher layer parameter maxLayersMIMO-r10 is configured to eightLayers, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-CategoryDL-v13xy. Otherwise, if the UE signals ue-CategoryDL-r12 indicating UE category 0, or if the UE signals ue-CategoryDL-r12 indicating UE category 14 and is configured by higher layers with altCQI-Table-r12 for the DL cell, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-CategoryDL-r12. Otherwise, if the UE signals ue-Category-v11a0, and is configured by higher layers with altCQI-Table-r12  or altCQI-Table-1024QAM-r15 for the DL cell, Nsoft is the total number of soft channel bits according to the UE category indicated by ue-Category-v11a0. Otherwise, if the UE signals ue-Category-v1020, and is configured with transmission mode 9 or transmission mode 10, or is configured with transmission mode 3 or transmission mode 4 and the higher layer parameter maxLayersMIMO-r10 is configured to fourLayers, for the DL cell, Nsoft is the total number of soft channel bits [4] according to the UE category indicated by ue-Category-v1020 [6]. Otherwise, Nsoft is the total number of soft channel bits [4] according to the UE category indicated by ue-Category (without suffix) [6].

If Nsoft = 35982720 or 47431680,

KC= 5,

elseif Nsoft = 303562752,

KC= 32,

elseif Nsoft = 14616576,

if the UE is configured by higher layers with altCQI-Table-r12,

KC = 3/2

else

KC = 2

end if.

elseif Nsoft = 19488768,

if the UE is configured by higher layers with altCQI-Table-1024QAM-r15,

KC = 8/5

elseif the UE is configured by higher layers with altCQI-Table-r12,

KC = 2

else

KC = 8/3

end if.

elseif Nsoft = 7308288 and the UE is configured by higher layers with altCQI-Table-r12,

if the UE is capable of supporting no more than a maximum of two spatial layers for the DL cell in the transmission mode configured for the UE, or if the configured maximum number of layers indicated by the maxLayersMIMO-r10 field is no more than two,

KC = 3

else

KC = 3/2

end if.

elseif Nsoft = 7308288 and the UE is configured by higher layers with altCQI-Table-1024QAM-r15,

if the UE is capable of supporting no more than a maximum of two spatial layers for the DL cell in the transmission mode configured for the UE, or if the configured maximum number of layers indicated by the maxLayersMIMO-r10 field is no more than two,

KC = 12/5

else

KC = 6/5

end if.

elseif Nsoft = 3654144 and the UE is capable of supporting no more than a maximum of two spatial layers for the DL cell, or if the configured maximum number of layers indicated by the maxLayersMIMO-r10 field is no more than two,

KC = 2

else

KC = 1

End if.

For initial PDSCH transmission with subframe duration, KMIMO is equal to 2 if the UE is configured to receive PDSCH transmissions based on transmission modes 3, 4, 8, 9 or 10 as defined in clause 7.1 of [3], and is equal to 1 otherwise. For BL/CE UE KMIMO is equal to 1. For initial PDSCH transmission with slot/subslot duration, KMIMO is equal to 1.

MDL_HARQ is the maximum number of DL HARQ processes as defined in clause 7 of [3].

Mlimit is a constant equal to 8.

Denoting by E the rate matching output sequence length for the r-th coded block, and rvidx the redundancy version number for this transmission (rvidx = 0, 1, 2 or 3 for DL-SCH and PCH transport channels, or rvidx = 0, 1,…, N-1 for MCH configured with higher layer parameter pmch-TimeInterleavingN), the rate matching output bit sequence is , k = 0,1,..., .

Define by G the total number of bits available for the transmission of one transport block.

Set where Qm is equal to 1 for π/2-BPSK, 2 for QPSK, 4 for 16QAM, 6 for 64QAM, 8 for 256QAM, and 10 for 1024QAM, and where

-	For transmit diversity:

-	NL is equal to 2,

-	Otherwise:

-	NL is equal to the number of layers a transport block is mapped onto

Set, where C is the number of code blocks computed in clause 5.1.2.

if

set

else

set

end if

Set for DL-SCH and PCH transport channels, or  for MCH configured with higher layer parameter pmch-TimeInterleavingN, where  is the number of rows defined in clause 5.1.4.1.1, and .

Set k = 0 and j = 0

while {k < E}

if

k = k +1

end if

j = j +1

end while