3GPP TSG-WG Meeting #122	R1- 2506674

# Bengaluru, India, August 25–29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | 0074 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Rel-19 AI/ML air-interface for TS38.215 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-09-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The information on what the linear average of the estimated channel response is based on is missing from the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the information on what the linear average of the estimated channel response is based on. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information on what the linear average of the estimated channel response is based on is missing from the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- unchanged text omitted ---

### 5.2.9	UL SRS time domain channel timing (UL SRS-TDCT)

| Definition | UL SRS time domain channel timing (UL SRS-TDCT) is defined as Nt values of timing information of the estimated channel response in time domain, Tn, corresponding to the resource elements carrying sounding reference signals (SRS), the Nt values are derived from Nt consecutive time instances with timing granularity 2k∙Tc that start from the first detected path rounded down with timing granularity 2k∙Tc, where Nt and Nt are given by higher layers and Tc is given in Clause 4.1 of [3, TS38.211], and k is the reporting granularity factor given by higher layer parameter timingReportingGranularityFactor. The Nt values of timing information are selected such that they correspond to the time instances with the highest power values among the Nt consecutive time instances where a power value for a time instance is determined as the power of linear average of the estimated channel response from resource elements carrying sounding reference signals (SRS) at time instance Tn. For frequency range 1 and 2, if receiver diversity is in use by the gNB for power of linear average of the estimated channel response measurement: - The measured received signal power, P1, value for the selected first time instance, T1, shall not be lower than the corresponding received signal power for the selected time instance, T1, of any of the individual receiver branches and the received signal power, Pn, shall be with the same receiver branch(es) as applied for the selected first time instance, T1. The timing information are reported relative to reference time, Tref. The reference time, Tref, is equal to the UL RTOA reference time T0+tSRS, where - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time [15, TS 38.455] - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. The reference point for UL SRS-TDCT shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. |
| --- | --- |

--- unchanged text omitted ---