3GPP TSG-RAN WG1 Meeting #122	R1-2506641

# Bengaluru, India, August 25 – 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0578 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IoT_NTN_Ph3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Ambiguous mapping of between reference signal sequence and OCC sequence for 3.75kHz SCS. Unnecessary text included in the specification. - Inconsistent phrasing of OCC enablement status. Unclear reference to DCI format. - Following a postponement due to an NPRACH occasion, when OCC is applied for a 3.75kHz SCS, the NPUSCH transmission should restart at the next slot satisfying (5n_f + n_s") mod 4 = 0 in order to allow multiplexing of UEs with UEs following the NPRACH occasion. - Incorrect or ambiguous definition of scrambling sequence reinitialization times for OCC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Definition of reference signal sequence linked directly to the OCC sequence. Text on start time and postponement of NPUSCH transmission that is a duplicate of text in TS36.213 is removed. Redundant text that replicates the main definition of reference signal sequence removed. - Phrasing of “OCC is indicated as enabled” is used. DCI is clarified as being DCI Format N0. - Following postponement due to an NPRACH occasion, the NPUSCH transmission restarts in the first slot that satisfies (5n_f + n_s) mod 4 = 0 - The scrambling sequence for OCC is reinitialized after xtransmissions of the codeword, as shown in the figure below, where the scrambling sequence S1 is reinitialized for each transmission of a codeword that is spread by a factor . The mapped symbols X in RU1 are identical to the mapped symbols Y in RU2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The mapping between reference signal sequence and OCC sequence would be ambiguous. - Inconsistent and imprecise specification. - There are cases where it is not possible to multiplex UEs with OCC when an NPUSCH straddles an NPRACH occasion. - Scrambling sequence when OCC is applied is incorrect or ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.6, 10.1.4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 10.1.3.1	Scrambling

Scrambling shall be done according to clause 5.3.1.

For a UE communicating over NTN in FDD operation and if the higher layer parameter npusch-OCC-Enabled is configured, OCC is indicated as enabled in DCI Format N0 as described in [3], and MrepNPUSCH≥2, the scrambling sequence shall be reinitialized with

cinit=nRNTI∙214+nf mod 2∙213+ns2∙29+NIDNcell

after every MidenticalNPUSCHNidenticalNPUSCH transmissions of the codeword with ns and nf set to the first slot and the frame, respectively, used for the transmission of the repetition. The quantity NidenticalNPUSCH is given by clause 10.1.3.6.

Otherwise, the scrambling sequence generator shall be initialised with

cinit=nRNTI∙214+nf mod 2∙213+ns2∙29+NIDNcell

where ns is the first slot of the transmission of the codeword. In case of NPUSCH repetitions, the scrambling sequence shall be reinitialised according to the above formula after every MidenticalNPUSCH transmissions of the codeword with ns and nf set to the first slot and the frame, respectively, used for the transmission of the repetition. The quantity MidenticalNPUSCH is given by clause 10.1.3.6.

#### 10.1.3.6	Mapping to physical resources

Each NPUSCH codeword can be mapped to one or more than one resource units, , as given by clause 16.5.1.2 of TS 36.213 [4], each of which shall be transmitted  times.

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in [4], and mapped in sequence starting with  to subcarriers assigned for transmission of NPUSCH. The mapping to resource elements  corresponding to the subcarriers assigned for transmission and not used for transmission of reference signals, shall be in increasing order of first the index , then the index, starting with the first slot in the assigned resource unit.

After mapping to slots, the  slots shall be repeated  additional times, before continuing the mapping of  to the following slot, where

For a UE communicating over NTN in FDD operation, if the higher layer parameter npusch-OCC-Enabled is configured, OCC is indicated as enabled in DCI Format N0 as described in [3], and MrepNPUSCH≥2,

-	For Δf=15 kHz, after mapping to a slot, the slot shall be repeated NidenticalNPUSCH-1 additional times and pairs of slots shall be multiplied by q(m) before continuing the mapping of z(∙) to the following slot, the process repeats for all the slots in the NPUSCH format 1 transmission before OCC is applied, where for NscRU=1, MOCC=2, NidenticalNPUSCH=MOCC, and q(m) equals +1+1 or +1-1 as indicated by the DCI Format N0 as described in TS36.212 [3].

-	For Δf=3.75 kHz, after mapping to a data symbol, the data symbol shall be repeated NidenticalNPUSCH-1 additional times and pairs of data symbols shall be multiplied by q(m) while skipping the DMRS symbol and guard period within the slot, before continuing the mapping of z(∙) to the following data symbol, the process repeats for all the data symbols in the NPUSCH Format 1 transmission before OCC is applied, where for NscRU=1, MOCC=2, NidenticalNPUSCH=MOCC, and q(m) equals +1+1 or +1-1 as indicated by the DCI Format N0 as described in TS36.212 [3].

For NPUSCH Format 1 and 2 on frame structure type 2 with ,

-	the NPUSCH transmission is carried out in the first set of  slots spanning over two contiguous uplink subframes not overlapping with any uplink subframe configured as invalid;

-	for TDD configuration 1 and 4, if the starting position for the NPUSCH is indicated as the second of the two contiguous uplink subframes, the NPUSCH transmission is postponed until the start of two consecutive uplink subframes.

If a mapping to  slots or a repetition of the mapping contains a resource element which overlaps with

-	any configured NPRACH resource according to nprach-ParametersList in SystemInformationBlockType2-NB, or

-	any configured NPRACH resource according to nprach-ParametersList given by ul-ConfigList in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH as supported, or

-	any configured NPRACH resource according to nprach-ParametersList given by ul-ConfigListMixed in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH and mixedOperationMode as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 in SystemInformationBlockType2-NB and if the UE indicates nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 given by ul-ConfigList in SystemInformationBlockType23-NB and if the UE indicates multiCarrier-NPRACH and nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 given by ul-ConfigListMixed in SystemInformationBlockType23-NB and if the UE indicates multiCarrier-NPRACH, mixedOperationMode and nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListTDD in SystemInformationBlockType2-NB, or

-	any configured NPRACH resource according to nprach-ParametersListTDD in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH as supported, or

-	any configured NPRACH resource configured for Early Data Transmission and if the NPUSCH transmission is during an Early Data Transmission procedure [12, Clause 7.3b],

then,

-	for ,

-	if a UE communicating over NTN in FDD operation, and the higher layer parameter npusch-OCC-Enabled is configured, and OCC is indicated as enabled in DCI Format N0 as described in [3], and MrepNPUSCH≥2, then the NPUSCH transmission in overlapped Nslots slots is postponed until the next Nslots slots starting with the first slot satisfying 5nf+ns mod 4=0 and not overlapping with any configured NPRACH resource,

-	otherwise,  the NPUSCH transmission in overlapped slots is postponed until the next  slots not overlapping with any configured NPRACH resource.

-	for  the NPUSCH transmission in overlapped  slots is postponed until the next  slots starting with the first slot satisfying ns mod 2=0 nsmod 2=0and not overlapping with any configured NPRACH resource.

NPRACH gaps as defined in clause 10.1.6.1 are not part of the NPRACH resource. For frame structure type 2, the valid uplink subframes which are not used for NPRACH transmission when it is not possible to map G symbol groups back-to-back are not part of the NPRACH resource. The mapping of  is then repeated until MrepNPUSCHNRUNslotsUL slots have been transmitted. After transmissions and/or postponements due to NPRACH of  time units, for frame structure type 1 for FDD, a gap of  time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

When higher layer parameter npusch-AllSymbols is set to false, resource elements in SC-FDMA symbols overlapping with a symbol configured with SRS according to srs-SubframeConfig shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH. When higher layer parameter npusch-AllSymbols is set to true, all symbols are transmitted.

If higher layer parameter resourceReservationConfigUL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space with the Resource reservation field in the DCI set to 1 including NPUSCH format 1 transmission without a corresponding NPDCCH, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space,

-	In a subframe for  or a slot for that is overlapping with any fully reserved uplink subframe as defined in clause 16.5 in [4],

-	for , the NPUSCH transmission is postponed until the next NB-IoT uplink subframe that is not fully reserved.

-	for , the NPUSCH transmission in the slot is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved.

-	In a subframe for  or a slot for that is not overlapping with any fully reserved uplink subframe, any SC-FDMA symbols overlapping with reserved symbols shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH.

For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1 for FDD, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ntn-SegmentedPrecompensationGaps-r17 as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled.

10.1.4.1.1.2	OCC reference signal sequence for NscRU=1 with Δf=3.75 kHz

For a UE communicating over NTN, the OCC reference signal sequence ru,OCC(n) for NscRU=1 is defined by

ru,OCCn=run'if nMOCC mod MOCC=mp 0otherwise

n=0,1,…,MrepNPUSCHNslotsULNRU-1

n'=n-nMOCC-mp

where

-	MOCC=2

-	MrepNPUSCH≥2

m=0, 1

-	p=1 if the DCI indicates OCC sequence 11, otherwise p=0

q(m)=11

q(m)=1-1

Δf=3.75 kHz5nf+ns mod 4=0

n mod 4∈0,1

ru,OCCnq(m)=11

q(m)=1-1

n mod 4∈2,3

q(m)=11

ru,OCCnq(m)=1-1