3GPP TSG-RAN WG1 Meeting # 122	R1- 2506647

# Bengaluru, India, Aug 25th - Aug 29th 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1451 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-09-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrections to IoT-NTN Phase 3 uplink capacity/throughput enhancement: Agreement for interleaved multi-TB NPUSCH transmission, the unit of interleaving includes the OCC length when OCC is enabled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When OCC is enabled, the interleaved TB for multi-TB NPUSCH format 1 is transmitted in the unit of g=CMOCCNRUNslotsUL. |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OCC is not supported when the NPUSCH transmission with multi-TB is configured with interleaved mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

NPUSCH format 1 transmission can be scheduled by a NPDCCH with DCI format N0, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14] , while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a NPDCCH with DCI format N0.

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of

-	n+k0+Koffset DL subframe for FDD or NTN TDD,

-	k0 NB-IoT UL subframes following the end of n+8 subframe for TN TDD,

a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and

-	, where the value of  is determined as specified in Clause 16.5.1.1, the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.5.1.1), the value of  is the number of NB-IoT UL slots of the resource unit (defined in clause 10.1.2.3 of [3]) corresponding to the  allocated number of subcarriers (as determined in Clause 16.5.1.1) in the corresponding DCI, and the value of is determined by the Number of scheduled TB for Unicast field, if present, in the corresponding DCI,  otherwise

-	for FDD,

-	if NPUSCH transmission with subcarrier spacing and the UE configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled in the corresponding DCI,

-	n0 is the first NB-IoT UL slot, ns, starting after the end of subframe n+k0+Koffset that fulfills (5nf+ns) mod 4=0

-	otherwise,

-	n0 is the first NB-IoT UL slot starting after the end of subframe n+k0+Koffset for FDD or NTN TDD

-	n0 is the first NB-IoT UL slot starting after k0 NB-IoT UL subframes following the end of n+8 subframe for TN TDD

-	value of k0 is determined by the scheduling delay field () in the corresponding DCI according to Table 16.5.1-1 for FDD or NTN TDD and Table 16.5.1-1A for TN TDD

-	For ,

-	if the UE is configured with higher layer parameter npusch-MultiTB-Config set to 'interleaved', and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and  where  for ,  otherwise.

-	NB-IoT UL slots  with l=0,1,…g-1,  c=0,1,…NRep/(CMOCC)-1, g=CMOCCNRUNslotsUL are associated with TBr+1 , .  MOCC=2 if NscRU=1 and the UE is configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled in the corresponding DCI, MOCC=1 otherwise.

-	otherwise,

-	NB-IoT UL slots  with  are associated with TBr+1 ,

Table 16.5.1-1: for DCI format N0 for FDD or NTN TDD.

| 0 | 8 |
| --- | --- |
| 1 | 16 |
| 2 | 32 |
| 3 | 64 |

Table 16.5.1-1A: for DCI format N0 for TN TDD.

| 0 | 0 |
| --- | --- |
| 1 | 8 |
| 2 | 16 |
| 3 | 32 |

If a NPUSCH transmission without a corresponding NPDCCH collides partially or fully with a NPDSCH transmission, the NPUSCH transmission is dropped.

<Unchanged parts are omitted>