3GPP TSG-RAN Meeting #109	RP-252663

Beijing, China, Sept 15th – 18th, 2025

Source:	Nokia

Title:	Considerations on RAN4-led REL-20 5G-Adv Demodulation Topics

Agenda item:	9.1.4.5

Document for:	Discussion

Introduction

In the last RAN plenary meeting #108 in Prague, the WF for RAN4 led REL-20 topics in [1] was endorsed. During the meeting a number of agreements were made while some aspects are still open for further discussions. In this paper we discuss these remaining open aspects regarding the demod aspects of the RAN4 led Rel-20 package.

Discussion

## 2.1	Demodulation performance evolution

The following was endorsed in [2]:

The high-level scope for WI on Demod enhancement (the exact scope and WID draft will be discussed and decided in RAN#109):

- Normative work on SCM based requirements for SU-MIMO in FR1

  - The exact scope also depends on the conclusion of R19 study

- Study on SCM for MU-MIMO

- Performance requirements for 6Rx UE with interference

  - Up to 4 MIMO layers for the target cell is considered

- No RAN1/2 impact is expected

- Target 0.5 TU allocation from Q1’26 and 1 TU from Q2’26.

In the Rel-19 SCM study (FS_NR_demod_SCM) the alignment and TE implementation feasibility of the CDL based SCM candidate (“TR 38.753 rCDL-C1”) was demonstrated. The study concluded in TR 38.753 that “this level of alignment is expected to allow RAN4 to work on normative requirements”:

Regarding rCDL-C1 the following observations can be drawn:

- The study has worked on alignment of rCDL-C1 implementations and has verified that there is a reasonable level of alignment.

- This level of alignment is expected to allow RAN4 to work on normative requirements.

- If the group considers the application of rCDL to settings that involve time-domain prediction it is necessary to further review the deterministic behaviour of rCDL and, if found necessary, identify countermeasures.

Furthermore, specific use cases for SCMs have been identified in setups with multiple codewords, advanced receivers, enhanced codebooks, and SDM based systems in general. Hence, we see a need for a first limited set of optional normative requirements for SU-MIMO in Rel-20, based on the Rel-19 rCDL aligned SCM. 
It is undisputed that this model covers typical spatial channel effects expected in real deployments and is aligned with RAN1 assumptions for feature development.

In order to manage workload, the Rel-19 study deprioritised the MU-MIMO aspects (including advanced receiver and some eTypeII use cases), and only includes some initial observations for information. However, MU MIMO is one of the use cases observed to be ill covered in legacy channel models, especially in combination with advanced receivers and eTypeII codebooks.	
As such it is necessary to continue the work on MU-MIMO and advanced receivers, as well as “eTypeII follow PMI” alignment.

It is our understanding that the introduction of SCM based requirements does not necessitate the development of new receivers; rather it provides assurance and lower bounds for the performance of existing (and future) receivers in typical MIMO environments that were blind spots in legacy channel models.	
Making requirements optional in 5GA can be a way forward to make it clear that the requirements should not be seen as implicitly requiring algorithmic updates in 5G-Advanced.

The introduction of SCM based requirements does not necessitate the development of new receivers; rather it provides assurance and lower bounds for the performance of existing (and future) receivers in typical MIMO environments that were blind spots in legacy channel models. Hence, the requirements may be optional in 5GA.

There is precedent in RAN4 demod with “optional UE requirements”, aka “UE declarations”, aka “UE capabilities without signalling” that are only known to the test equipment but not to the network in deployment, e.g. the advanced receiver and simplified 8Rx receiver features defined in TS 38.306. A similar approach can be used to make optional requirements for “UEs declaring to meet the SCM-based requirements”.

Concerning 6Rx UE demodulation performance with interference, we understand that the Rel-19 6Rx performance requirements (introduced in NR_ENDC_RF_Ph4) do not currently cover interference scenarios. With 6Rx a substantial gain of advanced receivers in high throughput scenarios is expected, due to larger spatial domain freedom compared to 2/4 Rx, which can be used to manage interference even with many serving cell layers.

Rel-19 6Rx performance requirements (introduced in NR_ENDC_RF_Ph4) do not currently cover interference scenarios. With 6Rx a substantial gain of advanced receivers in high throughput scenarios is expected.

Given the expected constraints on 5GA TUs in RAN4, the content of the 5GA Demod WI should be kept to a minimum and topics should be those that are foundational for 6G work in the future. The SCM work is a top priority for many network operators, and having SCM requirements is foundational to deliver confirmed feasibility for 6G.	
Priority should be given to the SCM topic.

The SCM topic is foundational for 6G and TUs are expected to be heavily limited for 5GA.

Following the above discussion and background from both FS_NR_demod_SCM and 6Rx discussions, we propose that Rel-20 demod work should have the following scope:

Rel-20 demod work shall be focused on specifying performance requirements using the rCDL based spatial channel model detailed in TR 38.753 for SU-MIMO in FR1; covering at least 8T8R 2CW, 4T4R 1CW and 8T4R eTypeII use cases. 
Additionally, rCDL based practical spatial channel modelling methodology extensions for MU-MIMO demodulation requirements and CSI reporting requirements shall be studied; covering at least eTypeII and advanced receiver use cases.

Additionally, Rel-20 demod work may be considered to define performance requirements for 6Rx UE with interference; at least the case with up to 4 MIMO layers for the target cell may be considered. Whether both inter cell and inter-user interference, and both PDSCH and PMI requirements are needed shall be discussed.

Where SCM is relevant for other use cases, the work should be done in the respective WIs.

## 2.2	Suggested WI Justification

In Rel-20, to verify the UE demodulation performance with a spatially consistent channel model (SCM) and to apply demodulation requirements to more practical network scenarios, the following enhancements are identified:

- Normative work on spatial channel model-based requirements for SU-MIMO in FR1.

- Study on spatial channel model for MU-MIMO

The Rel-19 study on spatial channel model for demodulation performance requirements for NR has concluded with recommendations on the scope for SU-MIMO UE demodulation performance requirements.

In particular, the study found that the TR 38.753 CDL based spatial channel model framework, is ready for use to specify normative demodulation and CSI performance requirements. Furthermore, it is aligned with RAN1 assumptions for core feature development and was found to exhibit typical spatially consistent spatial components and MIMO effects. Specific use cases for SCMs have been identified in setups with multiple codewords, advanced receivers, enhanced codebooks, and SDM reliant systems in general.

Currently, even if the UE conforms to the current minimum performance requirements, this does not guarantee that in the field the UE will make a reasonably efficient use of MIMO features specified since R15, e.g., providing appropriate channel state information to the network such that the network can make optimal use of the available MIMO layers. 
MNOs have invested heavily in roll-out of 5G NR MIMO systems with enhancements specified since R15. MIMO feature performance requirements need to be set under adequate test conditions that mimic MIMO effects observed in deployment as much as possible. The currently used TDL channels do not exhibit these effects nor match the expected statistics of a measured channel.

Additionally, to verify the 6Rx UE demodulation performance with interference, the following enhancements are identified:

The Rel-19 6Rx performance requirements (introduced in NR_ENDC_RF_Ph4) do not cover scenarios of interference. With 6Rx a substantial gain of advanced receivers in high throughput scenarios is expected, due to larger spatial domain freedom compared to 2/4 Rx, which can be used to manage interference even with many serving cell layers.

## 2.3	Suggested Objectives of Performance Part

The work item includes the following objectives:

- Specify performance requirements under a spatial channel model for SU-MIMO in FR1

  - Using the CDL based spatial channel model detailed in TR 38.753

    - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case.

    - Specify CSI performance requirements for the 4T4R and 8T4R eTypeII precoding use case.

- Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements and CSI reporting requirements

  - Base work on the rCDL based spatial channel model detailed in TR 38.753.

    - Consider spatially consistent approaches based both on creating a single channel for a single UE, as well as, creating channels for all UEs.

    - At least, consider the TPUT performance of advanced receivers, as well as the CSI selection performance of advanced UEs, both under MU interference.

- Define performance requirements for 6Rx UE with interference

  - Up to 4 MIMO layers for the target cell is considered.

    - Inter cell and inter user interference.

    - PDSCH and PMI.

Conclusion

Overall, the TU allocation for 5G-Adv topics in Rel-20 needs to take into account the time required for the 6G SI. Great care therefore needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items.

The following Observations and Proposals were made on the specific domains:

Observation 1: The introduction of SCM based requirements does not necessitate the development of new receivers; rather it provides assurance and lower bounds for the performance of existing (and future) receivers in typical MIMO environments that were blind spots in legacy channel models. Hence, the requirements may be optional in 5GA.

Observation 2: Rel-19 6Rx performance requirements (introduced in NR_ENDC_RF_Ph4) do not currently cover interference scenarios. With 6Rx a substantial gain of advanced receivers in high throughput scenarios is expected.

Observation 3: The SCM topic is foundational for 6G and TUs are expected to be heavily limited for 5GA.

Proposal 1: Rel-20 demod work shall be focused on specifying performance requirements using the rCDL based spatial channel model detailed in TR 38.753 for SU-MIMO in FR1; covering at least 8T8R 2CW, 4T4R 1CW and 8T4R eTypeII use cases.  Additionally, rCDL based practical spatial channel modelling methodology extensions for MU-MIMO demodulation requirements and CSI reporting requirements shall be studied; covering at least eTypeII and advanced receiver use cases.

Proposal 2: Additionally, Rel-20 demod work may be considered to define performance requirements for 6Rx UE with interference; at least the case with up to 4 MIMO layers for the target cell may be considered. Whether both inter cell and inter-user interference, and both PDSCH and PMI requirements are needed shall be discussed.

Proposed WI justification and objectives are provided above.

- References

- RP-251879; WF for RAN4 led REL-20 topics; RAN4 Chair

- RP-250906, Demod topics for 5G-Advanced Rel-20, Nokia

The only remaining CDL-based SCM candidate is agreed to be 38.901 CDLC (UMa delay spread) link-level tables based, with defined truncation procedure to 12 clusters and 2ns delay quantization. The convergence and alignment speedup framework from TR 38.827 is also included.