3GPP TSG-RAN Meeting #109                                           RP-252740

Beijing, China, September 15 – 18, 2025

Source: 	Huawei, HiSilicon

Title: 	Proposals for RAN4-led Rel-20 UE OTA topics

Agenda Item:	9.1.4.3

Document for:	Discussion and Decision

Introduction

In last RAN plenary meeting, Rel-20 RAN4 topics have been discussed and a WF can be found at [1]. There are still some remaining issues left for further consideration and discussion. In this contribution, we would like to share our views for UE OTA topics.

Discussion

UE OTA enhancement

Agreements in RAN#108:

The high-level scope of the WI on UE RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - The performance requirements of UE OTA, including TRP/TRS and MIMO OTA, for new bands - Handheld UE is the first priority, other UE types are 2nd priority if sufficient number devices is available - initial bands n7,n20, n79, needs operators input for other bands - the total number of band should be limited to [5] - prioritized UE type should be specified in the WID - Specify test method to support NTN VSAT - Define the measurement setup, test procedure, and preliminary MU - No RAN1/2 impact is expected - Target 0.5 TU allocation

While the main focus of UE OTA enhancement in Rel-20 becomes clear, detailed objectives need to be determined as well in order to stabilize the whole WI scope.

For MIMO OTA, only a limited number of bands would be covered under Rel-19 WI. However, such coverage is poor given that more frequency bands have already been deployed in the field. It is straightforward to select more representative bands for both FR1 and FR2. For multiple TRP test scenarios, it should be noted that multi-TRP is a key feature to improve cell edge performance. It is a Rel-16 feature and a number of commercial device already supported those functionalities. Furthermore, by the time this work item enters the measurement campaign phase to define performance requirement around Q4 2026, more commercial devices supporting Rel-16 features would become available. Unfortunately, current MIMO OTA test is only for single TRP. Therefore, MIMO OTA test covering multi-TRP can be considered together with the usage of dynamic channels which is completed in Rel-19 MIMO OTA study.

TRP TRS evolution could be another part of the WI. For instance, more FR1 bands and dedicated test procedure and requirements for wearable devices e.g. watch can be considered. Besides, since Rel-19 WI only completes OTA test procedure for DL 2CC CA, we think Rel-20 WI can cover UL 2CC CA test procedure and performance requirements for both UL and DL 2CC CA.

In conclusion, we have the following proposal on WI objective.

Proposal 1: Adopt following contents as the Rel-20 OTA enhancements WI objective.

Objective of SI or Core part WI or Testing part WI Objectives for this core part work item are as follows - Define test methodology for FR1 wearable devices - Define TRP and TRS test methodology and configuration - The performance metric of 1Rx and 2Rx devices - Testing time reduction solutions can be considered (further measurement grid optimization is precluded) - Study and specify the proper head phantom if needed, consider the coordination with CTIA on this aspect - Study and define test methodology and radiated performance metric for NTN VSAT devices - Study the usage scenarios and develop enhanced test methodology - Study and specify the proper performance metric - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system (RAN5) NOTE: During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic. Objective of Performance part WI Objectives for this performance part work item are as follows - Specify TRP and TRS requirements: - Specify FR1 TRP and TRS requirements for NR bands for UE based on operators’ demand - Handheld UEs with 1Tx at new bands n7, n20, n79 - Specify FR1 wearable TRP and TRS requirements - Specify MIMO OTA requirements: - Specify FR1 MIMO OTA requirements for new bands for handheld UE based on operators’ demand (2 layers or 4 layers) - Band n7, n20, n79 are first priority

Conclusion

In this paper we share our views on the Rel-20 WI scope of UE OTA topics based on the WF for RAN4 led REL-20 topics discussed in RAN#108 meeting.

Proposal 1: Adopt following contents as the Rel-20 OTA enhancements WI objective.

Objective of SI or Core part WI or Testing part WI Objectives for this core part work item are as follows: - Define test methodology for FR1 wearable devices - Define TRP and TRS test methodology and configuration - The performance metric of 1Rx and 2Rx devices - Testing time reduction solutions can be considered (further measurement grid optimization is precluded) - Study and specify the proper head phantom if needed, consider the coordination with CTIA on this aspect - Study and define test methodology and radiated performance metric for NTN VSAT devices - Study the usage scenarios and develop enhanced test methodology - Study and specify the proper performance metric - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system (RAN5) NOTE: During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic. Objective of Performance part WI Objectives for this performance part work item are as follows: - Specify TRP and TRS requirements: - Specify FR1 TRP and TRS requirements for NR bands for UE based on operators’ demand - Handheld UEs with 1Tx at new bands n7, n20, n79 - Specify FR1 wearable TRP and TRS requirements - Specify MIMO OTA requirements: - Specify FR1 MIMO OTA requirements for new bands for handheld UE based on operators’ demand (2 layers or 4 layers) - Band n7, n20, n79 are first priority

- References

[1] RP-251879, WF for RAN4 led REL-20 topics.