3GPP TSG-RAN Meeting # 109	RP-252746

# Beijing, China, Sep 15th – 18th, 2025

Title: 	Discussion on introduction of UE capability on max 6 DL MIMO layers

Source: 	Huawei, HiSilicon, MediaTek, Apple

Agenda item:	9.6.1.4

Document for:	Discussion

- Background

During RAN2#113, RAN2 technically endorsed two set of CRs on introduction of UE capability for max DL 6 MIMO layers [1, 7-10]:

RAN2 will provide two sets of technically endorsed CRs and it is up to the plenary to decide The CRs that capture general capability are technically endorsed in R2-2506259 and R2-2506260 The CRs that capture specific capability are technically endorsed in R2-2506466 and R2-2506467

The introduction of this UE capability is requested by RAN4 for one of UE features of WI NR_ENDC_RF_Ph4 [2]. As per RAN4 agreement, LS about the Rel-19 RAN4 UE feature list for NR was sent to RAN2 [5, 6] in RAN4#115 and RAN4#116. The feature 46-6 clearly states that “46-6 Support maximum 6 MIMO layers for DL reception indicates whether the UE supports maximum 6 spatial multiplexing layers for DL reception for FWA”, maybe some companies in RAN2 have misunderstood RAN4 agreement, in this paper, we share the related RAN4 agreements and the LS to help RAN2 to correctly understand RAN4’s agreement.

- Discussion

To ensure companies have correct understanding on RAN4 agreement and the RAN2 principle on introduction of UE capability signalling, we go through the whole discussion procedure in the order of time.

1: RAN4 latest updated WID RP-250780

As per RAN4 evaluation results on the gain and feasibility of support 6 MIMO layers for both handheld UE and FWA, RAN4 concluded that support of max 6 DL MIMO layers is an optional feature and only applicable for FWA. It was agreed in RAN#107 that 6MIMO layers is an optional feature only for FWA. This is also reflected by the updated WID RP-250780 approved in RAN#107 meeting as below:

During the following RAN#107 meeting it was agreed to: [3] - Introduce 6 MIMO layers support as an optional feature only for FWA, which is captured as the objective in the latest WID RP-250780. 6Rx for handheld and FWA UE [RAN4, RAN1] [2] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Introduce 6 MIMO layers support as an optional feature only for FWA. - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

From above RAN4 updated WID, we can directly get the following observation.

Observation 1: The scope in RAN4 WID clearly states to introduce 6 MIMO layers as an optional feature only for FWA

2: LS to RAN2 on request introduction of UE capability from RAN4

As per RAN4 evaluation results and the updated WID [2], RAN4 agreed to send a LS to RAN2 to request the introduction of UE capability signalling for support of max 6 DL MIMO layers for PDSCH [3]:

During the RAN4#114bis meeting the following further conclusions have been reached: - Introduce UE capability signalling to allow indication of the support of maximum 6 DL MIMO layers for PDSCH in addition to the existing capability of 2, 4 and 8 maximum DL MIMO layers. - The capability signalling shall be defined as optional. - The signalling details are up to RAN2.

3: LS to RAN4 on Rel-19 UE features implementation from RAN2

To give clear guidance/principle to RAN1/RAN4 on the introduction of UE capability signalling, RAN2 sent LS [4] to RAN1/RAN4. It clearly informs RAN1/RAN4 that RAN2 will only implement feature group that are included in RAN1/RAN4 feature list LS in RAN2 UE capability CR. Although early LS [3] can be sent to RAN2 to explain capabilities, but those capabilities still need to be captured in the feature list at the end. So RAN2 should introduce the UE capability signalling as per RAN4 feature list LS [5,6].

Besides, RAN2 also would like to inform RAN1 and RAN4 that, RAN2 will only implement feature groups that are included in RAN1/RAN4 feature list LS in UE capability CR. Additionally, UE feature groups with any FFS, highlighted yellow, [] and/or marked as FFS/TBD will not be captured into the UE capability CRs. Also, the capabilities that are dependent on FFS capabilities will not be implemented. RAN2 understands that RAN1 and RAN4 can still send early LSs to explain capabilities, but it is important that those capabilities are all captured in the feature list at the end.

From the LS from RAN2, we can know the work principle for RAN2 to implement the feature groups in RAN2 UE capability CR.

Observation 2: RAN2 will only implement feature groups that are included in RAN1/RAN4 feature list LS in UE capability CR.

4: LS to RAN2 about the Rel-19 RAN4 feature list

As per RAN4 discussion on Rel-19 feature list, RAN4 sent LS to RAN2 about the Rel-19 RAN4 feature list [5,6]. Feature 46-6 is included as per RAN4 agreement for the feature discussion for WI NR_ENDC_RF_Ph4 [2]:

As per RAN2 guidance on introduction of new UE capability [4] and the usual work procedure between 3GPP WG, RAN2 should implement the feature of support of maximum 6 MIMO layers for DL reception by introducing a UE capability signalling that is only applicable for FWA in RAN2 specifications TS 38.331 and TS 38.306. RAN4 agreement is very clear and is indisputable.

Observation 3: The new UE capability of support max 6 DL MIMO layers that is only applicable for FWA is clearly captured in RAN4 feature list LS to RAN2.

The set of CR R2-2506466 and R2-2506467 correctly reflect RAN4 agreement with the following yellow highlighted part description:

The maxNumberMIMO-LayersPDSCH-v1900 is only included for FWA UE as defined in TS 38.101-1 [2]. The UE supporting maxNumberMIMO-LayersPDSCH-v1900 shall also indicate support of maxNumberMIMO-LayersPDSCH for the corresponding component carrier. When maxNumberMIMO-LayersPDSCH and maxNumberMIMO-LayersPDSCH-v1900 are reported together for a component carrier, the network which is able to decode maxNumberMIMO-LayersPDSCH-v1900 ignores maxNumberMIMO-LayersPDSCH.

But another set of CRs doesn’t clearly capture the applicability of this UE capability signalling only for FWA and wants to make this UE capability generic to be applicable for both FWA and Handheld UE, but this is definitely inconsistent with RAN4 agreement.

Observation 4: The set of CRs R2-2506466 and R2-2506467 technically endorsed by RAN2 correctly reflects RAN4 agreement.

5: RAN4 specification on the restriction on max DL MIMO layers

As if the companies supporting another set of CRs R2-2506259 and R2-2506260 clearly know RAN4 agreement that the UE capability about max 6 DL MIMO layers should be only applicable for FWA, they just think that such applicability restriction can be reflected in RAN4 specification as did for 8Rx/8layers, i.e. RAN2 just introduces a generic signalling without differentiating FWA and Handheld UE in RAN2 specification. Here we would like to emphasize that RAN4 only introduced requirements for 8Rx targeted for FWA in Rel-18, but RAN4 introduced separate requirements for 6Rx targeted for FWA and Handheld UE [12]:

So people can’t recognize the feature of max 6 DL MIMO layers for 6Rx is only applicable for FWA purely from RAN4 specification point of view.

Observation 5: People can’t recognize the feature of max 6 DL MIMO layers for 6Rx is only applicable for FWA from RAN4 specification point of view.

RAN2 introduced many UE capabilities signalling for other features that are only applicable for certain device type as per RAN1/RAN4 agreements reflected in feature list. For example:

drx-Adaptation-r16, drx-Adaptation-r17 … In this version of the specification, this feature is not applicable in NTN.

powerClass, powerClass-v1610 … This capability is not applicable to IAB-MT or NCR-MT.

ncd-SSB-BWP-Wor-r18 … NOTE: This feature applies only to PCell and PSCell (if configured). It is not applicable to RedCap or eRedCap UEs.

powerBoosting-pi2BPSK Indicates whether UE supports power boosting for pi/2 BPSK, when applicable as defined in 6.2 of TS 38.101-1 [2] / TS 38.101-5 [34]. It is mandatory with capability signalling. This capability is not applicable to IAB-MT.

rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 … It is not applicable to RedCap or eRedCap UEs.

support3MHz-ChannelBW-Asymmetric-r18 support3MHz-ChannelBW-Symmetric-r18 … This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18.

…

It is RAN2’s duty to correctly introduce UE capability signalling as per the feature groups included in RAN1/RAN4 feature list LS in RAN2 UE capability CR. As per RAN4 Rel-19 feature list LS [5,6], it is undoubtful that the set of CR R2-2506466 and R2-2506467 should be agreed.

Observation 6: Many other UE capabilities signalling that are only applicable for certain device type were introduced by RAN2 as per RAN1/RAN4 feature list LS before.

Proposal: RAN formally approves the set of CRs R2-2506466 and R2-2506467 technically endorsed by RAN2.

- Conclusion

As per our analysis shown above, our observations and proposals are summarized as following:

Observation 1: The scope in RAN4 WID clearly states to introduce 6 MIMO layers as an optional feature only for FWA

Observation 2: RAN2 will only implement feature groups that are included in RAN1/RAN4 feature list LS in UE capability CR.

Observation 3: The new UE capability of support max 6 DL MIMO layers that is only applicable for FWA is clearly captured in RAN4 feature list LS to RAN2.

Observation 4: The set of CRs R2-2506466 and R2-2506467 technically endorsed by RAN2 correctly reflects RAN4 agreement.

Observation 5: People can’t recognize the feature of max 6 DL MIMO layers for 6Rx is only applicable for FWA from RAN4 specification point of view.

Observation 6: Many other UE capabilities signalling that are only applicable for certain device type were introduced by RAN2 as per RAN1/RAN4 feature list LS before.

Proposal: RAN formally approves the set of CRs R2-2506466 and R2-2506467 technically endorsed by RAN2.

- Reference

[1] R2_131_Chairnotes_25-08-29_17-00_final, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_131/Inbox/Chair_Notes/R2_131_Chairnotes_25-08-29_17-00_final.docx

[2] RP-250780 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon, RAN#107, Incheon, Korea

[3] R4-2505225 LS on DL MIMO layers capabilities for 6Rx UEs, Intel, RAN4#114bis, Wuhan, China

[4] R4-2505316 LS on Rel-19 higher layer parameter and UE Feature, RAN2, Malta, MT

[5] R4-2508078 LS on R19 RAN4 UE feature list for NR, CMCC, RAN4#115, Malta, MT

[6] R4-2511884 LS on R19 RAN4 UE feature list for NR, CMCC, RAN4#116, Bangaluru, India

[7] R2-2506466 Introduction of UE capability on 6 DL MIMO layers for TS 38.331, Huawei, HiSilicon, Samsung, MediaTek Inc., Apple, RAN2#131, Bangaluru, India

[8] R2-2506467 Introduction of UE capability on 6 DL MIMO layers for TS 38.306, Huawei, HiSilicon, Samsung, MediaTek Inc., Apple, RAN2#131, Bangaluru, India

[9] R2-2506259 Introduction of 6 DL MIMO layer for TS 38.331, Xiaomi, Intel Corporation, Oppo, Ericsson, Nokia, Qualcomm Incorporated, ZTE, Sanechips, CATT, T-mobile USA, CHTTL, RAN2#131, Bangaluru, India

[10] R2-2506260, Introduction of 6 DL MIMO layer for TS 38.306, Xiaomi, Intel Corporation, Oppo, Ericsson, Nokia, Qualcomm Incorporated, ZTE, Sanechips, CATT, T-mobile USA, CHTTL, RAN2#131, Bangaluru, India

[11] R2-2505303 Discussion on DL MIMO layer capability for 6Rx UE, Xiaomi, Intel Corporation, Oppo, Ericsson, Nokia, Qualcomm Incorporated, ZTE, Sanechips, CATT, T-mobile USA, CHTTL, RAN2#151, Bengaluru, India

[12] R4-2511763	Draft CR to TS38.101-1: Introduction of requirements for 6Rx, ZTE Corporation, Sanechips, Huawei, HiSilicon, AT&T, vivo, Apple, OPPO, Xiaomi, RAN4#116, Bengluru, India