3GPP TSG RAN Meeting #109	RP-252748

Beijing, China, September 15-18, 2025

Source:	BT, AT&T, Bouygues Telecom, Deutsche Telekom, Orange, SK Telecom, T-Mobile USA, Telefónica, TIM, Vodafone, Nokia

Title:	Discussion of WID objectives for NR demodulation performance for Rel-20

Agenda item:	9.1.4.5

Document for:	Discussion

Introduction

During RAN#108 the way forward regarding the Demodulation performance evolution has been endorsed [1]. This contribution proposes the scope for the WID draft for discussion and decision in RAN#109.

Justification

In Rel-20, to verify the UE demodulation performance with a spatially consistent channel model (SCM) and to apply demodulation requirements to more practical network scenarios, the following enhancements are identified:

•	Normative work on spatial channel model based requirements for SU-MIMO in FR1.

•	Study on spatial channel model for MU-MIMO

The Rel-19 study on spatial channel model for demodulation performance requirements for NR has concluded with recommendations on the scope for SU-MIMO UE demodulation performance requirements.

In particular, the study found that the TR 38.753 CDL based spatial channel model framework, is ready for use to specify normative demodulation and CSI performance requirements. Furthermore, it is aligned with RAN1 assumptions for core feature development and was found to exhibit typical spatially consistent spatial components and MIMO effects. Specific use cases for SCMs have been identified in setups with multiple codewords, advanced receivers, enhanced codebooks, and SDM reliant systems in general.

Currently, even if the UE conforms to the current minimum performance requirements, this does not guarantee that in the field the UE will make a reasonably efficient use of MIMO features specified since R15, e.g., providing appropriate channel state information to the network such that the network can make optimal use of the available MIMO layers.

MNOs have invested heavily in roll-out of 5G NR MIMO systems with enhancements specified since R15. MIMO feature performance requirements need to be set under adequate test conditions that mimic MIMO effects observed in deployment as much as possible. The currently used TDL channels do not exhibit these effects nor match the expected statistics of a measured channel.

Proposed Objective of Performance part WI

Based on Section 9 in the way forward from RAN#108 [1], the work item should include the following objectives:

- Specify performance requirements under a spatial channel model for SU-MIMO in FR1

  - Using the rCDL based spatial channel model detailed in TR 38.753

    - Specify PDSCH demodulation performance requirements for the 4T4R case (1 codeword) and the 8T8R (2 codeword) use case.

    - Specify CSI reporting performance requirements for the 4T4R, 8T4R and 32T4R eTypeII precoding use case as studied in the TR38.753.

- Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements and CSI reporting requirements

  - Base work on the rCDL based spatial channel model detailed in TR 38.753.

    - Consider spatially consistent approaches based both on creating a single channel for a single UE, as well as, creating channels for all UEs.

    - At least, consider the TPUT performance of advanced receivers, as well as the CSI selection performance of advanced UEs, both under MU interference.

Adopt the following as objectives of the WI:

- Specify performance requirements under a spatial channel model for SU-MIMO in FR1

  - Using the rCDL based spatial channel model detailed in TR 38.753

    - Specify PDSCH demodulation performance requirements for the 4T4R case (1 codeword) and the 8T8R (2 codeword) use case.

    - Specify CSI reporting performance requirements for the 4T4R, 8T4R and 32T4R eTypeII precoding use case as studied in the TR38.753.

- Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements and CSI reporting requirements

  - Base work on the rCDL based spatial channel model detailed in TR 38.753.

    - Consider spatially consistent approaches based both on creating a single channel for a single UE, as well as, creating channels for all UEs.

    - At least, consider the TPUT performance of advanced receivers, as well as the CSI selection performance of advanced UEs, both under MU interference.

Purely from organisational perspective, decoupling of normative (1st bullet) and study phase (2nd bullet) may be beneficial.

Decouple the work into normative (1st bullet) and study phase (2nd bullet).

Conclusion

This contribution proposes the following for the Release 20 NR Demodulation performance evolution:

- Adopt the following as objectives of the WI:

- Specify performance requirements under a spatial channel model for SU-MIMO in FR1

  - Using the rCDL based spatial channel model detailed in TR 38.753

    - Specify PDSCH demodulation performance requirements for the 4T4R case (1 codeword) and the 8T8R (2 codeword) use case.

    - Specify CSI reporting performance requirements for the 4T4R, 8T4R and 32T4R eTypeII precoding use case as studied in the TR38.753.

- Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements and CSI reporting requirements

  - Base work on the rCDL based spatial channel model detailed in TR 38.753.

    - Consider spatially consistent approaches based both on creating a single channel for a single UE, as well as, creating channels for all UEs.

    - At least, consider the TPUT performance of advanced receivers, as well as the CSI selection performance of advanced UEs, both under MU interference.

Decouple the work into normative (1st bullet) and study phase (2nd bullet).

References

- RP-251879, “Way forward on RAN4 R20 package”, RAN#108, June 2025