3GPP TSG-RAN2 Meeting #131	R2-2506604

# Bengaluru, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 1008 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-19 Evolution of NR duplex operation (SBFD) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-09-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of the SBFD feature in TS38.300 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 3.1, introducing the abbreviation of SBFD. - Introducing the clauses of SBFD. a). X.1 "General" describes SBFD operation, UE configuration, and Tx/Rx/measurement procedures for SBFD aware UEs; b). X.2 "SBFD Random Access" specifies Random Access procedures for SBFD operation. It defines supported RACH events, introduces two SBFD-specific RACH configuration options, and details PRACH occasion handling for both CFRA and CBRA. - Merge the changes from RAN1 (R1-2505081), including: a). in Clause 17.2: UE-to-UE CLI may also be present in case of SBFD operation; both L1-based and L3-based UE-to-UE CLI reporting are supported; b). in Clause 17.2: gNB-to-gNB CLI may be present when different TDD DL/UL patterns are used between neighbouring cells and in case of SBFD operation; UL resource muting can be configured to mitigate gNB-to-gNB CLI; c). in Clause X.1: a UE can be configured with SBFD sub-bands in downlink and flexible symbols provided by tdd-UL-DL-ConfigurationCommon and this UE is referred to as an SBFD-aware UE; the Tx/Rx/Measurement procedures for an SBFD-aware UE are described; refer to Clause 17.2 for CLI handling procedures. d). in Clause X.2: For CFRA triggered by PDCCH order, an SBFD aware UE can be explicitly indicated in the PDCCH order whether to use a PRACH occasion associated with either the first PRACH occasions or the second PRACH occasions, for the PRACH transmission that is initiated by the PDCCH order. - Merge the changes from RAN3 (R3-255986): Cross-Link Interference Management for Sub-band full duplex (SBFD) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG-RAN cannot support SBFD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, 17.2, X(new), X.1(new), X.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.321 CR 2106 TS 38.331 CR 5414 TS 38.331 CR 5403 TS 38.306 CR 1321 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R2-2505088 |  |  |  |  |  |  |  |  |

CHANGE START

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A2X	Aircraft-to-Everything

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

AR	Augmented Reality

ARP	Allocation and Retention Priority

ATG	Air to Ground

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BFD	Beam Failure Detection

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

BRID	Broadcast Remote Identification

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

cellDTRX-RNTI	Cell Discontinuous Transmission and Reception RNTI

CFR	Common Frequency Resource

CFRA	Contention Free Random Access

CG	Configured Grant

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAA	Detect And Avoid

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DCR	Direct Communication Request

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

DSR	Delay Status Report

DTX	Discontinuous Transmission

E-CID	Enhanced Cell-ID (positioning method)

EC	Energy Cost

EHC	Ethernet Header Compression

ePWS	enhancements of Public Warning System

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

FSA ID	Frequency Selection Area Identity

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

H-SFN	Hyper System Frame Number

HAPS	High Altitude Platform Station

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LBT	Listen Before Talk

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

LTM	L1/L2 Triggered Mobility

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

ML	Machine Learning

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MO-SDT	Mobile Originated SDT

MP	Multi-Path

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MT-SDT	Mobile Terminated SDT

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

N3C	Non-3GPP Connection

NB-IoT	Narrow Band Internet of Things

NCD-SSB	Non Cell Defining SSB

NCGI	NR Cell Global Identifier

NCL	Neighbour Cell List

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NES	Network Energy Savings

NGAP	NG Application Protocol

NGSO	Non-Geosynchronous Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDB	Packet Delay Budget

PDC	Propagation Delay Compensation

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PER	Packet Error Rate

PH	Paging Hyperframe

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PQI	PC5 5QI

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PRS	Positioning Reference Signal

PS-RNTI	Power Saving RNTI

PSDB	PDU Set Delay Budget

PSER	PDU Set Error Rate

PSI	PDU Set Importance

PSIHI	PDU Set Integrated Handling Information

PSS	Primary Synchronisation Signal

PTM	Point to Multipoint

PTP	Point to Point

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QMC	QoE Measurement Collection

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RLM	Radio Link Monitoring

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

RVQoE	RAN visible QoE

SBFD         	Sub-Band Full Duplex

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SDT	Small Data Transmission

SD-RSRP	Sidelink Discovery RSRP

SFI-RNTI	Slot Format Indication RNTI

SHR	Successful Handover Report

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SL-PRS	Sidelink Positioning Reference Signal

SL-RSRP	Sidelink RSRP

SMC	Security Mode Command

SMF	Session Management Function

SMTC	SS/PBCH block Measurement Timing Configuration

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SpCell	Special Cell

SPR	Successful PSCell Addition/Change Report

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRAP	Sidelink Relay Adaptation Protocol

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SSSG	Search Space Set Group

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TB	Transport Block

TCE	Trace Collection Entity

TNL	Transport Network Layer

TPC	Transmit Power Control

TRP	Transmit/Receive Point

TRS	Tracking Reference Signal

TSS	Timing Synchronization Status

U2N	UE-to-Network

U2U	UE-to-UE

UAV	Uncrewed Aerial Vehicle

UCI	Uplink Control Information

UDC	Uplink Data Compression

UDM	Unified Data Management

UE-Slice-MBR	UE Slice Maximum Bit Rate

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

VR	Virtual Reality

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

XR	eXtended Reality

17	Interference Management

<Unchanged parts omitted>

17.2	Cross-Link Interference Management

When different TDD DL/UL patterns are used between neighbouring cells, UL transmission in one cell may interfere with DL reception in another cell: this is referred to as UE-to-UE Cross Link Interference (CLI). In case of Sub-Band Full Duplex (SBFD) operation, UE-to-UE CLI can be present either within the same cell or across different cells: UL transmission may interfere with simultaneous DL reception within one cell or in another cell.

To mitigate UE-to-UE CLI, gNBs can exchange and coordinate their intended TDD DL-UL configurations over Xn and F1 interfaces; and the victim UEs can be configured to perform UE-to-UE CLI measurements. There are two types of UE-to-UE CLI measurements:

-	SRS-RSRP measurement in which the UE measures SRS-RSRP over SRS resources of aggressor UE(s);

-	CLI-RSSI measurement in which the UE measures the total received power observed over RSSI resources.

A gNB serving victim UEs may request neighbour gNBs to report SRS resources. The neighbour gNB may signal to neighbour gNBs information concerning SRS resources potentially causing UE-to-UE CLI.

Two types of UE-to-UE CLI reporting are supported in case of Sub-Band Full Duplex (SBFD) operation: L1-based reporting and L3-based reporting. A UE is not expected to be configured with both L1 CLI measurement and reporting and L3 CLI measurement and reporting simultaneously.

For L3-based UE-to-UE CLI reporting, layer 3 filtering applies to CLI measurement results and both event triggered and periodic reporting are supported.

For L1-based UE-to-UE CLI reporting, the configuration is dependent on the reporting quantity:

-	For SRS-RSRP, only aperiodic CSI reporting is supported.

-	For CLI-RSSI, the CSI reporting can be periodic or aperiodic.

In addition to UE-to-UE CLI, gNB-to-gNB CLI may also be present when different TDD DL/UL patterns are used between neighbouring cells or when SBFD operation is configured: DL transmission in one cell may interfere with UL reception in another cell.

To mitigate gNB-to-gNB CLI, a victim gNB can report gNB-to-gNB CLI related information of its serving cells to neighbour gNBs. The neighbour gNB should evaluate the received information and it may take CLI mitigation actions when necessary. Additionally, a victim gNB can configure a UE with UL resource muting. When UL resource muting is applied in a symbol, either even or odd sub-carriers of the frequency resource of the PUSCH are available, and the other sub-carriers are not used for the PUSCH transmission.

<Unchanged parts omitted>

X    SBFD

X.1 General

Sub-Band Full Duplex (SBFD) operation is supported for a TDD carrier, enabling simultaneous downlink transmission and uplink reception at the gNB on their non-overlapping respective sub-bands. From UE perspective, full duplex is not supported. The configurations of cell-specific SBFD time and frequency resources are provided through SIB1 or dedicated signalling.

A UE can be semi-statically configured with SBFD sub-bands in downlink symbols and flexible symbols provided by tdd-UL-DL-ConfigurationCommon and this UE is referred to as an SBFD aware UE. The maximum number of UL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is one. The maximum number of DL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is two.

In an SBFD symbol, except for cross-link interference measurements, a UE transmits or receives only in RBs that are both in the active UL BWP and in the UL sub-band, or both in the active DL BWP and the DL sub-band(s), respectively.

A UE can be configured to transmit or receive only in non-SBFD symbols, only in SBFD symbols, or across both SBFD symbols and non-SBFD symbols for multiple transmission or reception occasions.

X.2 SBFD Random Access

Random access procedure in SBFD symbols is supported for all existing RACH trigger events as described in clause 9.2.6, except for the event of Request for Other SI and Early UL synchronization with an LTM candidate cell. For the event of RACH-based LTM cell switch, random access procedure in SBFD symbols is only supported for intra-DU case.

Both CBRA and CFRA can be supported on SBFD sub-bands. Only the 4-step RA type using SBFD RACH resources can be supported.

Two RACH configuration options are specified for SBFD RA operation in TS 38.331 [12]. A cell can configure only one RACH configuration option. This can be either: 1) A single RACH configuration that supports both non-SBFD RA operation and SBFD RA operation, or 2) A dual RACH configuration where a RACH configuration is used for non-SBFD RA operation and an additional RACH configuration is designated for SBFD RA operation, as specified in TS 38.331 [12]. An SBFD aware UE that supports the RACH configuration option configured in the cell applies the corresponding RACH configuration. Otherwise, the SBFD aware UE applies the non-SBFD RA operation.

For CFRA triggered by PDCCH order, an SBFD aware UE can be explicitly indicated in the PDCCH order whether to use either the first PRACH occasions or the second PRACH occasions as specified in TS 38.213 [38], for the PRACH transmission.

For CBRA, an SBFD aware UE is permitted to switch from the first PRACH occasions to the second PRACH occasions and vice versa as specified in TS 38.213 [38] during a random access procedure. However, no further switch of PRACH occasions type (the first or second PRACH occasions) is allowed in the same random access procedure.

If a fall-back from CFRA to CBRA occurs, an SBFD aware UE uses the CBRA resource associated with the same PRACH occasions type.

CHANGE END