Core part objective:

- Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz

  - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements.

    - Consider impact on the following OTA co-location requirements:

      - Transmitter OFF power requirement

      - Transmitter co-location spurious emission requirement

      - Transmitter intermodulation requirement

      - Co-location OOBB requirement

  - Enhance OTA co-location requirements

    - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered

      - Option 1: Investigate and improve CLRA concept

      - Option 2: Investigate possibility to define requirements using existing OTA metrics (such TRP, EIRP, EIS), if any

Performance part objective:

- To define the conformance testing requirement of co-location requirements according to the core requirements.

Core part objective:

- SBFD BS to SBFD BS adjacent channel coexistence

  - Focus on FR1 WA.

  - Consider both scenarios including SBFD capable BS being co-located and non-collocated

  - Focus on bands: n104, n79, n41.

  - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include

    - TX unwanted emission

    - RX in-band blocking

  - Note 1: whether to comply with requirements is declared by BS.

  - Note 2: The existing TDD requirements apply for non-SBFD BSs

Performance part objective:

- Specify the conformance testing of core RF requirements for SBFD BS to SBFD BS coexistence.