Work item:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

  - Using rCDL-C1 channel model detailed in TR 38.753 as starting point

    - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case.

    - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding.

  - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

  - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference

  - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver

    - Use the Rel-17 interference model for 2/4Rx UE as a baseline

    - Up to 4 MIMO layers for target cell

  - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Note: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference.

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Use the existing interference model for 2/4Rx UE as a baseline

    - Up to 2 MIMO layers for each user

Study item:

- Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements and CSI reporting requirements

  - Base work on the rCDL based spatial channel model detailed in TR 38.753 as staring point.

    - Consider applicability to and support for AI/ML scenarios

  - Note 3: The methodology shall include both FR1 and FR2, with first priority for FR1.

  - Note 4: The channel modelling methodology should be applicable for 5G-A and can be considered 6G testing

  - Note 5: The study is aimed to consider both UL and DL.