Core requirement:

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 2} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4].

    - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

    - Case 2: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

    - Example band combinations: [CA_n12A-n29A and CA_n28-n67], additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed.

    - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible

  - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

    - If the clarification is needed, make the necessary modification by sending LS to RAN1/RAN2

- Define the corresponding UE capabilities [RAN4, RAN2].

- Consider the following deployment constraints:

  - The carrier frequency for all cases is <1GHz.

  - Co-located and synchronized network deployment for both carriers.

  - Both carriers are in a single TAG.

  - SCS 15Khz on both carriers.

  - No in-gap interference between DL carriers is expected

Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

Core part objective

- Scenario:

  - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km.

  - A direct radio link between BS on the ground and UAV UE.

- Specify RF requirements for UAV UE to support higher UL output power.

  - Example bands:

    - TDD with same TDD pattern, including n41, n77, n78, n79,

    - FDD: n3

  - Consider UAV UE with omni-direction antenna or with antenna array

  - Consider UAV UE with up to 33dBm max total conductive output power

  - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed.

  - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

- Specify RRM requirements, if needed

Note 1: No RAN1 impact are expected

Performance part:

Specify the RRM test cases corresponding to the RRM core requirements.