3GPP TSG-WG SA2 Meeting #170	S2-2506341

# 25-29 August 2025, Goteborg, Sweden		(revision of S2-2504777)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1608 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| * |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Update to Initial Call setup and functional support for UE-Satellite-UE communication in IMS |  |  |  |  |  |  |  |  |  |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-07-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Identify explicit PCRT triggers and explicit events used by the P-CSCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Identify event used by P-CSCF as being Access Network Information Notification, and the PCRT trigger set to Access Network Information report |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The call setup procedure not sufficiently clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.1, AE.3.2, AE.4.2, AE.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

# AE.1	General

This annex describes IMS architecture enhancements to support UE-Satellite-UE communication in IMS, i.e., optimized media routing via IMS user plane on-board satellite.

In this Release of the specification, this feature is supported only for IMS voice/video service and for UEs belonging to the same PLMN and in the non-roaming scenario.

The 5GS architecture for UE-Satellite-UE communication is defined in TS 23.501 [93]. Optimized media routing described in this annex refers to the routing of media between UEs under the coverage of the same or different serving satellites, using the 5GS network as the IP-CAN and with the media not transitioning through any network elements on the ground. The ground fallback routing refers to the case with the media transitioning through the ground segment.

The P-CSCFs serving the MO UE and the MT UE negotiate with each other to determine whether or not to activate UE-satellite-UE communication, that is, whether it is possible to apply media routing that only relies on IMS user plane on-board satellite.

## ---Start of the 2nd Change---

## AE.3.2	At call setup

The P-CSCF receives from the PCF the satellite identifier of the satellite serving the local UE and receives from the P-CSCF serving the remote UE the satellite identifier of the satellite serving the remote UE as per clause AE.5.1 and clause AE.5.3. Subject to operator policy, the P-CSCF uses this information to determine whether or not to activate optimized media routing. If the satellite identifier of the satellite serving either the local UE or the remote UE is not available, the P-CSCF does not activate optimized media routing.

NOTE:	In this Release of the specification, the P-CSCF usage of  the satellite identifier information to decide  whether optimized media routing is possible is not specified.

If P-CSCF activates optimized media routing, the P-CSCF instructs the PCF to authorize the necessary resources according to clause AE.5.1 and clause AE.5.3, so that the PCF proceeds to establish a 5GS user plane path for optimized media routing.

---Start of the 3rd Change---

## AE.4.2	At call setup

The P-CSCF serving the MO UE selects an IMS AGW on ground at call setup according to TS 23.334 [74], clause AE.5.1 and clause AE.5.3. The P-CSCFs serving the MO UE and MT UE proceed with the determination of whether UE-Satellite-UE communication is possible as described in clause AE.3.2. If the P-CSCF decides to activate optimized media routing, with consideration of whether early media is in progress or not, the P-CSCF releases the IMS AGW on ground if allocated, and selects instead an IMS AGW on satellite according to TS 23.334 [74], clause AE.5.1 and clause AE.5.2. If early media is in progress during call setup, the P-CSCF serving the MO UE does not release the IMS-AGW on ground until the IMS session is answered, then selects the IMS-AGW on satellite according to clause AE.5.3 for using the optimized media routing.

## ---Start of the 4th Change---

# AE.5	Procedures for optimizing media routing

## AE.5.1	At call setup

When the originating P-CSCF receives the initial INVITE request from the originating UE using regenerative NR satellite access, it selects the IMS-AGW on ground and triggers media negotiation by sending SDP offer to the terminating side indicating that satellite access is used on the originating side. If the terminating P-CSCF activates UE-satellite-UE communication, the originating P-CSCF re-selects an IMS-AGW on satellite. See Figure AE.5.1-1 that describes the procedure. In the procedure, UE A and P-CSCF A represent originating side function or entity, UE B and P-CSCF B represent terminating side function or entity.

NOTE 1:	To support the UE-satellite-UE communication, P-CSCFs (AF) needs to support N5 interface for AF influence on traffic routing and user plane management event subscription/notification.

Figure AE.5.1-1: Session establishment procedure with activation of optimizing media routing

1.	UE A sends a SIP INVITE request, containing an initial SDP offer, to P-CSCF A. The access information in the SIP INVITE request indicates that the UE is accessing from NR satellites access as defined in TS 24.229 [10a].

2.	After receiving the SIP INVITE request indicating NR satellite access, if UE A is not roaming and the P-CSCF A supports UE-Satellite-UE communication, the P-CSCF requests Access Network Information, which includes an indication for requesting satellite identifier information, for UE A from the PCF as described in TS 23.503 [95] via Npcf_PolicyAuthorisation_Create service operation; the event identifier set to Access Network Information Notification. The PCF requests Access Network Information, which includes the identifier of UE A's serving satellite, from the SMF via Npcf_SMPolicyControl_UpdateNotify service operation; the PCRT trigger set to Access Network Information report. Then the PCF receives the Access Network Information via Npcf_SMPolicyControl_Update service operation as described in TS 23.502 [94] and TS 23.503 [95]. This satellite identifier is subsequently conveyed to the P-CSCF in the Access Network Information Notification via Npcf_PolicyAuthorization_Notify service operation.

3.	The P-CSCF A allocates an IMS-AGW on the ground as default. P-CSCF A interacts with IMS-AGW on the ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 4-20 are performed if the UE-Satellite-UE communication is possible based on step 2 (i.e. UE A is using NR regenerative payload satellite access and is not roaming). Otherwise, procedures for IMS user plane traffic routing with IMS-AGW on ground are performed (details have been omitted from the diagram).

4.	P-CSCF A updates the SIP INVITE request inserting the identifier of the satellite serving UE A and updates SDP offer with the IMS-AGW transport addresses allocated in step 3. P-CSCF A forwards the updated SIP INVITE request to the terminating side via IMS core (details have been omitted from the diagram).

5.	After receiving the SIP INVITE request which contains a satellite identifier indicating that UE A is using regenerative satellite access, if UE B is not roaming and the P-CSCF B supports UE-Satellite-UE communication, the P-CSCF B requests Access Network Information, which includes an indication for requesting satellite identifier information, for UE B from PCF as described in TS 23.503 [95] via Npcf_PolicyAuthorization_Create service operation; the event identifier set to Access Network Information Notification. The PCF obtains Access Network Information, which may include an identifier of the satellite serving UE B, from the SMF via Npcf_SMPolicyControl_UpdateNotify; the PCRT trigger set to Access Network Information report and Npcf_SMPolicyControl_Update service operations as described in TS 23.502 [94] and TS 23.503 [95]. This satellite identifier, if received, is subsequently conveyed to the P-CSCF in the Access Network Information Notification via Npcf_PolicyAuthorization_Notify service operation.

NOTE 2:	P-CSCF B can decide to include an indication for requesting satellite identifier information based on e.g. access information in the SIP REGISTER request (or other SIP messages) which indicates that UE B is accessing from NR satellites access as defined in TS 24.229 [10a].

6.	Based on the Access Network Information received at step 5, if UE B is using regenerative payload satellite access and if the two satellites identified in steps 4 and 5 are the same or connected with ISL(s), P-CSCF B activates the UE-Satellite-UE communication in IMS.

If UE-Satellite-UE communications in IMS is not activated, P-CSCF B selects an IMS-AGW on ground for UE B; otherwise, P-CSCF B selects an IMS-AGW on satellite serving UE B.

NOTE 3:	How the terminating P-CSCF uses the satellite identifiers to determine whether the two satellites are connected with ISL(s) and are equipped with UPF and IMS-AGW is based on implementation or  operators' policy e.g. by querying transport network system or based on node level information reported by the IMS-AGW deployed on the satellites.

6a-6b.	Based on the decision in step 6, P-CSCF B interacts with IMS-AGW onboard satellite or IMS-AGW on ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 7-20 are performed if the UE-Satellite-UE communication is activated. Otherwise, SIP message routing procedures with IMS-AGW on the ground are performed after P-CSCF B selects a ground IMS-AGW for usage (details have been omitted from the diagram).

7-9.	P-CSCF B forwards the updated SIP INVITE request to UE B. UE B returns an 18X response with SDP Answer as normal SIP message routing procedures.

9a-9b.	After receiving the SDP Answer, P-CSCF B interacts with the IMS-AGW onboard the satellite to allocate transport resources based on the mechanism defined in TS 23.334 [74].

10.	P-CSCF B instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation. To enable the SMF to insert a UL CL/BP and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication, the P-CSCF B includes the  following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which includes the IP address of the IMS-AGW onboard the satellite;

-	A DNAI associated with the IMS-AGW onboard the satellite and optionally corresponding N6 traffic routing information.

NOTE 4:	DNAI value can be derived from the satellite identifier as per operator policy and implementation.

10a. To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AE.5.2, P-CSCF B subscribes to notification of User Plane management events (i.e., UP path change) via Npcf_PolicyAuthorization_Update service operation to the PCF, which in turn subscribes to the events from SMF via Npcf_SMPolicyControl_UpdateNotify service operation and receives the event notifications from the SMF via Nsmf_EventExposure_Notify service operation as specified in clause 4.3.6 of TS 23.502 [94]. The P-CSCF B inserts the following parameters in its subscription request (i.e. Npcf_PolicyAuthorization_Update service operation) as specified in clause 5.2.5.3.3 of TS 23.502 [94]:

-	Early and late notifications about UP path management events.

NOTE 5:	 If Rx is used by P-CSCF, implicit subscription is assumed as part of PCC rules setting.

11.	P-CSCF B updates the 18X response to carry the identifier of the satellite serving UE B to indicate that UE-Satellite-UE communication is activated. Then, P-CSCF B returns the updated 18X response to originating side via IMS core (details have been omitted from the diagram).

12.	Based on the identifier of the satellite serving UE B in step 11, P-CSCF A determines to perform UE-satellite-UE communication and select an IMS-AGW onboard satellite.

12a-12c.	P-CSCF A interacts with IMS-AGW onboard satellite to allocate transport resources for both IMS access and IMS Core network sides. P-CSCF A also releases the IMS-AGW on ground.

13.	P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation. To indicate to the SMF to insert a UL CL/BP and L-PSA UPF for the IMS PDU session to enable UE-Satellite-UE communication in IMS, P-CSCF B should send following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW onboard satellite;

-	A DNAI associated with the IMS-AGW onboard satellite and optionally corresponding N6 traffic routing information.

NOTE 6:	DNAI value can be derived from the satellite identifier as per operator policy and implementation.

13a. To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AE.5.2, P-CSCF A subscribes to notification of user plane management events (i.e., UP path change) via Npcf_PolicyAuthorization_Update service operation to the PCF, which in turn subscribes to the events from SMF via Npcf_SMPolicyControl_UpdateNotify service operation and receives the event notifications from the SMF via Nsmf_EventExposure_Notify service operation as specified in clause 4.3.6 of TS 23.502 [94]. The P-CSCF A inserts the following parameters in its subscription request (i.e. Npcf_PolicyAuthorization_Update service operation) as specified in clause 5.2.5.3.3 of TS 23.502 [94]:

-	Early and late notifications about UP path management events.

NOTE 7:	 If Rx is used by P-CSCF, implicit subscription is assumed as part of PCC rules setting.

14.	P-CSCF A updates the SDP answer in the 18X response with the IMS-AGW transport addresses allocated in step 12, and forwards the updated 18X response to UE A.

15.	UE A acknowledges the 18X Response and sends the Response Confirmation to P-CSCF A.

16-17.	P-CSCF A forwards the Response Confirmation to P-CSCF B. The Response Confirmation includes SDP offer modified/generated by P-CSCF A which contains the IMS-AGW transport address obtained in step 12. Upon receiving the SDP offer, P-CSCF B updates the allocated IMS-AGW on the satellite for UE B with the transport address in the SDP offer.

18.	P-CSCF B sends the Response Confirmation to UE B.

19.	Procedure continues to setup the call as defined in clauses 5.6 and 5.7.

## ---End of the Change---