3GPP TSG-SA WG6 Meeting #68	S6-253449

Goteborg, Sweden 25th – 29th August 2025	(revision of S6-253166)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0156 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe configurations correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5GProSe |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are several misconfigurations and wrong information captured in TS 23.289, which is not aligned with the concept specified in TS 23.304. Therefore, this CR corrects the configurations related to MC services over 5G ProSe UE-to-Network relay, and 5G ProSe UE-to-UE relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - in clauses 7.6.1 and 7.6.2 correct that the ProSe discovery group ID is no longer needed, and that ProSe Layer-2 ID is used to target user info in case of off-network private communication. - update clause A.3 and remove MC group ID. - add MC service group configuration data related to support utilizing 5G ProSe UE-to-UE and UE-to-Network relay. - add MC service UE configuration data related to support utilizing 5G ProSe UE-to-UE and UE-to-Network relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configurations to enable 5G ProSe UE-to-Network and UE-to-UE relay are not correct not aligned with TS 23.304. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.1, 7.6.2, A.3, A.4 and A.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.6	MC service over 5G ProSe

### 7.6.1	Off network group communication for MC service

The procedures and information flows used in 3GPP TS 23.280 [3], 3GPP TS 23.379 [6], 3GPP TS 23.281 [4], and 3GPP TS 23.282 [5] are reused for Off network group communication for MC service over 5G ProSe with following differences:

-	The ProSe discovery Group ID is not used for off-network group communication via 5G ProSe UE-to-UE relay as specified in 3GPP TS 23.304 [17].

-	The ProSe Layer-2 Group ID of the target MC service group is  used by the ProSe layer as the target group info, as specified in 3GPP TS 23.304 [17].

-	The Groupcast mode 5G ProSe Direct Communication procedure in clause 6.4.2 of 3GPP TS 23.304 [17] is utilized for the establishment of off-network group communication and the following media plane traffic exchanges.

### 7.6.2	Off-network private communication

The procedures and information flows used in 3GPP TS 23.280 [3], 3GPP TS 23.379 [6], 3GPP TS 23.281 [4], and 3GPP TS 23.282 [5] are reused for off-network private communication for MC service over 5G ProSe with following differences:

-	PPPP (ProSe Per-Packet Priority) is replaced with PQI, as specified in 3GPP TS 23.304 [17].

-	The ProSe Layer-2 ID of the target MC service user is used by the ProSe layer as the target user info, as specified in 3GPP TS 23.304 [17].

-	The ProSe discovery group ID is not used for off-network private communication via 5G ProSe UE-to-UE relay as specified in 3GPP TS 23.304 [17]. The Application layer group ID and the user info ID of the target MC service user are provisioned to the MC service client in the MC service user profile. The MC service client enables the ProSe layer to determine the IP address for the communication with the target MC service user by providing the Application layer group ID and user info ID of the target MC service user.

-	Group Member Discovery procedure in 3GPP TS 23.304 [17] is utilized to enable for the call initiator to determine whether the called MC service user is in the proximity of the calling user prior to the establishment of an off-network private communication.

-	The Unicast mode 5G ProSe Direct Communication procedure in 3GPP TS 23.304 [17] is utilized for the establishment of off-network private communication and the following media plane traffic exchanges.

# * * * * Second change * * * *

# A.3	MC service user profile configuration data

The configuration data defined in 3GPP TS 23.379 [6] in Annex A.3, in 3GPP TS 23.281 [4] in Annex A.3, and 3GPP TS 23.282 [5] in Annex A.3 apply, with the following exceptions:

-	The "ProSe discovery group ID" is not used;

The configuration data defined in 3GPP TS 23.281 [4] in Annex A.3, apply, with the following exceptions:

-	The "ProSe discovery group ID" is not used;

The configuration data defined in 3GPP TS 23.282 [5] in Annex A.3, apply, with the following exceptions:

-	The "Discovery group ID" is not used;

# * * * * Third change * * * *

# A.4	MC service group configuration data

The configuration data defined in 3GPP TS 23.379 [6] in Annex A.4, 3GPP TS 23.281 [4] in Annex A.4, 3GPP TS 23.282 [5] in Annex A.4, apply, with the following exceptions:

-	" Default PQI (as specified in 3GPP TS 23.304 [17]) values" instead of "Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values" shall be used;

Furthermore, Table A.4-1 and Table A.4-2 include the necessary configurations to enable 5G ProSe UE-to-Network Relay and 5G ProSe UE-to-UE Relay services (off-network communication), respectively.

Table A.4-1: Common group configuration data to enable 5G ProSe UE-to-network relay service (on-network)

| Reference | Parameter description | MC service UE | MC service Server | Group management server |
| --- | --- | --- | --- | --- |
| Subclause 4.8.2 | RSC(s) associated to 5G ProSe UE-to-Network Relay services (as specified in 3GPP TS 23.304 [17]) | Y | N | Y |
|  | > UE-to-Network Relay Layer indicator per RSC (NOTE) |  |  |  |
| NOTE: The UE-to-Network Relay Layer indicator indicates whether the associated RSC offers 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. |  |  |  |  |

Table A.4-2: Common group configuration data to enable 5G ProSe UE-to-UE relay service (off-network)

| Reference | Parameter description | MC service UE | MC service server | Group management server |
| --- | --- | --- | --- | --- |
| Subclause 4.10.2 | RSC(s) associated to 5G ProSe UE-to-UE Relay services (as specified in 3GPP TS 23.304 [17]) | Y | N | Y |
| Subclause 4.10.2 | ProSe layer-2 group ID (as specified in 3GPP TS 23.304 [17]) | Y | N | Y |
| NOTE: The UE-to-UE Relay Layer indicator indicates whether the associated RSC offers 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service. |  |  |  |  |

# * * * * Fourth change * * * *

# A.x 	MC UE configuration data

Annex A.2 in 3GPP TS 23.379 [6], Annex 2 in 3GPP TS 23.281 [4] and Annex 2 in 3GPP TS 23.282 [5] applies with the following updates in Table A.2-2 in 3GPP TS 23.379 [6], Table A.2-2 in 3GPP TS 23.281 [4] and Table A.2-2 in 3GPP TS 23.282 [5] to support 5G ProSe UE-to-Network Relay services as described in clause 4.8.

Table A.x-1: UE configuration data to enable 5G ProSe UE-to-Network Relay service (on network)

| Reference | Parameter description |
| --- | --- |
| Subclause 4.8.2 | Relay service (Y/N) |
| Subclause 4.8.2 | List of allowed relayed MCPTT groups, MCVideo groups, or MCData groups and their RSC(s) (as specified in 3GPP TS 23.304 [17]) (optional) (see NOTE) |
|  | > MCPTT group ID, MCVideo group ID, or MCData group ID |
|  | > RSC(s) for 5G ProSe UE-to-Network Relay service (as specified in 3GPP TS 23.304 [17]) |
| NOTE: When the value of the parameter Relay service is N, this parameter and its child parameters are not needed. |  |

# * * * * End of changes * * * *