3GPP TSG-SA WG6 Meeting #68	S6-253079

Gothenburg, Sweden, 25th – 29th August 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0053 | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix for clause 9.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There are missing mandtory parameters in procedure step 1. - There is Requestor identity & Requestor security credentials in Spatial localization service request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing mandtory parameters in procedure step 1. - Fix to requestor identifier & security credentials in Spatial localization service request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent with Spatial Anchors (SAn) Service |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.2, 9.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

9.4.2	Procedure

Figure 9.4.2-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to invoke spatial localization services. The service is provided by SEAL SM server.

Pre-condition:

1)	Spatial map is created and the authorized consumer is aware of the spatial map identity.

Figure 9.4.2-1: Spatial localization services

1)	The VAL server (or SEAL SM client) sends request to perform localization service in the spatial map to the SEAL SM server. The request includes a requestor identifier, security credentials, and identity of the map on which the action is to be performed. The request may include list of users (or UEs) to localize.

2)	The SEAL SM server authorizes the requestor (e.g. VAL server or SEAL SM client). If the requestor is not authorized then the SEAL SM server sends failure response. If the request is authorized: the SEAL SM server, identifies all users/UEs localization along with their three dimensional location and pose in the spatial map. If area of interest IE is present then the SEAL SM server identifies the target users/UEs within the area of interest only, otherwise, the SEAL SM server identifies the target users for whole map (according to the area associated with spatial map identifier).

3)	The SEAL SM server sends response to the VAL server (or SEAL SM client). The response includes the object localization and identity.

NOTE:	In this release, the localization of UE is considered. Localization of all objects will be considered in future release.

# * * * * Next change * * * *

9.4.3.1	Spatial localization service request

Table 9.4.3.1-1 describes information elements for the spatial localization service request from the SM client or VAL server to the SM server.

Table 9.4.3.1-1: Spatial localization service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL User or VAL UE, VAL server) |
| Security credentials | M | The security credentials of the requestor. |
| Spatial Map Id | M | Identity of the spatial map |
| Area of interest | O | Contains three dimensional area within spatial map |
| Target identities | O | List of identity of the users or objects to localize |

# * * * * End of changes * * * *