3GPP TSG-SA WG6 Meeting #68	S6-253359

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0252 | rev |  | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IE tables with Result return |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In a reponse message with result, some IEs are meaningness when the return result is failure. These meaningness IEs returned in a response message not only wasting the valueable network resources and alos might misguide the stage 3 development. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make corrections to the relevant information flows to clarify what some IEs shall be returned only in a success response message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect guidance to stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2.3, 7.1.2.2.4, 7.19.2.5, 7.19.2.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.1.2.2.3	Group call response

Table 7.1.2.2.3-1 describes the information flow group call response from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.3-1: Group call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is requested |
| SDP answer (NOTE) | O | Media parameters selected |
| Result | M | Result of the group call request (success or failure) |
| NOTE: This information element shall be included only when the Result is success. |  |  |

##### 7.1.2.2.4	Group call response (MCVideo client – MCVideo server)

Table 7.1.2.2.4-1 describes the information flow group call response from the MCVideo client to the MCVideo server.

Table 7.1.2.2.4-1: Group call response (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the target MCVideo group member |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is initiated |
| SDP answer (NOTE) | O | Media parameters selected |
| Result | M | Result of the group call request (success or failure) |
| NOTE: This information element shall be included only when the Result is success. |  |  |

# * * * * Next change * * * *

#### 7.19.2.5	Ad hoc group call response (MCVideo server – MCVideo client)

Table 7.19.2.5-1 describes the information flow ad hoc group call response from the MCVideo server to the MCVideo client.

Table 7.19.2.5-1: Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the group call request (success or failure) |
| Call resulting criteria for determining the participants (NOTE) | O | Call resulting criteria used by the MCPTT server to determine the list of participants to be invited |
| NOTE: This information element shall be included only when the Result is success. |  |  |

# * * * * Next change * * * *

#### 7.19.2.21	Modify ad hoc group call criteria response (MCVideo server – MCVideo client)

Table 7.19.2.21-1 describes the information flow Modify ad hoc group call criteria response from the MCVideo server to the MCVideo client.

Table 7.19.2.21-1: Modify Ad hoc group call criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the requesting MCVideo user who is authorized to modify the ad hoc group criteria |
| Functional alias | O | The functional alias of the requesting MCVideo user who is authorized to modify the ad hoc group criteria |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call participants request (success or failure) |
| Call resulting criteria for determining the participants (NOTE) | O | Call resulting criteria used by the MCPTT server to determine the list of participants to be invited. |
| NOTE: This information element shall be included only when the Result is success. |  |  |