3GPP TSG-SA WG6 Meeting #68	S6-253463

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253342)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0484 | rev | 1 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IE tables with Result return |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In a reponse message with result, some IEs are meaningless when the return result is failure. These meaningless IEs returned in a response message not only wasting the valueable network resources and alos might misguide the stage 3 development. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make corrections to the relevant information flows to clarify what some IEs shall be returned only in a success response message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect guidance to stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.2.10, 10.6.2.2.11, 10.6.2.2.12, 10.6.2.2.28, 10.6.2.2.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.6.2.2.10	Group call response (MCPTT server – MCPTT client)

Table 10.6.2.2.10-1 describes the information flow group call response from the MCPTT server to the MCPTT client.

Table 10.6.2.2.10-1: Group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is requested |
| SDP answer (NOTE) | O | Media parameters selected |
| Result | M | Result of the group call request (success or failure) |
| NOTE: This information element shall be included only when the Result is success. |  |  |

##### 10.6.2.2.11	Group call response (MCPTT server – MCPTT server)

Table 10.6.2.2.11-1 describes the information flow group call response between the MCPTT servers.

Table 10.6.2.2.11-1: Group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MCPTT group member |
| Functional alias | O | The functional alias of the target MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is requested |
| SDP answer (NOTE) | O | Media parameters selected |
| Result | M | Result of the group call request (success or failure) |
| NOTE: This information element shall be included only when the Result is success. |  |  |

##### 10.6.2.2.12	Group call response (MCPTT client – MCPTT server)

Table 10.6.2.2.12-1 describes the information flow group call response from the MCPTT client to the MCPTT server.

Table 10.6.2.2.12-1: Group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MCPTT group member |
| Functional alias | O | The functional alias of the target MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is initiated |
| SDP answer (NOTE) | O | Media parameters selected |
| Result | M | Result of the group call request (success or failure) |
| NOTE: This information element shall be included only when the Result is success. |  |  |

# * * * * Next change * * * *

##### 10.6.2.2.28	Group-broadcast group call response (MCPTT client – MCPTT server)

Table 10.6.2.2.28-1 describes the information flow group-broadcast group call response from the MCPTT client to the MCPTT server.

Table 10.6.2.2.28-1: Group-broadcast group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MCPTT group member |
| Functional alias | O | The functional alias of the target MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is initiated |
| SDP answer (NOTE) | O | Media parameters selected |
| Result | M | Result of the group-broadcast group call request (success or failure) |
| NOTE: This information element shall be included only when the Result is success. |  |  |

##### 10.6.2.2.29	Group-broadcast group call response (MCPTT server – MCPTT client)

Table 10.6.2.2.29-1 describes the information flow group-broadcast group call response from the MCPTT server to the MCPTT client.

Table 10.6.2.2.29-1: Group-broadcast group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is requested |
| SDP answer (NOTE) | O | Media parameters selected |
| Result | M | Result of the group-broadcast group call request (success or failure) |
| NOTE: This information element shall be included only when the Result is success. |  |  |