3GPP TSG-SA5 Meeting #162	S5-253718

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0364 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Clause 5.3.8 Utility Function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Figure 5.3.8.2-3 was previously not described accurately in the text and contained an error in the graph. The figure has been corrected to properly illustrate the example in which two different intents consider the same metric A but apply different utility functions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the updated figure, the circles in the left-hand graph have been swapped to align with the described scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | These changes address potential ambiguities that could have led to misinterpretation and incorrect implementation of the utility functions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.3.8	Intent Utility Function

#### 5.3.8.1	Introduction

In some cases, it may not be obvious to the MnS producer how to select from multiple candidate solutions for fulfilling an intent.  For example, an MnS consumer expressing multiple intents may want to provide further preference information to the MnS producer to improve overall fulfilment, rather than rely on simple satisfied/unsatisfied indication.

Extra information provided in an intent can indicate preferences between expectation targets and assist the MnS producer in balancing between acceptable solutions.

This is especially relevant when no solution fully satisfies all intents or meets the expectations of a particular intent. In such situations, balancing trade-offs among different intents becomes necessary.

For example, a network performance function could be used to quantify network performance relative to latency, providing a means for the MnS Consumer "to quantify an intent's relative value as higher when the latency is lower":

EXAMPLE 1:	U1 = f(throughput, latency, packet loss);  a single utility function defined for an intent

EXAMPLE 2:	U2 = f(latency) and U3=f(throughput); multiple utility functions defined for an intent, e.g. for different expectations

An MnS consumer could express this in the form of an Intent Utility Function along with their expected target value.  This assists the MnS producer(s) in fulfilling an MnS consumer's intents, as it provides an indicator to the MnS Producer of the satisfaction level of the intent fulfilment. The intent utility function can be for multiple intents or as a part of an intent.

Utility functions can be used for intent negotiation.

#### 5.3.8.2	Definition

Intent Utility Function defines a method by which consumers can express the relative value of an intent's expectations to assist the IDMS producer(s) in fulfilling their intents in the most acceptable manner.

Intent utility functions are mathematical expressions that quantify the satisfaction or utility derived from the various levels of fulfilment. Their basic components include:

-	variables: of the function to quantify specific aspects of the fulfilment, e.g., expectation target for network performance.

-	weights: to define the relative importance of each variable, e.g., for network performance, a variable representing low latency might be assigned a higher weight than throughput.

-	function: the mathematical function to be applied to the variables, e.g., linear, logarithmic, polynomials, scalars.

-	result: the output of the function  which represents the utility level achieved, i.e., the satisfaction of the current fulfilment based on the acceptability of potential solutions defined by the consumer.

MnS producer(s) can use Intent Utility Function information to assess the acceptability of potential solutions, in addition to information such as resource availability and performance targets.

Utility functions may be defined by the consumer and provided as part of the intent itself, i.e., the function is defined as part of the intent. A MnS consumer may also specify that an existing (i.e., predefined) utility function is used for one or multiple intent(s). Predefined utility functions may be vendor specified (e.g., system pre-installed) and/or specified by the MnS consumer.

A single intent may contain multiple intent expectations, each having its own utility function. In such cases, aggregation of these utility functions can be used to represent the overall utility of the intent. Normalization may also be needed when dealing with utility functions expressed in different scales (e.g., latency in milliseconds, throughput in Mbps, energy consumption in Joules).

Figure 5.3.8.2-1 represents a case where an MnS Consumer expresses an intent (A) with expectation which should be less than a, while expectation of another intent (B) should be greater than b.  There may be no solution to fulfil both expectations. However, different solutions may be available for each intent which leads to more or less preferred outcomes, as indicated by the black and red circles. While both solutions would fulfil intent A, neither would fulfil intent B, and there is limited ability for the MnS producer to decide which would be better from MnS consumer perspective.

Figure 5.3.8.2-1: Intents with binary fulfilment result

Figure 5.3.8.2-2 represents a case where an MnS Consumer defines Utility Functions A and B. Each provides the MnS producer with a means to calculate a utility value based on the  metrics A and B. These utility results can be used by the MnS producer to select from different solutions to improve the overall utility across multiple intents.

For example, the solution with the red circle yields a better outcome since it can maximize Utility Function A and has higher value for Utility Function B than the black solution. From the MnS consumer´s perspective, such a case is considered "quantitative fulfilment", as the MnS producer has a means by which to apply a quantitative value to each solution and use such information to better select on behalf of the consumer.

Figure 5.3.8.2-2: Example 1 of intent with differentiated solution preference through utility function

Figure 5.3.8.2-3 represents a case where two different intents consider the same metric A but use different utility functions (Utility Function A and Utility Function B). In this scenario, no solutions can fully maximize either of the utility results, and therefore, none of the intents can be completely satisfied. The MnS producer may choose to prioritize the red circle solution as its aggregate utility value (utility function A and utility function B) is higher than the one for the black circle solution. This would contribute to a better overall utility level across multiple intents.

Figure 5.3.8.2-3: Example 2 of intents with differentiated solution preference through utility function

End of  changes