# 3GPP TSG-SA Meeting #109	SP-251120

Beijing, China, September 16 – 19, 2025

# Source:	Nokia

# Title:	pCR TR 22.850: Updates related to AI/ML for SA WGs

# Document for:	Approval

# Agenda Item:	8

# 1	Introduction

5.2.2.8	Rel-18 SA WG3 WID - Security aspects of enablers for Network Automation for 5G - phase 3 (eNA_SEC_PH3)

A short summary is proposed for the SA3 WID.

5.2.2.12.2	Activities summary

The headline is not needed.

5.2.2.18.2	Activities summary

First editor’s note is not needed any longer since SA#108 introduce another editor’s note to indicate that:

Editor’s note:	Reference to TR 21.919 can be added when the work item summary is made available.

Tables 6.2.1.1-1, 6.2.1.2-1 and 6.2.1.5-1

Fixing a typo to only refer to TS 23.482.

Table 6.2.2-1

It is proposed to refer to TS 23.288 instead of TR 23.700-84.

6.3.1	Analysis on ML model training services

Editor’s note below can be removed, as no new services were introduced in SA2 nor SA5.

Editor's note:	This analysis is based on Release 18 and does not consider Release 19 for SA WG2 and SA WG5. Further analysis needs to be conducted as Release 19 matures and normative work progresses for these working groups.

6.3.2	Analysis on analytics related services

It is proposed to remove the following editor’s note as the analysis would go beyond the scope for the study:

Editor's note:	Further investigation is required to determine if similar analytics (e.g. radio resource related analytics) are defined across SA WG2, SA WG5 and SA WG6.

Editor’s note below can be removed as no new services were introduced.

Editor's note:	This analysis is based on Release 18 and does not consider Release 19 for SA WG5. Further analysis needs to be conducted as Release 19 matures and normative work progresses for these working groups.

6.3.3	Analysis on ML performance evaluation and monitoring

There is no dedicated AIML performance monitoring service defined in SA5, so the related editor’s note can be removed.

Editor's note:	SA WG5 services need to be added to the table.

6.3.4	Analysis on data collection and management for AI/ML

We propose to delete the following editor’s note with no additional change, the list of SA6 APIs in the table is complete:

Editors' note:	Some of the SA WG6 defined APIs in table above are defined in Rel-19 but are not complete yet.

Editor’s note below can be deleted since SA5 has a generic framework for data collection and nothing specific to AI/ML.

Editors' note:	This analysis is based on Release 18 and does not consider Release 19. Further analysis needs to be conducted as Release 19 matures and normative work progresses in SA5.

# 2	Detailed proposal

The following changes are proposed to be included into TR 22.850.

***** START OF CHANGES *****

## 5.2	AI/ML related activities in TSG SA & CT Working Groups

#### 5.2.2.8	Rel-18 SA WG3 WID - Security aspects of enablers for Network Automation for 5G - phase 3 (eNA_SEC_PH3)

##### 5.2.2.8.1	Description

The main objective of this work is to produce normative specification based on the conclusions from Rel-18 study. More specifically, the following objectives are expected to be specified:

-	Protection of data and analytics exchange in roaming case.

-	Security for AI/ML model storage and sharing.

-	Authorization of selection of participant NWDAF instances in the Federated Learning group.

##### 5.2.2.8.2	Activities summary

As part of this work, enhancements were specified for

- The protection of data and analytics exchange in roaming case including authorization and anonymization of data/analytics. Authorization at data and analytics level is enforced by the roaming entry NWDAF producer. The roaming entry NWDAF producer is responsible to control the amount of exposed data/analytics and to abstract or hide internal network aspects in the exposed data/analytics.

-	The authorization for selecting participant NWDAF instances in the Federated Learning (FL) group, using token-based authorization.

-	The procedure for secured and authorized AI/ML model sharing between different vendors and AI/ML model storing.

***** NEXT CHANGE *****

#### 5.2.2.12	Rel-19 SA WG5 SID - AI/ML management - phase 2 (FS_AIML_MGT_Ph2)

##### 5.2.2.12.1	Description

The objectives of this study item include:

-	Continuation of AI/ML Studies: Continue the study on AI/ML emulation, AI/ML inference coordination and ML knowledge transfer that are left over from Rel-18.

-	Management Aspects of AI/ML functionalities defined by other 3GPP WGs:

-	AI/ML Model Transfer/delivery in RAN: Study the management aspects (LCM, CM and PM) of AI/ML model transfer in RAN.

-	5GS Support for AI/ML-Based Services: Investigate the management aspects of 5GS support for AI/ML-based services, as defined in SA WG2.

-	Management Aspects of AI/ML Functionalities Defined by SA WG5:

-	Management Data Analytics (MDA): Study the management aspects (LCM, CM and PM) of AI/ML functionalities defined by SA WG5, including MDA Phase 3.

-	AI/ML Management and Operation Capabilities: Investigate the AI/ML management and operation capabilities to support different types of AI/ML technologies needed for AI/ML in 5GS, such as Federated Learning, Reinforcement Learning, Online and Offline Training, Distributed Learning and Generative AI.

-	Sustainability Aspects of AI/ML:

-	Energy consumption/efficiency impacts: Evaluate the energy consumption and efficiency impacts associated with AI/ML solutions for all operational phases (training, emulation, deployment, inference).

-	Trustworthiness Aspects Related to AI/ML Functionalities in 5GS:

-	Concept of Trustworthiness: Further study the concept of trustworthiness for AI/ML in the context of OAM.

-	Data for Trustworthiness Indicators: Identify and analyse data (e.g. measurements, events) to support the calculation of trustworthiness indicators.

The outcome of this study was used to support the Rel-19 SA WG5 WID on AI/ML management phase 2 (AIML_MGT_Ph2).

***** NEXT CHANGE *****

#### 5.2.2.18	Rel-19 SA WG6 WID - Application Data Analytics Enablement Service  (TEI19_ADAES)

##### 5.2.2.18.1	Description

The objectives of this work item include the following:

-	Clarify metrics related to the analytics inputs/outputs introduced in TS 23.436 [33].

-	Enhance the IEs for the request/response of data collection from Data Producer and/or A-ADRF (or via A-DCCF) in TS 23.436 [33].

-	Complete the definition of IEs for the data/analytics storage subscription request to A-ADRF in TS 23.436 [33].

##### 5.2.2.18.2	Activities summary

Editor’s note:	Reference to TR 21.919 can be added when the work item summary is made available.

***** NEXT CHANGE *****

# 6	Analysis on AI/ML across 3GPP

#### 6.2.1.1	Analysis on ML model

The term 'ML model' has been defined differently by SA WG5, SA WG6 and RAN WG1, as illustrated in Table 6.2.1.1-1.

Table 6.2.1.1-1: Definition of ML model as defined across 3GPP WGs

| TSG (TS/TR) | ML model |
| --- | --- |
| SA WG5 TS 28.105 [9] | A manageable representation of an ML model algorithm. NOTE 1: An ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output. NOTE 2: An ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification. NOTE 3: An ML model may include metadata. Metadata may include e.g. information related to the trained model and applicable runtime context. |
| SA WG6 TS 23.482 [34] | According to TS 28.105 [9], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information. |
| RAN WG1 TR 38.843 [3] | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |

The following unified definition for 'ML model' is proposed:

ML model:  A mathematical algorithm that applies ML techniques to generate a set of outputs based on a set of inputs. It may include metadata which consists of, e.g. information related to the model and applicable runtime context.

NOTE:	An ML model can be managed, stored and transferred as artifacts, which may be containers, images, or proprietary file formats.

***** NEXT CHANGE *****

#### 6.2.1.2	Analysis on ML model training

The term 'ML model training' has been defined differently by SA WG5, SA WG6 and RAN WG1, as illustrated in Table 6.2.1.2-1. RAN WG3 follows the definition of SA WG5.

Table 6.2.1.2-1: Definition of ML model training as defined across 3GPP WGs

| TSG (TS/TR) | ML model training |
| --- | --- |
| SA WG5 TS 28.105 [9] | A process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model. |
| SA WG6 TS 23.482 [34] | According to TS 28.105 [9], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss. |
| RAN WG1 TR 38.843 [3] | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference. |
| RAN WG3 TS 38.300 [11] | AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [9]. |
| RAN WG3 TS 38.401 | AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [9]. |

The following unified definition for 'ML model training' is proposed:

ML model training: A process to train an ML Model by learning the input/output relationship in a data driven manner and obtain the trained ML Model for e.g. inference.

***** NEXT CHANGE *****

#### 6.2.1.5	Analysis on ML model inference

The term 'ML model inference' has been defined differently by SA WG5, SA WG6 and RAN WG1, as illustrated in Table 6.2.1.5-1. RAN WG3 follows the definition of SA WG5.

Table 6.2.1.5-1: Definition of ML model inference as defined across 3GPP WGs

| TSG (TS/TR) | ML model inference |
| --- | --- |
| SA WG5 TS 28.105 [9] | A process of running a set of input data through a trained ML model to produce set of output data, such as predictions. |
| SA WG6 TS 23.482 [34] | According to TS 28.105 [9], ML model training includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference. |
| RAN WG1 TR 38.843 [3] | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs. |
| RAN WG3 TS 38.300 [11] | AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [9]. |
| RAN WG3 TS 38.401 | AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [9]. |

The following unified definition for 'ML model inference' is proposed:

ML model inference: A process of running a set of inputs through a trained ML model to produce a set of outputs.

***** NEXT CHANGE *****

### 6.2.2	Analysis on Federated Learning

The term 'Horizontal Federated Learning' and 'Vertical Federated Learning' have been defined in SA WG2 and RAN WG1 as well as SA WG5 defines 'Federated Learning', as illustrated in Table 6.2.2-1.

Table 6.2.2-1: Definition of Federated Learning as defined across 3GPP WGs

| TSG (TS/TR) | Federated Learning |
| --- | --- |
| SA WG2 TR 23.700-84 [4] | Horizontal Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs). |
| SA WG2 TS 23.288 [8] | Vertical Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). |
| RAN WG1 TR 38.843 [3] | Federated Learning: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g. UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| 3GPP SA5 TS 28.105 [9] | Federated Learning: a distributed machine learning approach where the ML model is trained collaboratively by multiple ML training functions. This includes multiple FL clients, which perform training on local data, and one FL server, which aggregates model outcomes from the clients iteratively without exchanging data samples. |

The definition of Federated Learning provided by RAN WG1 appears to only apply to Horizontal Federated Learning, as the phrase "each performing local model training using local data samples" implies that the data samples at individual nodes are distinct. The key difference between Horizontal Federated Learning and Vertical Federated Learning lies in the characteristics of the local datasets:

-	Horizontal Federated Learning: Local datasets have the same features but different samples.

-	Vertical Federated Learning: local data sets have different features but share same samples.

The definition of Federated Learning provided by SA WG5 highlights the collaborative training process among multiple FL participants, including an FL server and FL clients, without specifying the characteristics of the client datasets. This broader definition facilitates a more comprehensive understanding of both Horizontal Federated Learning (HFL) and Vertical Federated Learning (VFL) that are already defined in the specifications and also offers greater flexibility across more general scenarios, see definitions in clause 5.2.1.2.

The terms "distributed learning" and "federated learning" are often used together as "distributed/federated learning" in SA WG1 TS 22.261 [6]. "Distributed learning" typically refers to a broader set of learning techniques including "federated learning". Although the two terms are related, they are not identical and should be used appropriately based on the context.

The following unified definition for 'Federated Learning' is proposed:

Federated Learning: A distributed machine learning approach where the ML model(s) are collaboratively trained by multiple participants, including one acting as an FL server and multiple acting as FL clients, iteratively without exchanging data samples.

The following unified definition for 'Horizontal Federated Learning' is proposed:

Horizontal Federated Learning:  A federated learning technique without exchanging/sharing local data set, wherein the local data set in different HFL clients for local model training have the same feature space for different samples.

The following unified definition for 'Vertical Federated Learning' is proposed:

Vertical Federated Learning:  A federated learning technique without exchanging/sharing local data set and local ML models, wherein the local data set in different VFL clients for local model training have different feature spaces for the same samples.

***** NEXT CHANGE *****

## 6.3	AI/ML related features

### 6.3.1	Analysis on ML model training services

The analysis focuses on the specifications from SA WG2, SA WG5 and SA WG6, considering these are the working groups defining services and operations related to ML model training in 3GPP Release 18. SA WG1, SA WG3, SA WG4, RAN WG1, RAN WG2 and RAN WG3 have not defined any services or operations related to ML model training.

Table 6.3.1-1 provides a detailed overview of the specific services defined by each working group.

The key findings from the analysis are as follows:

-	SA WG2: Emphasizes a structured approach to ML model training services by defining a clear consumer-producer relationship. This enables specific entities to consume and produce these services, ensuring a well-defined and controlled environment for service utilization.

-	SA WG5: Offers a more flexible approach by defining generic ML model training services. This allows for greater adaptability in implementation and usage, without the constraints of a specific consumer-producer relationship.

-	SA WG6: Mirrors the approach of SA WG2, prioritizing a clear consumer-producer relationship for its defined services. This aligns with the structured approach advocated by SA WG2. Additional ML training features for applications are supported in the application enablement layer, such as: federated learning (horizontal, vertical), ML model training capability evaluation for selection of FL members, AI/ML task transfer to a suitable AIML enablement member, support for transfer learning enablement, support for FL member grouping, etc.

While SA WG2 and SA WG6 restrict the potential producers and consumers, SA WG5 emphasizes flexibility and adaptability. The choice of approach will depend on the specific needs and requirements of the individual service provider and consumer.

Table 6.3.1-1: ML model training related services and operations as specified across 3GPP WGs

| ML Model Training |  |  |  |
| --- | --- | --- | --- |
| TSG (TS/TR) | Service/API Type | Service/API/IOC Name | Description [Consumer, Producer] |
|  |  | Nnwdaf_MLModelProvision_Subscribe | The consumer subscribes to NWDAF ML model provision with specific parameters to receive a notification when an ML Model matching the subscription parameters becomes available. Consumer: NWDAF AnLF, LMF Producer: NWDAF MTLF |
|  | ML Model Provisioning Services | Nnwdaf_MLModelProvision_Unsubscribe | The consumer unsubscribes to NWDAF ML model provision. Consumer: NWDAF AnLF, LMF Producer: NWDAF MTLF |
|  |  | Nnwdaf_MLModelProvision_Notify | The NWDAF notifies the ML model information to the consumer which has subscribed to the NWDAF ML model provision service. Consumer: NWDAF AnLF, LMF Producer: NWDAF MTLF |
| SA WG2 TS 23.288 [8] | ML Model Information Services | Nnwdaf_MLModelInfo_Request | The consumer requests and gets NWDAF ML Model Information. Consumer: NWDAF AnLF, LMF Producer: NWDAF MTLF |
|  |  | Nnwdaf_MLModelTraining_Subscribe | The consumer subscribes to NWDAF ML model training with specific parameters. Consumer: NWDAF MTLF Producer: NWDAF MTLF |
|  | ML Model Training Services | Nnwdaf_MLModelTraining_Unsubscribe | The consumer terminates NWDAF ML model training. Consumer: NWDAF MTLF Producer: NWDAF MTLF |
|  |  | Nnwdaf_MLModelTraining_Notify | The NWDAF notifies about the trained ML model to the consumer which has subscribed to the NWDAF ML model training service. Consumer: NWDAF MTLF Producer: NWDAF MTLF |
|  | ML Model Training Information Services | Nnwdaf_MLModelTrainingInfo_Request | The consumer requests for the information about NWDAF ML model training with specific parameters. Consumer: NWDAF MTLF Producer: NWDAF MTLF |
|  | VFL Training Services | Nnwdaf_VFLTraining_Subscribe | The consumer subscribes to VFL ML Model training information. Consumer: NWDAF, AF, NEF Producer: NWDAF |
|  |  | Nnwdaf_VFLTraining_Unsubscribe | The consumer terminates NWDAF VFL ML Model training. Consumer: NWDAF, AF, NEF Producer: NWDAF |
|  |  | Nnwdaf_VFLTraining_Notify | The NWDAF notifies the consumer of client intermediate training result of the local ML model. Consumer: NWDAF, AF, NEF Producer: NWDAF |
|  |  | Nnwdaf_VFLTraining_Request | The consumer requests NWDAF VFL client to check if it can support requirements for VFL. Consumer: NWDAF, AF, NEF Producer: NWDAF |
|  |  | MLTrainingRequest | It represents the ML model training request to train an ML model which is triggered by the ML training MnS consumer towards the ML training MnS producer. Consumer: Any authorized network function, any authorized management function, operator Producer: Any function that is capable of training an ML model |
| SA WG5 TS 28.105 [9] | ML Training Management Services | MLTrainingReport | It represents the ML model training report provided by the ML training MnS producer to the ML training MnS consumer who has requested for ML model training. Consumer: Any authorized network function, any authorized management function, operator Producer: Any function that is capable of training an ML model |
|  |  | MLTrainingProcess | It represents the ML model training process. When a ML model training process starts, an instance of the MLTrainingProcess is created by the MnS Producer and notification is sent to MnS consumer who has subscribed to it. Consumer: Any authorized network function, any authorized management function, operator Producer: Any function that is capable of training an ML model |
| SA WG6 TS 23.482 [34] | ML Model Training APIs | Aimles_MLModelTraining Request | The consumer sends an ML model training request to the producer, requesting to assist in its ML model training. This request consists of ML model information or ML model requirement information, etc. Consumer: VAL server Producer: AIMLE Server |
|  |  | Aimles_MLModelTraining Response | If the consumer is authorized, the producer identifies and selects the appropriate ML model for training based on the ML model requirement information. The producer returns a success response indicating the selected ML model for training; otherwise, a failure response indicating the reason for failure. Consumer: VAL server Producer: AIMLE Server |

***** NEXT CHANGE *****

### 6.3.2	Analysis on analytics related services

This clause focuses on the specifications from SA WG2, SA WG5 and SA WG6, considering these are the working groups defining services and operations related to ML model inference in 3GPP Release 18. SA WG1, SA WG3, SA WG4, RAN WG1, RAN WG2 and RAN WG3 have not defined any services or operations related to ML model inference.

Table 6.3.2-1 provides a detailed overview of the specific services defined by each working group.

The key findings from the analysis are as follows:

-	SA WG2: Describes ML model inference by defining analytics services through a clear consumer-producer relationship. It defines several analytics types in TS 23.288 [8], each one supported by the NWDAF/RE-NWDAF AnLF and requested/subscribed by the NWDAF/RE-NWDAF AnLF consumer using the defined analytics services.

-	SA WG5: Describes ML model inference by defining generic analytics services without specific consumer-producer relationship in TS 28.105 [9] and TS 28.104 [71]. It defines several analytics types in TS 28.104 [71], each one supported by an MnS producer and requested by the MnS consumer using the defined analytics services.

-	SA WG6: Describes ML model inference by defining individual analytics services for each analytics type. It defines several analytics types in TS 23.436 [33].

While SA WG2 and SA WG6 restrict the potential producers and consumers, SA WG5 emphasizes flexibility and adaptability. Moreover, SA WG2 and SA WG5 defines several analytics types that can be supported by an entity and requested by another entity using the defined ML model inference services. In addition, in SA WG6, individual services are defined for each analytics type.

Table 6.3.2-1: Analytics and inference related services and operations as specified across 3GPP WGs

| ML Model inference |  |  |  |
| --- | --- | --- | --- |
| TSG (TS/TR) | Service/API Type | Service/API/IOC Name | Description [Consumer, Producer] |
|  | Network Data Analytics Subscription Services | Nnwdaf_AnalyticsSubscription_Subscribe | The consumer subscribes for network data analytics and optionally its corresponding analytics accuracy information with specific parameters. Consumer: PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF Producer: NWDAF AnLF |
|  |  | Nnwdaf_AnalyticsSubscription_Unsubscribe | The consumer unsubscribes for network data analytics. Consumer: PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF Producer: NWDAF AnLF |
|  |  | Nnwdaf_AnalyticsSubscription_Notify | The NWDAF notifies the analytics and optionally Analytics Accuracy Information to the consumer which has subscribed to the NWDAF analytics subscription service. Consumer: PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF Producer: NWDAF AnLF |
|  |  | Nnwdaf_AnalyticsSubscription_Transfer | The consumer NWDAF requests NWDAF for transferring analytics subscriptions from the consumer NWDAF. Consumer: NWDAF AnLF Producer: NWDAF AnLF |
| SA WG2 TS 23.288 [8] | Network Data Analytics Information Services | Nnwdaf_AnalyticsInfo_Request | The consumer requests NWDAF operator specific analytics and optionally Analytics Accuracy Information with specific parameters. Consumer: PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF Producer: NWDAF AnLF |
|  |  | Nnwdaf_AnalyticsInfo_ContextTransfer | The consumer NWDAF requests NWDAF to transfer context information related to analytics subscriptions. Consumer: NWDAF AnLF Producer: NWDAF AnLF |
|  |  | Nnwdaf_RoamingAnalytics_Subscribe | The consumer subscribes for network data analytics related to roaming UEs. Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  | Network Data Roaming Analytics Services | Nnwdaf_RoamingAnalytics_Unsubscribe | The consumer unsubscribes for network data analytics related to roaming UEs. Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  |  | Nnwdaf_RoamingAnalytics_Notify | The NWDAF notifies the analytics related to roaming UE(s) to the consumer which has subscribed to the NWDAF roaming analytics subscription service. Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  |  | Nnwdaf_RoamingAnalytics_Request | The consumer requests NWDAF operator specific related to roaming UEs. Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  | Nnwdaf_VFLInference | Nnwdaf_VFLInference_Subscribe | The consumer subscribes to VFL inference. Consumer: NWDAF, AF, NEF Producer: NWDAF |
|  |  | Nnwdaf_VFLInference_Unsubscribe | The consumer unsubscribes to VFL inference. Consumer: NWDAF, AF, NEF Producer: NWDAF |
|  |  | Nnwdaf_VFLInference_Notify | The consumer notifies VFL inference result. Consumer: NWDAF, AF, NEF Producer: NWDAF |
|  |  | Nnwdaf_VFLInference_Request | The consumer requests the NWDAF to perform a one-time VFL inference. Consumer: NWDAF, AF, NEF Producer: NWDAF |
| SA WG5 TS 28.104 [71] | Management Data Analytics Services | MDARequest | It represents the management data analytics output request which is created by an MDA MnS consumer towards the MDA MnS producer. Consumer: Any authorized network function, any authorized management function, operator Producer: Any function that is capable of producing management data analytics |
|  |  | MDAReport | It represents the management data analytics report containing the outputs for one or more MDA types delivered to the MDA consumer who has requested for management data analytics. Consumer: Any authorized network function, any authorized management function, operator Producer: Any function that is capable of producing management data analytics |
|  | SS_ADAE_VAL_performance_analytics | VAL_performance_analytics_subscribe | The consumer subscribes for VAL performance analytics. Consumer: VAL server Producer: ADAE server |
|  |  | VAL_performance_analytics_notify | The consumer is notified by ADAES on the VAL performance analytics. Consumer: VAL server Producer: ADAE server |
|  | SS_ADAE_slice_performance_analytics | slice_performance_analytics_subscribe | The consumer subscribes for slice specific performance analytics. Consumer: VAL server Producer: ADAE server |
|  |  | slice_performance_analytics_notify | The consumer is notified by ADAES on the slice specific performance analytics. Consumer: VAL server Producer: ADAE server |
|  | SS_ADAE_UE-to-UE_performance_analytics | UE-to-UE performance_analytics_subscribe | The consumer subscribes for UE-to-UE performance analytics. |
|  |  | UE-to-UE performance_analytics_notify | The consumer is notified by ADAES on the slice specific performance analytics. Consumer: VAL server Producer: ADAE server |
| SA WG6 TS 23.436 [33] | SS_ADAE_server-to-server_performance_analytics | server-to-server_performance_analytics_subscribe | The consumer subscribes to the ADAE server for Server-to-server performance analytics. Consumer: VAL server, EES Producer: ADAE server |
|  |  | server-to-server_performance_analytics_notify | The consumer is notified by the ADAE server on the Server-to-server performance analytics. Consumer: VAL server, EES Producer: ADAE server |
|  | SS_ADAE_location_accuracy_analytics | Location_accuracy_analytics_subscribe | The consumer subscribes for location accuracy analytics. Consumer: VAL server Producer: ADAE server |
|  |  | Location_accuracy_analytics_notify | The consumer is notified by ADAES on the location accuracy analytics. Consumer: VAL server Producer: ADAE server |
|  | SS_ADAE_service_API_analytics | Service_API_analytics_subscribe | The consumer subscribes for service API analytics. Consumer: VAL server, Subscriber, API invoker Producer: ADAE server |
|  |  | Service_API_analytics_notify | The consumer is notified by ADAES on the location accuracy analytics. Consumer: VAL server, Subscriber, API invoker Producer: ADAE server |
|  | SS_ADAE_slice_usage_pattern_analytics | slice_usage_pattern_analytics_subscribe | The consumer subscribes for slice usage pattern analytics. Consumer: VAL server, SEAL server Producer: ADAE server |
|  |  | slice_usage_pattern_analytics_notify | The consumer is notified by ADAES on the slice usage pattern analytics. Consumer: VAL server, SEAL server Producer: ADAE server |
|  | SS_ADAE_edge_analytics | edge_analytics_subscribe | The consumer subscribes for edge load analytics. Consumer: VAL server, ECS, EES Producer: ADAE server |
|  |  | edge_analytics_notify | The consumer is notified by ADAES on the edge load analytics. Consumer: VAL server, ECS, EES Producer: ADAE server |
|  |  | edge_analytics_get | The consumer requests edge analytics data. Consumer: VAL server, ECS, EES Producer: ADAE server |
|  | SS_ADAES_slice_usage_stats | slice_usage_stats_get | The consumer requests and receives slice usage statistics from ADAE server. Consumer: VAL server Producer: ADAE server |
|  | SS_ADAES_edge_preparation_analytics | edge_preparation_analytics_subscribe | The consumer subscribes for edge computing preparation analytics. Consumer: VAL server, ECS, EES Producer: ADAE server |
|  |  | edge_preparation_analytics_notify | The consumer is notified by the ADAE server on the edge computing preparation analytics. Consumer: VAL server, ECS, EES Producer: ADAE server |
|  |  | edge_preparation_analytics_get | The consumer requests edge computing preparation analytics Consumer: VAL server, ECS, EES Producer: ADAE server |
|  | SS_ADAE_collision_detection_analytics | collision_detection_analytics_subscribe | The consumer subscribes for collision detection analytics. Consumer: VAL server, LM server, UAE server, UAS application specific server Producer: ADAE server |
|  |  | collision_detection_analytics_notify | The consumer is notified by the ADAE server on collision detection analytics. Consumer: VAL server, LM server, UAE server, UAS application specific server Producer: ADAE server |
|  |  | collision_detection_analytics_get | The consumer requests collision detection analytics. Consumer: VAL server, LM server, UAE server, UAS application specific server Producer: ADAE server |
|  | SS_ADAE_location-related_UE_group_analytics | location-related_UE_group_analytics_subscribe | The consumer subscribes for location-related UE group analytics. Consumer: LM server Producer: ADAE server |
|  |  | location-related_UE_group_analytics_notify | The consumer is notified by the ADAE server on location-related UE group analytics. Consumer: LM server Producer: ADAE server |
|  |  | location-related_UE_group_analytics_get | The consumer requests location-related UE group analytics. Consumer: LM server Producer: ADAE server |
|  | SS_ ADAE_AIML_member_capability_analytics | AIML_member_capability_analytics_subscribe | The consumer subscribes for application layer AIML member capability analytics. Consumer: VAL server, AIMLE server Producer: ADAE server |
|  |  | AIML_member_capability_analytics_notify | The consumer is notified by the ADAE server on application layer AIML member capability analytics. Consumer: VAL server, AIMLE server Producer: ADAE server |
|  |  | AIML_member_capability_analytics_get | The consumer requests application layer AIML member capability analytics. Consumer: VAL server, AIMLE server Producer: ADAE server |
|  | SS_ADAE_UE_RAT_connectivity_analytics API | UE_RAT_connectivity_analytics_subscribe | The consumer requests UE RAT connectivity analytics. Consumer: VAL server Producer: ADAE server |
|  |  | UE_RAT_connectivity_analytics_notify | The consumer is notified by the ADAE server on UE RAT connectivity analytics. Consumer: VAL server Producer: ADAE server |

***** NEXT CHANGE *****

### 6.3.3	Analysis on ML performance evaluation and monitoring

The analysis focuses on the specifications from SA WG2, SA WG5 and SA WG6, considering these are the working groups defining services and operations related to ML performance evaluation in 3GPP Release 18 / Release 19. SA WG1, SA WG3, SA WG4, RAN WG1, RAN WG2 and RAN WG3 have not defined any services or operations related to ML performance evaluation.

Table 6.3.3-1 provides a detailed overview of the specific services defined by each working group.

The key findings from the analysis are as follows:

-	SA WG2: Dedicated ML model monitoring services are defined with a specific consumer-producer relationship. Additionally, analytics subscription and ML model training subscription services may indicate the performance requirements which the producer has to satisfy when providing the analytics or training the ML model. The focus in SA WG2 has been on the accuracy aspects of ML model performance.

-	SA WG5: SA WG5 defines the framework and mechanisms for performance assurance including performance metrics (ModelPerformance clause 7.4.1 of 28.105 [9]) on which the performance of an ML Model can be ascertained. ML training, ML testing and ML inference services indicate the performance requirements which the producer has to satisfy for the consumer when training the ML model or testing the ML model or providing the inferences. The achieved performance of ML Model is communicated to the consumer via MLTrainingReportMLTestingReport and AIMLInferenceReport.

-	SA WG6: Dedicated ML model monitoring services are defined with specific consumer-producer relationship. Additionally, ML model training APIs indicate the performance requirements that the producer has to satisfy when providing the ML model. No specific ML model performance metrics are standardized in SA WG6.

Table 6.3.3-1: ML model performance monitoring services and operations as specified in 3GPP WGs

| ML Model Training |  |  |  |
| --- | --- | --- | --- |
| TSG (TS/TR) | Service/API Type | Service/API/IOC Name | Description [Consumer, Producer] |
|  |  | Nnwdaf_MLModelMonitor_Subscribe | The consumer subscribes to NWDAF for the monitored ML Model accuracy information and Analytics Feedback Information for the analytics generated by the NWDAF with specific parameters. Consumer: NWDAF Producer: NWDAF |
|  |  | Nnwdaf_MLModelMonitor_Unsubscribe | The consumer unsubscribes to the NWDAF for the monitored ML Model accuracy information and Analytics Feedback Information for the analytics generated by the NWDAF. Consumer: NWDAF Producer: NWDAF |
| SA WG2 TS 23.288 [8] | ML Model Monitoring Services | Nnwdaf_MLModelMonitor_Notify | NWDAF notifies the monitored ML Model accuracy information and Analytics Feedback Information for the analytics generated by the NWDAF to the consumer who has subscribed to the specific NWDAF service. Consumer: NWDAF Producer: NWDAF |
|  |  | Nnwdaf_MLModelMonitor_Register | The consumer registers the use and monitoring capability for an ML Model at an NWDAF containing MTLF. Consumer: NWDAF Producer: NWDAF |
|  |  | Nnwdaf_MLModelMonitor_Deregister | The consumer deregisters, from an NWDAF containing MTLF, a previous MLModelMonitor registration, e.g. when the consumer is no longer using or monitoring the accuracy of the analytics generated using the ML Model. Consumer: NWDAF Producer: NWDAF |
| SA WG6 TS 23.482 [34] | ML Model Performance Monitoring APIs | MLModelPerfMonitor_Subscribe | The consumer subscribes for ML model performance monitoring. Consumer: VAL server Producer: AIMLE Server |
|  |  | MLModelPerfMonitor_Notify | The consumer is notified by ML repository on the ML model performance monitoring. Consumer: VAL server Producer: AIMLE Server |

***** NEXT CHANGE *****

### 6.3.4	Analysis on data collection and management for AI/ML

The analysis focuses on the specifications from SA WG2, SA WG6 and RAN WG3, considering these are the working groups defining services and operations related to data collection for AI/ML in 3GPP Release 18. SA WG1, SA WG3, SA WG4, SA WG5, RAN WG1 and RAN WG2 have not defined any services or operations related to data collection for AI/ML in Release 18. SA WG5 specifies data collection and performance measurement services that can be leveraged for AI/ML purposes (see TS 28.622 [72]). Table 6.3.4-1 provides a detailed overview of the specific services defined by each working group.

The key findings from the analysis are as follows:

-	SA WG2: Defines multiple network functions capable of producing data collection services and defines a function for data storage related services. For example, the DCCF coordinates and manages the collection of data from various network functions for purposes such as computation of analytics and Analytics/ML Model Accuracy monitoring.  Leverages event exposure framework and defines event exposure services for network functions that can be consumed by NWDAF (see clause 6.2.2.1 of TS 23.288 [8]) and DCCF (see clause 6.2.6.3 of TS 23.288 [8]). Defines data collection for AI/ML services through a clear consumer-producer relationship.

-	SA WG6: Defines network functions similar to those in SA WG2. Data collection services defined for A-DCCF as well as data storage services defined for A-ADRF are generic and applicable to any ADAE services; however, while A-DCCF APIs are defined in a generic manner, the A-ADRF APIs (for data storage and fetching) are defined in a per use case specific manner.

-	RAN WG3: Defines data collection messages exchanged between two gNBs over the Xn interface, in a P2P manner. It is to be noted that procedures used for AI/ML support in the NG-RAN shall be "data type agnostic", which means that the intended use of the data (e.g. input, output, feedback) shall not be indicated.

While SA WG2 and SA WG6 both define data collection services, their approaches to data storage and retrieval are different. SA WG2 defines generic data storage and retrieval services that can be supported by an entity (ADRF) and requested by another entity but SA WG6 defines both generic and individual services (related to each analytics type) for storing and retrieving data. RAN WG3 operates independently and is unrelated to services defined in SA WGs and therefore can coexist.

Table 6.3.4-1: Data Collection for AI/ML related services and operations as specified across 3GPP WGs

| Data Collection for AI/ML |  |  |  |
| --- | --- | --- | --- |
| TSG (TS/TR) | Service/API/Message Type | Service/API/IOC/Message Name | Description [Consumer, Producer] |
|  |  | Namf_EventExposure_Subscribe | The NWDAF uses this service operation to subscribe to or modify event reporting for one UE, a group of UE(s) or any UE. Producer: AMF |
|  |  | Namf_EventExposure_Unsubscribe | The NWDAF uses this service operation to unsubscribe for a specific event for one UE, group of UE(s), any UE. Producer: AMF |
|  |  | Namf_EventExposure_Notify | Provides the previously subscribed event information to the NWDAF which has subscribed to that event before. Producer: AMF |
|  |  | Nsmf_EventExposure_Subscribe | This service operation is used by an NWDAF to subscribe or modify a subscription for event notifications on a specified PDU Session or for all PDU Sessions of one UE, group of UE(s) or any UE. Producer: SMF |
| SA WG2 TS 23.288 [8] |  | Nsmf_EventExposure_UnSubscribe | This service operation is used by an NWDAF to unsubscribe event notifications. Producer: SMF |
|  | Event Exposure services | Nsmf_EventExposure_Notify | Report UE PDU Session related event(s) to the NWDAF which has subscribed to the event report service. Producer: SMF |
|  |  | Npcf_EventExposure_Subscribe | The NWDAF uses this service operation to subscribe to or modify event reporting for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI). Producer: PCF |
|  |  | Npcf_EventExposure_Unsubscribe | The NWDAF uses this service operation to unsubscribe for a specific event for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI). Producer: PCF |
|  |  | Npcf_EventExposure_Notify | This service operation reports the event to the NWDAF that has previously subscribed either using Npcf_EventExposure_Subscribe service operation or provided as part of the Data Set Application Data and Data Subset Service Parameters stored in UDR. Producer: PCF |
|  |  | Nudm_EventExposure_Subscribe | The NWDAF subscribes to receive an event. Producer: UDM |
|  |  | Nudm_EventExposure_Unsubscribe | The NWDAF deletes the subscription of an event if already defined in UDM. Producer: UDM |
|  |  | Nudm_EventExposure_Notify | UDM reports the event to the NWDAF that has previously subscribed. Producer: UDM |
|  |  | Nudm_EventExposure_ModifySubscription | The NWDAF requests to modify an existing subscription to event notifications. Producer: UDM |
|  |  | Nnef_EventExposure_Subscribe | The NWDAF subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated. Producer: NEF |
|  |  | Nnef_EventExposure_Unsubscribe | The NWDAF deletes an event if already defined in NEF. Producer: NEF |
|  |  | Nnef_EventExposure_Notify | NEF reports the event to the NWDAF that has previously subscribed. Producer: NEF |
|  |  | Naf_EventExposure_Subscribe | The NWDAF subscribes the event to collect AF data for UE(s), group of UEs, or any UE, or updates the subscription which is already defined in AF. Producer: AF |
|  |  | Naf_EventExposure_Unsubscribe | The NWDAF unsubscribes for a specific event. Producer: AF |
|  |  | Naf_EventExposure_Notify | The AF provides the previously subscribed event information to the NWDAF which has subscribed to that event before. Producer: AF |
|  |  | Nnsacf_SliceEventExposure_Subscribe | This service operation is used by the NWDAF to subscribe or modify a subscription with the NSACF for event based notifications of the current number of UEs registered for a network slice or the current number of PDU Sessions established on a network slice. Producer: NSACF |
|  |  | Nnsacf_SliceEventExposure_Unsubscribe | This service operation is used by the NWDAF to unsubscribe from the event notification. Producer: NSACF |
|  |  | Nnsacf_SliceEventExposure_Notify | This service operation is used by the NSACF to report the current number of UEs registered with a network slice or the current number of PDU Sessions established on a network slice in numbers or in percentage from the maximum allowed numbers, based on threshold or at expiry of periodic timer. Producer: NSACF |
|  |  | Nupf_EventExposure_Subscribe | This service operation reports the event and information to the NWDAF that has subscribed implicitly. Producer: UPF |
|  |  | Nupf_EventExposure_Unsubscribe | This service operation is used by an NWDAF to subscribe or modify a subscription to UPF event exposure notifications e.g. for the purpose of UPF data collection on a specified PDU Session or for all PDU Sessions of one UE or any UE. Producer: UPF |
|  |  | Nupf_EventExposure_Notify | The NF consumer uses this service operation to unsubscribe for a specific event. Consumer: Any NF Producer: UPF |
|  |  | Nscp_EventExposure_Notify | The NWDAF uses this service operation to unsubscribe for a specific event. Producer: SCP |
|  |  | Nscp_EventExposure_Subscribe | This service operation is used by an NWDAF to subscribe or modify a subscription to SCP event exposure notifications. Producer: SCP |
|  |  | Nscp_EventExposure_Unsubscribe | The NWDAF uses this service operation to unsubscribe from an existing subscription. Producer: SCP |
|  |  | Nnwdaf_DataManagement_Subscribe | The consumer subscribes to data exposed by an NWDAF. It can be historical data or runtime data. The subscription includes service operation specific parameters that identify the data to be provided and may include formatting and processing instructions that specify how the data is to be delivered to the consumer. Consumer: NWDAF, DCCF Producer: NWDAF |
|  | NWDAF Data Management services | Nnwdaf_DataManagement_Unsubscribe | The consumer unsubscribes to the data exposed by an NWDAF. Consumer: NWDAF, DCCF Producer: NWDAF |
|  |  | Nnwdaf_DataManagement_Notify | The NWDAF notifies the consumer of the requested data or notifies of the availability of previously subscribed data when delivery is via an NWDAF. The NWDAF may also notify the consumer when Data or Analytics is to be deleted. Consumer: NWDAF, DCCF, MFAF, ADRF Producer: NWDAF |
|  |  | Nnwdaf_DataManagement_Fetch | The consumer retrieves from the NWDAF subscribed data, as indicated by Fetch Instructions from Nnwdaf_DataManagement_Notify. Consumer: NWDAF, DCCF, MFAF, ADRF Producer: NWDAF |
|  |  | Nnwdaf_RoamingData_Subscribe | The consumer subscribes for input data related to roaming UE(s) for NWDAF analytics. The subscription includes service operation specific parameters that identify the data to be provided and may include formatting and processing instructions that specify how the data is to be delivered to the consumer. Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  | NWDAF Roaming Data services | Nnwdaf_RoamingData_Unsubscribe | The consumer unsubscribes to input data related to roaming UE(s). Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  |  | Nnwdaf_RoamingData_Notify | NWDAF notifies the consumer about input data related to roaming UE(s) that the consumer has subscribed to. Consumer: H-RE-NWDAF, V-RE-NWDAF Producer: H-RE-NWDAF, V-RE-NWDAF |
|  |  | Ndccf_DataManagement_Subscribe | The consumer subscribes to receive data or analytics from the DCCF. The subscription includes service operation specific parameters that identify the data or analytics to be provided and may include formatting and processing instructions that specify how the data is to be delivered to the consumer. The consumer may also request that data be stored in an ADRF or an NWDAF hosting ADRF functionality. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: DCCF |
|  |  | Ndccf_DataManagement_Unsubscribe | The consumer unsubscribes to DCCF for data or analytics. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: DCCF |
|  | DCCF Data Management Services | Ndccf_DataManagement_Notify | DCCF notifies the consumer instance of the requested data or analytics according to the request or notifies of the availability of previously subscribed Data or Analytics when data delivery is via the DCCF. The DCCF may also notify the consumer instance when Data or Analytics is to be deleted. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: DCCF |
|  |  | Ndccf_DataManagement_Fetch | The consumer retrieves from the DCCF, data or analytics as indicated by Ndccf_DataManagement_Notify Fetch Instructions. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: DCCF |
|  |  | Ndccf_DataManagement_Transfer | The Source DCCF transfers UE data subscription context to the target DCCF. Consumer: DCCF Producer: DCCF |
|  |  | Nmfaf_3daDataManagement_Configure | The consumer configures or reconfigures the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints and to format and process the out-bound data or analytics. Consumer: DCCF, NWDAF Producer: MFAF |
|  | MFAF Data Management Services | Nmfaf_3daDataManagement_Deconfigure | The consumer configures the MFAF to stop mapping data or analytics received by the MFAF to one or more out-bound notification endpoints. Consumer: DCCF, NWDAF Producer: MFAF |
|  |  | Nmfaf_3caDataManagement_Notify | MFAF provides data or analytics or notification of availability of data or analytics to notification endpoints. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: MFAF |
|  |  | Nmfaf_3caDataManagement_Fetch | The consumer retrieves from the MFAF, data or analytics as indicated by Nmfaf_3caDataManagement_Notify Fetch Instructions. Consumer: NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF Producer: MFAF |
|  |  | Nadrf_DataManagement_StorageRequest | The consumer NF uses this service operation to request the ADRF to store data or analytics. Data or analytics are provided to the ADRF in the request message. Consumer: DCCF, NWDAF, MFAF Producer: ADRF |
|  |  | Nadrf_DataManagement_StorageSubscriptionRequest | The consumer (NWDAF or DCCF) uses this service operation to request the ADRF to initiate a subscription for data or analytics. Data or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF. Consumer: NWDAF, DCCF Producer: ADRF |
|  | ADRF Data Management Services | Nadrf_DataManagement_StorageSubscriptionRemoval | The consumer NF uses this service operation to request that the ADRF no longer subscribes to data or analytics it is collecting and storing. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_RetrievalRequest | The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF. The Nadrf_DataManagement_RetrievalRequest response either contains the data or analytics or provides instructions for fetching the data or analytics. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_RetrievalSubscribe | The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF and to receive future notifications containing the corresponding data or analytics received by ADRF. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_RetrievalUnsubscribe | The consumer NF uses this service operation to request that the ADRF no longer sends data or analytics to a notification endpoint. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_RetrievalNotify | This service operation provides consumers with either data or analytics from an ADRF, or instructions to fetch the data or analytics from an ADRF. The notifications are provided to consumers that have subscribed using the Nadrf_DataManagement_RetrievalSubscribe service operation. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | Nadrf_DataManagement_Delete | This service operation instructs the ADRF to delete stored data. Consumer: NWDAF, DCCF Producer: ADRF |
|  |  | SS_AADRF_Data_Collection Subscribe | The consumer subscribes for offline data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_Data_Collection Notify | The consumer is receiving the offline data from A-ADRF as notification, based on subscription. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_ AADRF_Historical_ServiceAPI_Logs Get | The consumer requests API logs from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_NetworkSlice_Data Get | The consumer requests network slice data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_Location_Accuracy_Data Get | The consumer is receiving offline location analytics/data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
| SA WG6 TS 23.436 [33] | A-ADRF Data Collection APIs | SS_AADRF_EdgeData_Collection Subscribe | The consumer subscribes for offline edge data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_EdgeData_Collection Notify | The consumer is receiving the offline edge data from A-ADRF as notification, based on subscription. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_Edge_Preparation_Data Get | The consumer is receiving offline edge computing preparation data from the A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_Data_Storage Request Subscription | The consumer requests A-ADRF to subscribe for data or analytics from ADAE server or A-DCCF for store. This service operation provides parameters needed by the A-ADRF to initiate the subscription (to an ADAE server or A-DCCF). Consumer: ADAE server, A-DCCF Producer: A-ADRF |
|  |  | SS_AADRF_Data_Storage Store Data | The consumer requests A-ADRF to store data or analytics from ADAE server or A-DCCF. Data or analytics are provided to the A-ADRF in the request message. Consumer: ADAE server Producer: A-ADRF |
|  |  | SS_ADRF_ ServerToServer_Analytics Get | The consumer is receiving offline server-to-server analytics/data from A-ADRF. Consumer: ADAES Producer: A-ADRF |
|  |  | SS_AADRF_UE RAT connectivity analytics Get | The consumer is receiving offline UE RAT connectivity analytics/data from A-ADRF. Consumer: ADAE server Producer: A-ADRF |
|  |  | SS_ADCCF_Data_Collection Subscribe | The consumer subscribes to receive data or analytics from A-DCCF. The subscription includes service operation specific parameters that identify the data or analytics to be provided. Consumer: ADAE server Producer: A-DCCF |
|  | A-DCCF Data Collection APIs | SS_ADCCF_Data_Collection Notify | The A-DCCF notifies the consumer of the requested data or analytics according to the request or notifies of the availability of previously subscribed data or analytics when data delivery is via the A-DCCF. The A-DCCF may also notify the consumer when data or analytics is to be deleted. Consumer: ADAE server Producer: A-DCCF |
|  |  | SS_ADCCF_Data_Collection Get | The consumer retrieves data or analytics from the A-DCCF. Consumer: ADAE server Producer: A-DCCF |
|  |  | DATA COLLECTION REQUEST | NG-RAN node 1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node 2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node 2: shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop". Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node 2 performs measurements or predictions on. |
| RAN WG3 TS 38.423 [15] | Data Collection procedures | DATA COLLECTION RESPONSE | If NG-RAN node 2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node 1 and respond with the DATA COLLECTION RESPONSE message. If NG-RAN node 2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message. |
|  |  | DATA COLLECTION FAILURE | If none of the requested information can be initiated, NG-RAN node 2 shall send the DATA COLLECTION FAILURE message with an appropriate cause value. |
|  |  | DATA COLLECTION UPDATE | NG-RAN node 2 shall include in the DATA COLLECTION UPDATE message one or more of the following IEs based on the request: SSB Area Radio Resource Status List IE, Predicted Radio Resource Status, Predicted Number of Active UEs, Predicted RRC Connections, Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss, Energy Cost and Measured UE Trajectory. These IEs are specified in Rel. 18 to support three AI/ML for NG-RAN use cases, i.e. Energy Saving, Load Balancing and Mobility Optimization. |

***** END OF CHANGES *****