3GPP TSG-SA4 Meeting #134	S4-251788

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR | 0037 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD_PRO-MED] Corrections to ContentHostingConfiguration and ContentContributionConfiguration OpenAPI Schema |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD_PRO-MED |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The text describing the ContentHostingConfiguration and ContentPublishingConfiguration data models are not properly reflected within the OpenAPI YAML schema. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections to the ContentHostingConfiguration and ContentPublishing OpenAPI YAML to mirror the text within TS 26.510; and updates to TS 26.510 to properly reflect the OpenAPI YAML changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between normative specification text and OpenAPI YAML. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | [S4al250182]: Submitted for SWG ad hoc agreement. [S4aI250195]: Updates to address comments/recommendations from BBC, approved by SWG at 3GPP TSG-SA4 Meeting #-SA4-e (AH) MBS SWG post 133-e meeting. [S4-251788]: Resubmission of S4al250195 for SA4 approval. |  |  |  |  |  |  |  |  |

# Code changes

The code changes associated with this Change Request are available for review at the following URL on 3GPP Forge:

https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/13

https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/13/diffs?commit_id=29ac2702f4c6eaac2ba620c6166e6a0b1f2a7a98

The proposed changes are reproduced below for posterity.

## TS26510_Maf_Provisioning_ContentHosting.yaml

---a/TS26510_Maf_Provisioning_ContentHosting.yaml
+++b/TS26510_Maf_Provisioning_ContentHosting.yaml

@@ -1,7 +1,7 @@

1	1		openapi: 3.0.0

2	2		info:

3	3		  title: Maf_Provisioning_ContentHosting

4		-	  version: 1.1.0

4	+	  version: 1.2.0

5	5		  description: |

6	6		    Media Delivery: Content Hosting Provisioning API

7	7		    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

@@ -12,7 +12,7 @@ tags:

12	12		    description: 'Media Delivery: Content Hosting Provisioning API'

13	13

14	14		externalDocs:

15		-	  description: 'TS 26.510 V19.0.0; Media Delivery; Interactions and APIs for media session handling'

15	+	  description: 'TS 26.510 V19.1.0; Media Delivery; Interactions and APIs for media session handling'

16	16		  url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.510/'

17	17

18	18		servers:

@@ -310,6 +310,9 @@ components:

310	310		    IngestConfiguration:

311	311		      type: object

312	312		      description: 'A configuration for content ingest.'

313	+	      required:

314	+	        - mode

315	+	        - protocol

313	316		      properties:

314	317		        mode:

315	318		          $ref: 'TS26510_CommonData.yaml#/components/schemas/ContentTransferMode'

@@ -378,6 +381,7 @@ components:

378	381		          type: array

379	382		          items:

380	383		            $ref: '#/components/schemas/PathRewriteRule'

384	+	          minItems: 1

381	385		        cachingConfigurations:

382	386		          type: array

383	387		          items:

## TS26510_Maf_Provisioning_ContentPublishing.yaml

---a/TS26510_Maf_Provisioning_ContentPublishing.yaml
+++b/TS26510_Maf_Provisioning_ContentPublishing.yaml

@@ -1,7 +1,7 @@

1	1		openapi: 3.0.0

2	2		info:

3	3		  title: Maf_Provisioning_ContentPublishing

4		-	  version: 1.1.0

4	+	  version: 1.1.1

5	5		  description: |

6	6		    Media Delivery: Content Publishing Provisioning API

7	7		    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

@@ -12,7 +12,7 @@ tags:

12	12		    description: 'Media Delivery: Content Publishing Provisioning API'

13	13

14	14		externalDocs:

15		-	  description: 'TS 26.510 V19.0.0; Media Delivery; Interactions and APIs for media session handling'

15	+	  description: 'TS 26.510 V19.1.0; Media Delivery; Interactions and APIs for media session handling'

16	16		  url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.510/'

17	17

18	18		servers:

@@ -309,7 +309,7 @@ components:

309	309		  schemas:

310	310		    EgestConfiguration:

311	311		      type: object

312		-	      description: 'Parameters for egesting content from the Media AS at reference point M2.'

312	+	      description: 'Parameters for egesting content from the Media AS at reference point M2 or M10.'

313	313		      required:

314	314		        - mode

315	315		        - protocol

@@ -334,7 +334,6 @@ components:

334	334		      required:

335	335		        - contributionId

336	336		        - baseURL

337		-	        - entryPoint

338	337		      properties:

339	338		        contributionId:

340	339		          type: string

@@ -356,9 +355,6 @@ components:

356	355		          allOf:

357	356		            - $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId'

358	357		            - description: 'A reference to a Server Certificate resource to be presented by the Media AS at reference point M4.'

359		-	        canonicalDomainName:

360		-	          type: string

361		-	          description: 'Default Fully-Qualified Domain Name for use at reference point M4 or M10.'

362	358		        domainNameAlias:

363	359		          type: string

364	360		          description: 'Additional domain name nominated by the Media Application Provider and used by the Media AS to set appropriate CORS HTTP response headers at reference point M4.'

=====  CHANGE =====

### 8.8.3	Data model

#### 8.8.3.1	ContentHostingConfiguration resource

Table 8.8.3.1-1: Definition of ContentHostingConfiguration resource

| Property name |  |  |  | Data Type | Cardinality | Description |
| --- | --- | --- | --- | --- | --- | --- |
| name |  |  |  | string | 1..1 | A name for this Content Hosting Configuration. |
| ingestConfiguration |  |  |  | Ingest‌Configuration | 1..1 | Parameters for ingesting media content into the Media AS at reference point M2 or M10. |
|  | mode |  |  | Content‌Transfer‌Mode | 1..1 | Indicates whether media content is pulled by the Media AS from the Media Application Provider's origin server at reference point M2 or from another Media AS at reference point M10, or else pushed into the Media AS by the Media Application Provider at reference point M2 or from another Media AS at reference point M10 (see clause 7.3.4.5). |
|  | protocol |  |  | Uri | 1..1 | A fully-qualified term identifier URL that identifies the content ingest protocol. The controlled vocabulary of content ingest protocols is specified in clause 8 of TS 26.512 [6]. |
|  | baseURL |  |  | AbsoluteUrl | 0..1 | A base URL (i.e., one that includes a scheme, authority and, optionally, path segments) from which content is ingested by the Media AS at reference point M2 or M10 for this ingest configuration. - In the case of pull-based content ingest (mode is set to PULL), the base URL shall be provided to the Media AF by the Media Application Provider to indicate the location from which content is to be pulled. A request to a reference point M4 service location is mapped by the Media AS to a URL at reference point M2 or M10 whose base is the value of this property. - In the case of push-based content ingest (mode is set to PUSH), this property shall be populated by the Media AF and returned to the Media Application Provider to indicate the base URL to which content for this Content Hosting Configuration is to be published. |
| distributionConfigurations |  |  |  | array(Distribution‌Configuration) | 1..1 | Specifies the distribution method and configuration for the ingested content. The array shall contain at least one member. Hence, more than one distribution may be configured for the same ingested content, e.g. to offer different distribution configurations such as DASH and HLS, or to expose multiple service locations at reference point M4 or M10. |
|  | distributionId |  |  | string | 1..1 | An identification label, unique within the scope of this Content Hosting Configuration, that can be referenced by other resources in the Provisioning Session. The value is nominated by the Media Application Provider. |
|  | mode |  |  | ContentTransferMode | 0..1 | Indicates whether media content is: - pulled from the Media AS by a Media Access Client at reference point M4 or from another Media AS at reference point M10; or - pushed by the Media AS into a downstream Media AS at reference point M10. Default value if omitted: PULL. |
|  | affinityGroup |  |  | string | 0..1 | The Media Application Provider may assign an affinity group label indicating that the physical endpoint(s) of the reference point M4 service location exposed by this distribution configuration are to be deployed alongside those of service locations exposed by other distribution configurations declared in this Content Hosting Configuration with the same affinity group label. The physical endpoint(s) of service locations exposed by distribution configurations with different affinity group labels are intended to be deployed at mutually resilient network locations. If this property is omitted, deployment of physical endpoint(s) for the service location of this distribution configuration is at the discretion of the Media AF. |
|  | supplementary‌Distribution‌Networks |  |  | array(<Distribution‌NetworkType, DistributionMode> | 0..1 | Indicates that the content for this distribution configuration is also to be distributed via one or more supplementary networks. Each member of the array is a duple mapping a type of distribution network to a mode of distribution. The same DistributionNetworkType value shall appear at most once in this array. |
|  | edgeResources‌ConfigurationId |  |  | ResourceId | 0..1 | A reference to an Edge Resources Configuration resource (see clause 8.6.2). When present, indicates that the Media AS supporting this content distribution shall be realised as a set of one or more EAS instances configured per the referenced resource. |
|  | content‌Preparation‌TemplateId |  |  | ResourceId | 0..1 | A reference to a Content Preparation Template resource (see clause 8.5.2). Indicates that the referenced content preparation is required prior to distribution. |
|  | certificateId |  |  | ResourceId | 0..1 | A reference to a Server Certificate resource (see clause 8.4.3.2). When content is distributed using TLS [29], the referenced X.509 [10] certificate for the origin domain is presented by the Media AS in the TLS handshake at reference point M4 or M10. This attribute indicates the identifier of the certificate to use. - In the case of pull-based content distribution (content distribution mode is set to PULL), the referenced certificate shall be presented as a server certificate to the Media Client at reference point M4 or to the downstream Media AS at reference point M10. - In the case of push-based content distribution to a downstream Media AS (content distribution mode is set to PUSH), the referenced certificate shall be presented as a client certificate to the downstream Media AS at reference point M10. |
|  | canonical‌Domain‌Name |  |  | string | 0..1 | All resources exposed from the service location at reference point M4 or M10 shall be accessible through this default Fully-Qualified Domain Name. - In the case of pull-based content distribution at reference point M4 or M10 (content distribution mode is set to PULL), the value shall be assigned by the Media AF. - In the case of push-based content distribution to a downstream Media AS at reference point M10 (content distribution mode is set to PUSH), this property shall not be populated because the Media AS acts as the pushing client in this case. |
|  | domainNameAlias |  |  | string | 0..1 | The Media Application Provider may assign another Fully-Qualified Domain Name (FQDN) through which media resources within the scope of this distribution configuration are additionally accessible from the Media AS from the reference point M4 service location. This domain name is used by the Media AS to set appropriate CORS HTTP response headers sent from the reference point M4 service location. If this property is present, the Media Application Provider is responsible for providing in the DNS a CNAME record that resolves domainNameAlias to canonical‌Domain‌Name. If the certificateId property is also present in this distribution configuration, the provided domain name alias shall match one of the subjectAltName extension fields in the referenced Server Certificate resource, allowing for wildcard matching. This property shall be omitted if content distribution mode is set to PUSH because the Media AS acts as the pushing client in this case. |
|  | baseURL |  |  | AbsoluteUrl | 1..1 | A service location base URL (i.e., one that includes a scheme, authority and, optionally, path segments) from which content is made available to Media Clients at reference point M4 or to another Media AS at reference point M10 for this distribution configuration. - In the case of pull-based content distribution at reference point M4 or M10 (content distribution mode is set to PULL), the value is chosen by the Media AF when the Content Hosting Configuration is provisioned. It is an error for the Media Application Provider to set this. - In the case of push-based content distribution to a downstream Media AS at reference point M10 (content distribution mode is set to PUSH), this property shall be populated by the Media Application Provider with a base URL previously nominated by the Media AF managing that downstream Media AS. |
|  | entryPoint |  |  | Relative‌Media‌Entry‌Point | 0..1 | The Media Entry Point nominated by the Media Application Provider for this distribution configuration when it is used to describe a single content item and/or streaming session configuration (see clauses 5.2.8.2 and 7.3.3.12). This property may be omitted when this distribution configuration describes multiple content items or streaming session configurations. This property shall be omitted if content distribution mode is set to PUSH. |
|  |  | relativePath |  | RelativeUrl | 1..1 | A relative path (i.e., without a scheme or any leading forward slash characters) to the Media Entry Point document resource. The semantics are dependent on the value of ingestConfiguration.protocol. The path shall be valid at reference point M2 or M10 when appended to the ingest base URL and at reference point M4 when appended to the service location distribution base URL. |
|  |  | contentType |  | string | 1..1 | The MIME content type of the Media Entry Point. Used by the Media Client to select a Media Entry Point. |
|  |  | protocol |  | Uri | 0..0 | This property shall not be present in a distribution configuration. |
|  |  | profiles |  | array(Uri) | 0..1 | An optional list of conformance profile identifiers associated with the Media Entry Point, each one expressed as a URI. A profile URI may indicate an interoperability point, for example. Used by the Media Client to select a Media Entry Point. If present, the array shall contain at least one item. |
|  | pathRewriteRules |  |  | array(Path‌Rewrite‌Rule) | 0..1 | An ordered list of rules for rewriting the request URL paths of media resource requests handled by the Media AS at reference point M4 service location and translating them to URL paths at reference point M2 or M10. If multiple rules match a particular resource’s path, only the first matching rule, in order of appearance in this array, shall be applied. If present, the array shall have at least one member. |
|  |  | requestPathPattern |  | string | 1..1 | A regular expression [36] against which the path part of each Media AS request URL, including the leading “/”, and up to and including the final “/”, shall be compared. (Any leaf path element following the final “/” shall be excluded from this comparison.) In the case of pull-based content ingest, the M4 download request path is used in the comparison. In the case of push-based content ingest, the M2 or M10 upload request path is used in the comparison. In either case, if the request path matches this pattern, the path mapping specified in the corresponding mappedPath shall be applied. |
|  |  | mappedPath |  | string | 1..1 | A replacement for the portion of the Media AS request path that matches requestPathPattern. In the case of pull-based content ingest, ingestConfiguration.entryPoint is concatenated with the mapped path and any leaf path element from the original M4 download request to form the M2 or M10 origin request URL. In the case of push-based content ingest, canonical‌Domain‌Name (and, optionally, domain‌Name‌Alias) are concatenated with the mapped path and any leaf path element from the original M2 or M10 upload request to form the service location distribution URL(s) exposed at reference point M4 or M10. |
|  | cachingConfigurations |  |  | array(Caching‌Configuration) | 0..1 | A set of configurations of the Media AS content cache nominated by the Media Application Provider, each one affecting a matching subset of media resources ingested in relation to this Content Hosting Configuration. (See clause 7.3.3.13.) If present, the array shall have at least one member. |
|  |  | urlPatternFilter |  | string | 1..1 | A pattern used to match media resource URLs at reference point M2 or M10 to determine whether a given media resource ingested by the Media AS is eligible to be cached by it. The format of the pattern shall be a regular expression as specified in [36]. |
|  |  | cachingDirectives |  | object | 1..1 | If a urlPatternFilter applies to a resource, then the provided cachingDirectives shall be applied by the Media AS to resources served through the reference point M4 or M10 service location, potentially overwriting any origin caching directives provided by the Media Application Provider when that resource is ingested at reference point M2 or M10. |
|  |  |  | statusCodeFilters | array(integer) | 0..1 | The set of HTTP origin response status codes at reference point M2 or M10 to which these cachingDirectives apply. If the property is present, the array shall contain at least one item. If absent, the enclosing cachingDirectives shall apply to all HTTP origin response status codes. |
|  |  |  | noCache | boolean | 0..1 | If set to true, indicates that the media resources matching the filters shall be marked by the Media AS as not to be cached when it serves such media resources from a reference point M4 service location. Default value if omitted: false. |
|  |  |  | maxAge | Uint32 | 0..1 | The caching time-to-live period, expressed in seconds, of ingested media resources matching the filters. This determines the minimum period for which the Media AS shall cache matching media resources. If noCache is false, it also determines the time-to-live period signalled by the Media AS at reference point M4 service locations when it serves such media resources. The time-to-live for a given media resource shall be calculated relative to the time it was ingested by the Media AS. If noCache is false or omitted, ingested media resources shall be cached until the caching time-to-live period has been exceeded (if maxAge is present), indefinitely until the Content Hosting Configuration is destroyed by the Media Application Provider (if maxAge is omitted), until the Media Application Provider purges the cache, or until the available caching resources in the Media AS are exhausted, whichever is sooner. |
|  | geoFencing |  |  | object | 0..1 | Directives limiting access to the content to the indicated geographic areas (see NOTE 1). |
|  |  | locatorType |  | Uri | 1..1 | The type of the members of the locators array shall be indicated using a fully-qualified term identifier URI from the controlled vocabulary specified in clause B.1, or else from a vendor-specific vocabulary. |
|  |  | locators |  | array(string) | 1..1 | Array of locators from which access to the resources is to be allowed. The format of the locator strings shall be determined by the semantics of the term identifier indicated in locatorType. |
|  | urlSignature |  |  | object | 0..1 | Defines the URL signing scheme to be enforced by the Media AS at the reference point M4 service location (see NOTE 2). When present, only correctly signed and valid URLs are permitted to access the content resources within the scope of the enclosing distribution configuration. |
|  |  | urlPattern |  | string | 1..1 | A pattern that shall be used by the Media AS to match M4 media resource request URLs. The Media AS shall not serve a matching media resource from the reference point M4 service location unless it includes a valid authentication token calculated over the portion of the M4 request URL that matches this pattern. The format of the pattern shall be a regular expression as specified in [36]. |
|  |  | tokenName |  | string | 1..1 | The name of the query parameter that the Media Access Function shall use to present the authentication token in the M4 request URL when required to do so. |
|  |  | passphraseName |  | string | 1..1 | The name of the token parameter to be used to refer to the passphrase when constructing the M4 authentication token. |
|  |  | passphrase |  | string | 1..1 | A string of between 6 and 50 characters to be used as the shared secret between the Media Application Provider and the Media AS for this DistributionConfiguration. (This secret is used in the computation and verification of the M4 authentication token but is never sent in the cleartext part of the M4 request URL.) |
|  |  | tokenExpiryName |  | string | 1..1 | The name of the token parameter to be used to refer to the token expiry time point when constructing the M4 authentication token. The name of the query parameter that the Media Access Function shall use to present the token expiry time point in the cleartext part of the M4 request URL. |
|  |  | useIPAddress |  | boolean | 1..1 | If set to true, the IP address of the Media Access Function is included in the computation of the authentication token for resources that match urlPattern and access to matching media resources shall be allowed by the Media AF only when the M4 request is made from this IP address. |
|  |  | ipAddressName |  | string | 0..1 | The name of the token parameter that is encoded as part of the M4 authentication token if the useIPAddress flag is set to true. (The IP address is not passed in the cleartext part of the M4 request URL.) |
| NOTE 1: The geofencing feature used to restrict content requests to the Media AS at reference point M4 is specified in clause 7.6.4.6 of TS 26.512 [6]. NOTE 2: The format of the authentication token used to sign content requests to the Media AS at reference point M4 service locations is specified in clause 7.6.4.5 of TS 26.512 [6]. |  |  |  |  |  |  |

=====  CHANGE =====

Annex A (normative):
OpenAPI representation of HTTP REST APIs

# A.1	General

The normative code specifying the APIs defined in clauses 7.3, 8, 9 and 10 of the present document, including JSON Schema [38] representations of HTTP message bodies to be used with these APIs, is published on 3GPP Forge according to the OpenAPI 3.0.0 specification [32]. The YAML files corresponding to this version of the present document shall be published to the following location:

https://forge.3gpp.org/rep/all/5G_APIs/-/tags/TSG110-Rel19

Informative copies of these YAML files shall be distributed with the present document for convenience only. Where any discrepancy exists, the version on 3GPP Forge shall be considered definitive.