3GPP TSG-SA3 Meeting #125	S3-254724

Dallas, USA, 17 - 21 November 2025												    (revision of S3-254186)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.369 | CR | 0017 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIoT correction to the authentication procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, CATT, InterDigital, vivo, Huawei, HiSilicon, Lenovo, Ericsson, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-SEC |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Various terminologies are not consistent, and editor’s notes remain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixed terminology alignment and removal of several editor’s notes. Uses service operation in figure 5.2.1-1 and corresponding description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.2, 5.2.1, 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Merger of S3-253167, S3-253281, S3-253472, S3-253504, S3-253505, S3-253472, S3-253531, S3-253251, S3-253218 |  |  |  |  |  |  |  |  |

**************BEGIN OF CHANGES*************

#### 4.2.1.2	Requirements related to authentication between device and network

The AIoT device shall support:

-	a method for pseudo-random bit generation

* * * Next Change * * * *

## 5.2	Authentication procedure

### 5.2.1	General

This clause describes the authentication procedure for Ambient IoT devices for both Inventory procedure and Command procedure when authentication is triggered by the network. Device authentication shall always be performed for the Inventory Procedure

NOTE: KAIOT_root is the long-term key.

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of Clause 6.2.2 for the inventory procedure or Clause 6.2.3for the command procedure in TS 23.369 [2] is performed.

1. AIOTF shall invoke Nadm_SecRAND_Get service operation towards ADM. ADM shall generate and return RANDAIOT_n towards AIOTF.

2. AIOTF shall send Inventory Request message, including RANDAIOT_n in addition to the AIoT Identification Information specified in Clause 6.2.2 of TS 23.369 [2] to NG-RAN.

3. NG-RAN shall include RANDAIOT_n in the paging  message to the AIoT Device in addition to the AIoT Identification Information.

NOTE 1: An active attack may send a new paging message to the AIoT Device while there is an ongoing procedure in the AIoT Device. The AIoT Device will abort the ongoing procedure and respond to the new paging message. The security measure to such Denial-of-Service attack is not specified in present document.

NOTE 2: While a legitimate network is performing an inventory operation, an attacker may cause amplification of resource exhaustion at the legitimate network side by sending AIoT paging messages for all AIoT Ddevices or to a large group of AIoT Devices, which causes large number of AIoT Devices sending D2R messages to the legitimate network that the legitimate network does not expect to receive. The security measure to such amplification of resource exhaustion attack is not specified in present document.

4. Upon receiving the paging message, if the AIoT Device determines it needs to respond based on the AIoT Device Identification Information, the AIoT Device shall generate a pseudo-rancom number RANDAIOT_d, calculate RESAIOT using KAIoT_root, RANDAIOT_n, and RANDAIOT_d (see Annex A.2) for network authenticating the AIoT Device.

Editor’s Note: the randomness of RANDAIOT_d is FFS.

5. AIoT Device shall send a D2R message including an AIOT NAS message to the NG-RAN. The AIOT NAS message includes RESAIOT and RANDAIOT_d.

6.  NG-RAN shall send an Inventory Report message to AIOTF, including the AIOT NAS message containing RESAIOT and RANDAIOT_d.

7. AIOTF shall invoke Nadm_SecAuthentication_Get service operation with the AIoT Device Identification Information, RANDAIOT_n, and RANDAIOT_d towards ADM.

NOTE 3: The authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), which has load impact to ADM.

8. ADM shall calculate XRESAIOT using the same method as in AIoT Device (see Annex A.2).

9. ADM shall return XRESAIOT and AIoT Device Permanent Identifier if not included in step 7 to AIOTF.

10. AIOTF shall verify RESAIOT.

11.	If the verification is successful, for command case, the AIOTF shall invoke Nadm_SecSessionKey_Get service operation with AIoT Device Permanent Identifier, RANDAIOT_n, and RANDAIOT_d towards ADM. ADM shall calculate and return KAIoTF if it receives a request from AIOTF (see AnnexA.3).

The steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continue.

For the command procedure, the AIoT device implicitly authenticates the network via integrity check of the AIOT NAS Command Request message as specified in clause 5.3.2 of present document.

******************** CHANGES END******************