3GPP TSG-SA3 Meeting #123	S3-252542

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.326 | CR | 0003 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revision of 33.326 Rel 18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile US, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current TS has threat references that are TBD. The expected result for a AAA-S authorization in re-authentication and revocation scenarios is very confusing. Additionally, the SCAS is missing Clause Network Functions in service-based architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct TBD’s, improve the expected outcome and add the missing clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non 3GPP parties (e.g. labs) will not understand threat references and the expected outcome for AAA-S authorization in re-authentication and revocation scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.2.2, 4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of 1st Change ****************

### 4.2.1	Routes the S-NSSAI to the right place

Requirement Name: Routes the S-NSSAI to the right place

Requirement Reference: TS 33.501 [2], clause 6.8.1.2.3

Requirement Description: If the AAA-P is present (e.g. because the AAA-S belongs to a third party and the operator deploys a proxy towards third parties), the NSSAAF forwards the EAP ID Response message to the AAA-P, otherwise the NSSAAF forwards the message directly to the AAA-S. NSSAAF routes to the AAA-S based on the S-NSSAI, as specified in TS 33.501 [2], clause 6.13.

Threat Reference: TR 33.926 [4], clause P.3.2 – Threats related to NSSAAF.

Test Name: TC_NSSAAF_CORRECT_ROUTING

Purpose:

Verify that the NSSAAF forwards the NSSAA request to the right receiving end.

Pre-Conditions:

-	Test environment with AMF, AAA-S and AAA-P, which may be simulated. The NSAAF under test is connected with AMF, AAA-S and AAA-P.

-	A document describes the logic how the NSSAAF selects an AAA-S or AAA-P based on S-NSSAI.

-	Preconfigure the NSSAAF under test with two routing entries, each for a NSSAI. One of the slice is a part of MNO and the AAA-S can be directly found by the NSSAAF, while the other slice serves 3rd party and the AAA-P will be used for NSSAA procedure.

Execution Steps

1. The AMF sends Nssaaf_NSSAA_Authenticate Req to the NSSAAF including one of the S-NSSAI.

2. The NSSAAF sends AAA message to an AAA-P.

3. Repeat step 1 and 2 with the other S-NSSAI, and the NSSAAF sends AAA message to an AAA-S.

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

### 4.2.2	AAA-S authorization in re-authentication and revocation scenarios

Requirement Name: AAA-S authorization in re-authentication and revocation scenarios

Requirement Reference: TS 33.501 [2], clause 16.4

Requirement Description:  The NSSAAF checks whether the AAA-S is authorized to request the re-authentication and re-authorization by checking the local configuration of AAA-S address per S-NSSAI. If success, the NSSAAF requests UDM for the AMF serving the UE using the Nudm_UECM_Get (GPSI, AMF Registration) service operation. The UDM provides the NSSAAF with the AMF ID of the AMF serving the UE as specified in TS 33.501 [2], clause 6.13.

Threat Reference: TR 33.926 [4], clause P.3.2 – Threats related to NSSAAF.

Test Name: TC_NSSAAF_AAAS_AUTHORIZATION_REAUTH_REVOCATION

Purpose:

Verify that the AAA-S is authorized to send the re-authentication or revocation.

Pre-Conditions:

-	Test environment with AAA-S and AAA-P, which may be simulated. The NSAAF under test is connected with AAA-S and AAA-P.

-	A document describes the mapping between S-NSSAI and AAA-S server.

Execution Steps

1. The AAA-S sends Re-authentication or revocation message to the NSSAAF including the S-NSSAI and the GPSI.

2. The NSSAAF checks whether the AAA-S can be matched against with the S-NSSAI based on the mapping table.

Expected Results:

The NSSAAF rejects the re-authentication or revocation when the AAA-S and S-NSSAI are not mapped.

The NSSAAF passes the re-authentication or revocation when the AAA-S and S-NSSAI are mapped.

Expected format of evidence:

Save the logs and the communication flow in a .pcap file.

*************** End of 2nd Change ****************

*************** Start of 3rd Change ****************

### 4.3.6	Network Functions in service-based architecture

There are no NSSAAF additions to clause 4.3.6 of TS 33.117 [3].

*************** End of 3rd Change ****************