3GPP TSG-SA2 Meeting #171	S2-2509394

# 13 – 17 October 2025, Wuhan, China

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1586 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to Policy control for non-3GPP devices connecting behind a UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current wording as below may imply that multiple address(es) need to be provided for a single Non-3GPP Device Identifier. To make the wording consitent with all other places, user plane addresses should be changed to user plane address(es). The SMF may receive, from the UE, the Non-3GPP Device Identifier(s) and the corresponding user plane addresses in the UE initiated PDU session modification procedure, as specified in clauses 4.3.3.2 and 4.3.3.3 of TS 23.502 [3]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed addresses -> address(es) and PCC rules ->PCC rule(s) at couple of places - Other minor clarifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 6.1.3.31	Policy control for non-3GPP devices connecting behind a UE

The support of QoS differentiation for non-3GPP devices connecting behind a UE using a Non-3GPP Device Identifier is specified in clause 5.52 of TS 23.501 [2].

The SMF may receive, from the UE, the Non-3GPP Device Identifier(s) and the corresponding user plane address(es) in the UE initiated PDU session modification procedure, as specified in clauses 4.3.3.2 and 4.3.3.3 of TS 23.502 [3]. The SMF sends the Non-3GPP Device Identifier(s) and the user plane address(es) of the non-3GPP device(s) to the PCF. For the Non-3GPP Device Identifier(s) and the corresponding user plane address(es) received from the SMF, the PCF creates and/or updates PCC rule(s) accordingly. The PCF determines QoS parameters in the PCC rule(s) based on the Non-3GPP Device Identifier Information retrieved from the UDR and/or based on operator policy. The PCF subscribes to the modification of the Non-3GPP Device Identifier Information in the UDR. The PCF provides the PCC rule(s) to the SMF to enable differentiated QoS handling of the traffic to/from the non-3GPP device(s) behind the UE.

If any of the Non-3GPP Device Identifiers received in a PDU Session Modification Request is not present within the Non-3GPP Device Identifier Information as specified in clause 5.2.12.2.1 of TS 23.502 [3] for the UE, the PCF indicates to the SMF that one or more of the Non-3GPP Device Identifier(s) is not available for the UE. In such a case, the PCF shall reject the SM Policy Association Modification and based on the rejection received, the SMF shall reject the PDU Session Modification with a cause code to notify the UE that at least one of the Non-3GPP Device Identifier is not present for the UE.

When the Non-3GPP Device Identifier Information is updated/removed in the UDR for one or more Non-3GPP Device Identifiers, the UDR notifies the PCF, via the Nudr_DM_Notify service, of the update/removal of Non-3GPP Device Identifier Information (as defined in clauses 4.15.6.7.2 and 4.15.6.15 of TS 23.502 [3]). If there are PCC rule(s) associated to the traffic associated with the updated/removed Non-3GPP Device Identifier in a PDU session, the PCF shall remove or update the PCC rule(s) and start a PCF initiated SM Policy Association Modification, in which case the SMF removes/updates the corresponding QoS and N4 rule(s).

**** End of Changes ****