| 3GPP TR 23.700-44 V1.0.0 (2025-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Application Enablement to Support Energy Saving Phase 2 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Architectural Requirements and Assumptions	10

4.1	Architectural requirements	10

4.2	Architectural assumptions	10

5	Key Issues	11

5.1	Key issue #1: Enhance Application Enablement Layer Architecture and Services to Support Energy Saving	11

5.1.1	Description	11

5.1.2	Open issues	11

5.2	Key issue #2: Key Issue on monitoring and exposure of energy consumption	11

5.2.1	Description	11

5.2.2	Open issues	12

5.3	Key issue #3: EDGE application enablement enhancement for energy efficiency	12

5.4	Key issue #4: Support AIMLE enhancements for AIML services energy saving	12

5.4.1	Description	12

5.4.2	Open issues	13

5.5	Key issue #5: NSCE enhancement for energy efficiency	13

5.6	Key issue #6: Application Enablement Layer Services for Energy Saving	14

5.6.1	Description	14

5.6.2	Open issues	14

6	Solutions	14

6.0	Mapping of solutions to key issues	14

6.1	Solution #1: New Service on Energy Saving Assistance	15

6.1.1	Solution Description	15

6.1.1.0	General	15

6.1.1.1	Procedure	16

6.1.1.2	Information Flows	16

6.1.1.2.1	General	16

6.1.1.2.2	Energy saving assistance request	17

6.1.1.2.3	Energy saving assistance response	17

6.1.1.2.4	Energy saving assistance notify	17

6.1.2	Architecture impacts	17

6.1.3	Solution evaluation	17

6.2	Solution #2: Edge Application Server Discovery by Considering Renewable energy	18

6.2.1	Solution Description	18

6.2.2	Architecture impacts	18

6.2.3	Procedure	18

6.2.3.1	Procedure of subscription Energy information from EAS	18

6.2.3.2	Procedure of EAS discovery considering renewable energy information	19

8.5.2.2	Request-response model	20

6.2.4	Solution evaluation	21

6.3	Solution #3: Enhance Edge Services for Support Energy Saving	22

6.3.1	Solution Description	22

8.5.2.2	Request-response model	23

8.5.3.2	EAS discovery request	28

8.2.4	EAS Profile	30

6.3.2	Architecture impacts	31

6.3.3	Solution evaluation	31

6.4	Solution #4: Support of AIMLE client selection based on energy consumption	31

6.4.1	Solution Description	31

6.4.1.1	Impact to ML model information management	32

8.11.4.1	ML model information storage request	33

6.4.1.2	Impact to AIMLE client registration	34

8.7.2.2	AIMLE client registration	35

8.7.3.2	AIMLE client registration request	35

6.4.1.3	Impact to AIMLE client discovery	36

8.8.3.1	AIMLE client discovery request	37

6.4.1.4	Impact to AIMLE client selection	38

8.9.2.1	AIMLE client selection	39

6.4.1.5	Impact to AIMLE client selection subscription	40

8.13.2.2	AIMLE client selection subscription and notification	41

6.4.1.6	Impact to AIMLE client participation	42

8.10.3.1	AIMLE client participation request	42

6.4.2	Architecture impacts	42

6.4.3	Solution evaluation	43

6.5	Solution #5: AIMLE Enhancement of Considering Renewable Energy	43

6.5.1	Solution Description	43

6.5.2	Architecture impacts	44

6.5.3	Procedure	44

6.5.3.1	Procedure of FL member registration procedure	44

8.4.2	Procedure on FL member registration	45

6.5.3.2	Procedure of FL-related event subscription of renewable energy	45

8.5.2	Procedure on subscription for FL related events	46

6.5.3.3	Procedure of member selection during ML model training considering renewable energy	46

8.3.2	Procedure for ML model training	47

6.5.4	Solution evaluation	47

6.6	Solution #6: Support of ADAE analytics for AI/ML energy consumption	47

6.6.1	Solution Description	47

6.6.1.1	Procedure	47

6.6.1.2	Information flows	49

6.6.1.2.1	AI/ML energy consumption analytics subscription request	49

6.6.1.2.2	AI/ML energy consumption analytics subscription response	49

6.6.1.2.3	AI/ML energy consumption analytics notification	50

6.6.1.2.4	AI/ML energy consumption data collection subscription request	50

6.6.1.2.5	AI/ML energy consumption data collection subscription response	50

6.6.1.2.6	AI/ML energy consumption data notification	51

6.6.2	Architecture impacts	51

6.6.3	Solution evaluation	51

6.7	Solution #7: ADAES Support for AIMLE client energy sustainability analytics	52

6.7.1	Solution Description	52

6.7.2	Architecture impacts	53

6.7.3	Solution evaluation	53

6.8	Solution #8: Network Slice Energy Optimization based on Energy Saving VAL Server Policy	53

6.8.1	Solution Description	53

6.8.1.0	General	53

6.8.1.1	Procedure	54

6.8.1.2	Information Flows	56

6.8.1.2.1	General	56

6.8.1.2.2	Policy of Reducing Energy Consumption	56

6.8.1.2.3	Policy of Promoting renewable energy consumption	56

6.8.2	Architecture impacts	57

6.8.3	Solution evaluation	57

6.9	Solution #9: Enhancements to ADAE DN Energy Efficiency Analytics	57

6.9.1	Solution Description	57

8.18	Procedure for supporting DN Energy Efficiency analytics	58

8.18.1	General	58

8.18.2	Procedure	58

8.18.3	Information flows	59

8.18.3.1	General	59

8.18.3.2	DN energy analytics request/subscription request	59

8.18.3.3	DN energy analytics response/notification	60

8.18.3.4	Response to DN energy analytics request	61

6.9.2	Architecture impacts	61

6.9.3	Solution evaluation	61

6.10	Solution #10: Support of energy saving for location services	61

6.10.1	Solution description	61

6.10.1.1	Procedure of LMS subscription for the energy related information analysis	62

6.10.1.2	Procedure of LMS adjusting the location reporting configuration to save the energy	63

6.10.2	Architecture Impacts	64

6.10.3	Solution evaluation	64

6.11	Solution #11: Functional Architecture to Support Energy Saving	64

6.11.1	Solution Description	64

6.11.1.0	General	64

6.11.1.1	Functional Model	64

6.11.1.1.1	General	64

6.11.1.1.2	Network Functional Model	64

6.11.1.2	Functional Entities Description	65

6.11.1.2.1	General	65

6.11.1.2.2	SEAL client (Energy Saving)	65

6.11.1.2.3	SEAL server (Energy Saving)	65

6.11.1.3	Reference Points Description	65

6.11.1.3.1	General	65

6.11.2	Architecture impacts	65

6.11.3	Solution evaluation	65

6.12	Solution #12: New Service on Support Energy Data Collection and Processing	65

6.12.1	Solution Description	65

6.12.1.0	General	65

6.12.1.1	Procedure	66

6.12.1.2	Information Flows	67

6.12.1.2.1	General	67

6.12.1.2.2	Energy data collection and processing subscription request	67

6.12.1.2.3	Energy data collection and processing subscription response	68

6.12.1.2.4	Energy data collection and processing notify	68

6.12.2	Architecture impacts	68

6.12.3	Solution evaluation	68

6.13	Solution #13: Monitoring and exposing energy consumption within the application enablement layer	69

6.13.1	Solution Description	69

6.13.1.0	General	69

6.13.1.1	Energy monitoring procedure	69

6.13.1.2	Energy monitoring Information elements	70

6.13.1.2.1	Energy monitoring subscription request	70

6.13.1.2.2	Energy monitoring subscription response	71

6.13.1.2.3	Energy monitoring notification	71

6.13.2	Architecture impacts	72

6.13.3	Solution evaluation	72

6.14	Solution #14: Updating QoS according to renewable energy to improve user experience in EDGEAPP	72

6.14.1	Solution Description	72

6.14.1.1	General	72

6.14.1.2	Solution description	72

6.14.1.3	Procedure of QoS update related to renewable energy consumption	73

8.6.6.2.3	Update a session	74

6.14.2	Architecture impacts	74

6.14.3	Solution evaluation	75

6.15	Solution #15: Support energy saving in AI/ML task transfer	75

6.15.1	Solution Description	75

6.15.1.0	General	75

6.15.1.1	Energy saving in AI/ML task transfer procedure	75

6.15.1.2	Enhance the information flow for AI/ML task transfer procedure	76

6.15.2	Architecture impacts	77

6.15.3	Solution evaluation	77

6.16	Solution #16: Support Energy-aware split operation update	78

6.16.1	Solution Description	78

6.17	Solution #17: Support Energy-aware FL member availability update	80

6.17.1	Solution Description	80

6.18	Solution #18: Enhance Network Slice Adaptation Considering Energy Information for VAL Application	82

6.18.1	Solution Description	82

6.18.1.1	General	82

6.18.1.2	Procedure	82

9.11.2	Procedures	83

9.11.2.1	Procedure for VAL server-triggered and network-based network slice adaptation for VAL application - request and response model	83

6.18.1.3	Information flows	84

9.11.3.1	Network slice adaptation request	85

6.18.2	Architecture impacts	85

6.18.3	Solution evaluation	85

6.19	Solution #19: Enhance Multiple Slices Coordinated Optimization Considering Energy Information	85

6.19.1	Solution Description	85

6.19.1.1	General	85

6.19.1.2	Procedures	86

9.10.2	Procedure	87

9.10.2.1	Procedure on multiple slices coordinated resource optimization	87

9.10.3	Information flows	88

9.10.3.1	General	88

9.10.3.2	Multiple slices coordinated resource optimization request	88

9.10.3.3	Multiple slices coordinated resource optimization response	89

6.19.2	Architecture impacts	89

6.19.3	Solution evaluation	89

6.20	Solution #20: LM Service Enhancements to Location Reporting for Support Energy Saving	89

6.20.1	Solution Description	89

6.20.1.1	General	89

6.20.1.2	Enhance location reporting procedures	90

9.3.3.3	Location reporting	90

9.3.5	Client-triggered or VAL server-triggered location reporting procedure	91

6.20.1.3	Enhancements to information flows	92

9.3.2.4	Location reporting trigger	93

9.3.2.2	Location information report	95

6.20.2	Architecture impacts	95

6.20.3	Solution evaluation	95

6.21	Solution #21: Optimized Location Reporting based on Energy Saving	95

6.21.1	Solution Description	95

6.21.1.1	General	95

6.21.1.2	Procedure for optimized location reporting based on energy saving	96

6.21.1.2	Information Flows	97

6.21.1.2.1	Inform Location reporter VAL UE	97

6.21.1.2.2	Terminate Location reporter VAL UE	97

6.21.2	Architecture impacts	97

6.21.3	Solution evaluation	97

7	Overall Evaluation	98

7.1	Overall Evaluation of Key issue #1: Enhance Application Enablement Layer Architecture and Services to Support Energy Saving	98

7.2	Overall Evaluation of Key issue #2: Key Issue on monitoring and exposure of energy consumption	98

7.3	Overall Evaluation of Key issue #3: EDGE application enablement enhancement for energy efficiency	98

7.4	Overall Evaluation of Key issue #4: Support AIMLE enhancements for AIML services energy saving	98

7.5	Overall Evaluation of Key issue #5: NSCE enhancement for energy efficiency	98

7.6	Overall Evaluation of Key issue #6: Application Enablement Layer Services for Energy Saving	98

8	Conclusions	98

8.1	General conclusions	98

8.2	Conclusions of key issue #x	98

Annex A: Change history	99

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Application enablement layer can leverage the existing energy saving capabilities from core network and management system. In core network and network management system, the architecture work result in the design of architecture, functional, and services with consideration of energy efficiency and energy saving. Considering vertical-specific applications as the major consumers of 3GPP-provided services, the application enablement layer can play role on the exposure of energy saving services from different 3GPP domains to the vertical/ASP in a unified manner; and on defining, at an overarching layer, value-add support services for assisting energy saving services provided by either the VAL layer or the application enablement layer.

This technical report identifies the key issues and corresponding application architecture potential enhancements/new design and related solutions with recommendations for the normative work.

# 1	Scope

The present document is a technical report which identifies the application enabling layer architecture, capabilities, and services to support energy saving services at the application layer.

The aspects of the study continue the investigation of application enablement impacts, cover the application enablement layer architecture potential enhancements/new design and solutions needed, to support energy saving operations at the application enablement layer and provide assistance to energy saving operations at the VAL layer.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[8]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS)".

[10]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[11]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[12]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[13]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[14]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL)".

15]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Renewable Energy: As defined by the 3GPP TS 28.310 [4].

Renewable energy factor: As defined by the 3GPP TS 28.310 [4].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Architectural Requirements and Assumptions

## 4.1	Architectural requirements

[AR-4.1-a] The SEAL enabler layer (Energy Saving) shall support the provisioning of energy saving related assistance information to the application layer consumer.

[AR-4.1-b] The SEAL enabler layer (Energy Saving) shall support energy saving service event report from the SEAL enabler layer to the application layer.

[AR-4.1-c] The SEAL enabler layer (Energy Saving) shall support energy saving event-triggered and/or periodic event reporting.

[AR-4.1-d] The SEAL enabler layer (Energy Saving) shall support authentication of the application layer consumer.

[AR-4.1-e] The SEAL enabler layer (Energy Saving) shall support authorization of access and operation request from application layer consumer.

[AR-4.1-f] The SEAL enabler layer (Energy Saving) shall support exposing the value-added energy information to the the application layer consumer.

[AR-4.1-g] The SEAL enabler layer (Energy Saving) shall support provisioning and monitoring of energy information/status.

## 4.2	Architectural assumptions

[AA-4.2-a] The architecture for the energy saving services in the application enablement layer shall support consumption of the energy consumption information collection and exposure services as specified in clause 4.29 of 3GPP TS 23.502 [7].

[AA-4.2-b] The architecture for the energy saving services in the application enablement layer shall support consumption of the energy consumption information collection and measurement services as specified in clause 5.1 of 3GPP TS 28.552 [5] and in clause 6.7 of 3GPP TS 28.554 [6].

[AA-4.2-c] The architecture for the energy saving services in the application enablement layer shall use the CAPIF framework as specified in 3GPP TS 23.222 [15].

# 5	Key Issues

## 5.1	Key issue #1: Enhance Application Enablement Layer Architecture and Services to Support Energy Saving

### 5.1.1	Description

The new SID "study on Application Enablement to support Energy Saving Phase 2" lists the objective of the study that include:

=	Potential enhancements to the application enablement layer (e.g. SEALDD, AIMLE, LM, ADAE) to support energy saving for the network resources used by the applications.

Potential enhancements to the exisiting application enablement layer services or introduction of a new application enabler may be required to support energy saving of the resources used by the applications. Therefore, whether there is need on common functionalities from application enablemebt layer services for supporting energy saving operations need to be studied.

### 5.1.2	Open issues

To enhance application enablement layer architecture and services to support energy saving, the key issue will study:

-	Whether enhancements to existing application enablement layer services or a new application enabler service is required to support energy savings operations in the application enablement layer.

NOTE:	The study will consider the underlying 3GPP system’s existing capabilities such as leveraging the capabilities provided by core network and management system.

## 5.2	Key issue #2: Key Issue on monitoring and exposure of energy consumption

### 5.2.1	Description

Use cases and service requirements for energy saving have been specified in 3GPP TS 22.261 [2], including use cases and requirements requiring application enablement layer support.

In 3GPP TS 23.501 [3], 5GS enhancements for energy saving include framework for data volume information collection, network energy consumption collection, calculation, and exposure.

Measurement of the energy efficiency of NG-RAN, 5G Core and network slicing and the optimization of the energy saving have been defined in 3GPP TS 28.310 [4], 3GPP TS 28.552 [5], and 3GPP TS 28.554 [6].

Energy consumption used by application enablement services may vary based on the services’ performance level, usage level and localization. Furthermore, the energy consumption associated with application enablement services may fluctuate over time due to different conditions (e.g., varying traffic load) and hence may need to be monitored.

Consumers of application enablement services may need exposure of the energy consumption information associated to consumed application enablement services and may need to be informed about energy consumption. For example, a consumer of application enablement services may be informed about energy consumption related to its usage of the services and may reduce its interactions to lower energy consumption.

This key issue proposes to study support monitoring of energy consumption measurements and exposure of energy consumption information associated with usage of the application enablement layer.

### 5.2.2	Open issues

This key issue will study the following aspects:

1.	Whether and how the application enablement layer can obtain and use data related to energy consumption for energy consumption monitoring.

2.	Whether and how to support the monitoring of energy consumption measurements associated with the usage of the application enablement layer.

3.	Whether and how the application enablement layer can support exposing energy consumption results related to monitoring energy consumption.

## 5.3	Key issue #3: EDGE application enablement enhancement for energy efficiency

Energy efficiency and energy saving is a critical feature in 5G. As use case studied in 3GPP TR 22.882, clause 5.14, use case on reducing GHG footprint of Application Services, green services could be provided.

The related requirements defined in 3GPP TS 22.261 [2] including:

-	Subject to operator policy and agreement with 3rd party, the 5G system shall provide a mechanism to support the selection of an application server based on energy consumption information associated with a set of application servers.

As for now, EDGE service as defined in 3GPP TS 23.558 [8], is provided mostly based on service KPI, without fully considering the energy efficiency and energy saving, therefore, this key issue will study:

1.	Whether and how the EDGE application enablement layer needs to be enhanced to support energy saving and energy efficiency?

2.	How EDGE application enablement service could be enhanced to support energy saving and energy efficiency, including how to support the selection of an edge application server based on energy consumption information.

## 5.4	Key issue #4: Support AIMLE enhancements for AIML services energy saving

### 5.4.1	Description

SA1 identified use cases and requirements for energy efficiency and energy saving in Release-19 and Release-20 (in 3GPP TS 22.261 [2]). Energy efficiency and energy saving is a critical feature in 5G. Such use cases and requirements have application layer impacts. AI/ML requires more energy consumption at the device side as well as the network side. 3GPP TS 22.261 [2] has defined energy related information as a service criterion for multiple service including AIML.

3GPP SA6 in Rel-18 specified an application data analytics enablement functionality (ADAES, 3GPP TS 23.436 [11]). ADAES is a SEAL functionality for providing end to end performance analytics (e.g. VAL server performance). One analytics service of ADAES, as introduced in Rel-19, is the Support for DN energy Analytics. This feature supports a logical functionality at the ADAES to provide analytics on the energy consumption /efficiency of an edge platform (including the EESs / EASs). The DN energy analytics is performed per DNN/ DNAI and may be used to trigger the application server migration to different cloud. The analytics are based on NWDAF analytics and UPF/DN measurements on user plane load as well as edge/app side measurements on the energy consumption.

In addition, in Rel-19 AIML Enablement (AIMLE) service is described in 3GPP TS 23.482 [10]. AIMLE server is a SEAL server which includes of a common set of services for comprehensive enablement of AIML functionality. AIMLE server defines the following group of capabilities:

-	Support for application-layer ML model related aspects, including model retrieval, model training, model monitoring, model selection, model update and model storage / discovery.

-	Assistance in AI/ML task transfer and split AI/ML operations.

-	Support HFL/VFL operations, including FL member registration, FL grouping and FL-related events notification, VFL feature alignment, HFL training.

-	Support for AIMLE client registration, discovery, participation and selection.

Given the fact that ADAES supports ML-enabled analytics using AIMLE service (as defined in 3GPP TS 23.482 [10]), and in particular the requirement for an ADAE/AIMLE Client undertaking part of an ML task, such as ML model training in an FL operation, the energy consumption / efficiency analytics would help selecting the most appropriate UE as ML/FL member in the application layer ML task (e.g. training, inference).

### 5.4.2	Open issues

The key issue aims to study:

1.	How to collect and utilize energy consumption data (and which data) from the UE side to support an application layer ML task (or ML model lifecycle / workflow operation).

2.	How to enhance functionality and architecture of AIMLE to support energy saving.

3.	Whether and how to use energy consumption information to fulfill FL member selection, AIMLE client selection, split AI/ML operation, etc.

## 5.5	Key issue #5: NSCE enhancement for energy efficiency

SA1 identified use cases and requirements for energy efficiency and energy saving in Release-19 and Release-20 (in 3GPP TS 22.261 [2]). Energy efficiency and energy saving is a critical feature in 5G. Such use cases and requirements have application layer impacts.

3GPP TS 22.261 [2] has defined that network slice support for alternative service for energy saving reasons, energy consumption monitoring at per network slice and per subscriber granularity, expose information on energy consumption. These requirements may have effect on NSCE.

3GPP TS 23.501 [3] has defined following information of energy consumption information which can exposure to AF:

-	For UE level energy exposure, the consumer NF provides UE ID (SUPI/GPSI).

-	For S-NSSAI of the UE level exposure, the consumer NF provides UE ID (SUPI/GPSI), S-NSSAI.

-	For PDU session level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI.

-	For Service Data Flow level exposure (e.g. per UE per application), the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and application information, e.g. Application Identifier, or Flow description(s).

Based on information above, this key issue will study:

1.	Whether and how to enhance functionalities of NSCE to support for energy efficiency and energy saving.

2.	Whether and how to use energy consumption information to fulfill network slice selection, modification, adaptation, etc.

## 5.6	Key issue #6: Application Enablement Layer Services for Energy Saving

### 5.6.1	Description

The new SID "study on Application Enablement to support Energy Saving Phase 2" lists the objective of the study include:

-	Potential enhancements to the application enablement layer (e.g. SEALDD, AIMLE, LM, ADAE) to support energy saving for the network resources used by applications.

SA6 defines services which may be enhanced to support energy saving capabilities in order to satisfy the request from consumer (e.g. VAL server). Examples include (but not limited to):

-	Data delivery via SEALDD server and client consumes energy. For example, enhancements to the SEALDD server on adjustment of data delivery configuration with consideration of energy contains is needed.

-	LM client reports location information consumes energy. LM service on location information collection can be adjusted, for example, adjustment of LCS QoS to reduce energy consumption and/or switch LM client location reporting to network location reporting to saving energy at the UE side.

-	There are existing ADAE analytics on energy efficiency, for example, DN Energy Efficiency analytics, can be used for supporting energy saving of the network resources in some applications. To support wider range of applications, potential enhancements to the ADAE services may needed, e.g., enhance existing ADAE analytics and/or introduce new analytics.

Therefore, whether existing application enablement layer services including SEALDD, LM, ADAE, etc., can be enhanced to support energy saving of the resources used by applications need to be studied.

### 5.6.2	Open issues

To enhance existing application enablement layer services to support energy saving, the key issue will study:

-	Identify whether and how existing application enablement layer services (e.g. SEALDD, LM, ADAE) need be enhanced to support energy savings.

NOTE 1:	The existing application enablement layer services considered in this Key Issue are the ones which are not covered in the other Key Issues.

NOTE 2:	The study will consider the underlying 3GPP system’s existing capabilities such as leveraging the capabilities provided by core network and management system.

# 6	Solutions

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |
| Sol #2 | X |  | X |  |  |  |
| Sol #3 |  |  | X |  |  |  |
| Sol #4 |  |  |  | X |  |  |
| Sol #5 | X |  |  | X |  |  |
| Sol #6 |  |  |  | X |  | X |
| Sol #7 |  |  |  | X |  | X |
| Sol #8 |  |  |  |  | X |  |
| Sol #9 |  |  |  |  |  | X |
| Sol #10 |  |  |  |  |  | X |
| Sol #11 | X |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |
| Sol #13 |  | X |  |  |  |  |
| Sol #14 |  |  | X |  |  |  |
| Sol #15 |  |  |  | X |  |  |
| Sol #16 |  |  |  | X |  |  |
| Sol #17 |  |  |  | X |  |  |
| Sol #18 |  |  |  |  | X |  |
| Sol #19 |  |  |  |  | X |  |
| Sol #20 |  |  |  |  |  | X |
| Sol #21 |  |  |  |  |  | X |

## 6.1	Solution #1: New Service on Energy Saving Assistance

### 6.1.1	Solution Description

#### 6.1.1.0	General

The following clauses specify the procedure and information flow on energy saving assistance provisioning.

#### 6.1.1.1	Procedure

Figure 6.1.1.1-1: Energy saving assistance

1.	A consumer (e.g., VAL server, SEAL server/client) sends energy saving assistance request to SEAL server. The request includes information as described in Table 6.1.1.2.2-1.2.	The SEAL server authenticates and authorizes the request. If authorized, the SEAL server determines the next actions (e.g. collect energy-related data, monitor energy-related information/status) for assisting energy saving. The ESE server can consolidate similar requests from Consumers (e.g., data collection from same source into single southbound requests, avoid re-fetching data that it already has if the request parameters allow it such as time frame matches, etc.).

3.	The SEAL server sends energy-related data collection response to the consumer. The response includes information as described in Table 6.1.1.2.3-1.4.	The SEAL server collects energy consumption information, monitor energy status, and generates assistance information for energy saving based on the data/information obtained in step 4.

5.	The SEAL server sends energy saving assistance notify to the consumer with the assistance information. The notify includes information as described in Table 6.1.1.2.4-1.

Editor's Note:	The update of the solution is FFS.

#### 6.1.1.2	Information Flows

##### 6.1.1.2.1	General

The following information flows are specified for energy saving assistance.

##### 6.1.1.2.2	Energy saving assistance request

Table 6.1.1.2.2-1 shows the request sent by a SEAL service consumer to a SEAL server for energy saving assistance procedure.

Table 6.1.1.2.2-1: Energy saving assistance request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| VAL service ID | M | The identity of the VAL service for which the request applies. |
| Target energy saving entity(ies) | M | The identity(ies) of the target energy saving entity(ies) (e.g. VAL UE ID(s), SEAL server ID(s)) |
| Assistance requirements | M | Identifies the requirement for the assistance on energy saving. |
| >Assistance information type | M | Identifies the assistance information type required for energy saving (e.g. information on entity which consume the most/least energy, assistance information for selection of entity(ies)). |

##### 6.1.1.2.3	Energy saving assistance response

Table 6.1.1.2.3-1 shows the response sent by the SEAL server to the consumer for energy saving assistance procedure.

Table 6.1.1.2.3-1: Energy saving assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement). |
| Subscription ID | O | Identifier of the subscription. It shall be provided if the request if the Result is positive acknowledgement. |

##### 6.1.1.2.4	Energy saving assistance notify

Table 6.1.1.2.4-1 shows the notification sent by the SEAL server to the consumer for energy saving assistance procedure.

Table 6.1.1.2.4-1: Energy saving assistance notify

| Information element | Status | Description |
| --- | --- | --- |
| Success status | O (NOTE) | Indicates that energy saving assistance was successful. |
| >Assistance information | M | Provides energy saving assistance information corresponding to the request. |
| Failure response | O (NOTE) | Indicates that energy saving assistance was failure. |
| >Cause | M | Reason for the failure. |
| NOTE: One of the information elements shall be provided in the output. |  |  |

### 6.1.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.1.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.2	Solution #2: Edge Application Server Discovery by Considering Renewable energy

### 6.2.1	Solution Description

The proposed solution solves the problem listed in KI#1 and KI#3 and proposes to consider Renewable energy in edge computing.

In Edge Computing deployment, an application service may be served by multiple Edge Application Servers typically deployed in different sites. These multiple Edge Application Servers that host service may use a single IP address (anycast address) or different IP addresses. To start such a service, the UE needs to know the IP address(es) of the Application Server(s) serving the service. The UE may do a discovery to get the IP address(es) of a suitable Edge Application Server (e.g. the closest one), so that the traffic can be locally routed to the Edge Application Server and service latency, traffic routing path and user service experience can be optimized.

During the EAS discovery procedure, that the renewable energy of EAS is not considered. In order to save the energy consumption and reduce the carbon emission, the operators may consider using renewable energy or green energy to replace traditional energy.

### 6.2.2	Architecture impacts

In this solution, it doesn’t change the overall architecture of enabling edge applications defined in section 6.2 of 3GPP TS 23.558 [8]. But the following enhancements are added:

-	The EES can subscribe to EAS whether renewable is consumed by EAS. If the EAS consumes the renewable energy, for example in the morning time that solar energy can be consumed, the EES may select this EAS to save the energy.

The EAS can request the energy supply information from OAM.

### 6.2.3	Procedure

#### 6.2.3.1	Procedure of subscription Energy information from EAS

Figure 6.2.3.1-1: EAS energy information subscription

1.	The EES sends an EAS energy information subscription request to the EAS. The EAS energy information subscription request includes the EES ID along with the security credentials, Event ID and time period to subscribe to information about energy information of EAS.

The energy information includes Renewable energy of EAS. The Renewable energy of EAS includes that either the indication of the EAS consumes renewable energy or not, or the Renewable energy factor of EAS.

The time period indicates the EAS to notify the Energy information of EAS in the certain time period in the future.

2.	Upon receiving the request from the EES, the EAS checks if the EEC is authorized to subscribe for information of the requested EAS(s).

3.	If the processing of the request was successful, the EAS sends an EAS energy information subscription response to the EES, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire.

4.	The EAS sends an EAS renewable energy utilization notification to the EES.

The EAS can act as AF to subscribe or get the information of energy supply of EAS itself from OAM as defined in 3GPP TS 28.533 clause 5.6.

#### 6.2.3.2	Procedure of EAS discovery considering renewable energy information

The procedure listed here are based on the procedure defined in section 8.5.2.2 of 3GPP TS 23.558 [8].

#### 8.5.2.2 Request-response model Pre-conditions: 1. The EEC has received information (e.g. URI, IP address) related to the EES; 2. The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and 3. The EES is configured with ECSP's policy for EAS discovery. NOTE 1: Details of ECSP's policy are out of scope. Figure 8.5.2.2-1: EAS Discovery procedure 1. The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters, EEC service continuity support, and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route. The request may include an EAS selection request indicator. If an e2e tunnel is used in the UE for applications (e.g. user configured tunnel service), and if the tunnel service provider and the ECSP are from the same organization and only if user grants the permission, then the EEC provides the tunnel information for the associated applications in the request to the EES considering user consent. NOTE 2: The e2e tunnel case is limited to case where the tunnel service is deployed in home domain in the present release. 2. The same as step 2 in section 8.5.2.2. 3. If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs and Instantiable EAS Information. For discovered EASs, this includes endpoint information. If the EES perform traffic influence for EAS(s) in step 2, then the discovered EAS(s) is with optimized traffic route. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc. The EAS discovery response may contain a list of EASs and Instantiable EAS Information with EAS instantiation completion time. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC. When the discovered EAS is for a certain application group, then the Application Group ID is also included in the response message. If the discovered EAS is registered to another EES, then the EES endpoint of the EES where the discovered EAS is registered is also included in the response message. For the selected EAS, the capability of Renewable energy supporting is prioritized. When the EES accepts the request and determines to trigger the EAS instantiation, then the response may indicate that the EAS instantiation is in progress so that the detailed EAS profile information will be available later. Then the EES determines the remaining EAS instantiation time or EAS instantiation completion time based on the timing receiving EAS instantiation in progress ECSP management system and the predicted EAS deployment time. When EEC receives the EAS instantiation in progress indication, the EEC may send EAS discovery subscription request message if not subscribed yet or send EAS discovery request message later to the EES for obtaining updated EAS information. If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause. If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required. If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request. Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiated (i.e., an EAS profile was provided), the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE. Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiable but not yet instantiated (i.e. an EAS profile is not provided), the EEC sends the EAS information provisioning request indicating the selected EASID as in clause 8.15. If the EAS discovery response message contains the EAS(s) information along with r EAS instantiation completion time which the EAS instantiation in progress, then the EEC determines whether to identify the EAS which instantiation in progress as T-EAS based on Prediction expiration time and remaining EAS instantiation time/EAS instantiation completion time. If the EEC determines to identify the EAS for which instantiation is in-progress, the EEC may retry to discover the EAS and send the EAS discovery request again after waiting for the time as included in the response message, otherwise, the EAS discovery may fail. NOTE 10: Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command). NOTE 11: The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document. NOTE 12: The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required. If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request. NOTE 13: As long as a proper EAS (e.g. considering expected AC service KPIs included in EAS discovery request) is discovered and selected by the EES, EEC of a constraint UE can stop sending EAS discovery to rest candidate EES(s), and provide the selected EAS information to AC.

### 6.2.4	Solution evaluation

Editor's Note:	This part is for further update.

## 6.3	Solution #3: Enhance Edge Services for Support Energy Saving

### 6.3.1	Solution Description

According to the key issue #3, there is a need for EDGE application enablement layer enhance to support energy saving. The services procedures and information flows introduced in 3GPP TS 23.558 [8] for edge services, can be enhanced to support the energy saving requirements.

The procedure for EAS discovery as specified in clause 8.5.2.2 of 3GPP TS 23.558 [8] is enhanced as follows (new text in bold italics):

#### 8.5.2.2 Request-response model Pre-conditions: 1. The EEC has received information (e.g. URI, IP address) related to the EES; 2. The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and 3. The EES is configured with ECSP's policy for EAS discovery. NOTE 1: Details of ECSP's policy are out of scope. Figure 8.5.2.2-1: EAS Discovery procedure - The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters, EEC service continuity support, and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route. The request may include an EAS selection request indicator. If an e2e tunnel is used in the UE for applications (e.g. user configured tunnel service), and if the tunnel service provider and the ECSP are from the same organization and only if user grants the permission, then the EEC provides the tunnel information for the associated applications in the request to the EES considering user consent. The EAS discovery filter may include energy requirements to the EAS, e.g., Energy mode, Energy type supports. The Energy type is about requirement to the EAS on its supported energy type (e.g. non-renewable energy and/or renewable energy). NOTE 2: The e2e tunnel case is limited to case where the tunnel service is deployed in home domain in the present release. 2. Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. If EAS discovery filters contain energy requirements, the EES checks whether the EAS satisfy the requirements on Energy type supports in the EAS discovery requests. If the EAS fulfil the energy requirements, then EES adds the identified EAS(s) into the list of discovered EAS(s). If UE's location information is not already available, the EES obtains the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.3. If EAS discovery filters are provided by the EEC, but it does not contain Application group profile, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location. For the identified EAS(s) if EAS profile indicates the list of associated devices required in order to serve the UE, then EES checks whether the UE has required associated devices with it or not based on AC profile received in EAS discovery request. If the UE has the required associated devices then EES adds the identified EAS(s) into the list of discovered EAS(s). When the bundle EAS information is provided, then - If bundle EAS information includes EAS bundle identifier, the EES identifies all or part of the EAS(s) associated with the same EAS bundle identifier. - If bundle EAS information includes a list of EASIDs, the EES identifies the EASs which are all or part of the EAS bundle. If the EEC indicates that service continuity support is required, when identifying the EAS, the EES shall take the indication which ACR scenarios are supported by the AC, the EEC, the EES and the EAS and which of these are preferred by the AC into consideration. The EES may select one EAS and determine whether to perform application traffic influence for this EAS in advance based on AC's service KPI or EAS’s service KPI in desired response time, when the EAS does not perform traffic influence in advance. If the EES determines to perform application traffic influence for this EAS in advance, then the EES will applies the AF traffic influence of the EAS in the 3GPP Core Network before sending EAS discovery response. If the Prediction expiration time is provided then the EES may determine whether to identify the instantiable but not instantiated EAS as T-EAS based on Prediction expiration time and the predicted EAS deployment time information obtained from ADAES as specified in clause 8.11 of TS 23.436 [28] or from local configured maximum EAS deployment time. The EES determines remaining EAS instantiation time or EAS instantiation completion time based on the timing receiving EAS instantiation is in progress from ECSP management system and the predicted EAS deployment time. Furthermore, if EES received the indication which the EAS instantiation is in progress, then the EES determines whether to identify the EAS which instantiation in progress as T-EAS based on Prediction expiration time and remaining EAS instantiation time information. When EAS discovery filters are not provided, then: - if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location; - EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location), When EAS discovery filters contain Application group profile, the EES checks whether information about common EAS and related Application Group ID is available or not. If the common EAS information related to the Application Group ID is: - not available at the EES, then based on the policy if EES needs to select the common EAS, the EES identifies an EAS for the Application Group ID based on the provided EAS discovery filters such as KPIs, UE-specific service information or the ECSP policy. Furthermore, the EES stores the common EAS information and related Application Group ID. The EES may also subscribe and get the notifications of candidate DNAIs for the UEs of the group as described in 3GPP TS 23.548 [20], 3GPP TS 23.501 [2], the EES identifies the EAS(s) taking the candidate DNAIs into account, e.g., an EAS where its DNAI in the EAS profile is common to all UEs in the group. - available at the EES, then the EES provides information of that EAS as result for EAS discovery, or the EES identifies the EAS based on the provided EAS discovery filters and the UE location when E2E response time is received, furthermore: - when the UE in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE can connect either common EAS or EAS registered to this EES which is UE is in the common EAS service area), the EES identifies either common EAS or EAS registered to this EES for the UE based on the UE location, EAS discovery filters, application group profile, common EAS KPI as specified in table 8.2.5 and EDN information, registered EAS KPI as specified in table 8.2.5 and EDN information, and inter-EAS communication performance of these EASs and checks whether the common EAS or EAS registered to this EES can provide better E2E response time. - When the UE is not in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE is not in the common EAS service area), then the EES identifies candidate EAS(s) for the group from its registered EASs based on UE location, EAS discovery filters and application group profile and checks if the candidate EAS(s) can satisfy the E2E response time considering the response time of the candidate EAS(s), the response time of available common EASs in other EDNs and the inter-EAS communication latency. If the E2E response time cannot be satisfied, the EES will respond the EEC in step 3 indicating such a rejection reason; otherwise, the EES selects a candidate EAS and interacts with the ECS-ER to store it as common EAS as described in clause 8.20.2.3. NOTE 3: The EES can use ADAE analytics service in 3GPP TS 23.436 [28] or local configuration to obtain application performance (e.g. latency) for inter-EAS communication. NOTE 4: If the current EDN has overlapping area (where UE resides) with other EDN and both EDNs has available common EAS which can satisfy E2E response time, which EAS to select can take number of UEs in the group connected to available common EAS into consideration. NOTE 5: The EES may have previously determined and stored the common EAS for Application group ID, or the EES may have received the common EAS selection information for Application group ID during the common EAS announcement procedure. When the ECS-ER is not available and the EES selects the common EAS, the selected common EAS shall be announced to other EES(s) as per procedure specified in clause 8.19. When the ECS-ER is available, if the common EAS information related to the Application Group ID is: - not available at the ECS-ER, the EES identifies one EAS for the group and interacts with the ECS-ER to store the common EAS information as described in clause 8.20.2.3. The EES may also subscribe and get the notifications of candidate DNAIs for the UEs of the group as described in 3GPP TS 23.548 [20], 3GPP TS 23.501 [2], the EES identifies the EAS(s) taking the candidate DNAIs into account, e.g., an EAS where DNAI in the EAS profile is common to all UEs in the group. - available at the ECS-ER, then the ECS-ER returns the common EAS information to the EES as described in clause 8.20.2.3, or the EES identifies the EAS based on the provided EAS discovery filters and the UE location when E2E response time is received, furthermore: - When the UE is in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE can connect either common EAS or EAS registered to this EES, which is UE is in the common EAS service area), the EES identifies either common EAS or EAS registered to this EES for the UE based on the UE location, EAS discovery filters, application group profile, common EAS KPI as specified in table 8.2.5 and EDN information, registered EAS KPI as specified in table 8.2.5 and EDN information, and inter-EAS communication performance of these EASs and checks whether the common EAS or EAS registered to this EES can provide better E2E response time. - When the UE is not in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE is not in the common EAS service area), the EES checks with the ECS-ER about all available common EASs and their corresponding EDNs by using procedure defined in clause 8.20.2.2. If no common EAS is available for the group, EES identifies one EAS for the group and interacts with the ECS-ER to store the common EAS information as described in clause 8.20.2.3. If current EDN already has available common EAS, the EES selects such common EAS considering UE location, EAS discovery filters, application group profile and the required E2E response time. If there are available common EASs in other EDNs, the EES identifies candidate EAS(s) for the group from its registered EASs and checks if the candidate EAS(s) can satisfy the E2E response time considering UE location, EAS discovery filters, application group profile, and the response time of the candidate EAS(s), the response time of available common EASs in other EDNs and the inter-EAS communication latency. If the E2E response time cannot be satisfied, the EES will respond the EEC in step 3 indicating such a rejection reason; otherwise, the EES selects a candidate EAS and interacts with the ECS-ER to store it as common EAS as described in clause 8.20.2.3. NOTE 6: The EES can use ADAE analytics service in 3GPP TS 23.436 [28] or local configuration to obtain application performance (e.g. latency) for inter-EAS communication. NOTE 7: Details of the UE-specific service information and how it is available at the EES is out of scope. NOTE 8: Both steps are evaluated prior to sending a response. Upon receiving the request from the EEC, the EES may also collect edge load analytics from ADAES (as specified in clause 8.8.2 of TS 23.436 [28]) or performance data from OAM to find whether the EAS(s) satisfies the Expected AC service KPIs or the Minimum required AC Service KPIs. Upon receiving the request from the EEC, if the EEC does not indicate EAS Instantiation Triggering Suppress in the EAS Discovery request, the EES may trigger the ECSP management system to instantiate the EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12. Otherwise, upon receiving the request from the EEC, if the EEC indicates EAS Instantiation Triggering Suppress in the EAS Discovery request and the EES supports such capability, the EES determines not triggering the ECSP management system to instantiate the EAS and may determine Instantiable EAS Information for EAS(s) that are instantiable but not yet instantiated and match the EAS discovery filter IEs. Instantiable EAS Information is provided in the EAS Discovery response and includes the EASID(s) and, for each EASID, the status indicating whether the EAS is instantiated or instantiable but not yet instantiated. If the EEC provides in the EAS discovery request the EAS selection request indicator, the EES selects EAS satisfying the EAS discovery filter or based on other information (e.g. ECSP policy) as described above (if no EAS discovery filter received), and then provides the selected EAS information to the EEC in the discovered EAS list of EAS discovery response. NOTE 9: Without EAS selection request indication, the EES handling is as per R17 procedure. If the tunnel information is received, the EES additionally takes it into consideration in identifying EAS(s). If no tunnel information is received, the EES additionally takes the N6 tunnel (e.g. L2TP) information from 3GPP core network (via NEF user plane path management service as described in 3GPP TS 23.501 [2], clause 5.6.7) or NATed UE IP address (e.g. by local UPF based on local configuration) and EAS IP address information into consideration in identifying EAS(s). For instance, the IP address(es) of identified EAS(s) needs to be topologically close to the IP address of the tunnel server, local UPF (based on NATed UE IP address) or NATed UE for optimal N6 route. 3. If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs and Instantiable EAS Information. For discovered EASs, this includes endpoint information. If the EES perform traffic influence for EAS(s) in step 2, then the discovered EAS(s) is with optimized traffic route. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc., and regarding satisfied energy requirements, e.g., supported energy type. The EAS discovery response may contain a list of EASs and Instantiable EAS Information with EAS instantiation completion time. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC. When the discovered EAS is for a certain application group, then the Application Group ID is also included in the response message. If the discovered EAS is registered to another EES, then the EES endpoint of the EES where the discovered EAS is registered is also included in the response message. When the EES accepts the request and determines to trigger the EAS instantiation, then the response may indicate that the EAS instantiation is in progress so that the detailed EAS profile information will be available later. Then the EES determines the remaining EAS instantiation time or EAS instantiation completion time based on the timing receiving EAS instantiation in progress ECSP management system and the predicted EAS deployment time. When EEC receives the EAS instantiation in progress indication, the EEC may send EAS discovery subscription request message if not subscribed yet or send EAS discovery request message later to the EES for obtaining updated EAS information. If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause. If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required. If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request. Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiated (i.e., an EAS profile was provided), the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE. Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiable but not yet instantiated (i.e. an EAS profile is not provided), the EEC sends the EAS information provisioning request indicating the selected EASID as in clause 8.15. If the EAS discovery response message contains the EAS(s) information along with r EAS instantiation completion time which the EAS instantiation in progress, then the EEC determines whether to identify the EAS which instantiation in progress as T-EAS based on Prediction expiration time and remaining EAS instantiation time/EAS instantiation completion time. If the EEC determines to identify the EAS for which instantiation is in-progress, the EEC may retry to discover the EAS and send the EAS discovery request again after waiting for the time as included in the response message, otherwise, the EAS discovery may fail. NOTE 10: Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command). NOTE 11: The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document. NOTE 12: The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required. If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request. NOTE 13: As long as a proper EAS (e.g. considering expected AC service KPIs included in EAS discovery request) is discovered and selected by the EES, EEC of a constraint UE can stop sending EAS discovery to rest candidate EES(s), and provide the selected EAS information to AC.

The information flow for EAS discovery as specified in clause 8.5.3.2 of 3GPP TS 23.558 [8] is enhanced as follows (new text in bold italics):

#### 8.5.3.2 EAS discovery request Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element. Table 8.5.3.2-1: EAS discovery request Information element; Status; Description. Requestor identifier; M; The ID of the requestor (e.g. EECID). UE Identifier; O; The identifier of the UE (i.e., GPSI). Security credentials; M; Security credentials resulting from a successful authorization for the edge computing service.. EAS discovery filters; O; Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2.. UE location; O; The location information of the UE. The UE location is described in clause 7.3.2.. List of UE IDs; O; List of UE IDs in an Application Group, applicable for S-EAS or S-EES triggered EAS discovery request.. Serving MNO information (NOTE 2); O; The serving MNO information (e.g. MNO name, PLMN ID) which is serving the subscriber.. Target DNAI (NOTE 1); O; Target DNAI information which can be associated with potential T-EAS(s). EEC Service Continuity Support; O; Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR.. EES Service Continuity Support (NOTE 1); O; The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR.. EAS Service Continuity Support (NOTE 1); O; The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR.. EAS Instantiation Triggering Suppress; O; Indicates to the EES that EAS instantiation triggering should not be performed for the current request, and Instantiable EAS Information (e.g. instantiated, instantiable but not be instantiated yet) is to be provided in response.. EAS selection request indicator; O; Indicates the request for EAS selection support from the EES (e.g., for constrained device).. Indication of service continuity planning; O; Indicates that this EAS discovery request is triggered for service continuity planning.. Prediction expiration time; O; The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest. This IE is used by EES as analytics input to get edge load analytics information from ADAES service as described in clause 8.8 of TS 23.436 [28].. Tunnel information; O; It includes service provider ID, the endpoint address (e.g. IP address) of the tunnel server associated with application(s).. NOTE 1: This IE shall not be included when the request originates from the EEC. NOTE 2: This IE shall be included if edge node sharing is used.. Table 8.5.3.2-2: EAS discovery filters Information element; Status; Description. List of AC characteristics (NOTE 1); O; Describes the ACs for which a matching EAS is needed.. > AC profile (NOTE 2); M; AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1.. > Application group profile (NOTE 6); O; Application group profile associated with the AC Profile, as defined in Table 8.2.11-1.. List of EAS characteristics (NOTE 1, NOTE 3); O; Describes the characteristic of required EASs.. > EASID; O; Identifier of the required EAS.. > Application Group ID; O; Application group identifier as defined in 7.2.11.. > EAS content synchronization support; O; Indicates if the EAS content synchronization support is required or not.. > Bundle ID (NOTE 5); O; A list of EASIDs or a bundle ID as described in clause 7.2.10.. > List of EASIDs (NOTE 5); O; A list of EASIDs specific to a particular EAS bundle.. > Bundle type (NOTE 4); O; Type of the EAS bundle as described in clause 7.2.10. > EAS bundle requirements (NOTE 4); O; Requirements associated with the EAS bundle as described in clause 8.2.10.. > EAS provider identifier; O; Identifier of the required EAS provider. > EAS type; O; The category or type of required EAS (e.g. V2X, UAV, application enabler). > EAS schedule; O; Required availability schedule of the EAS (e.g. time windows). > EAS Geographical Service Area (NOTE 6); O; Location(s) (e.g. geographical area, route) where the EAS service should be available.. > EAS Topological Service Area (NOTE 6); O; Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1.. > Service continuity support; O; Indicates if the service continuity support is required or not.. > Service permission level; O; Required level of service permissions e.g. trial, gold-class. > Service feature(s); O; Required service features e.g. single vs. multi-player gaming service. > Energy type; O; Indicates what energy type support is required, e.g. non-renewable energy, renewable energy.. NOTE 1: Either "List of AC characteristics" or "List of EAS characteristics" shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. NOTE 4: When EAS discovery request is sent by the EEC, this IE shall not be included. NOTE 5: “Bundle ID" and "List of EASIDs" shall not both be present. NOTE 6: If application group profile IE is present, the expected group geographic service area IE present in the application group profile is preferentially used. If there is no EAS satisfied the expected group geographic service area or EAS does not provide better E2E response time, then the UE location and other service area IEs are considered..

The information flow of EAS Profile for EAS registration and discovery as specified in clause 8.2.4 of 3GPP TS 23.558 [8] is enhanced as follows (new text in bold italics):

### 8.2.4 EAS Profile An EAS Profile includes information about an EAS used to describe services and service characteristics offered. NOTE: Information elements in the EAS Profile are provided by the ASP. Table 8.2.4-1: EAS Profile Information element; Status; Description. EASID; M; The identifier of the EAS. EAS Endpoint; M; Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS.. List of EAS bundle information; O; List of EAS bundles to which the EAS belongs and related bundling requirements.. > Bundle ID (NOTE 3); O; Bundle ID as described in clause 7.2.10.. > List of EAS IDs (NOTE 2, NOTE 3); O; List of the EAS IDs of the EASs to be invoked by the EAS for an EAS driven association of EASs.. > Bundle type; M; Type of the EAS bundle as described in clause 7.2.10. > Main EASID; O; Indicate which EAS in a bundle takes the main EAS service role.. > EAS bundle requirements; O; Requirements associated with the EAS bundle as described in clause 8.2.10.. ACID(s); O; Identifies the AC(s) that can be served by the EAS. EAS Provider Identifier; O; The identifier of the ASP that provides the EAS.. Allowed MNO information (NOTE 4); O; Information of the allowed operator (e.g. MNO name, PLMN ID) from which its subscriber can consume the EAS services. EAS Type; O; The category or type of EAS (e.g. V2X, UAV, application enabler). EAS description; O; Human-readable description of the EAS. EAS Schedule; O; The availability schedule of the EAS (e.g. time windows). EAS Geographical Service Area; O; The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served.. EAS Topological Service Area; O; The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1.. EAS Service KPIs; O; Service characteristics provided by EAS, detailed in Table 8.2.5-1. EAS service permission level; O; Level of service permissions e.g. trial, gold-class supported by the EAS. EAS Feature(s); O; Service features e.g. single vs. multi-player gaming service supported by the EAS. EAS content synchronization support; O; Indicates if the EAS supports content synchronization between EASs.. EAS Service continuity support; O; Indicates if the EAS supports service continuity or not. This IE indicates which ACR scenarios are supported by the EAS, also indicates the EAS ability (e.g. EAS bundle information) of handling bundled EAS coordinate ACR.. EAS Transport layer service continuity support; O; This IE indicates the EAS service continuity support for seamless transport layer (e.g. TCP/TLS/QUIC) relocation. General context holding time duration (NOTE 1); O; The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC.. List of EAS DNAI(s); O; DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides.. List of N6 Traffic Routing requirements; O; The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI.. EAS Availability Reporting Period; O; The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration.. EAS Status; O; The status of the EAS (e.g. enabled, disabled, overload warning etc.). List of associated devices; O; List of associated devices (e.g. haptic device, joy stick) required along with UE in order to provide service to the user. It includes device type.. Energy type; O; Indicates what energy type is supported by the EAS, e.g. non-renewable energy, renewable energy.. NOTE 1: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). NOTE 2: This IE may be provided when only bundle ID is provided, and the bundle type indicates the proxy bundle. NOTE 3: At least one of the IEs shall be present if EAS bundle information is provided. NOTE 4: For edge node sharing scenario, in order to restrict the access to the subscriber of the partner operator, this IE should only include MNO information of the leading operator.. NOTE: The EAS Transport layer service continuity support can be used in EAS discovery, e.g. as described in 3GPP TS 23.433 [26] for SEALDD server acting as EAS, which can further support the EAS IP replacement function.

Editor's Note:	How EES gets the information of energy type supported by an EAS is FFS.

### 6.3.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.3.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.4	Solution #4: Support of AIMLE client selection based on energy consumption

### 6.4.1	Solution Description

This solution addresses Key Issue #4 – open issue #3 on how to use energy consumption information for AIMLE client selection. The solution proposes enhancements to the ML model information management procedure, and to procedures involving AIMLE client selection, to enable the storage and use of energy consumption information for AIMLE client selection.

The enhancements enable an AIMLE client to provide energy consumption capabilities during registration. Energy consumption capabilities include the maximum energy budget for performing AIML operations, and the VAL UE’s power profile. The VAL UE power profile indicates the degree of energy consumption of a VAL UE for performing AI/ML operations.

The enhancements also enable an ML model provider to include energy consumption information during ML model storage. The energy consumption information includes expected energy consumption values for performing AIML operations using an ML model. The expected energy consumption values are provided per power profile.

Both the AIMLE energy budget and the ML model energy consumption information are used for AIMLE client selection (e.g., request or subscription). During AIMLE client selection, the AIMLE server receives energy consumption requirements from a requestor. The energy consumption requirements include an indication to consider energy consumption during AIMLE client selection, and the maximum total energy consumption that is acceptable to the requestor. The AIMLE server uses the energy budget and energy consumption information to identify and select the ML models and AIMLE clients that meet the energy consumption requirements of the requestor.

#### 6.4.1.1	Impact to ML model information management

The ML model information management procedure and information flows in 3GPP TS 23.482 [10] are enhanced (highlighted in bold italics) as follows.

#### 8.11.4.1 ML model information storage request Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository or from the AIMLE client/VAL server to the AIMLE server as a request for the ML model information storage. Table 8.11.4.1-1: ML model information storage request Information element; Status; Description. Requestor Identity; M; The identity of the requestor performing the request.. Security Credentials; O (NOTE 1); Security credentials of the requestor performing the request.. ML model; O (NOTE 2); The ML model to be stored in the ML repository.. ML model address; O (NOTE 2); The address (e.g., a URL or an FQDN) of the ML model file.. ML model information; M; Provides information of the ML model, as described in Table 8.11.4.1-2.. NOTE 1: This information is needed if the requestor is at the VAL service provider domain. NOTE 2: At least one of these information elements shall be provided.. Table 8.11.4.1-2: ML model information Information element; Status; Description. ML model identifier; M; An identifier for the ML model. ADAE Analytics ID; O; Represents ADAE analytics ID for which the model can be used.. ML model size; O; Indicates the size of the ML model.. ML model source identifier; O; The identifier of ML model source (e.g., VAL server ID, VAL client ID) that stored the model in the ML repository.. VAL service ID(s); O; Identify the VAL service ID(s).. Domain; O; Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location prediction, etc.).. List of allowed vendors; O (NOTE 1); Indicates which vendors that are allowed to use the ML model and thereby also are interoperable to the model.. ML model interoperability information; O (NOTE 1); Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, input/output parameters of the ML model, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.. ML Model phase; O (NOTE 1); Represents the ML model phase, e.g., in training, trained, re-training, deployed.. > Observed performance; O (NOTE 2); Provides information on the performance of the model e.g. accuracy, or application-specific performance metrics (if ML model is in trained or deployment phase).. > Training information; O (NOTE 2); If the ML model is in trained or deployed phase: Information on the data that has been used to train the model (e.g. data sources, volume, freshness), and the base model ID in case of Transfer Learning.. > Indication of continuous model training; O; Indicates whether the model can be continuously trained or not.. > Continuous model training parameter; O; Parameters required for continuous model training.. ML model storage and discovery requirements; O (NOTE 1); Represents the requirements for the ML repository for the ML model storage and discovery.. > Storage duration; O; Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted.. > Security and access requirements; O; Represents the information on security requirements for storing the ML model information and the ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs or VAL client IDs, time period and location access). If the access requirement is private use only, then the model is not discoverable by other consumers.. ML model usage requirements; O; Represents the requirements for using the ML model (e.g. for inference or for training). The requirements are used by the AIMLE server to determine whether an AIMLE client is capable of using the model based on comparing the requirements with information in the AIMLE client profile in Table 8.7.3.1-2.. Energy consumption information; O; Energy consumption information associated with the ML model.. > List of AI/ML operations; M; The AI/ML operations (e.g., ML model training, model transfer, model inference, model offload and model split).. >> Expected energy budget; M; The expected energy budget (e.g., watt-hour, joules) associated with performing an AI/ML operation. The expected energy budget relates energy consumption values to a VAL UE power profile as described in Table 8.7.3.2-2.. NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available..

#### 6.4.1.2	Impact to AIMLE client registration

The AIMLE client registration procedure and information flows in 3GPP TS 23.482 [10] are enhanced (highlighted in bold italics) as follows.

#### 8.7.2.2 AIMLE client registration Pre-conditions: 1. The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server. 2. The AIMLE client has been pre-configured with an AIMLE client profile. Figure 8.7.2.2-1: AIMLE client registration 1. The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.2-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AIMLE client task capabilities with compute and task performance capabilities, and energy consumption capabilities to assist with performing AIMLE client discovery and AIMLE client selection. 2. The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. Upon successful authorization, the AIMLE server saves the context of the AIMLE client registration in the ML repository. 3. The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request. #### 8.7.3.2 AIMLE client registration request Table 8.7.3.2-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration request. Table 8.7.3.2-1: AIMLE client registration request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. List of supported profiles; M; Supported AIML client profile(s). For each client profile provided in the list, the supported service information is provided.. > AIMLE client profile; M; Information about the capability of the AIMLE client to support AI/ML operations for the VAL service ID as detailed in Table 8.7.3.2-2.. > List of supported services; M; List of VAL service IDs with corresponding permissions.. > VAL service ID; M; The identifier of the VAL service.. > Service permission level; O; Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage).. Table 8.7.3.2-2: AIMLE client profile Information element; Status; Description. Supported AI/ML model types; O; AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks).. Supported AI/ML operations; M; AI/ML operations supported by the AIMLE client such as ML model training, model transfer, model inference, model offload, model split, continue perform intermediate AI/ML operation/task).. AIMLE client time schedule configurations; O; Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations (e.g. ML model training) in the given time slot(s) and/or day(s) of the week.. AIMLE client location configurations; O; Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID.. AIMLE client capabilities; M; AIMLE client capability information (e.g. ML application type, allowed resource usage level).. Dataset availability; O; Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers.. Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions.. AIMLE client task capability; O (NOTE 1); Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs). Energy consumption capabilities; O; The energy consumption capabilities of the AIMLE client.. > Maximum energy budget; O (NOTE 2); The maximum energy budget (e.g., watt-hour, joules) permitted for performing AI/ML operations.. > Power profile; O (NOTE 2); The power rating of the VAL UE that represents the VAL UE’s processing capabilities (e.g., entry-level, mid-tier, premium, high performance, high efficiency). The power rating is used to determine the usage rate of energy per unit time that factors into the determination of energy consumption and is static.. NOTE 1: The Green and Energy-efficient task performance may not be applicable to a UE. NOTE 2: At least one of these information elements shall be present..

#### 6.4.1.3	Impact to AIMLE client discovery

The AIMLE client selection procedure and information flows in 3GPP TS 23.482 [10] are enhanced (highlighted in bold italics) as follows.

#### 8.8.3.1 AIMLE client discovery request Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure. Table 8.8.3.1-1: AIMLE client discovery request Information element; Status; Description. Requestor identity; M; The identifier of the requestor (e.g., VAL server).. AIMLE client discovery criteria; M; Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. Number of required AIMLE clients; O; Indicates the requested number of AIMLE clients to be discovered based on the discovery criteria.. Table 8.8.3.1-2: AIMLE client discovery criteria Information element; Status; Description. Service requirement; M; Information about the required service. > VAL Service ID; M; VAL Service ID that the client is required to support. This identifies the service associated with the requester.. > Service permission level; O; Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage).. Requested ML model types; O; Requested ML model types (decision trees, linear regression, neural networks).. Requested AIML operations; M; Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split.. Application layer AIMLE client capabilities; M; Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate).. Dataset requirements; O; Information about dataset.. > Dataset availability; O; Dataset availability, including dataset identifiers, dataset size, age, list of dataset features.. > Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions.. AIML client task capability requirements; O; Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs). AIMLE client velocity; O; Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static.. Location information; O; Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID) to discover the AIMLE clients.. AIMLE client QoS requirements; O; Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients.. Energy consumption requirements; O (NOTE 1); The energy consumption requirements of the requestor.. > Maximum energy budget; M; The maximum energy budget (e.g., watt-hour, joules) that is acceptable to the requestor.. NOTE 1: This information element applies to AIMLE client selection..

#### 6.4.1.4	Impact to AIMLE client selection

The AIMLE client selection procedure and information flows in 3GPP TS 23.482 [10] are enhanced (highlighted in bold italics) as follows.

#### 8.9.2.1 AIMLE client selection Pre-conditions: 1. AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services. 2. The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with the AIMLE clients. Figure 8.9.2.1-1: AIMLE client selection 1. A VAL server sends an AIMLE client selection request to an AIMLE server to select AIMLE clients available for participation in AI/ML operations (e.g., is available and have required data to train an ML model). The AIMLE client selection request includes information as described in Table 8.9.3.1-1. 2. The AIMLE server performs authentication and authorization checks to determine if the requestor is able to select AIMLE clients. 3. If the requestor is authorized, the AIMLE server performs AIMLE client selection based on the provided inputs as described below. For VAL server selection, the AIMLE server receives a list of AIMLE client IDs in the request and selects the AIMLE clients in the list as candidate AIMLE clients. For AIMLE server selection, the AIMLE server retrieves a list of clients with AI/ML capabilities from the ML repository. From the list of clients, the AIMLE server selects a list of candidate AIMLE clients, whose client profiles fulfil the AIMLE client selection criteria. The AIMLE server may use SEAL (LM service), NEF (e.g. MonitoringEvent API), and NWDAF (e.g. UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [2]) capabilities to assist the AIMLE client selection. The AIMLE server determines UEs by using their identifiers to determine their location and then selects only those that fulfil the location requirements specified in the AIMLE client selection criteria. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the UEs which fulfil the location requirement. The AIMLE server may then determine the QoS parameters for the AIML traffic session between the requestor and the candidate AIMLE client(s) and configure the AI/ML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). The AIMLE server determines the application QoS parameters based on the VAL Service ID. NOTE 1: To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated AIMLE client QoS requirements in the AIMLE client selection criteria provided in step 1. If the AIMLE client selection criteria include energy consumption requirements, the AIMLE server retrieves a list of ML model information from the ML repository using the ML model information discovery procedure as described in clause 8.11.3. The AIMLE server uses information from the AIMLE client selection criteria as filtering criteria for ML model information discovery. If energy consumption requirements are included, the AIMLE server selects the candidate AIMLE clients that fulfil the expected energy consumption needs for the retrieved ML models. The expected energy consumption needs are based on the requested AI/ML operations and on the AIMLE client’s power profile. If the energy consumption requirements include a maximum energy budget, the AIMLE server identifies and selects the ML models and candidate AIMLE clients that can perform the requested AI/ML operations without exceeding the maximum energy budget. NOTE 2: If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server can discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the AIMLE client selection response message. NOTE 3: The AIMLE server can reuse SEAL group management for any necessary group management. 4. The AIMLE server performs AIMLE client participation procedure with each candidate AIMLE client as described in clause 8.10. For all candidate AIMLE clients that agreed to participate in AI/ML operations, the AIMLE server selects the AIMLE clients and assigns an AIMLE client set identifier for the selected clients. The AIMLE client set may then be used for training a ML model. NOTE 4: If a required minimum number of AIMLE clients for AI/ML operation is provided in the AIMLE client selection request and the number of AIMLE clients that agreed to participate in AI/ML operations is less than this number, an AIMLE client set identifier will not be assigned and the status will be set to fail in the AIMLE client selection response in step 5. 5. The AIMLE server sends an AIMLE client selection response that includes information in Table 8.9.3.2-1.

#### 6.4.1.5	Impact to AIMLE client selection subscription

The AIMLE client selection subscription procedure and information flows in 3GPP TS 23.482 [10] are enhanced (highlighted in bold italics) as follows.

#### 8.13.2.2 AIMLE client selection subscription and notification Figure 8.13.2.2-1: AIMLE client selection subscription and notification 1. A VAL server sends an AIMLE client selection subscription request to the AIMLE Server. The AIMLE client selection subscription request includes information as described in Table 8.13.3-1 which includes selection criteria. 2. The AIMLE server validates the AIMLE client selection subscription request. The AIMLE server further performs authentication and authorization checks to determine if the requestor is able to subscribe to the selected AIMLE client selection subscription request. 3. The AIMLE server sends the AIMLE client selection subscription response to the VAL server. 4. The AIMLE server monitors AIMLE clients whether they fulfil the selection criteria as provided in step 1. The AIMLE server interacts with the NEF and/or SEAL services (including SEALDD) to establish monitoring. The AIMLE server utilizes SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.11, or 3GPP TS 23.434 [5] clause 9.3.12) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to establish monitoring of UEs entering or present in the target location provided in the location information in the selection criteria. 5. The AIMLE Server obtains the identifiers of the AIMLE clients from the monitoring and selects the clients that fulfil the selection criteria and remove the AIMLE clients which do not fulfil the selection criteria. The AIMLE server uses the location monitoring for selecting UEs that fulfil the location criteria and removing UEs which cease to fulfil the location criteria as provided in the location information in the AIMLE client selection criteria. The AIMLE Server may determine the application QoS parameters (e.g. bandwidth, latency, jitter) for the AIML traffic session between the VAL server and the selected AIMLE client and configure the AIML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed. The AIMLE server may determine the application QoS parameters based on the VAL Service ID. If the AIMLE client selection criteria include energy consumption requirements, the AIMLE server retrieves a list of ML model information from the ML repository using the ML model information discovery procedure as described in clause 8.11.3. The AIMLE server uses information from the AIMLE client selection criteria as filtering criteria for ML model information discovery. If energy consumption requirements are included, the AIMLE server selects the candidate AIMLE clients that can fulfil the expected energy consumption needs of the ML model. The expected energy consumption needs are based on the requested AI/ML operations and on the AIMLE client’s power profile. If the energy consumption requirements include a maximum energy budget, the AIMLE server identifies and selects the ML models and candidate AIMLE clients that can perform the requested AI/ML operations without exceeding the maximum energy budget. If a desired service in the selection criteria is ceased to be provided by the client or its profile change so that it no longer meets the selection criteria, the AIMLE server removes the AIMLE clients which ceases to fulfil the criteria and selects other clients that fulfil selection criteria. NOTE 1: To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated AIMLE client QoS requirements in the AIMLE client selection criteria provided in step 1. NOTE 2: In case of AIMLE server determining the application QoS parameters based on the VAL service ID, how it does this is up to implementation. 6. The AIMLE Server notifies the VAL server about the selected and re-selected AIMLE clients e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B.

#### 6.4.1.6	Impact to AIMLE client participation

The AIMLE client participation procedure and information flows in 3GPP TS 23.482 [10] are enhanced (highlighted in bold italics) as follows.

#### 8.10.3.1 AIMLE client participation request Table 8.10.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure. Table 8.10.3.1-1: AIMLE client participation request Information element; Status; Description. Requestor identity; M; The identifier of the requestor.. AIMLE client set identifier; M; An identifier for the AIMLE client set.. Add/remove indicator; M; Indicator for adding/removing the AIMLE client to/from the AIMLE client set.. ML model ID; M; The ML model ID to use for the AI/ML operation.. AI/ML operations; M; A list of AI/ML operations (e.g., training) required to be performed.. Expected energy budget; O; The expected energy budget (e.g., watt-hour, joules) associated with performing the AI/ML operations.. Operational schedule; O; Schedule for the AI/ML operations.. Dataset requirement; M; Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features.. Service requirement; M; Information about the required Service including its VAL service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage).

### 6.4.2	Architecture impacts

None.

### 6.4.3	Solution evaluation

This solution addresses Key Issue #4 – open issue #3 on how to use energy consumption information for AIMLE client selection. The solution proposes enhancements to 3GPP TS 23.482 [10] to enable the storage and use of energy consumption information for AIMLE client selection.

The solution enhances the ML model information to include energy consumption information associated with performing AI/ML operations using an ML model. The solution enhances the AIMLE client profile and the AIMLE client registration procedure to include and store the energy consumption capabilities of an AIMLE client. The solution enhances the AIMLE client discovery, selection, selection subscription and client participation procedures to include and consider the energy consumption requirements of the requestor and the energy consumption capabilities of the AIMLE clients.

The solution has no architectural impact.

## 6.5	Solution #5: AIMLE Enhancement of Considering Renewable Energy

### 6.5.1	Solution Description

The proposed solution solve the problem listed in KI#1 and KI#4, and proposes to consider renewable energy in AIMLE.

As indicated in 3GPP TS 28.310 [4], the renewable energy and renewable energy factor have the following definitions:

Renewable energy: energy from renewable non-fossil sources. NOTE 15: This definition is taken from 3GPP TS 22.261 [31]. NOTE 16: Examples of renewable energy sources include wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases NOTE 17: A renewable energy source is an energy source not depleted by extraction as it is naturally replenished at a rate faster than it is extracted ISO/IEC 30134-3:2016 [33]. NOTE 18: Criteria to categorize an energy as renewable can differ among jurisdictions, based on local environmental or other reasons ISO/IEC 30134-3:2016 [33]. Renewable energy factor: ratio of the renewable energy to the total energy. NOTE 19: This definition is taken from ISO/IEC 30134-3:2016 [33].

Energy can be classified into two major types: renewable energy and non-renewable energy. Renewable energy sources include solar energy, hydropower, wind energy, biomass energy, wave energy, tidal energy, ocean thermal energy, geothermal energy, etc. They can be recycled and regenerated in nature. It is an inexhaustible and endless energy source that regenerates automatically without human intervention.

In order to save the energy consumption and reduce the carbon emission, the operators may consider using renewable energy or green energy to replace traditional energy. So, in the current AIMLE procedure, the following enhancements can be added:

For AIMLE related procedure:

-	The FL member capabilities can include whether renewable energy support or not. It is more preferred to select the FL members that support renewable energy to perform such as AI model training. This capability can be indicated during the registration procedure.

-	Also, the capability that support renewable energy can be set as the Member selection criteria. During the ML training procedure, the member that supports the renewable energy can have the priority selection.

### 6.5.2	Architecture impacts

In this solution, it doesn’t change the overall architecture of Application enablement architecture of AIMLE in section 5 of 3GPP TS 23.482 [10]. But the following enhancements are added:

-	The FL member can register itself of the new capability that support renewable energy supply.

-	The subscriber can be notified with that new FL member is added to the ML repository with the energy supply of renewable energy.

-	And during member selection procedure, the Member selection criteria can be enhanced that whether renewable energy support or not.

One possible way is to request the energy supply information from OAM.

### 6.5.3	Procedure

#### 6.5.3.1	Procedure of FL member registration procedure

This procedure is the same as the procedure in section 8.4.2 of 3GPP TS 23.482 [10] with the following enhancement:

### 8.4.2 Procedure on FL member registration Figure 8.4.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry. Figure 8.4.2-1: Procedure for registration on FL member registry - The candidate FL member (e.g., VAL server via AIMLE server or AIMLE server) sends an FL member registration request to the ML repository for registering to the ML repository which acts as the AIML service registry. The request may include capability whether this FL member supports the renewable energy as energy supply or not. The FL member can act as AF to subscribe or get the information of energy supply of FL member itself from OAM as defined in 3GPP TS 28.533 clause 5.6. 2. The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the registration request. 3. The ML repository sends the generated information in the FL member registration response message to the candidate FL member. The procedure for AIMLE client to register to ML repository as FL member is introduced in clause 8.7.2.2.

#### 6.5.3.2	Procedure of FL-related event subscription of renewable energy

This procedure is the same as the procedure in section 8.5.2 of 3GPP TS 23.482 [10] with the following enhancement:

### 8.5.2 Procedure on subscription for FL related events This procedure, as illustrated in Figure 8.5.2-1, describes the subscription for events related to FL member availability. Pre-conditions: 1. The AIMLE server has the authorization to subscribe for the FL-related events (events described in clause 8.5.4). Figure 8.5.2-1: Procedure for FL-related event subscription - The subscriber (AIMLE server or VAL server (via AIMLE server)) sends an FL-related event subscription request to the ML repository to receive notification of FL related events and in particular the availability of FL member for a target area and time. The subscription includes indicator whether the FL member supports the capability of renewable energy supporting or not. 2. Upon receiving the event subscription request from the AIMLE server, the ML repository checks for the relevant authorization for the event subscription. If the authorization is successful, the ML repository stores the subscription information. 3. The ML repository sends an FL-related event subscription response to the subscriber indicating successful operation.

#### 6.5.3.3	Procedure of member selection during ML model training considering renewable energy

This procedure is the same as the procedure in section 8.3.2 of 3GPP TS 23.482 [10] with the following enhancement:

### 8.3.2 Procedure for ML model training Figure 8.3.2-1 illustrates the procedure for AIMLE server to support ML model training based on the request from the VAL server. Figure 8.3.2-1: ML model training 1. The VAL server sends an ML model training request to AIMLE server, requesting to assist in its ML model training. This request consists of ML model information or ML model requirement information, capability of renewable energy supporting or not, etc. 2. The AIMLE server checks whether the VAL server is authorized to perform the ML model training request. If no model information is provided but only the model requirement information is provided in step 1, the AIMLE server identifies and selects the appropriate ML model for training based on the ML model requirement information using the procedure as specified in clause 8.11.3. 3. If the VAL server is authorized, AIMLE server returns the success response, otherwise a failure response indication the reason for failure. 4. The AIMLE server notifies the VAL server to update the list FL/ML clients selected or de-selected for the ML model training or to share the training output or any errors during the training process.

### 6.5.4	Solution evaluation

Editor’s Note:	This part is for further update in the future.

## 6.6	Solution #6: Support of ADAE analytics for AI/ML energy consumption

### 6.6.1	Solution Description

This solution addresses Key Issue #4 – open issue #1 on how to collect and use energy consumption data. The solution proposes a new ADAE service procedure in 3GPP TS 23.436 [11] to support AI/ML energy consumption analytics.

#### 6.6.1.1	Procedure

Figure 6.6.1.1-1 illustrates the procedure for a consumer (e.g., VAL server, AIMLE server) to subscribe for AI/ML energy consumption analytics generated from data collected from VAL UEs while the VAL UEs are performing AI/ML operations.

Pre-conditions:

1.	ADAE Client (ADAEC) is connected to the ADAE Server (ADAES).

2.	Data producers (e.g., VAL Clients) may be pre-configured with data producer profiles for the data they can provide. ADAES and ADAEC have discovered available data producers and their data producer profiles.

Figure 6.6.1.1-1: ADAES support for AI/ML energy consumption analytics

1.	An analytics consumer (e.g. VAL Server, AIMLE Server) sends an AI/ML energy consumption analytics subscription request to an ADAES. The subscription request includes an Analytics ID “AI/ML energy consumption analytics”, ML model identifier, AIML operation, energy consumption metrics, and other information as defined in Table 6.6.1.2.1-1.

2.	The ADAES validates the subscription request and performs authentication and authorization checks to determine if the consumer is authorized to subscribe for the requested analytics. If authorization is successful, the ADAES stores the subscription request information, assigns a subscription ID, and sends a subscription response indicating successful subscription and includes the subscription ID.

3.	The ADAES maps the analytics ID to a list of data collection event identifiers and determines a list of data producer IDs for the data collection event identifiers. The determination of data producer IDs is based on data producer profiles.

4.	The ADAES sends an AI/ML energy consumption data collection subscription request to the ADAECs determined in step 3. The subscription request includes information as defined in Table 6.6.1.2.4-1. Data collection at the VAL UE(s) reuses the EVEX mechanism defined in 3GPP TS 26.531 [12].

5.	Each ADAEC configures the data producers (i.e., VAL client or AIMLE client) for collecting data (e.g., duration of AI/ML operation, % resource utilization for AI/ML operation) for AI/ML energy consumption analytics requested in the data collection subscription request.

6.	The ADAEC sends an AI/ML energy consumption data collection subscription response to acknowledge the ADAES. The response includes a status and a subscription ID for the data collection.

7.	The data producers perform AI/ML operations when triggered (e.g., ML model training) using the indicated ML model and collect data for AI/ML energy consumption analytics based on the configured data collection requirements.

8.	At the completion of AI/ML operation, the ADAEC sends an AI/ML energy consumption data collection notification to the ADAES. The data notification includes the data collection subscription ID and the information as defined in Table 6.6.1.2.6-1.

9.	The ADAES performs analytics relevant operations to generate the AI/ML energy consumption analytics based on the data received from the ADAECs. The ADAES uses the data collected by the ADAECs to calculate the AI/ML energy consumption analytics.

NOTE:	How the collected data are used to calculate energy consumption is up to implementation.

10.	The ADAES sends an AI/ML energy consumption analytics notification to the consumer. The analytics notification includes the analytics subscription ID and the information as defined in Table 6.6.1.2.3-1.

#### 6.6.1.2	Information flows

##### 6.6.1.2.1	AI/ML energy consumption analytics subscription request

Table 6.6.1.2.1-1 describes the information flow from the consumer (e.g. VAL server, AIMLE server) as a request or update request for AI/ML energy consumption analytics.

Table 6.6.1.2.1-1: AI/ML energy consumption analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer (e.g., VAL server, AIMLE client). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example "AI/ML energy consumption analytics". |
| Analytics type | M | The type of analytics for the event (e.g., statistics or predictions). |
| ML model identifier | M | The identifier of the ML model for which analytics are determined. |
| AIML operation | M | The desired AIML operations (e.g., ML model training, inference) for which analytics are determined. |
| Energy consumption metrics | M | The formula and necessary metrics for calculating the energy consumption based on factors that impact energy consumption for an AIML operation using a particular ML model. |
| Target VAL UE IDs | O | The identifier(s) of VAL UE(s) for which the analytics subscription applies. |
| Target data producer IDs | O | The identifier(s) of VAL clients, AIMLE clients or FL members to target for collecting data for energy consumption analytics. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time validity | O | The time validity of the subscription request. |

##### 6.6.1.2.2	AI/ML energy consumption analytics subscription response

Table 6.6.1.2.2-1 describes the information elements for the AI/ML energy consumption analytics subscription response from the ADAES to the consumer.

Table 6.6.1.2.2-1: AI/ML energy consumption analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement). |
| Subscription ID | M | An identifier for the subscription. |

##### 6.6.1.2.3	AI/ML energy consumption analytics notification

Table 6.6.1.2.3-1 describes the information flow from the ADAES to the consumer (e.g. VAL Server, AIMLE Server) as a notification for the AI/ML energy consumption analytics.

Table 6.6.1.2.3-1: AI/ML energy consumption analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | The identifier for the subscription. |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example " AI/ML energy consumption analytics". |
| ML model identifier | M | The ML model identifier for which analytics are determined. |
| Energy consumption information | M | The range of energy consumption values for a particular ML model with a certain AIML operation. The output can be an average, minimum, and/or maximum value for the energy consumption. The output may be a prediction or a statistical value. |
| List of AIML operations | M | The AIML operations for which the analytics are determined. |
| > Energy consumption | M | The range of energy consumption values for performing an AI/ML operation. |
| >> Maximum consumption | O | The maximum energy consumption value. |
| >> Average consumption | O | The average energy consumption value. |
| >> Minimum consumption | O | The minimum energy consumption value. |
| Target analytics period | O | The time validity of the analytics. |
| Confidence level | O | The level of accuracy for the analytics output (e.g. prediction). |

##### 6.6.1.2.4	AI/ML energy consumption data collection subscription request

Table 6.6.1.2.4-1 describes information elements for the AI/ML energy consumption data collection subscription request from the ADAES to the ADAEC.

Table 6.6.1.2.4-1: AI/ML energy consumption data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data collection requirements | M | The requirements for data collection and for information to include in the data notification. |
| > ML model identifier | M | The identifier of the ML model for which data is collected. |
| > AIML operations | M | The desired AI/ML operations (e.g., ML model training, inference) for which data is collected. |
| > Time period | M | The time period of data collection. |
| > Location or area information | O | The location or area of the data collection. |

##### 6.6.1.2.5	AI/ML energy consumption data collection subscription response

Table 6.6.1.2.5-1 describes information elements for the AI/ML energy consumption data collection subscription response from the ADAEC to the ADAES.

Table 6.6.1.2.5-1: AI/ML energy consumption data collection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the AI/ML energy consumption data collection subscription request (positive or negative acknowledgement). |
| Subscription ID | M | An identifier for the subscription. |

##### 6.6.1.2.6	AI/ML energy consumption data notification

Table 6.6.1.2.6-1 describes information elements for the AI/ML energy consumption data notification from the ADAEC to the ADAES.

Table 6.6.1.2.6-1: AI/ML energy consumption data collection notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID |  | M | The identifier for the subscription. |
| Data Collection Event ID |  | M | The identifier of the data collection event. |
| Data Producer ID |  | M | The identity of Data Producer. |
| Collected data |  | M | The collected data. |
| > ML model identifier |  | M | The identifier of the ML model for which data was collected. |
| > AIML operation |  | M | The AI/ML operation (e.g., ML model training, inference) for which data was collected. |
| > Power profile |  | M | The power rating of the VAL UE that represents the VAL UE’s processing capabilities (e.g., entry-level, mid-tier, premium, high performance, high efficiency). The power rating is used to determine the usage rate of energy per unit time that factors into the determination of energy consumption and is static. |
| > Elapse time for AI/ML operation |  | M | The time duration for performing the AI/ML operation. |
| > % resource utilization for AI/ML operation |  | M | The percentage utilization of hardware resources (e.g., compute, memory, storage) determined for performing the AI/ML operation. |

### 6.6.2	Architecture impacts

None.

### 6.6.3	Solution evaluation

This solution addresses Key Issue #4 – open issue #1 on how to collect and use energy consumption data for AI/ML operations. The solution proposes enhancements to ADAE service described in 3GPP TS 23.436 [11] to support AI/ML energy consumption analytics.

The solution enhances the ADAE service by generating analytics for energy consumption associated with performing AI/ML operations using an ML model. The solution provides a new procedure for collecting data from ADAECs required for generating AI/ML energy consumption analytics. The solution manages subscriptions for AI/ML energy consumption analytics and notifies consumers of the analytics outputs.

The solution has no architectural impact.

## 6.7	Solution #7: ADAES Support for AIMLE client energy sustainability analytics

### 6.7.1	Solution Description

This solution introduces a new analytics event for “AIMLE client energy sustainability analytics” as requested from the Consumer who can be the VAL Server or AIMLE server to the ADAES. These analytics may be used for predicting whether an AI/ML client at the VAL UE can be considered a candidate in a ML training or inference task, or FL process based on its expected energy status/capability. Such energy analytics can be used for identifying whether the energy consumption is sustainable for a given AIMLE service, or a recommendation on whether the AIMLE client is applicable or suitable for the AI/ML task based on the energy prediction or sustainability indicator.

This clause describes the procedure for supporting the AIMLE client energy analytics in ADAES as new analytics service.

Figure 6.7.1-1: ADAES support for AIMLE client energy analytics

1.	The Consumer (AIMLE or VAL server) sends an AIMLE client energy sustainability analytics request to ADAES to perform analytics on the AIMLE client Energy Consumption who is expected to undertake an AIML task, Event ID= “AIMLE client energy sustainability analytics”, for a given service area and a given time window. This request includes a VAL service ID or AIMLE service ID for which the analytics apply, as well as the consumer identifier and the analytics KPI. Such request can include a predicted or expected route of the VAL UE hosting the AIMLE client, and the type of analytics requested. The analytics type can be requesting whether the energy consumption is sustainable for a given AIMLE service.

Such request can also include an AI/ML task or operation or process (ML training or inference or FL task) for which the prediction/stats apply.

2.	The ADAES authorizes the consumer’s request.

3.	The ADAES maps the AIMLE client energy analytics event ID to a list of data collection event identifiers, and a list of data producer IDs. Such mapping may be preconfigured by OAM or may be determined by ADAES based on the analytics event type/vertical type and/or data producer profile.

ADAES determines and maps the analytics event to the energy data to acquire and the data producers which can be functionalities in the target VAL UE(s) and/or data stored in A-ADRF.

Optionally, the ADAES (or A-ADRF) keeps per AIMLE client. This energy profile keeps stats or energy rating of the AIMLE client or the data source in the UE (e.g. AI app) which can be used as a metric to identify whether the AIMLE client at the VAL UE is energy demanding source; hence applicable or not to be considered as capable of undertaking an AI/ML task.

NOTE:	If Energy Profile is used, this information may either be provided by the VAL UE (or the VAL server) as part of the energy data or can be quantified at the ADAES.

4a.	The ADAES requests and receives from the VAL UE(s) of interest energy data using the AIMLE client Energy Data Collection API. Such Energy data collection can include measurements or stats on the energy consumption per AIMLE client.

Editor's Note:	Further details on the energy data collection from the VAL UE is FFS.

4b.	In addition, or complementary to AIMLE client energy data collection, the ADAES may also request and receive from the ADAEC of the UE, performance statistics related to the application layer or enabler layer sessions (based on existing ADAE capabilities on VAL session analytics).

5.	The ADAES calculates the expected AIMLE client energy consumption based on the received energy/ performance data in step 4, based on the request.

6.	The ADAES obtains the corresponding trained ML model based on procedure in 3GPP TS 23.482 [10] clause 8.3.2 and performs analytics to derive the predicted UE energy consumption sustainability at the target area and time horizon.

7.	The ADAES sends the analytics output via the AIMLE client energy sustainability analytics API to the consumer (VAL or AIMLE server) as analytics response with the analytics output data (based on the type of analytics in the request). The analytics output may include:

-	identification on whether the energy consumption is sustainable for a given VAL or AIMLE service, where this service can be an AI/ML service

-	a recommendation on whether the AIMLE client is applicable or suitable for the AI/ML task based on the energy prediction or sustainability indicator.

The ADAES and/or the AIMLE (as consumer) may also store the energy data and/or analytics to a repository being A-ADRF or ML repository (if this applies to an ML task).

Based on step 7, the consumer (AIMLE or VAL server) can use these analytics as input to trigger the pro-active selection or re-selection or removal of the AIMLE client from the list of UEs which are capable of undertaking an AI/ML process/task/operation, given the energy factor.

### 6.7.2	Architecture impacts

This clause will provide the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 6.7.3	Solution evaluation

This clause will provide an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 6.8	Solution #8: Network Slice Energy Optimization based on Energy Saving VAL Server Policy

### 6.8.1	Solution Description

#### 6.8.1.0	General

This solution does not add new procedure. Instead, it proposes introducing a new VAL server policy in clause 9.5, 'Network slice optimization based on VAL server policy', 3GPP TS 23.435 [13]. The new policies enable NSCE server performs slice modifications based on monitoring energy-related information, such as energy consumption and the renewable energy ratio, at the slice level.

The high-level principles are as follows:

1.	The VAL server provisions energy saving policies to the NSCE server.

-	Based on the monitored energy consumption of a network slice - categorized by energy source (renewable or non-renewable) - over a specific duration (such as one hour) or as accumulated over a longer period (for example, a calendar month), slice modification is triggered when the measured value exceeds or falls below a defined threshold. This modification may involve reducing the slice's maximum throughput, slightly increasing allowable latency, or decreasing redundancy levels (such as removing a redundant UPF instance or redundant RAN cell).

-	Based on the monitored renewable energy ratio of a network slice, a slice modification with predefined parameters is triggered when the energy ratio or crosses a specified threshold. Such modifications may include increasing the slice’s maximum throughput, raising allowable latency, expanding the maximum number of supported UEs, or increasing the maximum number of PDU sessions.

2.	The NSCE server monitors a slice's energy usage by gathering energy-related data from the slice's OAM system.

3.	Based on the defined triggers, the NSCE server carries out the actions outlined in energy-saving policies, aiming to either reduce the slice’s energy consumption or encourage the use of renewable energy sources.

The following clauses specify network slice energy optimization based on energy saving VAL server policy.

#### 6.8.1.1	Procedure

Figure 6.8.1.1-1 outlines the procedure, where Steps 1-4 represent existing operations. Step 5 introduces a new function in which the NSCE server monitors energy-related information for a network slice. Step 6 involves the NSCE server taking action to adjust the slice, either to conserve energy or to encourage the use of renewable energy sources.

Figure 6.8.1.1-1: VAL server policy provision and enforcement for energy optimization

- The consumer (VAL, SEAL Server) sends a VAL server policy provisioning request to the NSCE server by following clause 9.5 of 3GPP TS 23.435 [13]. New energy saving related policies (see examples in clause 6.8.1.2) are added:

  - Based on the monitored energy consumption of a network slice - categorized by energy source (renewable or non-renewable) - over a specific duration (such as one hour) or as accumulated over a longer period (for example, a calendar month), slice modification is triggered when the measured value exceeds or falls below a defined threshold. This modification may involve reducing the slice's maximum throughput, slightly increasing allowable latency, or decreasing redundancy levels (such as removing a redundant UPF instance or redundant RAN cell).

  - Based on the monitored renewable energy ratio of a network slice, a slice modification with predefined parameters is triggered when the energy ratio or crosses a specified threshold. Such modifications may include increasing the slice’s maximum throughput, raising allowable latency, expanding the maximum number of supported UEs, or increasing the maximum number of PDU sessions.

The action of the above policy may include charging policy update when slice SLA is modified to save energy to attract the VAL server to use the policies.

- The NSCE server performs a VAL server policy check. This is the same as step 2 in clause 9.5.2.1, 3GPP TS 23.435 [13].

- The NSCE server performs Slice Harmonization. This step is the same as clause 9.5.2.1.4 Policy harmonization, 3GPP TS 23.435 [13].

- The NSCE server sends back a VAL server policy provisioning response to the Consumer (VAL/ASP), confirming policy provision. This is the same as step 2 in clause 9.5.2, 3GPP TS 23.435 [13].

- The NSCE server initiates the retrieval of energy consumption data from the slice OAM (3GPP TS 28.554 [6]) to monitor energy-related information, including but not limited to the following parameters:

  - slice accumulated energy consumption refers to summing the average energy usage recorded during each slice OAM measurement interval (for example, every 15 minutes) by the OAM system and aggregating it into the total energy consumption over an extended period, such as a calendar month

  - slice average energy consumption: e.g. Energy-Consumption/hour

  - slice renewable energy ratio: e.g. 30%

Editor's Note:	Whether the NSCE collect the energy data directly or by using existing enabler service (e.g. A-DCCF) is FFS.

- The NSCE server carries out slice modifications in accordance with the policy action defined in step 1, through its interaction with the Slice OAM and the 5GC.

#### 6.8.1.2	Information Flows

##### 6.8.1.2.1	General

The following information flows are specified for VAL server policy.

##### 6.8.1.2.2	Policy of Reducing Energy Consumption

Table 6.8.1.2.2-1 shows the request sent by a consumer to an NSCE server for VAL server policy provisioning procedure.

Table 6.8.1.2.2-1: Policy of reducing energy consumption

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Reduce Energy Consumption |
| >Area of interest | M | The service area for which the policy profile applies, which can be expressed as a geographical area (e.g. geographical coordinates), or a topological area (e.g. a list of TA). |
| >Trigger event | M | Threshold information, i.e. slices accumulate energy consumption, slices average energy consumption reached the threshold |
| >Recommended action | M | Examples: - Modification of PDU sessions / max number of UEs (step for decrease in %). - Decrease slice capacity (step for decrease in %). - Decrease packet delay, and increase packet error rate etc. - Update the energy efficiency parameter. - Decrease GBR requirements. - Apply better charging offering for the slice after adjustment. |
| Lifetime or number of events | M | Time duration or number of times the policy can take action. |
| Priority | O | Indicates the priority of the policy. |
| Scheduling period | O | Indicates the scheduling of policy in terms of time. |
| >Start time | M | Indicates the scheduled start time. |
| >End time | M | Indicates the scheduled end time. |
| Preemption | O | Indicates the pre-empt capability of the policy. |

##### 6.8.1.2.3	Policy of Promoting renewable energy consumption

Table 6.8.1.2.3-1 shows the request sent by a consumer to an NSCE server for VAL server policy provisioning procedure.

Table 6.8.1.2.3-1: Policy of Promoting renewable energy consumption

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Promote renewable energy usage |
| >Area of interest | M | The service area for which the policy profile applies, which can be expressed as a geographical area (e.g. geographical coordinates), or a topological area (e.g. a list of TA). |
| >Trigger event | M | Threshold information, i.e. when renewable energy reached a threshold % |
| >Recommended action | M | Examples: - Modification of PDU sessions/max number of UEs (step for increase in %). - Increase slice capacity. |
| Lifetime or number of events | M | Time duration or number of times the policy can take action. |
| Priority | O | Indicates the priority of the policy. |
| Scheduling period | O | Indicates the scheduling of policy in terms of time. |
| >Start time | M | Indicates the scheduled start time. |
| >End time | M | Indicates the scheduled end time. |
| Preemption | O | Indicates the pre-empt capability of the policy. |

### 6.8.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.8.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.9	Solution #9: Enhancements to ADAE DN Energy Efficiency Analytics

### 6.9.1	Solution Description

In 3GPP TS 23.436 [11], the DN Energy Efficiency analytics, ADAES provides analytics on the energy consumption /efficiency of an edge platform (including the EESs / EASs). The DN energy analytics is performed per DNN/ DNAI and may be used to trigger the application server migration to different cloud. The analytics are based on NWDAF analytics and UPF/DN measurements on user plane load as well as edge/app side measurements on the energy consumption. Information/analytics on renewable energy used for providing application services is needed to support flexibility adjustment of the execution of workloads with GHG emissions. However, analytics relevant to renewable energy used for providing application services is missing. Enhancements to the DN Energy Efficiency analytics is required.

The DN Energy Efficiency analytics as specified in clause 8.18 of 3GPP TS 23.436 [11] is enhanced as follows (new text in bold italics):

## 8.18 Procedure for supporting DN Energy Efficiency analytics ### 8.18.1 General This clause describes the procedure for DN energy consumption/efficiency analytics, where the analytics are performed based on data collected from one or more DNs and A-ADRF. ### 8.18.2 Procedure Figure 8.18.2-1 illustrates the procedure for DN energy efficiency analytics enablement solution. Pre-conditions: 1. Data producers (e.g. A-ADRF, EAS, EES) may be pre-configured with data producer profiles (as in Table 8.2.4.8-1) for the data they can provide. ADAES has discovered available data producers and their data producer profiles. Figure 8.8.2.1-1: ADAES support for DN energy analytics 1. The VAL server sends a DN energy analytics request/subscription request to ADAES to perform analytics on the DN Energy Consumption/Efficiency for one or more DNs/EDNs, Event ID= “DN energy analytics”, “Energy analytics per application service”, or “Ratio of renewable energy analytics”, for a given DN service area (or subarea) and a given time window. The message in the request/subscription request is as described in Table 8.18.3.2-1. 2. The ADAES authorizes the VAL request. The ADAES responses to the analytics request. The message in the response is as described in Table 8.18.3.4-1. 3. The ADAES requests and receives from the EAS /VAL servers hosted at the serving and target DNs (within the VAL service area). 3a. (If Event ID= “DN energy analytics” in step 1) expected application service load and traffic schedules for the ongoing or future sessions within the area. Such data include traffic schedule report for the VAL Server, and this step re-uses the step 3 to 10 of clause 8.8.2.1. 3b. (If Event ID= “Energy analytics per application service”, or “Ratio of renewable energy analytics” in step 1) energy usage data at the DN per application, and/or in a time period, and/or the type of energy resources used. 4. The ADAES calculates the expected energy consumption or efficiency based on the received traffic and load data for the given DNN/DNAI based on the request. NOTE: How the collected data are used to calculate energy efficiency metric is up to implementation. 5. (Optional) The ADAES obtains the corresponding trained ML model based on procedure in 3GPP TS 23.482 clause 8.3.2 and performs analytics to derive the predicted energy consumption at the target area and time horizon. The analytics outputs can be the statistics or predicted energy consumption / efficiency for the given DNN/DNAI. 6. The ADAES sends a DN energy analytics response/notifications with the energy consumption/efficiency analytics output data to the VAL server. The message in the analytics response/notifications is as described in Table 8.18.3.3-1. Based on 6, the VAL server can use these analytics as input to trigger pro-actively: - an application server migration to a different edge cloud or to a centralized cloud as a way of reducing the energy consumption for the edge (if consumption is expected to be very high (e.g. higher than a pre-configured threshold)). - an application server offboarding and the instantiation of a new server at the target edge/centralized cloud to minimize energy consumption of the edge platform (taking into account the system wide energy efficiency). ### 8.18.3 Information flows #### 8.18.3.1 General The following information flows are specified for DN energy analytics based on clause 8.18.2. #### 8.18.3.2 DN energy analytics request/subscription request Table 8.18.3.2-1 describes information elements for the DN energy analytics request/subscription request from the VAL server to the ADAE server. Table 8.18.3.2-1: DN energy analytics request/subscription request Information element; Status; Description. Analytics Consumer ID; M; The identifier of the analytics consumer (VAL server, EAS).. Analytics ID; M; The identifier of the analytics event. This ID can be for example “DN energy analytics”, “Energy analytics per application service”, “Ratio of renewable energy analytics”.. Analytics type; M; The type of analytics for the event, e.g. statistics or predictions.. VAL service ID; O; The identifier of the VAL service for which the analytics is requested. This IE shall be provided when the Analytics ID is “Energy analytics per application service”.. DNN/DNAI; M; DNN or DNAIs information for which the subscription applies.. Energy Efficiency/Consumption metrics; O; The formula and necessary metrics for calculating the EE based on load and traffic information per DN.. Target data producer profile criteria; O; Characteristics of the data producers to be used.. Renewable energy type; O; Indicate the type of renewable energy for which the analytics applies (e.g., solar, wind). This IE may be provided when the Analytics ID is “Ratio of renewable energy analytics”. If omit, it indicates all types of renewable energy.. Preferred confidence level; O; The level of accuracy for the analytics service (in case of prediction).. Area of Interest; O; The geographical or service area for which the subscription request applies.. Time period of Interest; O; The time period for which the analytics applies. This IE shall be provided when the Analytics ID is “Ratio of renewable energy analytics”.. Time validity; O; The time validity of the subscription request.. Reporting requirements; O; It describes the requirements for the energy analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, the event in case of event triggered (e.g. the energy consumption/efficiency crosses a threshold, the ratio of renewable energy crosses a threshold), and reporting thresholds.. Notification endpoints ID or address; O; The identifier or address of the notification endpoints (e.g. VAL server, VAL client, or other network entities).. #### 8.18.3.3 DN energy analytics response/notification Table 8.18.3.3-1 describes information elements for the DN energy analytics response/notification request from the VAL server to the ADAE server to the VAL server (or to the notification endpoints provided in the request/subscription request). Table 8.18.3.3-1: DN energy analytics response/notification Information element; Status; Description. Result; M; The result of the analytics request (positive or negative acknowledgement).. Analytics ID; O; The identifier of the analytics event.. Analytics Output; O; The predictive or statistical parameter, which can be stats or prediction related to the energy consumption or efficiency for the edge platform for a given area/time and based on the event type.. > DNN; M; Identifies the data network name for which analytics information is provided.. > DNAI; M; Identifier of a user plane access to one or more DN(s) of the DN.. > Energy metrics; O; The predicted energy metrics.. >> Energy Consumption (NOTE); O; The predicted energy consumption per DNAI based on network and edge resource usage. >> Energy Efficiency (NOTE); O; The energy efficiency per DNAI based on network and edge resource usage (given a certain optimal energy consumption metric, which can be pre-configured).. >> DN Data Volume (NOTE); O; The predicted data volume per DNAI.. >> Energy per Application service (NOTE); O; The statistics or predicted energy consumption or energy efficiency per application service based on network and edge resource usage.. >>> VAL service ID; M; The identifier of the VAL service for which the analytics applied.. >> Ratio of renewable energy (NOTE); O; The statistics or predicted ratio of renewable energy analytics per DNAI or per application service based on network and edge resource usage.. >>> Renewable energy type; O; Indicate the type of renewable energy used (e.g., solar, wind). If omit, it indicates all types of renewable energy.. >>>> Renewable energy ratio; O; Ratio of the renewable energy.. > Area of Interest; O; The area (topological or geographical or edge area) where the analytics apply.. > Applicable time period; O; The time period that the analytics applies to.. >Confidence level; O; For predictive analytics, the achieved confidence level can be provided.. NOTE: At least one of these shall be present.. Editor’s Note: How to obtain the renewable energy information is FFS. #### 8.18.3.4 Response to DN energy analytics request Table 8.18.3.4-1 describes information elements for the ADAES responses to the analytics request to the consumer (e.g. VAL server). Table 8.18.3.4-1: Response to DN energy analytics request Information element; Status; Description. Successful response (NOTE); O (NOTE); Indicates that the request was successful.. > Subscription Identifier; O; The unique identifier for the event subscription. This IE shall be provided when it’s subscription request.. Failure response (NOTE); O (NOTE); Indicates that the request failed.. > Cause; O; Indicates the cause of request failure (e.g. the energy usage data is not available).. NOTE: One of these IEs shall be present in the message..

### 6.9.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.9.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.10	Solution #10: Support of energy saving for location services

### 6.10.1	Solution description

This solution addresses the KI#6.

The following are the procedures to introduce how LMS subscribes the energy related information analysis from ADAES (clause 6.10.1.1) and how LMS utilizes the obtained energy related information analysis to reduce the energy consumption for the location services (clause 6.10.1.2).

#### 6.10.1.1	Procedure of LMS subscription for the energy related information analysis

Figure 6.10.1.1-1 illustrates the high-level procedure of the LMS subscription for the energy related information analysis.

The LMS may subscribe to the ADAES for the analysis of energy related information. And ADAES needs to support the new energy related information (i.e. energy consumption information) analytics.

Pre-condition:

-	The A-ADRF may have stored the historical energy consumption analytics for the corresponding VAL UEs or VAL service.

Figure 6.10.1.1-1: Procedure of LMS subscription for the energy related information analysis

- The consumer (i.e. LMS) may subscribe the energy information analysis related to location services from ADAES, including the analysis ID (e.g., energy consumption information analytics), the analytics type (e.g., statistics or predictions), the service ID, the target area, the energy type (e.g., energy consumption), the energy information granularity (e.g., per VAL UE per location service), the filtering information (e.g., maximum energy consumption per VAL UE), the target UE ID, the trigger conditions, the time duration, preferred confidence level, etc.

- The ADAES check if the LMS is authorized to request the energy information analysis. If the request is authorized, the ADAES responds to the LMS.

- The ADAES sends the energy information analysis subscription response to the LMS.

- The ADAE server optionally requests the energy consumption historical analytics/information from A-ADRF for the corresponding VAL UEs or VAL service (e.g. location service).

- Based on the request, the A-ADRF checks the local storage and sends the energy consumption historical analytics/information to ADAES if there is any.

- If there is no historical energy consumption analytics/information received from A-ADRF, the ADAES may subscribe the energy related information (e.g., energy consumption information) from 3GPP Core network as described in clause 5.51.2.4 of 3GPP TS 23.501 [3], with the parameters received in service request in step 1.

- When the trigger condition is met, the 3GPP Core network provides the energy related information (e.g., energy consumption information) to the ADAES per requested granularity (e.g., per VAL UE per location service).

- Upon receiving the energy related information from step 5 and step 7, the ADAES utilizes and generates the analytics for energy related information per VAL UE per location service, including the energy consumption information statistic (e.g., maximum/average/minimum energy consumption information over a certain period of time, etc.), historical energy information, energy consumption information predication (e.g., the time point of peak energy consumption), confidence level, etc.

- The ADAES notifies the energy information analysis to the LMS as service requested, including the analysis results generated in step 8.

#### 6.10.1.2	Procedure of LMS adjusting the location reporting configuration to save the energy

Figure 6.10.1.2-1 illustrates the high-level procedure of LMS adjusting the location reporting configuration to save the energy consumption.

Figure 6.10.1.2-1: Procedure of LMS adjusting the location reporting configuration to save the energy

- Similar with step 1 of clause 9.3.5 in 3GPP TS 23.434 [14], the VAL server sends a location reporting trigger to the LMS to activate a location reporting procedure for obtaining the location information of LMC with the parameters defined in clause 9.3.2.4 of 3GPP TS 23.434 [14]. The location reporting trigger is periodical and the request also includes the energy saving indicator which means the LMS should consider saving the energy when performing the location services for the target UE.

- Upon receiving the service request, the LMS decides to subscribe the energy information analysis from the ADAES for the target UE, following the procedure as described in clause 6.10.1.1.

The ADAES replies the energy related information analysis for the target UE to the LMS, and may include the energy information statistic (e.g., maximum/average/minimum energy consumption information over a certain period of time), energy information predication (e.g., the time point of peak energy consumption), etc.

- Based on the energy related information analysis received from ADAES and the service request from VAL server, the LMS determines the UE location reporting configuration. For example, the LMS may decrease the reporting frequency when the UE’s energy consumption almost reaches its daily maximum, and vice versa. Or the LMS may pend the location reporting when the time point of peak energy consumption is coming based on the UE's energy information predication.

- The LMS may interact with the VAL server regarding the determined UE location reporting configuration and sends the adaptive location reporting configuration provisioning messages as defined in clause 9.3.20 of 3GPP TS 23.434 [14].

- If the VAL server approves the suggested location reporting configuration, the LMS sends such configurations to the LMC as defined in clause 9.3.3 of 3GPP TS 23.434 [14]. If the VAL server rejects the suggested location configuration, the LMS may discard the configuration.

- Considering the received location reporting configuration, the LMC reports the location information to the LMS dynamically when the location reporting trigger is met.

- The LMS reports the received location information for the target UE to the VAL server.

### 6.10.2	Architecture Impacts

This solution has no impact on the existing architecture.

### 6.10.3	Solution evaluation

This solution addresses the KI#6 which introduces how LMS in SEAL subscribes the energy consumption information analytics from ADAES as described in clause 6.10.1.1 and how LMS reduces the energy consumption for the location services per VAL UE to support the energy saving via utilizing the obtained energy consumption information analytics  as described in clause 6.10.1.2.

This solution has no impact on the existing architecture. The ADAES needs to support the new energy consumption information analytics requested from the consumers. The LMS needs to be enhanced to adjust the location reporting configuration for the location management client to save the energy.

## 6.11	Solution #11: Functional Architecture to Support Energy Saving

### 6.11.1	Solution Description

#### 6.11.1.0	General

The following clauses specify options on functional architecture to support application layer energy saving services.

#### 6.11.1.1	Functional Model

##### 6.11.1.1.1	General

The functional architecture enhancement for SEAL Enabler (Energy Saving) is based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [14]. It is organized into functional entities to describe an architecture enhancement which addresses the support for energy saving aspects for vertical applications.

##### 6.11.1.1.2	Network Functional Model

Figure 6.11.1.1.2-1: Network functional model

Figure 6.11.1.1.2-1 illustrates the network functional model with SEAL Enabler (Energy Saving). In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. The SEAL functional entities with energy saving enabler function on the UE and the server are grouped into SEAL client(s) (Energy Saving) and SEAL server(s) (Energy Saving) respectively. The SEAL Enabler (Energy Saving) consists of a common set of services and reference points. The SEAL Enabler (Energy Saving) offers its services to the vertical application layer (VAL), and other SEAL servers.

Editor's Note:	Whether a standalone architecture for energy saving enablement (e.g. server-side functionality on energy saving for providing common services (e.g. energy data collection and processing, energy information/status monitoring, energy saving assistance information provisioning), and client-side functionality on support energy data collection) or integrate energy saving functionality with the existing SEAL enablers (e.g. enhance EES/EAS, server/client of AIMLE, NSCE, SEALDD, ADAE, and LM to support energy saving functionality on energy data collection and processing, etc.) will be decided during solution evaluation and conclusion stage of the study phase.

Editor's Note:	Whether energy data collection via new energy saving enablement service/functionality or via existing enablement service (e.g. A-DCCF) is FFS.

#### 6.11.1.2	Functional Entities Description

##### 6.11.1.2.1	General

The SEAL Enabler (Energy Saving) functional entities with energy saving assistance and management functions are described in the following subclauses.

##### 6.11.1.2.2	SEAL client (Energy Saving)

The SEAL client (Energy Saving) interacting with the VAL client to obtain such energy data/information. It also interacts with the SEAL server (Energy Saving).

##### 6.11.1.2.3	SEAL server (Energy Saving)

The SEAL server (Energy Saving) functional entity provides for energy saving assistance and management supported within the vertical application layer.

#### 6.11.1.3	Reference Points Description

##### 6.11.1.3.1	General

The reference points for the functional model for assistance and management of energy saving are described in the following subclauses.

Editor's Note:	The description of reference points is FFS.

### 6.11.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.11.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.12	Solution #12: New Service on Support Energy Data Collection and Processing

### 6.12.1	Solution Description

#### 6.12.1.0	General

The following clauses specify the procedure and information flow of SEAL service on energy data collection for support energy saving.

#### 6.12.1.1	Procedure

Pre-condition:

- The VAL UEs, VAL server, VAL service, and consumer in the procedure belong to the same party.

Figure 6.12.1.1-1: Energy data collection and processing

- A consumer (e.g., VAL server, SEAL server/client) sends energy data collection and processing subscription request to SEAL server (Energy Saving). The request includes information as described in Table 6.12.1.2.2-1.

Editor's Note:	Whether the SEAL server (Energy Saving) is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

Editor's Note:	Whether energy data collection via new energy saving enablement service/functionality or via existing enablement service (e.g. A-DCCF) is FFS.

- The SEAL server (Energy Saving) authenticates and authorizes the request. If authorized, the SEAL server (Energy Saving) determines the data source (e.g. EIF, SEAL client (Energy Saving), and/or other network entities (e.g. OAM, 3rd party energy entity)) for collecting the energy data. It is assumed that SEAL server (Energy Saving) has a registry with the data sources from which it can collect energy data and what kind of data they can provide.

- The SEAL server (Energy Saving) sends energy data collection and processing subscription response to the consumer. The response includes information as described in Table 6.12.1.2.3-1.

4.	The SEAL server (Energy Saving) collects energy data from the data source.

4a.	The SEAL server (Energy Saving) may interact with 5GC NF for energy data collection, by using the EIF service introduced in 3GPP TS 23.501 [3].

4b.	The SEAL server (Energy Saving) may collect energy data from SEAL client(s) (Energy Saving).

Editor's Note:	Whether energy information can be collected from other network entities (e.g. OAM, 3rd party energy entity) is FFS.

Editor's Note:	Whether any issue in user privacy and consent aspects for energy data collection should be coordinated with SA3.

5.	The SEAL server (Energy Saving) aggregates the collected energy data. Aggregation includes combining or performing a statistical operation on the received data according to the data processing requirements in step 1. If necessary, steps 4-5 are repeated to complete all the required data collection.

6.	The SEAL server (Energy Saving) notifies the consumer the energy data collection and processing result. The notification includes information as described in Table 6.12.1.2.4-1.

#### 6.12.1.2	Information Flows

##### 6.12.1.2.1	General

The following information flows are specified for energy data collection and processing.

##### 6.12.1.2.2	Energy data collection and processing subscription request

Table 6.12.1.2.2-1 shows the request sent by a SEAL service consumer to a SEAL server (Energy Saving) for energy data collection and processing procedure.

Table 6.12.1.2.2-1: Energy data collection and processing subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| VAL service ID | M | The identity of the VAL service for which the request applies. |
| Target entity ID(s) | M | The identity(ies) of the target data collection entity(ies) (e.g. VAL UE ID(s), SEAL server ID(s)) |
| Data collection requirements | M | Identifies the requirement for energy data collection. |
| >Energy consumption | O | Identifies the requirement on energy consumption to be collected (e.g. energy consumption per application, energy consumption per task operation). |
| >Time of Interest | M | Indicate the interested time for the data collection. |
| Area of Interest | O | The geographical or service area for which the request applies. |
| Reporting requirements | O | It describes the requirements for reporting. This requirement may include e.g. the type of reporting (periodic or one-time report), the reporting periodicity in case of periodic. |
| Data processing requirements | M | Indicates the requirements on data processing, as defined in Table 6.12.1.2.2-2. |
| Additional info | O | Identifies addition requirement of the data to be collected (e.g. time stamp of the data, quality of the data). |

Table 6.12.1.2.2-2: Data processing requirements

| Information element | Status | Description |
| --- | --- | --- |
| Processing type | M | Identifies the processing type, e.g. aggregation, shape data format. |
| Aggregation requirements | O | Identifies the requirements for aggregation based on the energy data. |
| >Average | O (NOTE) | Indicates that calculate the average value of the energy data. |
| >Minimum | O (NOTE) | Indicates that calculate the minimum value of the energy data. |
| >Maximum | O (NOTE) | Indicates that calculate the maximum value of the energy data. |
| Granularity | M | Identifies the granularity for the processing, e.g. per UE, or per application level, or per application service level. |
| Format requirement | O | Indicates the requirement on format for the energy data. |
| NOTE: At least one of the IEs shall be provided if the Processing type is calculation. |  |  |

##### 6.12.1.2.3	Energy data collection and processing subscription response

Table 6.12.1.2.3-1 shows the response sent by the SEAL server (Energy Saving) to the consumer for energy data collection and processing procedure.

Table 6.12.1.2.3-1: Energy data collection and processing subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement). |
| Subscription ID | O | Identifier of the subscription. It shall be provided if the Result is positive acknowledgement. |

##### 6.12.1.2.4	Energy data collection and processing notify

Table 6.12.1.2.4-1 shows the notification sent by the SEAL server (Energy Saving) to the consumer for energy data collection and processing procedure.

Table 6.12.1.2.4-1: Energy data collection and processing notify

| Information element | Status | Description |
| --- | --- | --- |
| Success status | O (NOTE) | Indicates that data collection was successful. |
| >Data processed result | M | Provides outputs for data collection and processing result (e.g. aggregated result, energy data with required format), it may also include the additional information of the processed data based on request. |
| Failure response | O (NOTE) | Indicates that data collection was failed. |
| >Cause | M | Reason for the failure. |
| NOTE: One of the information elements shall be provided in the output. |  |  |

### 6.12.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.12.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.13	Solution #13: Monitoring and exposing energy consumption within the application enablement layer

### 6.13.1	Solution Description

#### 6.13.1.0	General

According to key issue #2, there is a need for a service that can monitor and expose the energy information/status (e.g. energy source type change, anomaly energy consumption, etc.). This solution provides energy saving monitoring service, that can be used by the consumer (e.g., VAL server, SEAL client (Energy Saving), SEAL server) to receive notifications regarding energy consumption information/status, which in turn can be useful for energy savings.

#### 6.13.1.1	Energy monitoring procedure

Figure 6.13.1.1-1 illustrates the procedure for a consumer (e.g., VAL server, SEAL client (Energy Saving), SEAL server) to monitor energy information/status.

Pre-condition:

-	The VAL UEs, VAL server, VAL service, and Conumser in the procedure belong to the same party.

Figure 6.13.1.1-1: Energy monitoring subscription and notification

1.	The consumer (e.g., VAL server, SEAL client (Energy Saving), other SEAL server) sends an Energy Monitoring Subscription request to the SEAL Server (Energy Saving) with an energy monitoring profile that contain energy monitoring requirements, and reporting requirements. The reporting requirements may include the frequency of reporting is periodic reporting, or triggering event (e.g. Reporting energy threshold event, Reporting energy anomaly event) for event triggered reporting.

The subscription request includes information as defined in Table 6.13.1.2.1-1.

Editor's Note:	Whether the SEAL server (Energy Saving) is a standalone energy saving enablement server or integrates energy saving functionality with an existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

2.	The SEAL server (Energy Saving) checks if the consumer is authorized for the energy monitoring subscription.

3.	The SEAL server (Energy Saving) obtains the energy monitoring requirements from the energy monitoring profile and reporting requirements provided by the consumer in the subscription request. Based on the obtained energy monitoring and reporting requirements, the SEAL server (Energy Saving) determines the data source(s) (e.g., EIF, SEAL server/client) to monitor, and applies for subscription for energy information with the selected data source(s).

Editor's Note:	Whether any issue in user privacy and consent aspects for energy data collection should be coordinated with SA3.

4.	The SEAL server (Energy Saving) sends an Energy Monitoring Subscription response to the consumer to indicate the subscription status. The subscription response includes information as defined in Table 6.13.1.2.2-1.

5	The SEAL server (Energy Saving) receives notifications about energy information from the subscribed data source(s), for example, energy consumption per UE from EIF via NEF, energy consumption at the VAL UE (via SEAL client (Energy Saving)).

6.	The SEAL server (Energy Saving) determines whether to send a notification to the consumer based on the reporting requirements in step 1. The reporting can be periodic, or event triggered (e.g. the energy consumption at the VAL UE exceeds the threshold provided in step 1, which triggers SEAL server (Energy Saving) to send notification to the consumer).

7.	The SEAL server (Energy Saving) sends an Energy Monitoring notification to the consumer. The notification includes information as defined in Table 6.13.1.2.3-1.

The consumer may adjust the network requirements (e.g., QoS requirements) to the VAL client for energy saving, based on the energy monitoring data provided by the SEAL server (Energy Saving).

#### 6.13.1.2	Energy monitoring Information elements

##### 6.13.1.2.1	Energy monitoring subscription request

Table 6.13.1.2.1-1 shows request sent by the consumer (e.g. VAL server, SEAL client (Energy Saving), other SEAL server) to the SEAL server (Energy Saving) for the energy monitoring subscription.

Table 6.13.1.2.1-1: Energy monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the consumer (e.g. VAL server, SEAL client (Energy Saving), SEAL server). |
| Target VAL UE ID(s) | M | List of VAL UE(s) for which the energy monitoring applies. |
| Target VAL service ID | O | The identifier of the VAL service for which the energy monitoring applies. |
| Target Slice ID | O | The slice ID (S-NSSAI) for which the energy monitoring applies. |
| Notification endpoint | M | Indicates the address (e.g., IP address, FQDN, URI) where the notifications should be sent. |
| Energy Monitoring event | M | Represents the energy monitoring event, and corresponding energy monitoring requirements of the consumer. |
| > Reporting energy threshold event | O (NOTE) | Indicates the reporting threshold with the corresponding threshold reaching direction (e.g., corresponding energy source type exceeds the limit, slices accumulated energy consumption excess, slice average energy consumption excess, average energy consumption (e.g., per hour) excess, maximum renewable energy excess, slice renewable energy ratio excess). |
| > Reporting energy anomaly event | O (NOTE) | Indicates the anomaly events for energy consumption or usage (e.g., spike in energy consumptions/usage, high-consumption for a longer interval, black-out). |
| Time validity | O | The time validity of the subscription request. |
| NOTE: Only one of the IEs shall be present. |  |  |

Editor's Note:	Whether Data Source, Energy Consumption Category or other IEs to be contained in the request is FFS.

##### 6.13.1.2.2	Energy monitoring subscription response

Table 6.13.1.2.2-1 shows response sent by the SEAL server (Energy Saving) to the consumer (e.g. VAL server, SEAL client (Energy Saving), other SEAL server) for the energy monitoring subscription procedure.

Table 6.13.1.2.2-1: Energy monitoring subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result. |
| Subscription ID | O | Represents the identifier of the subscription. This IE shall be present if the subscription status is success. |

##### 6.13.1.2.3	Energy monitoring notification

Table 6.13.1.2.3-1 shows the notification sent by the SEAL server (Energy Saving) to the consumer (e.g. VAL server, SEAL client (Energy Saving), other SEAL server) for the energy monitoring procedure.

Table 6.13.1.2.3-1: Energy monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Represents the identifier of the subscription. This IE shall be present if the subscription status is success. |
| Slice ID | O | The network slice whose energy monitoring data is provided. |
| Energy monitoring result | M | Energy monitoring result is generated based on aggregation of energy consumption related data collected from the data source, e.g., reporting for energy threshold event, reporting for energy anomaly event. |
| Triggering event | O (NOTE) | The event that triggered the monitoring notification. |
| NOTE: This information element applies to event triggered reporting. |  |  |

### 6.13.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.13.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.14	Solution #14: Updating QoS according to renewable energy to improve user experience in EDGEAPP

### 6.14.1	Solution Description

This paper proposes the solution to solve the problem listed in KI#3 and proposes to improve the user experience if UE or QoS flow consumes renewable energy.

#### 6.14.1.1	General

According to the section 5.51.2.2.1 of 3GPP TS 23.501 [3], the Energy Information Function (EIF) collects the UE related Energy Consumption information including Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e. S-NSSAI, PDU Session and/or Service Data Flow) from UPF (via SMF). The EIF can expose the energy consumption at the requested granularity over a certain time window (i.e. the total energy consumed from a start time to an end time) or periodically (i.e. every reporting period of e.g. 10 minutes). And NEF/AF can subscribe the energy consumption.

Energy can be classified into two major types: renewable energy and non-renewable energy. Renewable energy sources include solar energy, hydropower, wind energy, biomass energy, wave energy, tidal energy, ocean thermal energy, geothermal energy, etc. They can be recycled and regenerated in nature. It is an inexhaustible and endless energy source that regenerates automatically without human intervention.

#### 6.14.1.2	Solution description

In order to save the energy consumption and reduce the carbon emission, the operators may consider using renewable energy or green energy to replace traditional energy. For example, some servers of operators can be powered by electricity converted from renewable energy to reduce the carbon emission. Considering the renewable energy can be recycled and regenerated in nature, if the UE has renewable energy in energy consumption, from operator's policy, the operator can improve the QoS to a certain extent.

In Edge Computing deployment, as described in section 8.6.6.2.3 of 3GPP TS 23.558 [8], the EAS can trigger the session with QoS update request. And if in the notification from EIF that renewable energy is consumed by UE or QoS flow, the EAS can act as AF to trigger another session with QoS update to 5GC to improve the QoS parameters.

#### 6.14.1.3	Procedure of QoS update related to renewable energy consumption

The procedure listed here are based on the procedure defined in section 8.6.6.2.3 of 3GPP TS 23.558 [8].

##### 8.6.6.2.3 Update a session Figure 8.6.6.2.3-1 illustrates the session with QoS update request operation between the EAS and the EES. It is used to request a modification of the QoS of the data session between AC and EAS. Figure 8.6.6.2.3-1: Session with QoS API: update operation 0. The EAS act as AF to subscribe and receive the energy consumption information from 5GC as described in clause 4.29 of TS 23.502[7], or get renewable energy information from OAM as defined in TS 28.533. And if there is the existence of renewable energy in the energy consumption information of target VAL UE or QoS flow, the EAS may act as AF to improve QoS parameters to allow the UE to have higher user experience. 1. The EAS requests a modification of the QoS of the data session between the AC and the EAS, e.g. by including a different QoS reference or different bandwidth value. The EAS shall include the Context ID. The EAS may also request a modification of the event monitoring by subscribing to new events and updating or removing subscriptions to existing events. The EAS may consider updating the QoS according to the energy related information received in step 0. There are the examples below: - Increase the requested maximum bitrate or aggregate bit rates of both UL/DL to improve the data rate of target UE. - Adjust Priority to give higher priority to target VAL UE to increase the pre-emption capability. - Adjust the Requested Packet Error Rate of target VAL UE to further reduce the possibility of packet loss. 2. The EES checks if there is a context for the Context ID available and whether the EAS is authorized for this operation. If yes and the UE IP address is known by the EES, the EES invokes the Policy Authorization Update service or AF Session with QoS Update service with the 3GPP Core Network (PCF or NEF, respectively) as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], providing the updated information to the PCF or NEF, respectively. The EES may subscribe to additional events or unsubscribe to certain events, if necessary. 3. If the operation is successful, the EES responds with a Context ID and a Result. If the EAS is not authorized or any other failure happens during the operation, the EES provides a rejection response with cause information. NOTE: The EES will report the resource allocation outcome, e.g. the successful modification of the Service Data Flow(s) related to the data session, with a separate session with QoS notify operation (see 3GPP TS 23.503 [12], clause 6.1.3.18).

### 6.14.2	Architecture impacts

In this solution, it doesn’t change the overall architecture of enabling edge applications defined in section 6.2 of 3GPP TS 23.558 [8]. But the following enhancements are added:

-	The EAS can act as AF to subscribe the energy consumption information of certain UE or certain QoS from 5GC. And if renewable energy is consumed by UE or QoS flow, the EAS can act as AF to trigger another session with QoS update to 5GC to improve the QoS parameters.

### 6.14.3	Solution evaluation

Editor's note: This part is for further update.

## 6.15	Solution #15: Support energy saving in AI/ML task transfer

### 6.15.1	Solution Description

#### 6.15.1.0	General

According to the key issue #4, there is a need for AIMLE enhancements for AIML services such as AI/ML task transfer. This solution enables the AIMLE Server to select the target AI/ML member(s) for the AI/ML task transfer, based on energy consumption related information of the target AI/ML member(s) and the energy saving assistance information from the SEAL server (Energy Saving).

#### 6.15.1.1	Energy saving in AI/ML task transfer procedure

Pre-condition:

- The source and target AI/ML members belong to the same operator

Figure 6.15.1.1-1: Support energy saving in AI/ML task transfer

Figure 6.15.1.1-1 illustrates the procedure for support energy saving in AI/ML task transfer. The detailed corresponding procedure is detailed as follows:

- The source AI/ML member (e.g., an AIMLE Client) sends an AI/ML task transfer assist request to AIMLE Server for assisting with the AI/ML task transfer based on energy saving. To select a target AI/ML member(s), the source AI/ML member set energy consumption criteria for the target AI/ML member(s), such as energy consumption level, and maximum energy consumption.

2.	The AIMLE Server discovers other AI/ML members (e.g., other AIMLE Clients) directly or via ML repository, and selects one or more target AI/ML member(s) based on the energy criteria and other requirements from the source AI/ML member.

3.	The AIMLE Server interacts with the SEAL server (Energy Saving) for the energy saving assistance information for the list of target AI/ML member(s) that is selected in step 2. The SEAL server (Energy Saving) generates energy saving assistance information for the list of target AI/ML member(s) and provides it to the AIMLE Server.

4.	Based on step 2 in clause 8.6.2.1 of 3GPP TS 23.482 [10] and the information collected from the SEAL server (Energy Saving), the AIMLE Server selects the AI/ML member with low energy consumption as the target AI/ML member. The AIMLE Server sends request to the selected target AI/ML member(s) for the AI/ML task transfer. Then, the AIMLE Server determines the transfer mode, i.e., transfer with server-controlled or transfer directly from the source AI/ML member to the target AI/ML member. For example, if the task is model training, the AIMLE Server determines the target AI/ML member(s) based on AI/ML member(s)’s capability in the AIMLE client profile, task's AI/ML model training requirement, and the energy information.

If target AI/ML member(s) is selected, the AIMLE Server calculate the total energy consumption for the AI/ML task transfer, and generates assistance information for the AI/ML task transfer from the source AI/ML member to the target AI/ML member(s).

5.	The AIMLE Server sends an AI/ML task transfer assist response to the source AI/ML member with the information on the target AI/ML member(s), the assistance information, and the energy consumption generated in step 4.

Editor's Note:	Whether the SEAL server (Energy Saving) is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

#### 6.15.1.2	Enhance the information flow for AI/ML task transfer procedure

The information flow for AI/ML task transfer as specified in clause 8.6.3 of 3GPP TS 23.482 [10] is enhanced as follows (new text in bold italics):

#### 8.6.3.2 AI/ML task transfer assist request Table 8.6.3.2-1 describes information elements for the AI/ML task transfer assist request from the source AI/ML member to the AIMLE Server. Table 8.6.3.2-1: AI/ML task transfer assist request Information element; Status; Description. Requestor identity; M; The identifier of the source AI/ML member (e.g. identifier of the source AIMLE client or the VAL UE).. VAL service ID; O; The identifier of the VAL service for which the assistance information is requested.. AI/ML Task Type; M; The type of the AI/ML operation (e.g. ML model training).. AI/ML Information Type; M; The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results).. > AI/ML model remaining training requirement (NOTE); O; Indicate the requirement for AI/ML model training including, required remaining training resource, required remaining training number of iterations,. > AI/ML intermediate information (NOTE); O; Indicate the AI/ML intermediate information for intermediate AI/ML operation and result (e.g. ML model training), including AI/ML intermediate model, AI/ML intermediate model used training time, used training resource, used training number of iterations.. Target AI/ML member energy criterion; M; Specifies the energy consumption criteria (e.g. energy consumption level, max energy consumption) of the target AI/ML member.. Time validity; O; The time validity of the request.. NOTE: The IE only present when AI/ML Task Type is ML model training.. #### 8.6.3.3 AI/ML task transfer assist response Table 8.6.3.3-1 describes information elements for the AI/ML task transfer assist response from the AIMLE Server to the source AI/ML member. Table 8.6.3.3-1: AI/ML task transfer assist response Information element; Status; Description. Transfer Mode; O; Indication the transfer mode (e.g. direct transfer).. Target AI/ML member(s); M; The identifier of the target AI/ML member (e.g. identifier of the target AIMLE client or the VAL UE).. Assistance Information; M; Assistance information for the AI/ML task transfer (e.g. time window for the transfer)..

### 6.15.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.15.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.16	Solution #16: Support Energy-aware split operation update

### 6.16.1	Solution Description

This solution addresses Key Issue #4.

Stage 1 has introduced the notion of ‘energy credit’ as per 3GPP TR 22.882 and 3GPP TS 22.261 [2], which is a quantity of credit associated with the subscriber (end user, ASP) that can be used for credit control by the 5G system. Energy credit in the 3GPP context refers to a policy control mechanism within mobile networks to manage and limit a subscriber's energy consumption, allowing operators to apply policies like "energy credit limits" to services. Energy credit can be associated to the ability for the network operator to create a 'maximum energy credit' policy, after which services are gated. Such energy credit or budget may be for a particular application service (end to end) or VAL UE.

A Split AI/ML operation pipeline is a workflow for ML model inference in which AI/ML endpoints are organized and collaborate to process ML models in sequential stages, where processing at each stage involves ML model inference on the output of the previous stage. The AIMLE capability for supporting a split AI/ML operation update has been specified in 3GPP TS 23.482 [10] clause 8.14.2.6.

In this solution, the reason for split AI/ML operation pipeline update is the energy criteria/conditions, and particularly the energy credit or budget per VAL UE (or per application or aggregately for the entire pipeline) which is reaching a threshold. Hence, the AI/ML client detects that due to high energy consumption, it needs to modify the pipeline to offload the ongoing split ML operation to a target AI/ML member which can be an edge AIMLE server or another AIMLE client.

Figure 6.16.1-1 illustrates the procedure for an AIMLE client to update an instance of a split operation pipeline at the AIMLE server. The AIMLE client or the VAL client determines that some of the nodes in the split operation pipeline may not be able to perform the required operation (due to VAL UE energy credit/budget starvation/limitation) and decides to modify the pipeline either to add or remove the nodes.

Pre-conditions:

- The AIMLE client has received information related to split operation pipeline profile as specified in clause 8.14.2.3.

- VAL client is aware of the energy credit/budget for the VAL UE.

- AIMLE server has provided in advance to AIMLE clients configuration for reporting trigger events related to energy status changes.

Figure 6.16.1-1: Split operation pipeline update due to energy criteria

1.	AIMLE client detects an energy trigger event based on step 1 and optionally monitoring the energy status at the VAL UE. This can be done by monitoring the energy data/analytics (e.g. as in Solution 7) per VAL client / AIMLE client, and application traffic schedule or pattern from the VAL application.

Such detection can be also triggered by VAL client providing in advance a VAL UE energy credit/budget report to AIMLE client (via AIMLE-C API) to indicate the current status of the energy credit (remaining) and/or the maximum credit limit for the UE / application.

2.	The AIMLE client sends an energy-aware split pipeline update trigger message to the AIMLE server. The trigger includes a status update and in particular:

-	An energy credit limit/budget per VAL UE is expected to reach, or

-	high predicted or actual or expected energy usage or credit/budget for VAL UE/application, or

-	high Energy Consumption indicator (based on a pre-defined threshold.

3.	The AIMLE server requests ESE Server to provide energy information (e.g. consumption/usage status) for the list of alternative AIMLE servers/clients in this service area,

4.	The AIMLE server receives the energy status (or credit/budget status) information for the list of alternative AIMLE clients and servers.

In this step, the AIMLE server can alternatively fetch the energy credit information from a charging function in the operator’s charging domain.

5.	Then, the AIMLE server identifies the split operation pipeline(s) for which the VAL UE is applicable and determines whether the split operation pipeline needs to be updated based on the split operation pipeline identifier and the energy status trigger.

If the split operation pipeline profile is determined, the AIMLE server updates the split operation pipeline profile and notifies the appropriate processing nodes indicated in the request about their inclusion or exclusion in the split operation pipeline as described in clause 8.14.2.5 of 3GPP TS 23.482 [10]. The AIMLE server also notifies the involved nodes about modification of the existing pipeline as described in clause 8.14.2.5 of 3GPP TS 23.482 [10].

Then, the AIMLE server determines to check alternative entities (AIMLE clients or AIMLE servers) to undertake the task and discovers the entities (AIMLE clients or another AIMLE server using existing discovery capability as described in of 3GPP TS 23.482 [10].

Editor's Note: It is FFS how the split operation update procedure needs to be enhanced if the alternative entity is an AIMLE client.

At the discovery of alternative entities, the energy capability (energy credit limit, energy usage information) of the list of alternative candidate AIMLE clients or AIMLE servers is also fetched by the ML repository.

6.	For the list of AIMLE servers (deployed at the edge DN), AIMLE server may also consume the DN energy analytics service from ADAES (using the corresponding analytics service as in 3GPP TS 23.436 [11]). Additionally, for the AIMLE clients the AIMLE server may also collect analytics based on Solution 7.

7.	AIMLE server evaluates (by rating or ranking) the alternative candidate entities (AIMLE clients or AIMLE servers) for undertaking the task using the performance for the split operation as well as the energy sustainability factor (if ADAE analytics from Solution 7 are used).

AIMLE server determines one or more entities to be selected or considered as alternative to the AIMLE client of interest for becoming part of the updated split operation pipeline.

8.	The AIMLE server sends a split operation pipeline update notification message to the AIMLE client. If the AIMLE server has updated an instance of a split operation pipeline profile, the notification includes the corresponding split operation profile.

The notification also includes the list of the alternative entities and their energy rating / credit (as part of the split operation profile or as separate IEs including the entity ID/address (VAL UE ID or AIMLE client ID or AIMLE server ID) the rating/ranking information per entity, the time and area for validity for the availability of the candidate entities and optionally whether this is based on prediction/analytics.

9.	AIMLE server sends to the ML repository the updated list of AIMLE clients and servers which will be part of the updated pipeline and may also provide the energy status/credit to ML repository to store this information for future updates.

Editor's Note: Whether the ESE (aka SEAL server (Energy Saving) in other solutions) is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

## 6.17	Solution #17: Support Energy-aware FL member availability update

### 6.17.1	Solution Description

This solution addresses Key Issue #4.

Stage 1 has introduced the notion of ‘energy credit’ as per 3GPP TR 22.882 and 3GPP TS 22.261 [2], which is a quantity of credit associated with the subscriber (end user, ASP) that can be used for credit control by the 5G system. Energy credit in the 3GPP context refers to a policy control mechanism within mobile networks to manage and limit a subscriber's energy consumption, allowing operators to apply policies like "energy credit limits" to services. Energy credit can be associated to the ability for the network operator to create a 'maximum energy credit' policy, after which services are gated. Such energy credit or budget may be for a particular application service (end to end) or VAL UE.

This solution describes the event notification procedure for the FL member availability, since the FL member may be a VAL user who has a maximum energy credit for supporting FL operations. The notification can be triggered based on a change at the availability or capabilities of the candidate FL members. The enhancement in this procedure (shown in bold italics) is the addition of the energy credit/budget status event notification to the subscribers. This will allow the pro-active modification of the FL member in case the credit is low, or the consumption / usage of energy is high at the AIMLE or VAL server or EAS acting as candidate FL member.

Pre-conditions:

1.	The subscription procedure in clause 8.5.2 of 3GPP TS 23.482 [10] is performed by the AIMLE server or VAL server (via AIMLE server).

2.	The FL member has registered its capabilities (including energy credit / budget status) or availability to the ML repository based on the support for FL registration capability.

Figure 8.5.3-1: Procedure for FL-related event notifications

1.	The ML repository detects a change related to the availability of an FL member and generates events to be consumed by the AIMLE server, based on the notification, or based on a received updated registration from FL member, who has already registered at the ML repository.

The energy aware trigger event can be based on the registration update request message for the respective FL member(s) or by monitoring the energy status at the AIMLE or VAL / EAS server side. As part of the detection, the ML repository may request an energy monitoring function (e.g. EIF or other energy monitoring function at OAM or SEAL ESE server) to provide the energy credit status and limit / remaining value for the respective candidate FL member.

2.	For the generated event, the ML repository retrieves the list of corresponding subscriptions.

3.	The ML repository sends FL-related event notification to all entities (e.g., AIMLE servers) that have subscribed for the FL-related event matching the criteria. If a notification reception information is available as part of the FL-related event subscription, then the notification reception information is used by the ML repository to send event notifications to the corresponding AIMLE servers.

In case on energy-related event, the information about the event related to the FL member energy status change can include:

- the FL member credit status (remaining energy credit) or energy usage information (e.g. percentage or qualitative value e.g. high, medium) for the respective EAS or VAL server or AIMLE server acting as FL member.

- the actual or expected absolute energy consumption or efficiency metric (bits / Joule) if the credit is not provided.

NOTE: Any user privacy/consent aspects related to fetching energy consumption information from the AIMLE client side will be handled by SA3.

4.	The notified subscriber (s) sends an event notification acknowledgement to the ML repository.

The definition of the FL related events is provided in Table 8.5.4-1 of 3GPP TS 23.482 [10] (new event in bold italics).

Table 8.5.4-1: Definition of FL related events

| Events | Events Description |
| --- | --- |
| Availability changes of FL member | The event type relates to the availability change of a FL member. Such availability change can be an indication of availability or unavailability for a given service area, or for a given ML model ID/profile. |
| FL model information | Information such as accuracy, time schedule and latency for the FL training. Latency is considering the target latency, i.e., when the FL model training shall be completed. |
| FL member load information | The event type relates to the monitoring of the computational load for the requested FL member. Such load can be the estimated (based on measurements) or expected/predicted based on the tasks that the FL member undertakes. This event may be triggered on-demand or can be monitored periodically. |
| FL member energy status information. | Information on the energy credit or budget or usage for the FL member. Such energy information can be the estimated (based on energy credit monitoring) or expected based on the tasks that the FL member undertakes. This event may be triggered on-demand or can be monitored periodically. |

Editor's Note: Whether the Energy Saving Enabler (aka SEAL server (Energy Saving) in other solutions) is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

## 6.18	Solution #18: Enhance Network Slice Adaptation Considering Energy Information for VAL Application

### 6.18.1	Solution Description

#### 6.18.1.1	General

This solution proposes to enhance the NSCE service on VAL server-triggered and network-based network slice adaptation for VAL application (in clause 9.11 of 3GPP TS 23.435 [13]). The enhancement enables NSCE to guide UEs to use different slices to improve energy efficiency and promote the use of renewable energy.

The high-level principles are as follows:

1.	VAL server/VAL requests NSCE server to perform network-based network slice adaptation for VAL application with information considering network slice energy related information.

2.	NSCE service retrieves slices’ status data including energy related information of a slice from 5GC/OAM.

3.	Based on energy related information, the NSCE server performs network slice modification considering energy-related information to improve energy efficiency or promote the use of renewable energy.

#### 6.18.1.2	Procedure

The procedure on VAL server-triggered and network-based network slice adaptation for VAL application in clause 9.11.2 of 3GPP TS 23.435 [13] is enhanced as follows (new text in bold italics):

### 9.11.2 Procedures #### 9.11.2.1 Procedure for VAL server-triggered and network-based network slice adaptation for VAL application - request and response model Figure 9.11.2.1-1 illustrates the VAL server-triggered and network-based procedure where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application. Figure 9.11.2.1-1: Network slice adaptation for VAL application 1. The VAL server sends a network slice adaptation request to the NSCE server for the VAL application (and the VAL UEs within the VAL application). This request may be in the form of exact requested network slice (and optionally DNN) for all the VAL UEs of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN). The request optionally includes the adaptation threshold of network slice adaptation as defined in Table 9.11.3.1-1. For example: - slice average energy consumption: EnC per reported period - slice energy efficiency KPI: DL DV/EnC, UL DV/EnC - slice renewable energy ratio or renewable energy is used or not - renewable energy schedule Editor’s Note: Whether renewable energy related information in bullet 3 and 4 is available from OAM needs to be coordinated with SA5. 2. [Optional] NSCE server collects the network slice status information, including network slice performance measurements in clause 5.1.1.1, clause 5.1.1.2, clause 5.1.1.3, in 3GPP TS 28.552 [19] and key performance indicators in clause 6.3, energy performance in clause 6.7 (e.g. slice energy consumption, slice energy efficiency), in 3GPP TS 28.554 [20], and network slice related E2E latency analytics report in clause 8.4.2.4.3, in 3GPP TS 28.104 [21] from network slice management functions by utilizing MnS of create MOI operation defined in clause 11.1 and MnS of streaming data reporting service or file data reporting service defined in clause 11.5 and 11.6, 3GPP TS 28.532 [7], and renewable energy related information. 3. The NSCE server processes the request and triggers the network slice configuration per VAL UE within the VAL Application. If network slice status from step 2 is considered, the NSCE server analyses the network slice status information before triggering the network slice configuration. If the threshold is crossed for the current network slice of adaptation and the objective network slice satisfy the requests, NSCE server triggers the network slice configuration per VAL UE within the VAL Application. For example, the slice energy consumption for the current network slice of adaptation crossed the enegy threshold or the renewable energy used for the current newtork slice of adaptation is lower than a energy threshold, the NSCE server triggers network slice modification considering energy-related information to improve energy efficiency or promote the use of renewable energy. 4. The NSCE server acting as AF provides the updated S-NSSAI and DNN per VAL UE. In particular, NSCE server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in 3GPP TS 23.502 [12] clause 4.15.6.10, 3GPP TS 23.503 [17] clause 6.6.2.2, 3GPP TS 23.548 [18] clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic. The guidance may update RSD priority for the slice according to energy threshold info, i.e. the NSCE sets the higher priority of the RSD for the slice which has higher slice energy efficiency KPI or lower priority for the slice which has higher average energy consumption. NOTE: NSCE server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping 5. Upon successful adaptation of the route selection parameters, the NSCE server provides a network slice adaptation response to the VAL server, providing information on the fulfilment of the network slice adaptation request per VAL application.

#### 6.18.1.3	Information flows

The information flows on network slice adaptation request in clause 9.11.3 of 3GPP TS 23.435 [13] is enhanced as follows (new text in bold italics):

#### 9.11.3.1 Network slice adaptation request Table 9.11.3.1-1 describes the information flow network slice adaptation request from the VAL server to the NSCE server. Table 9.11.3.1-1: Network slice adaptation request Information element; Status; Description. VAL service ID; M; The VAL service ID of the VAL application for which the network slice adaptation may corresponds to.. List of VAL UE IDs; M; List of the VAL UE IDs within the VAL service for which the slice adaptation request corresponds. Requested Network slice related identifier(s); O; Identifier of network slice for which the VAL server requests to use for adaptation. Requested monitored Network slice related identifier(s); O (see NOTE 1); Identifier of the provisioned network slice(s) which are provisioned for the listed UE(s) and requested to be monitored by NSCE server. Requested DNN; O; Indication of the new DNN which is requested.. Requested Adaptation threshold; O (see NOTE 2); The threshold of network slice adaptation. >Requested adaptation threshold of the delay of network slice; O; The network slice delay defined clause 5.1.1.1, 5.1.1.2, 5.1.1.3, in 3GPP TS 28.552 [19] and key performance indicators in clause 6.3 and clause 6.7 (e.g. slice energy consumption in clause 6.7.6 and slice energy efficiency in clause 6.7.3), in TS 28.554 [20], and network slice related analytics report in clause 8.4.2.4.3, in 3GPP TS 28.104 [21], and renewable energy related information.. NOTE 1: If this IE is not present then the NSCE server monitors all the slices provisioned for the listed UE(s) mentioned in this request. If this IE is present, the NSCE server monitors the network slice(s) only indicated by the identifier(s). NOTE 2: The NSCE is requested to adapt UE to the requested network slice only when the status of the provisioned network slice for the listed UE(s) crosses the requested threshold..

### 6.18.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.18.3	Solution evaluation

Editor's Note:	The solution evaluation of the solution is FFS.

## 6.19	Solution #19: Enhance Multiple Slices Coordinated Optimization Considering Energy Information

### 6.19.1	Solution Description

#### 6.19.1.1	General

This solution proposes enhancing multiple slices coordinated resource optimization (clause 9.10 of 3GPP TS 23.435 [13]) to consider slice energy related information, enable slice modification between multiple network slices for energy saving and promotion of the use of renewable energy.

The high-level principles are as follows:

1.	VAL server requests NSCE server to perform Multiple slices coordinated resource optimization providing the optimization target in the request.

2.	NSCE service retrieves slices’ status data including energy related information of a slice from 5GC/OAM.

3.	Based on energy related information, the NSCE server performs network slice modification considering energy related information or/and resource usage related information according to the optimization target.

#### 6.19.1.2	Procedures

The procedure and information flows on multiple slices coordinated resource optimization in clause 9.10 of 3GPP TS 23.435 [13] is enhanced as follows (new text in bold italics):

### 9.10.2 Procedure #### 9.10.2.1 Procedure on multiple slices coordinated resource optimization Figure 9.10.2.1-1 illustrates the procedure of multiple slices coordinated resource optimization service from VAL server to NSCE server. Pre-conditions: 1. The network slice enabler layer is capable to interact with PLMN 5GC or OAM system to handle slices of PLMN and its PNI-NPNs. 2. PNI-NPNs are deployed as network slices of the PLMN. Figure 9.10.2.1-1: Multiple slices coordinated resource optimization process 1. The VAL server initiates multiple slices coordinated resource optimization request towards the NSCE server. The request includes VAL server ID, VAL service ID. The message also includes preferred optimization zone, Optimization Target (see Table 9.10.3.2-1). 2. Upon receiving the request from the VAL server to make the network slice resource optimization, the NSCE server makes authentication and authorization of the VAL server and if VAL server is not authorized to send multiple slices coordinated resource optimization request, the NSCE server replies with failure response. 3. The NSCE server makes mapping from VAL service ID that received from VAL server to slice identities (S-NSSAIs allocated in multiple networks) and retrieves the network slice related status information from 5GC or OAM of PLMN for PNI-NPN slices, such as NF(s) load in 5GC and network utilization in access network, and slice energy related information, such as slice energy consumption and slice energy efficiency, as defined in TS 28.535 [11]. The NSCE server may retrieve from 5GC or OAM renewable energy usage information for network slice, such as renewable energy ratio or renewable energy is used or not. The PLMN operator can choose to deploy NSCE server acting as single entry of PLMN capability exposure which can be optional. Editor Note: Whether renewable energy related information is available from OAM needs to be coordinated with SA5. 4. The NSCE server verifies and analyses status data including slice energy related information of network slices instance as well as network slice performance and energy monitoring response (optional), then NSCE server makes resource and energy optimization (e.g. shared radio resources, etc.) decision among different kinds of slices in specific location zone. 5-6. The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi request as specified in TS 28.531 [8] to PNI-NPN slice. Based on decision made by NSCE server, the network slice management entity (such as NSMF) performs the corresponding operation(s) and sends LCM response including slice resource adjustment result according to the optimization target. - If the Optimization Target is energy, according to the energy related information such as slice average energy consumption goes above a threshold, the NSCE requests slice modification triggering slice scaling in or reduce the slice capacity etc. 7-8. The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi request as specified in TS 28.531 [8] to PLMN slice. Based on decision made by NSCE server, the network slice management entity (such as NSMF) performs the corresponding operation(s) and sends LCM response including slice resource adjustment result according to the optimization target similar to step 5-6. The PLMN operator can choose to deploy NSCE-S acting as single entry of PLMN capability exposure which can be optional. 9. The NSCE server sends the multiple slices coordinated resource optimization response towards VAL server. ### 9.10.3 Information flows #### 9.10.3.1 General The following information flows are specified: - Multiple slices coordinated resource optimization request and response. #### 9.10.3.2 Multiple slices coordinated resource optimization request Table 9.10.3.2-1 describes information elements for the multiple slices coordinated resource optimization request from the VAL server to the NSCE server. Table 9.10.3.2-1: Multiple slices coordinated resource optimization request Information element; Status; Description. Requestor Identifier; M; Unique identifier of the requestor (i.e. VAL server ID).. Security credentials; M; Security credentials resulting from a successful authorization.. VAL service identity; M; Identifier of the VAL service application to be monitored.. Optimization Zone; O; The preferred location area of the performance monitoring and optimization.. Optimization Target; O; The target for the slices’ optimization: - energy: optimize the energy KPIs, e.g. reduce energy consumption, increase energy efficiency and promote the use of renewable energy. - improve energy efficiency - reduce energy consumption - promotion of the use of renewable energy. Requested S-NSSAI(s); O; Indication of the S-NSSAI(s) which are requested.. #### 9.10.3.3 Multiple slices coordinated resource optimization response Table 9.10.3.3-1 describes the information elements for the multiple slices coordinated resource optimization response from the NSCE server to the VAL server. Table 9.10.3.3-1: Multiple slices coordinated resource optimization response Information element; Status; Description. Result; M; Indicates the success or failure of the multiple slices resource optimization request.. > VAL service identity; O (see NOTE 1); Identifier of the VAL service application to be monitored.. > Cause; O (see NOTE 2); Indicates the cause of multiple slices resource optimization request failure.. NOTE 1: Shall be present if the result is success and shall not be present otherwise. NOTE 2: Shall be present if the result is failure and shall not be present otherwise..

### 6.19.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.19.3	Solution evaluation

Editor's Note:	The solution evaluation of the solution is FFS.

## 6.20	Solution #20: LM Service Enhancements to Location Reporting for Support Energy Saving

### 6.20.1	Solution Description

#### 6.20.1.1	General

Definition of energy saving mode:

-	Economy mode: an energy saving mode when the service provider (e.g., SEAL Server, etc.) minimizes energy consumption prioritized over performance.

-	Balanced mode: an energy saving mode when the service provider (e.g., SEAL Server, etc.) balances the energy consumption and performance.

-	Performance mode: an energy saving mode when the service provider (e.g., SEAL Server, etc.) provides high performance without energy saving constrains.

The services procedures and information flows introduced in 3GPP TS 23.434 [14] for location reporting, can be enhanced to support the energy saving requirements. For the enhancements, the energy saving modes for LM services:

-	Economy mode indicates that the consumer (LMC, VAL server) requests to minimize the energy consumption at UE side for location reporting.

-	Balanced mode indicates that the consumer (LMC, VAL server) requests to balance the energy consumption at UE side and satisfy the location reporting requirements.

-	Performance mode indicates that consumer (LMC, VAL server) requests to provide location reporting from UE side without energy saving constrains.

#### 6.20.1.2	Enhance location reporting procedures

The procedure for Location reporting as specified in clause 9.3.3.3 of 3GPP TS 23.434 [14] is enhanced as follows (new text in bold italics):

#### 9.3.3.3 Location reporting Figure 9.3.3.3-1 illustrates the procedure for location reporting. Figure 9.3.3.3-1: Location reporting procedure 1. The location management client sends a location information report to the location management server, containing location information identified by the location management server and available to the location management client. The location management client may include low battery indication if experience the low battery charge. 2. Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client. If low battery indication is received, the location management server may initiate the location reporting adaptation if the adaptive reporting is requested in step 1 of clause 9.3.5.

The procedure for Location reporting as specified in clause 9.3.5 of 3GPP TS 23.434 [14] is enhanced as follows (new text in bold italics):

### 9.3.5 Client-triggered or VAL server-triggered location reporting procedure Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting. Figure 9.3.5-1: Client-triggered location reporting procedure 1. Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1. The location reporting event triggers as specified in table 9.3.2.4-1, e.g. minimum time between consecutive reports, SAI changes, access RAT changes, or ECGI changes for reporting the location of the VAL UE, are included. NOTE: Step 1 can be performed when location management client 2 or VAL server require to update the location reporting trigger corresponding to location management client 1. 2. Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. If the immediate report indicator is included or the triggering criteria is periodical reporting in step 1 and there is valid location report of the target UE (i.e. the timer for the stored location report is not expired), the location management server reuses the stored UE location report and sends the report to the location management client 2 or VAL server. a. The location management server determines the location information source (i.e. location management client 1 and/or 3GPP CN (e.g. NEF service for location, access type and roaming status monitoring as specified in 3GPP TS 23.502 [11])), to satisfy VAL server request in step 1 (e.g. if energy saving mode is set to "Economy mode", the location management server may select 3GPP CN as location information source to save energy at location management client). b. If the adaptive reporting is requested in step 1, the location management server interacts with NWDAF (e.g., UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [34]) to obtain the location management client 1 moving statistic and prediction i. If "DIRECT UPDATE" is requested, the location management server determines the reporting configuration based on VAL service ID, VAL server ID, Energy Saving mode, and the retrieved location management client 1 moving statistic and prediction. In order to adapt the configuration, the location management server may determine, for example, whether to increase or decrease the frequency of the report based on the UE's mobility pattern, or update location change condition (e.g., to change the location accuracy). ii. If "SUGGESTIVE UPDATE" is requested, the location management server initially uses the configuration provided by VAL server and later dynamically adjusts the reporting configuration to determine adaptive location configuration based on VAL service ID, VAL server ID, Energy Saving mode, and by analysing the retrieved location management client 1 moving statistic and prediction and received location information. In order to adapt the configuration, the location management server may determine, for example, whether to increase or decrease the frequency of the report based on the UE's mobility pattern, or update the location change condition (e.g., to change the location accuracy). c. If the valid location report is not available and 3GPP CN was selected as a source of location data in step 2.a, the location management server requests 3GPP CN to monitor the location management client 1 location change. d. If the valid location report is not available and location management client was selected as a source of location data in step 2.a, depending on the information specified by the location reporting trigger or determined reporting configuration (if "DIRECT UPDATE" is requested), location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1. 3. Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server. Further, if adaptive reporting is enabled by the VAL server with "SUGGESTIVE UPDATE", the location management server dynamically adjusts the reporting configuration as specified above (in step 2.a.ii). Once location configuration is adjusted and if it is required to update the triggers to the SEAL client, the SEAL LMS may suggest the adaptive location configuration to the VAL server (or authorized SEAL location management client) as specified in clause 9.3.19. If VAL server accepts the suggested adaptive location configuration, then the location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1. If VAL server rejects the suggested adaptive location configuration, then the location management server will discard the updated configuration.

#### 6.20.1.3	Enhancements to information flows

The information flow for Location reporting trigger as specified in clause 9.3.2.4 of 3GPP TS 23.434 [14] is enhanced as follows (new text in bold italics):

#### 9.3.2.4 Location reporting trigger Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure. Table 9.3.2.4-1: Location reporting trigger Information element; Status; Description. Requestor Identity; M (see NOTE 1); Identity of the requesting authorized VAL user or VAL UE or VAL server. Identity; M (see NOTE 1); Identity of the requested VAL user or VAL UE. VAL service ID; O; Identity of the VAL service for which the location reporting trigger is set.. Immediate Report Indicator; O (see NOTE 2); Indicates whether an immediate location report is required. Requested location information; O (see NOTE 2); Identifies what location information is requested. This information element may be represented as VAL service area ID(s) when this information flow from VAL server to the location management server.. Triggering criteria; O (see NOTE 2); Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE as specified in table 9.3.2.4-2.. Minimum time between consecutive reports; O (see NOTE 2); Defaults to 0 if absent otherwise indicates the interval time between consecutive reports. Endpoint information; O; Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required.. Adaptive reporting; O; It indicates the request for an adaptive location reporting by dynamically adjusting the configuration. Following are possible values: - "DIRECT UPDATE", indicates the SEAL LMS to directly update the SEAL LM client once configuration is adjusted; or - "SUGGESTIVE UPDATE", indicates that the VAL Server subscribes to receive the suggested configurations from the SEAL LMS. The related notifications shall be sent by SEAL LMS before updating to the SEAL LM client.. Energy Saving mode; O; It indicates the Energy Saving mode (i.e. Economy mode, Balanced mode, or Performance mode) for the SEAL LM client reporting and corresponding thresholds. - Economy mode indicates that the VAL server requests to constrain the energy consumption at UE side for location reporting providing best efforts to satisfy provided location reporting requirements. - Balanced mode indicates that the VAL server requests the best effort for energy saving at UE side to satisfy the location reporting requirements. - Performance mode indicates that VAL server requests to provide location reporting from UE side without energy saving constrains. The default value is Performance mode if omitted.. NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present.. Editor’s Note: How to use the threshold for energy saving mode is FFS. Table 9.3.2.4-2: Location reporting Triggering criteria Information element; Status; Description. Frequency of reporting; O; It indicates the requested frequency of reporting. The reporting frequency may be periodic or event-triggered.. Reporting periodicity; O; If the Frequency of reporting is periodic, the reporting periodicity shall be provided.. Location change condition; O (see NOTE); It indicates the location change condition when the reporting shall occur, e.g., VAL user or VAL UE changes a cell, NodeB, TA, RA, WLAN access network, civic address, GPS coordinates, SAI, ECGI, RAT, VAL service area. If the Frequency of reporting is event-triggered, this IE shall be provided.. Inside or outside indication; O (see NOTE); It indicates the condition when the reporting shall occur, i.e., the VAL user or VAL UE is inside or outside the locations represented by the VAL service area ID(s).. Reporting schedule; O; It indicates the requested reporting schedule, e.g., days of the week and/or time period for the location reporting.. NOTE: Triggering criteria based on the VAL service area represented by the VAL service area ID(s) may be provided when this information flow is from the VAL server to the location management server..

The information flow for Location information report as specified in clause 9.3.2.2 of 3GPP TS 23.434 [14] is enhanced as follows (new text in bold italics):

#### 9.3.2.2 Location information report Table 9.3.2.2-1 describes the information flow from the location management client to the location management server for the location information reporting or from the location management server to the requesting location management client or VAL server to report location information. Table 9.3.2.2-1: Location information report Information element; Status; Description. Set of identities; M; Set of identities of the reporting VAL users or VAL UEs. Triggering event; M; Identity of the event that triggered the sending of the report. The triggering event may be e.g., SAI changes, ECGI changes, RAT changes.. Location Information; M; Location information. For LM-UU reference point, it may include UE location information via non-3GPP positioning technologies (e.g. GNSS, Sensor, TBS, WLAN, Bluetooth) as described in 3GPP TS 37.355 [53].. Velocity information; O (NOTE 1); Velocity information of the reporting VAL users or VAL UEs.. Failure Reason; O (NOTE 2); Indicates the failure reason when the LM Client cannot obtain the requested UE’s location information (e.g. the target VAL UE has moved away, or required positioning method is not supported by target VAL UE). Low battery indication; O; Indicates that battery is low.. Timestamp; O; Timestamp of the location report. NOTE 1: The definition of the velocity refers to 3GPP TS 23.271 [60]. NOTE 2: This element is only applicable when the Location information IE is null..

### 6.20.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.20.3	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 6.21	Solution #21: Optimized Location Reporting based on Energy Saving

### 6.21.1	Solution Description

#### 6.21.1.1	General

The existing location reporting optimization for different devices sharing the same location is not based on the energy saving criteria. Consider the example scenario where a user with different devices sharing the same location needs to report the location. In this case, a device can be selected as the location reporting device based on the energy information of the different devices and their location range proximity. This will help to save energy saving in location reporting. This solution provides dynamic role switching for the location reporter using the information collected from the SEAL Server (Energy Saving). This enables the location management server to reselect the device for the reporter role based on the information from the energy saving enabler server.

#### 6.21.1.2	Procedure for optimized location reporting based on energy saving

Pre-conditions:

1.	The LMS has the information that all UEs share the same location.

2.	The UEs can be aware of each other's location.

Figure 6.21.1.2-1: Optimized location reporting based on energy saving

1.	The Location Management Server (LMS) sends track energy information subscription request to SEAL Server (Energy Saving) for energy information of the VAL UEs.2.	The SEAL Server (Energy Saving) collects energy information (e.g. energy consumption for location reporting over period of time) for the VAL UEs (via SEAL client (Energy Saving)) based on the request from the LMS.

NOTE:	The procedures for energy information collection need to take user consent into account.

Editor's Note:	How SEAL Server (Energy Saving) collects the energy consumption information for location reporting from the VAL UE is FFS.

3.	The SEAL Server (Energy Saving) sends the collected energy information of each VAL UE to the LMS.

4.	Based on the energy information of VAL UE collected from the SEAL Server (Energy Saving), the LMS select VAL UE with less energy consumption for location reporting as a location reporter.

5.	LMS informs the selected VAL UE to report on the location for the other VAL UEs in the same location.

6.	The location reporter VAL UE informs other VAL UEs in the same location via application layer or PC5 communication links, that it enables other VAL UEs to stop reporting location to save energy.

7.	The location reporter VAL UE starts to report on location for the VAL UEs in the same location, the VAL UE list which shares the same location may be included in the report.

8.	The SEAL Server (Energy Saving) sends a new notification for the collected energy information of each VAL UE to the LMS.

9.	LMS detects the change at the location reporter VAL UE for e.g., energy change or location change and re-selects the location reporter VAL UE for location reporting based on energy information and the location information of the VAL UEs.

10.	LMS server informs the new location reporter VAL UE to report on the location of other VAL UEs in the same location, and terminates the previous location reporter VAL UE to stop reporting location.

Editor's Note:	Whether the SEAL server (Energy Saving) is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

Editor's Note:	The evaluation of energy consumed by the energy collection and the final energy being saved is FFS.

#### 6.21.1.2	Information Flows

##### 6.21.1.2.1	Inform Location reporter VAL UE

Table 6.21.1.2.1-1: Inform Location Reporter VAL UE

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | Indicates the requestor’s identity (i.e., identifier of LMS). |
| VAL service ID | O | Identity of the VAL service for which the location service is registered. |
| Identity of VAL UE(s) | M | Indicates the identity of other VAL UE(s) whose location is reported by the reporter UE. |
| Location reporter | M | Indicates the selection of VAL UE to be the location reporter. |

##### 6.21.1.2.2	Terminate Location reporter VAL UE

Table 6.21.1.2.2-1: Terminate Location reporter VAL UE

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | Indicates the requestor’s identity (i.e., identifier of LMS). |
| VAL service ID | O | Identity of the VAL service for which the location service is registered. |
| Stop location report | M | Indicates to stop the location reporting and change in the location reporter role. |
| >Reason | O | Indicates the reason for stopping location reporting (e.g., energy change, location change). |

### 6.21.2	Architecture impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 6.21.3	Solution evaluation

Editor's Note:	The solution evaluation of the solution is FFS.

# 7	Overall Evaluation

## 7.1	Overall Evaluation of Key issue #1: Enhance Application Enablement Layer Architecture and Services to Support Energy Saving

This clause will provide an overall evaluation of the key issue #1.

## 7.2	Overall Evaluation of Key issue #2: Key Issue on monitoring and exposure of energy consumption

This clause will provide an overall evaluation of the key issue #2.

## 7.3	Overall Evaluation of Key issue #3: EDGE application enablement enhancement for energy efficiency

This clause will provide an overall evaluation of the key issue #3.

## 7.4	Overall Evaluation of Key issue #4: Support AIMLE enhancements for AIML services energy saving

This clause will provide an overall evaluation of the key issue #4.

## 7.5	Overall Evaluation of Key issue #5: NSCE enhancement for energy efficiency

This clause will provide an overall evaluation of the key issue #5.

## 7.6	Overall Evaluation of Key issue #6: Application Enablement Layer Services for Energy Saving

This clause will provide an overall evaluation of the key issue #6.

# 8	Conclusions

## 8.1	General conclusions

This clause will provide general conclusions for the study.

## 8.2	Conclusions of key issue #x

This clause will provide conclusions for the specific key issue.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA6#68 |  |  |  |  | TR skeleton as approved by SA6 in S6-253267. | 0.0.0 |
| 2025-08 | SA6#68 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-253641, S6-253642, S6-253698, S6-253715, S6-253716, S6-253753, S6-253754, S6-253791. | 0.1.0 |
| 2025-10 | SA6#69 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-254687, S6-254689, S6-254706, S6-254754, S6-254771, S6-254772, S6-254774, S6-254775, S6-254776, S6-254791. | 0.2.0 |
| 2025-11 | SA6#70 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-255027, S6-255084, S6-255503, S6-255506, S6-255507, S6-255511, S6-255512, S6-255513, S6-255205, S6-255609, S6-255610, S6-255628, S6-255629, S6-255630, S6-255631, S6-255632, S6-255633, S6-255640. | 0.3.0 |
| 2025-12 | SA#110 | SP-251463 |  |  |  | Presentation for information at SA#110 | 1.0.0 |