TSG SA Meeting #109	SP-251211

16 – 19 September, 2025, Beijing, China

# Source:	SA WG2

# Title:	New WID on Deferred AM Policies in the 5GC

# Document for:	Approval

# Agenda Item:	6.4.2

3GPP TSG SA2 Meeting #170	S2-2508046

# Goteborg, Sweden, August 25-29, 2025 				         was S2-2507961

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Deferred AM Policies in the 5GC

Acronym:	TEI20_DAMP

Unique identifier: 1090006

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The access and mobility related policy control encompasses the management of service area restrictions, the management of the RFSP Index, the management of the UE-AMBR, the management of the UE Slice-MBR, the slice replacement management and the management of SMF selection.

UE policy encompasses the management of ANDSP, URSP, V2XP, ProSeP, RSLPP, A2XP (see 23.503 clause 6.1.2.2). In the context of this document, UE policies refer to URSP only.

The following two policy mechanisms exist(ed) in EPC.

-	Policy rule activation/deactivation in a deferred mode – The ability of the policy server to ask the enforcement to enforce policy decisions (session rules or/and pcc rules) in a future time and/or stop enforcing policy decisions at a future time; and the corresponding ability of the enforcement to follow through with that.

These two policy mechanisms were carried over as is to 5GC Session management related policy control (Npcf_SMPolicyControl).

Some scenarios however require the use of these mechanisms also for AM or/and UE policy (Npcf_AMPolicyControl or/and Npcf_UEPolicyControl). UE policy (specifically URSP) already has a “built-in” mechanism of activation/deactivation via the time window of the RSD.

Consequently, we need to:

-	Add the mechanism of policy rule activation/deactivation in a deferred mode to AM policy.

The following scenarios have been identified:

Scenario 1: Promotions – An operator can offer a promotion on higher UE Slice-MBR, UE AMBR, different service area restrictions, different RFSP, URSP rule with a dedicated slice/DNN combination etc. Promotions are usually associated with time interval. For example, a promotion may be active for 24 hours or a promotion may be active for the rest of the billing cycle etc. These promotions use rule activation time (i.e. rules becomes active upon start of a promotion), rule deactivation time (i.e. rules stops being active upon end of a promotion).

Scenario 2: Time of Day policy actions. For example, during busy hours an operator may want to lower UE Slice-MBR or change the RFSP, while during off peak hours the operator may want a higher UE Slice-MBR/RFSP.

Scenario 3: Planned events - Same as Scenario 2, but wrt/ planned events such as a game or a concert. This may also involve pushing event specific AM policies, e.g., instructing the UE/RAN/AMF about slice related restrictions, allowed TAIs etc.

# 4	Objective

- WT-1: The objective is to enhance the applicability of ‘PCC rule activation/deactivation in a deferred mode’ mechanism to AM policy.

The scope of this Work Item in the context of AM policies applies to either non-roaming or roaming. Only impacted NFs are PCF for a UE and AMF.

## TU estimates and dependencies

| Work Task ID | TU Estimate | TU Estimate | RAN Dependency | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
|  | (Study) | (Normative) | (Yes/No/Maybe) |  |
| WT-1 |  | 0.5 | No |  |

The expected time for completing this work is 1 TUs. 0.25 time slots in one SA2 meeting to define the functional description and 0.25 time slots in next SA2 meeting to define the procedures and services.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.503 | Define the PCF/AMF impacts. 6.1.2.X(new) - Update Access and mobility policy control such that they will include activation/deactivation of policies in a deferred mode. 6.5 - Add attributes related to activation/deactivation of policies in a deferred mode to Access and mobility related policy information table. | TSG#111 (Mar 2026) |  |
| TS 23.502 | 4.15.6.9.2, 4.15.6.9.3 - Add the ability of AF/NEF to influence AM policy at a deferred time 5.2.5.8.2 – Add deferred time parameter 5.2.6.23.2 - Add deferred time parameter | TSG#111 (Mar 2026) |  |

# 6	Work item Rapporteur(s)

Baniel, Uri, Oracle, uri dot baniel at oracle dot com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None.

# 9	Supporting Individual Members

Supporting IM name

Oracle

Verizon UK Ltd

China Telecom

SK Telecom

LG Electronics

Nokia

Huawei

HiSilicon

AT&T

Ericsson