3GPP TSG SA WG 1 Meeting #112 	S1-254074

17-21 November 2025, Dallas, Texas, USA	(revision of S1-25xxxx)

Source:	China Mobile

pCR Title:	Pseudo-CR on Consolidation of Data-Related Potential Requirements in Y.1.10 Integrated Sensing and Communication (ISAC)

Draft Spec:	3GPP TR 22.870 V0.4.1

Agenda item:	8.1.10

Document for:	Approval

Contact:	Haoyue Zheng, zhenghaoyue@chinamobile.com

Pengtai Qin, qinpengtai@chinamobile.com

Abstract: This document focuses on the consolidation of data-related PRs in Y.1.10 Integrated Sensing and Communication (ISAC).

1. Introduction

It is proposed to update Clause Y.1.10	Integrated Sensing and Communication (ISAC).

This contribution focuses on the consolidation of data-related Potential Requirements (PRs) for Integrated Sensing and Communication (ISAC) in 3GPP TR 22.870. Recognizing that Sensing Data is fundamental to ISAC, as sensing results and services are derived from the processing and analysis of sensing measurement data, this document systematically categorizes and consolidates the relevant PRs.

The consolidation is performed from a comprehensive data lifecycle perspective, addressing the following key aspects of Sensing Data handling within the 6G system:

- Sensing Data Collection: the aggregation of data from base stations (3GPP sources) and third-party non-3GPP sources.

- Sensing Data Processing: the computation and analysis required to generate valuable sensing results.

- Sensing Data Storage: the storage of sensing data for future use and service provision.

- Sensing Data Exposure: the secure and authorized access of sensing results to UEs and third-party applications.

- Sensing Data Security and Privacy Protection: Ensuring the security and privacy of sensing data and targets throughout the sensing service.

By organizing the PRs into this structured framework, this contribution aims to provide a clear and consolidated view of the data-related PRs for ISAC, thereby facilitating the ongoing standardization work for ISAC in 3GPP SA1.

2. Reason for Change

This contribution proposes the following changes:

1.Consolidate the data-related PRs in Y.1.10 Integrated Sensing and Communication (ISAC).

3. Conclusions

With these changes, progress can be made for Clause Y.1.10.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

Editor’s Note:  PRs with associated Editor’s Note(s) will not be considered for consolidation until Editor’s Note(s) associated with the PR are resolved.

Editor’s Note: CPR Table(s) can be modified (e.g. split, merged, renamed, moved) during the consolidation process.

Y.1	Consolidated potential Functional Requirements

Y.1.10	Integrated Sensing and Communication (ISAC)

Editor’s Note: This includes (but not limited to) PRs from clause 7.

Editor’s Note: PR 7.17.6-1, PR 7.17.6-2, PR 7.17.6-3, PR 7.17.6-4, and PR 7.25.6-1 have Editor’s Notes associated with them, so they are not included for consolidation into Table Y.1.10-1 at this time.

Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Table Y.1.10-1 – ISAC

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.10-1-1 | Subject to operator policy, regulation, and user consent, the 6G network shall enable the base station to send sensing measurement data to the core network, and enable the core network to - aggregate, collect, process, and store sensing measurement data from base stations, - collect non-3GPP sensing data from third party, and - collect the sensing results with a specified level of accuracy. | PR 7.5.6-1 PR 7.16.6-1 PR 7.2.6-1 | Sensing Ops Measure, Aggregate, Collect, Process, Store Sensing Data Collection of 3GPP and non-3GPP Sensing Data Collection Accuracy Table Y.1.5-1 – Data Collection and Consumption? |
| Y.1.10-1-2 | Subject to operator policies, the 6G Network shall provide mechanisms to - provide a means to enable efficient use of sensing resources for stable sensing operation, - provide sensing service to detect, classify and count one or more sensing targets (e.g. detect UAVs, distinguish UAVs from birds, identify specific UAV characteristics.). - configuring a sensing operation with a single or multiple sensing modes from all sensing modes supported (e.g. bistatic, monostatic, multistatic). NOTE: Classification refers to the identification of specific characteristics of the detected target objects and grouping together of the detected target objects with similar characteristics. For example, once a UAV is detected, the sensing service can be used to determine the type of UAV it is (e.g. quadcopter or hexacopter) and then group together UAVs with similar characteristics in each class. NOTE: Mechanisms for sensing operation configuration are in scope of ongoing 5GA work, without explicit 5GA service requirements. | PR 7.8.6-2 PR 7.14.6-1 PR 7.23.6-1 | (Stable) Sensing Ops Sensing Modes Multiple targets |
| Y.1.10-1-3 | The 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity. NOTE: The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11]. | PR 7.5.6-3 | Sensing Service Capacity Multiple targets |
| Y.1.10-1-4 | Subject to regulation and operator's policy, 6G system shall efficiently support sensing target density requested by the third party for a given sensing service. | PR 7.7.6-1 | Target Density |
| Y.1.10-1-5 | Subject to operator’s policy and regulation, the 6G network shall be able to provide a sensing service to derive predicted location and/or velocity of sensing target(s). | PR 7.10.6-1 | Prediction |
| Y.1.10-1-6 | Subject to regulatory requirements, operator’s policy, and user consent, 6G network shall support the use of stored sensing data to provide a sensing service and ensure that only authorised entities are able to access the stored sensing data and results. | PR 7.11.6-1 PR 7.12.6-1 | Sensing Data Storage Usage & Security |
| Y.1.10-1-7 | The 6G system shall be able to prioritize communication, sensing and positioning together used in Network Assisted Smart Transportation. | PR 7.19.6-2 | Prioritization |
| Y.1.10-1-8 | Subject to operator’s policy, the 6G system shall provide exposure mechanism(s) to support - the exposure of sensing results in a synchronised manner with other types of traffic (e.g. audio, video, haptics) to the sensing service consumer, - activate and deactivate exposing sensing results to a UE (AMR) that are used for prediction in a given sensing area of interest at a particular time of interest to nearby UEs at the request of a trusted third party - a latency upper-bound requested by trusted third party, when providing sensing results to nearby UEs (e.g. AMRs) about environment situation. NOTE 1: This requirement is intended to describe multiple UEs (AMRs) collaborating to provide useful information for each other in areas shared by human and AMRs. For example, a UE (AMR) will have an ample time to stop or slow down, using the information on presence of multiple human workers in a few seconds. NOTE 2: The latency depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. | PR 7.8.6-1 PR 7.14.6-2 PR 7.8.6-3 | Exposure Mechanism Enhancement Third party support Delivery synchronization latency upper-bound |
| Y.1.10-1-9 | Subject to operator’s policy, regulation and user consent, the 6G network shall - atuhorise and expose sensing results to UE or an application on the UE for a specific service and - provide sensing results to a UE for a specific service, where the UE is authorized by mobile network operator providing sensing service. NOTE: As an example, UE could use the provided sensing results (e.g. environment characteristics around UE) to optimize communication service. | PR 7.20.6-1 PR 7.21.6-1 PR 7.5.6-4 | Sensing Results Exposure to UE |
| Y.1.10-1-10 | Subject to operator’s policy and regulation, the 6G system shall be able to link sensing results with communication service area for communication service. | PR 7.20.6-2 | ISAC Linking |
| Y.1.10-1-11 | Subject to regulatory requirements and user consent, the 6G network shall be able to use the 6G sensing service to monitor and recognize gestures of a worker such as hand gestures, body and limb positions, head positions, and facial expressions of a worker. NOTE: The sensing resolution is proportional to the accuracy of recognition. No angular resolution for the sensor is given, since it depends on the distance from the subject. A combination of different sensors could be implemented to meet these requirements. | PR 7.24.6-1 | Gesture Recognition |
| Y.1.10-1-12 | Subject to regulatory requirements and user consent, the 6G network shall be able to use the 6G sensing service to monitor and recognize gestures of a worker such as hand gestures, body and limb positions, head positions, and facial expressions of a worker. NOTE: The sensing resolution is proportional to the accuracy of recognition. No angular resolution for the sensor is given, since it depends on the distance from the subject. A combination of different sensors could be implemented to meet these requirements. The 6G system shall be able to support the KPIs from Table 7.3.6-1: KPIs for safety assistance for vulnerable pedestrians | PR 7.3.6-1 | KPI Safety Assistance Vulnerable Peds |
| Y.1.10-1-13 | The 6G system shall be able to support the KPIs. From Table 7.4.6-1: KPIs for high-resolution topographical maps | PR 7.4.6-1 | KPI Mapping |
| Y.1.10-41-1 | The 6G system shall be able to provide sensing with KPIs [9] [11] from Table 7.5.6-1: Performance requirements of sensing results for low-altitude UAV supervision | PR 7.5.6-5 | KPI Low Alt UAV supervision |
| Y.1.10-1-15 | The 6G system shall be able to provide sensing service by detecting and tracking the characteristics of individual environmental objects as described in the Table 7.6.6-1: KPIs for environment object reconstruction | PR 7.6.6-1 | KPI Environmental object reconstruction |
| Y.1.10-1-16 | The 6G system shall be able to support the KPIs from Table 7.7.6-1: Performance requirements for road digitalization | PR 7.7.6-2 | KPI Road digitalization |
| Y.1.10-1-17 | The 6G system shall be able to provide sensing service with larger coverage as described in Table 7.9.6-1: Performance requirements for 6G ship detection and tracking | PR 7.9.6-1 | KPI Large coverage (ship detection/tracking) |
| Y.1.10-1-18 | The 6G system shall support the KPIs in Table 7.13.6-1: Performance requirements of sensing results for enhanced XR navigation | PR 7.13.6-2 Should be -1 | KPI Enhanced XR nav |
| Y.1.10-1-19 | The 6G system shall be able to support the KPIs from Table 7.14.6-1: Proposed KPIs for collaborative robots using digital twinning | PR 7.14.6-3 | KPI Cobots using DT |
| Y.1.10-1-20 | The 6G system needs to be capable of sensing the infrastructure collapse with KPIs from Table 7.15.6-1: Performance requirements of sensing results for infrastructure collapse monitoring | PR 7.15.6-1 | KPI Infrastructure monitoring |
| Y.1.10-1-21 | The 6G system shall be able to provide sensing service by enabling multi-sensor fusion based sensing with KPIs from Table 7.16.6-1: Performance requirements for Multi-Sensor Fusion based sensing for UAV takeoff | PR 7.16.6-2 | KPI Multi-sensor fusion |
| Y.1.10-1-22 | The 6G system shall support the KPIs in Table 7.18.6-1: Performance requirements for Safe and Economic UAV transport | PR 7.18.6-1 | KPI UAV Transport |
| Y.1.10-1-23 | The 6G system shall be able to offer this Network assisted Smart Transportation service by ensuring the KPIs for communication, location and sensing according to Table 7.19.6-1: Performance requirements are fulfilled simultaneously. | PR 7.19.6-1 | KPI Simultaneous performance metrics |
| Y.1.10-1-24 | The 6G system shall be able to support the KPIs in Table 7.22.6-1: Performance requirements of sensing results for structural health monitoring | PR 7.22.6-1 | KPI Structural Monitoring |
| Y.1.10-1-25 | Table 7.23.6-1: Performance requirements for UAV Detection, Classification and Counting | Clause 7.23 (no PR#) | KPI |
| Y.1.10-1-26 | The 6G System shall support the sensing KPIs for gesture recognition in industrial environments from Table 7.24.6-1: KPIs for gesture recognition in industrial environments [94] | PR 7.24.6-2 | KPI Gesture Recognition |
| Y.1.10-1-27 | Subject to user consent, regulation, and operator’s policy, the 6G network shall be able to provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party, while maintaining the privacy of the sensing target(s). | PR 7.10.6-1 PR 7.10.6-2 | Sensing Data Security and Privacy Protection For Sensing Data, Prediction, Results and Targets |
| Y.1.10-1-28 | The 6G system should support energy-efficient sensing operations. | PR 7.5.6-2 | Energy-efficient Sensing Operations |

* * * End of Change * * * *