3GPP TSG-SA WG1 Meeting #112 	S1-254085

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254020)

Title:		Proposed editorial and rephrasing to rapporteurs’ proposals for Y.1.1-1-4 to Y.1.1-1-6

Agenda Item:	8.1.10 - Consolidation of service and performance requirements

Source:	Charter communications, Lenovo, CableLabs

Contact:	inma.carrion@charter.com

Abstract: typo in Y.1.1-1-4 and improved wording for Y.1.1-1-6 to be inline with the text in 5.3 first paragraph. Improved wording reflects the fact that the requirements is for non-3GPP access.

* * * First Change * * * *

## Y.1	Consolidated potential Functional Requirements

### Y.1.1	Basic Services and Capabilities

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to):	Support for Legacy (i.e., 5G and earlier) services and capabilities, Enhancements to Legacy services and capabilities, Legacy services and capabilities not supported in 6G, and device support. This includes (but not limited to) PRs from clauses 5.2, 5.3, 5.4, 5.7, 5.9, and 5.10.

Table Y.1.1-1 – Support for legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-1-1 | The 6G system shall support new 6G services and capabilities and existing 5G services and system requirements, unless explicitly identified. This includes requirements in [14] clause 5.1.2.2, related to inter-RAT capabilities not to be supported by 5GS, apply similarly to 6GS with the following modification: - voice service continuity from the radio access network of the 6G system to UTRAN CS shall not be supported. | Clause 5.1 Clause 5.2 PR 5.5.10.2-1 PR 5.7.4.2-1 PR 5.7.6.2-1 PR 5.9.1.3-1 | The following four PRs “should be” implicitly included in Y.1.1-1 and not needed to be identified separately. |
|  | The 6G system shall have a means to ensure that the visited network provides services to an inbound roamer only once the subscriber identity (e.g. SUPI, IMSI) is obtained from the home network. | PR 5.5.10.2-1 | Roaming |
|  | The 6G system shall support network sharing, based on 5GS network sharing requirements. | PR 5.7.4.2-1 | Network Sharing |
|  | The 6G system shall support Unified Access Control requirements as specified in [14]. | PR 5.7.6.2-1 | Unified Access Control |
|  | Subject to operators' policy, the 6G system shall provide a mechanism for the support of Femtocell deployment. | PR 5.9.1.3-1 | Femtocells |
| Y.1.1-1-2 | The 6G system shall be able to support the services listed and those identified as not needing to support (i.e., CS Fallback, CS based voice call) in Clause 5.4.2. | Clause 5.4.2 |  |
| Y.1.1-1-3 | The 6G system shall be able to support 5G system requirements (functional and performance requirements) defined by the specifications listed in Clause 5.4.3 for 3GPP and non-3GPP accesses. These requirements do not apply to requirements involving the legacy systems/RATs (e.g. E-UTRAN, UTRAN, GERAN). | Clause 5.4.3 Clause 5.3 |  |
| Y.1.1-1-4 | Subject to operator’s policy, the 6G system shall support mobility procedures - between the core network (CN) of the 6G system and a 5G core network, and - between the core network of the 6G System and the evolved Packet Core Network (EPC), and - between the 6G 3GPP access and non-3GPP access with minimum impact to the user experience (e.g. Quality of Service (QoS), Quality of Experience (QoE)). NOTE 1: Complexity on introducing the above interworking requirement needs to be minimized. | Clause 5.2 Clause 5.3 |  |
| Y.1.1-1-5 | The 6G system shall be able to provide 3GPP services to a UE using various 3GPP and non-3GPP access networks (e.g. satellite, WLAN or Wireline). | Clause 5.3 Clause 5.4.1 |  |
| Y.1.1-1-6 | The 6G system shall be able to select the most appropriate 3GPP or non-3GPP access technology for a service and support multiple access technologies (i.e. 3GPP and non-3GPP access) to be used simultaneously for one or more services active on a UE. | Clause 5.3 |  |

Table Y.1.1-2 – Enhancements to legacy services and capabilities

* * * End of change * * * *