3GPP TSG-SA WG1 Meeting #112                           S1-254096

17-21 November 2025, Dallas, USA                (revision of S1-25xxxx)

Title:	CPR proposal for basic services and capabilities Y.1.1

Agenda Item:	8.1.10

Source:	ZTE

Contact:	XuLing xu.ling@zte.com.cn

Abstract: it proposes the basic services and capabilities related CPRs for Clause Y.1.1 based on the Rapporteurs draft (S1-254020).

Y.1.1	Basic Services and Capabilities

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to):	Support for Legacy (i.e., 5G and earlier) services and capabilities, Enhancements to Legacy services and capabilities, Legacy services and capabilities not supported in 6G, and device support. This includes (but not limited to) PRs from clauses 5.2, 5.3, 5.4, 5.7, 5.9, and 5.10.

Table Y.1.1-1 – Support for legacy services and capabilities

NOTE:	Terms referring to 5G (e.g. “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g. “6G system”, “radio access network of the 6G system”) in requirements in Table Y.1.1-1.

New editor’s note: Clause 5.1 and 5.4.1 are introduction or description parts. It is suggested not to include in the CPR table.

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-1-1 | The 6G system shall support requirements in [14] clause 5.1.2.2, related to inter-RAT capabilities not to be supported by 5GS, apply similarly to 6GS with the following modification: - voice service continuity from the radio access network of the 6G system to UTRAN CS shall not be supported. | Clause 5.2 not supported IWK | The following four PRs “should be” implicitly included in Y.1.1-1 and not needed to be identified separately. [ZTE] only include 5.2 not supported PR |
|  | The 6G system shall have a means to ensure that the visited network provides services to an inbound roamer only once the subscriber identity (e.g. SUPI, IMSI) is obtained from the home network. | PR 5.5.10.2-1 | Roaming security [ZTE]Move to table Y.1.2 |
|  | The 6G system shall support network sharing, based on 5GS network sharing requirements. | PR 5.7.4.2-1 | Network Sharing [ZTE] Merged into Y.1.1-1-3(clause 5.4.3 legacy requirements) |
|  | The 6G system shall support Unified Access Control requirements as specified in [14]. | PR 5.7.6.2-1 | Unified Access Control [ZTE]Merged into Y.1.1-1-3(clause 5.4.3 legacy requirements) |
|  | Subject to operators' policy, the 6G system shall provide a mechanism for the support of Femtocell deployment. | PR 5.9.1.3-1 | Femtocells [ZTE]Merged into Y.1.1-1-3(clause 5.4.3 legacy requirements) |
| Y.1.1-1-2 | The 6G system doesn’t need to support CS related telephony services, (i.e., CS Fallback, CS based voice call) | Clause 5.4.2 not supported services |  |
| Y.1.1-1-3 | The 6G system shall be able to support 5G system requirements (functional and performance requirements) defined e.g. in TS 22.261 [14], TS 22.104 [64], TS 22.011 [60], TS 22.173 [59], TS 22.071 [61], TS 22.262 [57], TS 22.185 [65], TS 22.186 [66], TS 22.125 [35], TS 22.263 [67], TS 22.115 [68], TS 22.101 [58], TS 22.153 [63], TS 22.289 [69], TS 22.468 [70], TS 22.368 [71], TS 22.156 [28], TS 22.137 [6]. for 3GPP and non-3GPP accesses. NOTE: These requirements do not apply to requirements involving the legacy systems/RATs (e.g. E-UTRAN, UTRAN, GERAN). | Clause 5.4.3 Clause 5.3 PR 5.7.1.2-1 PR 5.7.1.2-2 PR 5.7.4.2-1 PR 5.7.6.2-1 PR 5.9.1.3-1 | [ZTE] non-3GPP access PRs in clause 5.3 and FWA general PRs in clause 5.7.1 could be merged into Y.1.1-1-3 |
|  | The 6G system shall be able to support a user to access network services via 3GPP and/or non-3GPP access (e.g. WLAN or Wireline). | Clause 5.3 | [ZTE] It could be merged into Y.1.1-1-3 |
| Y.1.1-1-4 | Subject to operator’s policy, the 6G system shall support mobility procedures - between the core network (CN) of the 6G system and a 5G core network, and - between the core network of the 6G System and the evolved Packet Core Network (EPC), - between the 6G 3GPP access and non-3GPP access with minimum impact to the user experience (e.g. Quality of Service (QoS), Quality of Experience (QoE)). NOTE 1: Complexity on introducing the above interworking requirement needs to be minimized. | Clause 5.2 Clause 5.3 |  |
|  | The 6G system shall be able to provide 3GPP services to a UE using various 3GPP and non-3GPP access networks (e.g. satellite, WLAN or Wireline). | Clause 5.3 Clause 5.4.1 | [ZTE] non-3GPP access PRs and FWA general PRs could be merged into Y.1.1-1-3 |
|  | The 6G system shall support multiple access technologies to be used simultaneously for one or more services active on the UE. | Clause 5.3 | [ZTE] The original sentence is not PR |
| Y.1.1-1-5 | The 6G system shall be able to support the following services: - Mission Critical Services, i.e. MCPTT, MCData, MCVideo, ref TS 22.179 [53], TS 22.280 [54], TS 22.281 [55]. and TS 22.282 [56], - Message service, ref TS 22.262 [57], - Short Message Service (SMS), ref TS 22.101 [58], - Multimedia communication services, ref TS 22.101 [58], TS 22.261 [14], TS 22.173 [59], - IMS Multimedia Telephony Service, ref TS 22.261 [14], TS 22.173 [59], - Roaming services, ref TS 22.011 [60], TS 22.101 [58], TS 22.261 [14], - Location and positioning services, ref TS 22.261 [14], TS 22.071 [61], - Broadcast and Multicast Services, ref TS 22.261 [14], - Emergency Services, ref TS 22.101 [58], - Public Warning System (PWS), ref TS 22.268 [62], - Multimedia Priority Service (MPS), ref TS 22.153 [63], - Lawful Interception, ref TS 22.261 [14] and TS 33.126 [312], and - Other regulatory services, based on regional/national regulatory requirements. | 5.4.2 supported legacy services | [ZTE] The supported services in Clause5.4.2 need to be included. |

Table Y.1.1-2 – Enhancements to legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-2-1 | The 6G network shall provide a mechanism to support event triggered network sharing (e.g. disaster occurrence, network failure, overloaded situation, resource constraints). | PR 5.7.4.2-2 | Network Sharing |
| Y.1.1-2-2 | Subject to regulatory requirements or operator policy, the 6G network shall support sharing of radio access network with sensing capability among operators. | PR-5.7.10.6-1 | Network Sharing |
| Y.1.1-2-3 | The 6G system shall support suitable access categories to manage the access attempt for the new services (such as sensing, AI application, computing) in congestion scenarios. | PR 5.7.6.2-2 | Unified Access Control |
|  | The 6G system shall provide optimized network capabilities for FWA (e.g. support stationary devices) in relevant bands taking into consideration the regulatory requirements for each specific band. | PR 5.7.1.2-1 PR 5.7.1.2-2 | Fixed Wireless Access [ZTE] merged into Y.1.1-1-3 |
| Y.1.1-2-4 | Based on operator policy, the 6G system shall support means to aware of user service characteristics (e.g. data rate, latency,predicted changes for each traffic flow component of its service/application to the 6G network.) and dynimically adjust and optimize network resource . | PR 5.7.1.2-3 PR-5.9.8.2-1 PR-5.9.8.2-2 | Service awareness including Fixed Wireless Access |
| Y.1.1-2-5 | The 6G system shall enhance the Short Message Service to enable a network operator to verify the identity of the SMS sender and information concerning operator verified SMS sender information to the recipient of a SMS. NOTE 1: Operator-verified SMS sender information is used to inform the recipient of a SMS that the identity of the SMS sender is operator-verified and support displaying additional information (e.g. brand name, logo, etc.) of the SMS sender. Human interface aspects are out of scope of this requirement. NOTE 2: Indication that the identity of the SMS sender is operator-verified, any additional information about the SMS sender and the message itself is assumed to be integrity protected. NOTE 3: Based on interworking agreements and trust relationships, the requirements above apply also when the SMS recipient is roaming or receives a SMS from a sender served by other operators. NOTE 4: The requirements above apply to A2P SMS and may apply to Person-to-Person SMS. | PR 5.7.3.2-1 PR 5.7.3.2-2 | SMS |
| Y.1.1-2-6 | Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. | PR 5.9.6.6-1 | 6G LAN [ZTE] It is suggested to set up a new group for 6G LAN, refer S1-25XXXX |
| Y.1.1-2-7 | Subject to operator policies, and agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to - authorize PLMN’s users to access a subscribed service provided by an authorized 3rd party via a local area network (deployed by the PLMN operator), and - minimize service interruption when the serving network changes between the local area network and the PLMN network. | PR 5.9.6.6-2 | 6G LAN [ZTE] It is suggested to set up a new group for 6G LAN, refer S1-25XXXX |
| Y.1.1-2-8 | Subject to operator’s policy and regulation, for an operator with multiple 6G core networks, when there is UE mobility from one 6G core network to another 6G core network of the same PLMN, the 6G network shall support efficient traffic routing for the traffic from UE to data network. NOTE 1: The above term does not imply any architectural assumption, e.g. whether 6G CN is a new or evolved CN (compared to 5G). NOTE 2: This requirement only impacts core network. | PR 5.9.7.6-1 | Multiple Core Networks |
|  | Based on operator policy, the 6G network shall support the ability to allow an authorized 3rd party service provider to provide information of the service characteristics for each traffic flow component of its service/application to the 6G network. | PR-5.9.8.2-1 | Service Awareness [ZTE]merged into Service awareness CPR |
|  | Based on operator policy, the 6G network shall support mechanisms to dynamically adjust and optimize network resources based on the service characteristics, including their predicted changes, provided by the service or application. | PR-5.9.8.2-2 | Service Awareness [ZTE]merged into Service awareness CPR |
| Y.1.1-2-9 | Subject to operator policy and user consent, the 6G system shall support means to provide users with differentiation of QoS and charging based on users’ digital identity information issued by a third party and users’ subscription information. | PR 5.5.9.6-1 | Digital Identity |
| Y.1.1-2-10 | The multimedia telephony service [144] provided by IMS shall be able to minimise user perception of the transition during codec modification of an ongoing voice call, e.g. a codec change during communication link fluctuation. | PR 5.7.8.2-1 | IMS Codec change QoE |

NEW Editor’s Note:  PRs 5.7.1.2-5, PR 5.7.2.2-1, PR 5.7.5.2-1, and PR 5.7.7.6-1 have editor’s notes associated with them, so they are not included in consolation into Table Y.1.1-2 at this time.

Table Y.1.1-3 – Device Support

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-3-1 | The 6G system shall support UEs with different characteristics such as data rate, latency, etc. | PR 5.10.1.3-1 | Editor's Note: additional considerations on the above requirement is FFS. |
| Y.1.1-3-2 | The 6G system with satellite access shall support UEs with the performance requirements in Table 5.10.2.2-1 | PR 5.10.2.2-1 | Table values are FFS |

NEW Editor’s Note:  Both PRs for Table Y.1.1-3 have editor’s notes associated with them, but neither of the ENs are against the actual text of the PR. PR 5.10.1.3-1’s EN is about “additional considerations” and PR 5.10.2.2-1’s EN is against the associated Table values.  They are provided for discussion/consideration.