3GPP TSG-SA WG1 Meeting #112 	S1-254213

Dallas, USA, 17th - 21st November, 2025	(revision of S1-253304)

Title:	Use case on 6G AI Agent collaboration for disaster rescue

Draft Spec:	3GPP TR 22.870

Agenda Item:	8.1.3-Artifcial Intelligence

Source:	BUPT, Pengcheng Laboratory, ZGC Institute of Ubiquitous-X Innovation and Application, CMCC

Document for:	Approval

Contact:	Xiaodong Xu (xuxiaodong@bupt.edu.cn), Mengying Sun (smy_bupt@bupt.edu.cn), Nan Ma (manan@bupt.edu.cn), Sheng Jiang (shengjiang@bupt.edu.cn), Yuankang Chen (ykchen@bupt.edu.cn), Zechuan Fang (zechuan@bupt.edu.cn); Zhe Zheng (zhengzh02@pcl.ac.cn)

Abstract: This document proposes a Use case on 6G AI Agent collaboration for disaster rescue.

* * * First Change* * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- Xu M, Niyato D, Kang J, et al. When large language model agents meet 6G networks: perception, grounding, and alignment[J]. IEEE Wireless Communications, 2024, 31(6): 63-71.

- Jiang F, Peng Y, Dong L, et al. Large language model enhanced multi-agent systems for 6G communications[J]. IEEE Wireless Communications, 2024, 31(6): 48-55.

- Ray Forbes, Chair ETSI ISG ENI ,“AI Agents and Policy, an AI Architecture,” ETSI AI Conference 2025 , Feb. 2025.

- Q. Yu, et al, An Image is Worth 32 Tokens for Reconstruction and Generation, 38th Conference on   Neural Information Processing Systems (NeurIPS), 2024.

- Wang J, Jiang Y, Yuan Z, et al. Omnitokenizer: A joint image-video tokenizer for visual generation[J]. Advances in Neural Information Processing Systems, 2024, 37: 28281-28295.

…

[x] 	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change (all the text is new)* * * *

## 6.X	Use case on 6G AI Agent collaboration for disaster rescue

### 6.X.1	Description

In recent years, AI agent technology has achieved breakthrough advances in perception, decision-making, and execution. Leveraging paradigms such as reinforcement learning, transfer learning, and large-scale language models, AI agents can autonomously interpret complex intents and decompose intricate tasks, demonstrating significant potential across smart manufacturing, smart cities, and public safety domains.

Public safety scenarios impose stringent requirements on timeliness and reliability. In a 6G environment, a 3rd party AI agent constructs a near–real-time situational scene and recommends actions that are visualized on the firefighter’s device. UE AI agents (e.g., UAVs and ground robots) act as on-site actors for sensing and task execution.

At the same time, disaster scenes generate large volumes of multimodal data, creating risks of congestion and latency. Token is unified representation but more compact for various modalities of data, and can be processed and generated by AI agents directly. Therefore, UE AI agents utilize on-board tokenizer/de-tokenizer to perform tokenization on collected data so that only essential information is reported, reducing load and shortening the decision loop.

This use case focuses on fire rescue to demonstrate the deep integration of tokenization by UAV Agents, 6G network resource orchestration for token traffic, and precise instruction execution by ground robot Agents, thereby establishing an efficient closed-loop rescue workflow.

### 6.X.2	Pre-conditions

1. The 6G network is functioning normally and accessible, supporting advanced capability for priority-based protection of token traffic.

2. UAV agents have completed 6G network registration and subscribed to communication, sensing, and edge computing services. Each UE AI agent hosts a tokenizer/de-tokenizer on the device, supporting local multimodal data tokenization to support low-latency transmission.

3. The ground rescue team is equipped with rescue, fire-suppression, and logistics robot agents, and has subscribed to the 6G network service provided by Operator A, enabling instruction tokens reception, task execution, and dynamic resource orchestration.

4. The 3rd party AI agent (accessible via the operator or a trusted third party) is available to reconstruct the incident scene and provide decision support, and the firefighter’s command device is provisioned to visualize the situational picture and submit intents.

### 6.X.3	Service Flows

1. A fire breaks out in a certain area. Upon receiving the alarm, the firefighter uses the command device to issue a rescue intent. The 3rd-party AI agent derives the rescue plan and issues tasking instructions to UE AI agents (e.g., UAVs and ground robots) for execution.

2. UAV AI agents collect multimodal data such as video, audio, and thermal imagery over the fire zone. These multimodal data are then translated into tokens by the tokenizer that is part of the UE AI agents, and reported via the 6G network to the 3rd party AI agent. The 3rd-party AI agent also deployed corresponding de-tokeninzer to process the received tokens, as well as to execute tasks.

3. The 6G network forwards the uploaded tokens from UE AI agents to the 3rd party AI agent for scene reconstruction and inference process for the rescue plan. The reconstructed situational picture and recommended actions are presented on the firefighter’s command device for human review.

4. The firefighter confirms or adjusts the recommended plan, and sends to the 3rd party AI agent for task instruction refinement. The agreed plan enumerates task categories-victim rescue, fire suppression and evacuation, post-disaster clearance and supply delivery, and continuous monitoring-with victim rescue set as the highest priority.

5. The 3rd party AI agent issues precise task instruction tokens to the selected UAV agents and ground robot agents, which start the corresponding rescue actions.

6. The 6G network is made aware of the characteristics of the transmitted tokens (e.g., priority, tolerable error rate). The 6G network performs priority-based protection transmission strategy to the transmitted tokens, ensuring that high-priority data (e.g., victim localization coordinates) meets strict performance requirements.

7. During the rescue, UAV agents provide thermal/localization cues and other updates to ground robot agents to support precise operations such as clearing debris, targeted fire suppression, and evacuation guidance. The 6G network keeps resources prioritized for victim rescue.

8. UAV agents and ground robot agents continuously report progress and scene updates via the 6G network, and when needed, invokes the 3rd party AI agent for plan refinement. Updates are reflected on the firefighter’s command device.

9. After the operation concludes, the 3rd party AI agent respectively archives the session and refines the strategies based on collected status data to provide more accurate decision support for future missions.

Figure 6.X.3-1 Use case on 6G AI Agents collaboration for disaster rescue

### 6.X.4	Post-conditions

Enabled by the situational picture and recommendations from the 3rd party AI agent, the firefighter’s human-in-the-loop decisions, and the 6G network with priority-based protection transmission strategy, the UAV agents and ground robot agents executed the plan effectively: all trapped individuals were evacuated to designated safe zones, the fire was contained within the affected area, debris was removed and access paths were restored, and emergency supplies were delivered to the affected population.

### 6.X.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 clause 6.40.2.2:

Based on user consent, operator policy and trusted 3rd party request, the 5G system shall support a means to authorize specific UEs to transmit data (e.g. AI-ML model data for a specific application,) via direct device connection in a certain location and time.

3GPP TR 22.874 and TR 22.876:

In 5G and 5G Advanced (Rel-18 and Rel-19), SA1 has studied use cases and requirements of AI/ML model transfer Phase 1 and Phase 2, which mainly focused on model transfer and big data transfer at the application layer.

### 6.X.6	Potential New Requirements needed to support the use case

[PR 6.X.6-1] Based on user consent and operator policy, the 6G networks shall support differentiated transmission of AI-processed data formats (e.g., tokens) to enable efficient interaction among the 3rd party AI agents.

[PR 6.X.6-2] Based on user consent and operator policy, the 6G networks shall support priority-based dynamic network resource management to meet the performance requirements of diverse services.

* * * End of Change * * * *