3GPP TSG-SA WG1 Meeting #112 	S1-254410

17-21 November 2025, Dallas, USA

Title:	Initial Consolidation Status after SA1 #112

Agenda Item:	TBD

Source:	6G Study Rapporteurs

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Provides the results of the initial round of consolidation for TR 22.870 after SA1 #112.

* * * First Change * * * *

Y		Consolidated potential functional and performance requirements

Editor’s Note:  PRs with associated Editor’s Note(s) will not be considered for consolidation until Editor’s Note(s) associated with the PR are resolved.

Editor’s Note: CPR Table(s) can be modified (e.g. split, merged, renamed, moved) during the consolidation process.

Green indicates there was consensus at this meeting to include the CPR for inclusion into the TR.

Yellow indicates that there the CPR was discussed and some additional work is needed.

* * * Second Change(all new text) * * * *

Y.1	Consolidated Potential Functional Requirements

Y.1.1	Basic Services and Capabilities

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to):	Support for Legacy (i.e., 5G and earlier) services and capabilities, Enhancements to Legacy services and capabilities, Legacy services and capabilities not supported in 6G, and device support. This includes (but not limited to) PRs from clauses 5.2, 5.3, 5.4, 5.7, 5.9, and 5.10.

Table Y.1.1-1 – Support for legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-1-1 | The 6G system shall be able to support the following services: - Mission Critical Services, i.e. MCPTT, MCData, MCVideo, ref TS 22.179 [53], TS 22.280 [54], TS 22.281 [55]. and TS 22.282 [56], - Message service, ref TS 22.262 [57], - Short Message Service (SMS), ref TS 22.101 [58], - Multimedia communication services, ref TS 22.101 [58], TS 22.261 [14], TS 22.173 [59], - IMS Multimedia Telephony Service, ref TS 22.261 [14], TS 22.173 [59], - Roaming services, ref TS 22.011 [60], TS 22.101 [58], TS 22.261 [14], - Location and positioning services, ref TS 22.261 [14], TS 22.071 [61], - Broadcast and Multicast Services, ref TS 22.261 [14], - Emergency Services, ref TS 22.101 [58], - Public Warning System (PWS), ref TS 22.268 [62], - Multimedia Priority Service (MPS), ref TS 22.153 [63], - Lawful Interception, ref TS 22.261 [14] and TS 33.126 [312], and - Other regulatory services, based on regional/national regulatory requirements. NOTE: The following services do not need to be supported by the 6G system: CS related telephony services, e.g. CS Fallback, CS based voice call. | Clause 5.4.2 |  |
| Y.1.1-1-2 | The 6G system shall be able to support 5G system requirements (functional and performance requirements) defined e.g. in TS 22.261 [14], TS 22.104 [64], TS 22.011 [60], TS 22.173 [59], TS 22.071 [61], TS 22.262 [57], TS 22.185 [65], TS 22.186 [66], TS 22.125 [35], TS 22.263 [67], TS 22.115 [68], TS 22.101 [58], TS 22.153 [63], TS 22.289 [69], TS 22.468 [70], TS 22.368 [71], TS 22.156 [28], TS 22.137 [6]. NOTE: Terms referring to 5G (e.g. “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g. “6G system”, “radio access network of the 6G system”) in those requirements. NOTE: These requirements do not apply to requirements involving the legacy systems/RATs (e.g. E-UTRAN, UTRAN, GERAN). Requirements mentioned in the above standards that include E-UTRAN, UTRAN, and GERAN are not automatically included. | Clause 5.4.3 | There was general support for the NOTE, but the text in the NOTE is still needs some work/under discussion |
| Y.1.1-1-3 | Subject to operator’s policy, the 6G system shall support mobility procedures between the core network (CN) of the 6G system and a 5G core network with minimum impact to the user experience (e.g. Quality of Service (QoS), Quality of Experience (QoE)). | Clause 5.2 |  |
| Y.1.1-1-4 | Subject to operator’s policy, the 6G system shall support mobility procedures between the core network of the 6G System and the evolved Packet Core Network (EPC) with minimum impact to the user experience (e.g. QoS, QoE). | Clause 5.2 |  |
| Y.1.1-1-6 | The 6G system shall be able to support a user to access network services via 3GPP and/or non-3GPP access (e.g. WLAN or Wireline). | Clause 5.3 | There was a proposal to merge/include this requirement with CPR Y.1.1-1-2 |
| Y.1.1-1-7 | Subject to operator’s policy, the 6G system shall support mobility between the 6G 3GPP access and non-3GPP access, with minimum impact to the user experience (e.g. QoS, QoE). | Clause 5.3 | There was a proposal to merge/include this requirement with CPR Y.1.1-1-2 |
| Y.1.1-1-5 | Requirements in TS 22.261 clause 5.1.2.2, related to inter-RAT capabilities not to be supported by 5GS, shall apply similarly to 6G System with the following modification: - voice service continuity from the radio access network of the 6G system to UTRAN CS shall not be supported. NOTE 2: Terms referring to 5G (e.g. “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g. “6G system”, “radio access network of the 6G system”) in those requirements. | Clause 5.2 | There was a comment that the NOTE” may need to be revised as TSG RAN is using “6GR”. |

Table Y.1.1-2 – Enhancements to legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-2-1 | The 6G network shall provide a mechanism to support event triggered network sharing (e.g. disaster occurrence, network failure, overloaded situation, resource constraints). | PR 5.7.4.2-2 | Network Sharing |
| Y.1.1-2-2 | Subject to regulatory requirements or operator policy, the 6G network shall support sharing of radio access network with sensing capability among operators. | PR-5.7.10.6-1 | Network Sharing |
| Y.1.1-2-3 | The 6G system shall support suitable access categories to manage the access attempt for the new services (such as sensing, AI application, computing) in congestion scenarios. | PR 5.7.6.2-2 | Unified Access Control |
| Y.1.1-2-4 | The 6G system shall provide optimized network capabilities for FWA (e.g. support stationary devices) in relevant bands taking into consideration the regulatory requirements for each specific band. | PR 5.7.1.2-1 PR 5.7.1.2-2 | Fixed Wireless Access |
| Y.1.1-2-5 | The 6G system shall enable the means to provide awareness of user service characteristics (e.g. data rate, latency) to support the RAN and CN in making real time resource allocation for FWA. | PR 5.7.1.2-3 | Fixed Wireless Access |
| Y.1.1-2-6 | The 6G system shall enhance the Short Message Service to enable a network operator to verify the identity of the SMS sender and information concerning operator verified SMS sender information to the recipient of a SMS. NOTE 1: Operator-verified SMS sender information is used to inform the recipient of a SMS that the identity of the SMS sender is operator-verified and support displaying additional information (e.g. brand name, logo, etc.) of the SMS sender. Human interface aspects are out of scope of this requirement. NOTE 2: Indication that the identity of the SMS sender is operator-verified, any additional information about the SMS sender and the message itself is assumed to be integrity protected. NOTE 3: Based on interworking agreements and trust relationships, the requirements above apply also when the SMS recipient is roaming or receives a SMS from a sender served by other operators. NOTE 4: The requirements above apply to A2P SMS and may apply to Person-to-Person SMS. | PR 5.7.3.2-1 PR 5.7.3.2-2 | SMS |
| Y.1.1-2-7 | Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. | PR 5.9.6.6-1 | 6G LAN |
| Y.1.1-2-8 | Subject to operator policies, and agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to - authorize PLMN’s users to access a subscribed service provided by an authorized 3rd party via a local area network (deployed by the PLMN operator), and - minimize service interruption when the serving network changes between the local area network and the PLMN network. | PR 5.9.6.6-2 | 6G LAN |
| Y.1.1-2-9 | Subject to operator’s policy and regulation, for an operator with multiple 6G core networks, when there is UE mobility from one 6G core network to another 6G core network of the same PLMN, the 6G network shall support efficient traffic routing for the traffic from UE to data network. NOTE 1: The above term does not imply any architectural assumption, e.g. whether 6G CN is a new or evolved CN (compared to 5G). NOTE 2: This requirement only impacts core network. | PR 5.9.7.6-1 | Multiple Core Networks |
| Y.1.1-2-10 | Based on operator policy, the 6G network shall support the ability to allow an authorized 3rd party service provider to provide information of the service characteristics for each traffic flow component of its service/application to the 6G network. | PR-5.9.8.2-1 | Service Awareness |
| Y.1.1-2-11 | Based on operator policy, the 6G network shall support mechanisms to dynamically adjust and optimize network resources based on the service characteristics, including their predicted changes, provided by the service or application. | PR-5.9.8.2-2 | Service Awareness |
| Y.1.1-2-11 | Subject to operator policy and user consent, the 6G system shall support means to provide users with differentiation of QoS and charging based on users’ digital identity information issued by a third party and users’ subscription information. | PR 5.5.9.6-1 | Digital Identity |
| Y.1.1-2-12 | The multimedia telephony service [144] provided by IMS shall be able to minimise user perception of the transition during codec modification of an ongoing voice call, e.g. a codec change during communication link fluctuation. | PR 5.7.8.2-1 | IMS Codec change QoE |

Editor’s Note:  PRs 5.7.1.2-5, PR 5.7.2.2-1, PR 5.7.5.2-1, and PR 5.7.7.6-1 have editor’s notes associated with them, so they are not included in consolation into Table Y.1.1-2 at this time.

Editor’s Note: SA1 #112 discussions on Consolidation of Y.1.1 stopped here. The rapporteur’s initial consolidation proposals for the remaining CPR Tables have been removed and will be resubmitted in a separate TDOC. The CPR numbering will be adjusted as consolidation progresses.

Table Y.1.1-3 – Device Support

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-3-1 |  |  |  |
| Y.1.1-3-2 |  |  |  |

Editor’s Note:  Both PRs for Table Y.1.1-3 have editor’s notes associated with them, but neither of the ENs are against the actual text of the PR. PR 5.10.1.3-1’s EN is about “additional considerations” and PR 5.10.2.2-1’s EN is against the associated Table values.  They are provided for discussion/consideration.

Y.1.2	Security and Privacy

Editor’s Note: This clause contains consolidated potential requirements for 6G that include security, privacy and lawful interception. This includes (but not limited to) PRs from clauses 5.5 and X.1.

Editor’s Note:  PR 5.5.1.2-5, PR 5.5.4.2-10, and PR 5.5.6.3-1 have editor’s notes associated with them, so they are not included in consolation into Table Y.1.2-1 at this time.

Table Y.1.2-1 – Security and Privacy

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.2-1-1 |  |  |  |
| Y.1.2-1-2 |  |  |  |
| Y.1.2-1-3 |  |  |  |
| Y.1.2-1-4 |  |  |  |
| Y.1.2-1-5 |  |  |  |
| Y.1.2-1-6 |  |  |  |
| Y.1.2-1-7 |  |  |  |
| Y.1.2-1-8 |  |  |  |
| Y.1.2-1-9 |  |  |  |
| Y.1.2-1-10 |  |  |  |
| Y.1.2-1-11 |  |  |  |
| Y.1.2-1-12 |  |  |  |
| Y.1.2-1-13 |  |  |  |
| Y.1.2-1-14 |  |  |  |
| Y.1.2-1-15 |  |  |  |
| Y.1.2-1-16 |  |  |  |

Table Y.1.2-2– Lawful Interception

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.2-2-1 |  |  |  |
| Y.1.2-2-2 |  |  |  |
| Y.1.2-2-3 |  |  |  |
| Y.1.2-2-4 |  |  |  |
| Y.1.2-2-5 |  |  |  |
| Y.1.2-2-6 |  |  |  |
| Y.1.2-2-7 |  |  |  |

Y.1.3	Resilience

Editor’s Note: This clause contains consolidated potential requirements for 6G that address resiliency. This includes (but not limited to) PRs from clauses 5.6 and 5.9.

Table Y.1.3-1 – Resilience

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.3-1-1 |  |  |  |
| Y.1.3-1-2 |  |  |  |
| Y.1.3-1-3 |  |  |  |
| Y.1.3-1-4 |  |  |  |
| Y.1.3-1-5 |  |  |  |

Y.1.4	Energy-related aspects

Editor’s Note: This clause contains consolidated potential requirements for 6G that address energy efficiency and sustainability. This includes (but not limited to) PRs from clause 5.8.

Editor’s Note:  PR 5.8.2.6-2 has an editor’s note associated with it (concerning energy consumption information), so it is not included in consolation into Table Y.1.4-1 at this time.

Table Y.1.4-1 – Energy-related aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.4-1-1 |  |  |  |
| Y.1.4-1-2 |  |  |  |
| Y.1.4-1-3 |  |  |  |
| Y.1.4-1-4 |  |  |  |
| Y.1.4-1-5 |  |  |  |
| Y.1.4-1-6 |  |  |  |
| Y.1.4-1-7 |  |  |  |
| Y.1.4-1-8 |  |  |  |
| Y.1.4-1-9 |  |  |  |

Y.1.5	Data Collection and Consumption

Editor’s Note: This clause contains consolidated potential requirements for 6G that address data collection, management, and consumption. This includes (but not limited to) PRs from clauses 5.9.2 and W.

Editor’s Note:  PR 5.9.2.2-2, -3, and -6 have editor’s notes associated with them (concerning user consent), so they are not included in consolation into Table Y.1.5-1 at this time.

Table Y.1.5-1 – Data Collection and Consumption

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.5-1-1 |  |  |  |
| Y.1.5-1-2 |  |  |  |
| Y.1.5-1-3 |  |  |  |

Y.1.6	Charging

Editor’s Note: This includes (but not limited to) PRs on charging throughout the document.

Editor’s Note: PR 7.17.6-4 is for FFS, so it is not included in consolation into Table Y.1.6-1 at this time.

Table Y.1.6-1 – Charging

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.6-1-1 |  |  |  |
| Y.1.6-1-2 |  |  |  |
| Y.1.6-1-3 |  |  |  |
| Y.1.6-1-4 |  |  |  |
| Y.1.6-1-5 |  |  |  |
| Y.1.6-1-6 |  |  |  |
| Y.1.6-1-7 |  |  |  |
| Y.1.6-1-8 |  |  |  |
| Y.1.6-1-9 |  |  |  |
| Y.1.6-1-10 |  |  |  |
| Y.1.6-1-11 |  |  |  |
| Y.1.6-1-12 |  |  |  |
| Y.1.6-1-13 |  |  |  |
| Y.1.6-1-14 |  |  |  |
| Y.1.6-1-15 |  |  |  |

Y.1.7	Other operational aspects

Editor’s Note: This includes (but not limited to) PRs from clause 5.5.8, 5.8.7, 5.9.3, throughout the document.

Table Y.1.7-1 – NDT

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.7-1-1 |  |  |  |
| Y.1.7-1-2 |  |  |  |
| Y.1.7-1-3 |  |  |  |
| Y.1.7-1-4 |  |  |  |
| Y.1.7-1-5 |  |  |  |
| NOTE: TBD. |  |  |  |

Table Y.1.7-2 – OAM

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.7-2-1 |  |  |  |
| Y.1.7-2-2 |  |  |  |

Y.1.8	AI

Editor’s Note: This includes (but not limited to) PRs from clause 6.

Table Y.1.8-1 – General AI requirements for 6G system

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-1-1 | Based on operator policy, the 6G system shall support AI capabilities. NOTE: Example of AI capabilities is the system ability to predict the UE behaviour (based on UE type, historical data, mobility patterns, etc.) and use that for the allocation and planning resources efficiently. | PR 6.4.6-1 | General, prediction |
| CPR Y.1.8-1-2 | Based on operator policy, the network entities supporting AI capabilities shall be able to collaborate upon request. | PR 6.4.6-2 | General |
| Alternative CPR Y.1.8.-1-1 | Subject to operator policy and user preference, the 6G system shall be able to support mechanisms (e.g. AI capabilities) to predict UE behavior (e.g. based on device UE type, historical data, mobility patterns, etc.) for efficient resource allocation and planning. | PR 6.4.6-1 PR 6.4.6-2 | Proposed merged CPR on AI capabilities for prediction of UE behavior |
| CPR Y.1.8-1-3 | Based on operator policy, the 6G system shall be able to support mechanisms (e.g. AI capabilities in the network and UEs) allowing the network and UEs to negotiate communication parameters for a communication service. | PR 6.4.6-3 | General, performance |
| CPR Y.1.8-1-4 | Subject to operator policy, the 6G network shall support the use of AI capabilities for the operations and management (OAM) of the 6G network for energy efficiency and carbon emissions reduction. | PR 6.16.6-1 |  |

Table Y.1.8-2 – Network AI Agent

Editor’s Note: This includes requirements related to network AI.

NOTE:	The mention of AI capabilities such as AI Agent doesn’t imply or preclude any architecture assumption or solution.

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-2-1 | Subject to operator’s policy, the 6G network shall support mechanism, e.g. AI capabilities such as AI Agent, to provide suitable 3GPP service or combination of multiple 3GPP services in response to received service request (e.g. expressed via intent(s)) from subscriber/users, an authorized third party,. | PR 6.6.6-4 PR 6.20.6-1 PR 6.21.6-2 PR 6.32.6-1 PR 6.43.6-1 PR 6.51.6-1 PR 6.6.6-2 – to be separate PR 6.44.6-2 – to be separate PR 6.43.6-2 – to be separate | Proposed merged CPR on Network AI agent, provide service based on intent |
| CPR Y.1.8-2-5 | Subject to operator policy, the 6G network shall support mechanisms (e.g. AI capabilities such as AI Agent) to send intent(s) related to 3GPP services towards a third-party AI Agent (e.g. proactively or in response to a received intent), also taking into account information related to user mobility context, subscription information. | PR 6.6.6-3 | Network AI agent, intent |
| CPR Y.1.8-2-6 | Based on operators’ policy and user consents, the 6G system shall support mechanisms (e.g. AI capabilities such as AI agent) to translate intent received (e.g. from subscribers) into service and service performance requirements. | PR 6.21.6-1 | Network AI agent, intent translation |
| CPR Y.1.8-2-7 | The 6G system shall support a mechanism to identify and associate the source (e.g. home robot, or specific AI Agent) of the intent with a subscriber and with the actions resulting from the handling/processing of the intent. | PR 6.21.6-3 | Network AI agent, intent |
| CPR Y.1.8-2-8 | Subject to operator policy, the 6G network shall support mechanisms (e.g. Al capabilities such as Al agent) to authorize the user who sends the intent. The 6G network shall support mechanisms to ensure the user is authorized to request the intent. | PR 6.44.6-1 | Network AI agent, intent |
| CPR Y.1.8-2-9 | Subject to operator policy, the 6G network shall support a mechanism (e.g. AI capabilities such as AI agent) to monitor and evaluate the quality of the provided 3GPP service and enhance the service if needed. | PR 6.21.6-4 | Network AI agent, quality optimizations |
| CPR Y.1.8-2-10 | Subject to user consent and operator policy, the 6G system shall be able to support mechanisms (e.g. AI capabilities such as AI agent) to enable real-time call quality analytics and dynamic optimizations. | PR 6.54.6-1 | Network AI agent, quality optimizations |
| CPR Y.1.8-2-11 | Subject to user consent, and operator policy, the 6G network shall be able to receive and analyse aggregated call quality data, and apply enhancements using mechanisms (e.g. AI capabilities such as AI agent) in the 6G network. | PR 6.54.6-2 | Network AI agent, quality optimizations |
| CPR Y.1.8-2-12 | Subject to user consent and operator policy, the 6G system shall be able to support continuous enhancement of mechanisms (e.g. AI capabilities such as AI agent) in 6G network, from user experience feedback. | PR 6.54.6-3 | Network AI agent, quality optimizations |

Table Y.1.8-3 – 3rd party AI Agent

Editor’s Note: This includes requirements related to 3rd party AI agent.

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-3-1 | Based on regulatory requirements, operators’ policy and user consent, 6G network shall support trusted network access for 3rd party AI agent and support a mechanism to expose 3rd party AI agent’s attributes (e.g. related users, sensing capabilities, AI capabilities, service features) to other 3rd party AI agents. | PR 6.7.6-1 PR 6.23.6-3 | Proposed merged CPR on 3rd party AI agent, exposure between AI agent |
| CPR Y.1.8-3-2 | Based on regulatory requirements, user consent, operators’ policy and agreement with authorized 3rd party, the 6G network shall be able to support mechanisms to enable secure identification of 3rd party AI agents associated with a user (e.g. AI agents belonging to a customer). | PR 6.7.6-2 PR 6.55.6-1 | Proposed merged CPR on 3rd party AI agent, identification |
| CPR Y.1.8-3-3 | Based on regulatory requirements, operators’ policy and user preference, 6G network shall support mechanisms for 3rd party AI agents to provide their attributes to 6G network, and discover other authorized 3rd party AI agents. NOTE: Attributes can include e.g. capabilities, associated authorized users | PR 6.7.6-3 PR 6.46.6-1 | Proposed merged CPR on 3rd party AI agent, register and discover |
| CPR Y.1.8-3-4 | Based on operator's policy and user preference, the 6G network shall support a mechanism to manage the attributes (e.g. sensing capabilities, AI capabilities, service features, associated authorized users) of AI application (e.g. AI agent application) on UE (e.g. related user). | PR 6.23.6-1 | 3rd party AI agent, management |
| CPR Y.1.8-3-5 | Based on user consent and operator's policy, the 6G system shall support secure interoperability and efficient mechanism between AI applications (e.g. AI Agents) on multiple UEs. NOTE 1: Interoperability between AI agents refers to the ability to discover, authenticate and authorize AI Agents to communicate, exchange data, and work together seamlessly. NOTE 2: Collaborative task refers to an activity, action requiring the involvement of two or more AI agents. | PR 6.9.6-2 PR 6.23.6-4 PR 6.30.6-2 PR 6.40.6-1 | Proposed merged CPR on 3rd party AI agent, collaboration |
| CPR Y.1.8-3-6 | Based on regulatory requirements and operators’ policy and user preference, the 6G network shall provide means to support efficient and secure communication (including multi-modality exchange) between multiple e.g. AI applications /3rd party AI agents on UEs over a target area considering data characteristics. NOTE: This requirement can apply to 3rd party AI agents of same users or different users. It is expected that the required communication service would be provisioned in the range of minutes to days, depending on use case. Lower for temporary task and higher for long term task. | PR 6.7.6-4 PR 6.8.6-3 PR 6.23.6-4 | Proposed merged CPR on 3rd party AI agent, communication |
| CPR Y.1.8-3-7 | Based on operators’ policy and user consent, the 6G system shall support hosting of large amount of 3rd party AI agents managed and controlled by the 6G network and/or multiple AI Agent applications on a UE. | PR 6.9.6-1 | 3rd party AI agent, general |
| CPR Y.1.8-3-8 | Based on operator policy, the 6G network shall be able to support secure means to expose 3GPP services (e.g. 6G computing service in Service Hosting Environment) to the authorised third-party AI agent based on its intent. | PR 6.6.6-1 PR 6.7.6-5 | Proposed merged CPR on 3rd party AI agent, service exposure |
| CPR Y.1.8-3-9 | Based on user consent and operator policy, the 6G system shall provide a suitable means for an AI agent application on UE to invoke some 3GPP services (e.g. IMS service, AI service) upon request. | PR 6.8.6-1 PR 6.14.6-2 PR 6.30.6-1 | Proposed merged CPR on 3rd party AI agent, service invoke |
| CPR Y.1.8-3-10 | The 6G network shall be able to provide a suitable means to allocate network resources (e.g., network slice) to a group of trusted third parties (e.g., applications running on multiple UEs/robots or third party AI agents), considering dynamic changes of traffic demand and QoS characteristics. | PR 6.52.6-2 | 3rd party AI agent, resource |
| CPR Y.1.8-3-11 | Based on the operator's policy and user consent, the 6G network shall support a mechanism to authenticate and authorize 3rd party AI agent on the UE. | PR 6.23.6-2 PR 6.41.6-1 PR 6.41.6-2 | Proposed merged CPR on 3rd party AI agent, authentication and authorization |
| CPR Y.1.8-3-12 | Based on user consent, operator policy and regulatory requirements, the 6G system shall provide an efficient way to expose information (e.g. change of QoS) to authorized 3rd party AI agents (e.g. by means suitable for prompt augmentation). | PR 6.8.6-2 PR 6.13.6-1 PR 6.9.6-3 | Proposed merged CPR on 3rd party AI agent, exposure to AI agent |
| CPR Y.1.8-3-13 | Based on the user consent and operator’s policy, the 6G system shall be able to support means for the network to invoke the 3rd party AI agent (application) on the UE. | PR 6.46.6-2 | 3rd party AI agent, network invoke AI agent |
| CPR Y.1.8-3-14 | Subject to operator’s policy, regulatory requirements and user consent, the 6G system shall support a mechanism for a user-authorized AI application (e.g. AI agent) in the Service Hosting Environment to autonomously initiate communication with emergency services on behalf of the user, i.e., the communication session is associated with the user's identity and location. | PR 6.47.6-1 | 3rd party AI agent, emergency service |
| CPR Y.1.8-3-15 | Subject to operator’s policy, regulatory requirements and user consent, the 6G network shall support a mechanism to log the data (e.g., sensor data, context information) used by an AI application (e.g., AI agent) in the Service Hosting Environment, which autonomously decided to initiate communication with emergency services on behalf of the user. | PR 6.47.6-2 | 3rd party AI agent, emergency service |
| CPR Y.1.8-3-16 | Subject to operator’s policy, regulatory requirements and user consent, the 6G system shall support a mechanism for a user to provide policies which define the autonomous actions that can be taken by their authorized AI applications (e.g. AI agents) in the Service Hosting Environment. NOTE: For example, a user could provide a policy authorizing their AI agent to share location data with a pre-defined family member. A separate policy could authorize their AI agent to initiate communication with emergency services on the user's behalf when the agent identifies a critical safety event | PR 6.47.6-3 | 3rd party AI agent, emergency service |

Editor’s Note: SA1 #112 discussions on Consolidation of Y.1.8 stopped here. The rapporteur’s initial consolidation proposals for the remaining CPR Tables have been removed and will be resubmitted in a separate TDOC. The CPR numbering will be adjusted as consolidation progresses.

Table Y.1.8-4 – IMS based AI services / AI related IMS services

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-4-1 |  |  |  |
| CPR Y.1.8-4-2 |  |  |  |
| CPR Y.1.8-4-3 |  |  |  |
| CPR Y.1.8-4-4 |  |  |  |
| CPR Y.1.8-4-5 |  |  |  |
| CPR Y.1.8-4-6 |  |  |  |
| CPR Y.1.8-4-7 |  |  |  |
| CPR Y.1.8-4-8 |  |  |  |
| CPR Y.1.8-4-9 |  |  |  |
| CPR Y.1.8-4-10 |  |  |  |
| CPR Y.1.8-4-11 |  |  |  |
| CPR Y.1.8-4-12 |  |  |  |
| CPR Y.1.8-4-13 |  |  |  |
| CPR Y.1.8-4-14 |  |  |  |

Table Y.1.8-5 – AI model training and inferencing

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-5-1 |  |  |  |
| CPR Y.1.8-5-2 |  |  |  |
| CPR Y.1.8-5-3 |  |  |  |
| CPR Y.1.8-5-4 |  |  |  |
| CPR Y.1.8-5-5 |  |  |  |
| CPR Y.1.8-5-6 |  |  |  |
| CPR Y.1.8-5-7 |  |  |  |
| CPR Y.1.8-5-8 |  |  |  |
| CPR Y.1.8-5-9 |  |  |  |
| CPR Y.1.8-5-9 |  |  |  |
| CPR Y.1.8-5-10 |  |  |  |
| CPR Y.1.8-5-11 |  |  |  |
| CPR Y.1.8-5-12 |  |  |  |
| CPR Y.1.8-5-13 |  |  |  |
| CPR Y.1.8-5-14 |  |  |  |
| CPR Y.1.8-5-15 |  |  |  |
| CPR Y.1.8-5-16 |  |  |  |
| CPR Y.1.8-5-17 |  |  |  |
| CPR Y.1.8-5-18 |  |  |  |
| CPR Y.1.8-5-19 |  |  |  |
| CPR Y.1.8-5-20 |  |  |  |
| CPR Y.1.8-5-21 |  |  |  |

Table Y.1.8-6 – AI traffic characteristics

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-6-1 |  |  |  |
| CPR Y.1.8-6-2 |  |  |  |
| CPR Y.1.8-6-3 |  |  |  |
| CPR Y.1.8-6-4 |  |  |  |

Table Y.1.8-7 – Other

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-7-1 |  |  |  |
| CPR Y.1.8-7-2 |  |  |  |
| CPR Y.1.8-7-3 |  |  |  |

Y.1.9	Computing

Editor’s Note: This includes (but not limited to) PRs from clause 6, 9, 11, W.

Table Y.1.9-1 – Computing

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.9-1-1 |  |  |  |
| CPR Y.1.9-1-2 |  |  |  |
| CPR Y.1.9-1-3 |  |  |  |
| CPR Y.1.9-1-4 |  |  |  |
| CPR Y.1.9-1-5 |  |  |  |
| CPR Y.1.9-1-6 |  |  |  |
| CPR Y.1.9-1-7 |  |  |  |
| CPR Y.1.9-1-8 |  |  |  |
| CPR Y.1.9-1-9 |  |  |  |
| CPR Y.1.9-1-10 |  |  |  |
| CPR Y.1.9-1-11 |  |  |  |
| CPR Y.1.9-1-12 |  |  |  |
| CPR Y.1.9-1-13 |  |  |  |
| CPR Y.1.9-1-14 |  |  |  |
| CPR Y.1.9-1-15 |  |  |  |
| CPR Y.1.9-1-16 |  |  |  |
| CPR Y.1.9-1-17 |  |  |  |
| CPR Y.1.9-1-18 |  |  |  |
| CPR Y.1.9-1-19 |  |  |  |
| CPR Y.1.9-1-20 |  |  |  |
| CPR Y.1.9-1-21 |  |  |  |
| CPR Y.1.9-1-22 |  |  |  |
| CPR Y.1.9-1-23 |  |  |  |
| CPR Y.1.9-1-24 |  |  |  |
| CPR Y.1.9-1-25 |  |  |  |
| CPR Y.1.9-1-26 |  |  |  |

Y.1.10	Integrated Sensing and Communication (ISAC)

Editor’s Note: This includes (but not limited to) PRs from clause 7.

Editor’s Note: PR 7.17.6-1, PR 7.17.6-2, PR 7.17.6-3, PR 7.17.6-4, and PR 7.25.6-1 have Editor’s Notes associated with them, so they are not included for consolidation into Table Y.1.10-1 at this time.

Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Table Y.1.10-1 – ISAC

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.10-1-1 | Subject to regulatory requirements, operator’s policy, the 6G network should support suitable means to collect non-3GPP sensing data from third party if available. | PR 7.16.6-1 | [ZTE] Non-3GPP sensing data collection Considering the sensing data collection is specific, it is suggested to remain it in Y.1.10 |
| Y.1.10-1-2 | Subject to operator policies, the 6G Network shall provide sensing service to detect, classify and count one or more sensing targets (e.g. detect UAVs, distinguish UAVs from birds, identify specific UAV characteristics.). NOTE: Classification refers to the identification of specific characteristics of the detected target objects and grouping together of the detected target objects with similar characteristics. | PR 7.23.6-1 | (Stable) Sensing Ops Sensing Modes Multiple targets |
| Y.1.10-1-3 | Subject to operator policies, the 6G Network shall provide mechanisms for configuring a sensing operation with a single or multiple sensing modes from all sensing modes supported (e.g. bistatic, monostatic, multistatic). | PR 7.14.6-1 | Sensing modes configuration |
| Y.1.10-1-4 | Subject to regulation and operator’s policy, the 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity or/and a latency upper-bound to nearby UEs (e.g. AMRs), requested by the trusted third party. NOTE: The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11]. NOTE 2: The latency depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. | PR 7.5.6-3 PR 7.7.6-1 PR 7.12.6-2 | Sensing QoS: capacity/density/latency upper-bound |
| Y.1.10-1-5 | Subject to regulation and operator's policy, 6G system shall support sensing target density requested by the third party for a given sensing service. | PR 7.7.6-1 | Target Density |
| Y.1.10-1-7 | Based on operator policy, regional and/or national regulations, the 6G network in the area of the disaster shall provide secure mechanisms to collect sensing results with a requested level of accuracy that can be used to generate real time maps. | PR 7.2.6-1 | Accuracy Target Density Latency 3rd Party Support |
| Y.1.10-1-8 | Subject to operator’s policy and regulation, the 6G network shall be able to provide a sensing service to derive predicted location and/or velocity of sensing target(s). | PR 7.10.6-1 | Prediction |
| Y.1.10-1-9 | Subject to regulation and operator policy, the 6G network shall provide a target prediction capability to derive predicted target characteristics (e.g. size, shape, location, velocity), while maintaining the privacy of the sensing target(s) and means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. | PR 7.10.6-1 PR 7.10.6-2 | Prediction Exposure Privacy Third Party support |
| Y.1.10-1-10 | Subject to regulatory requirements, operator’s policy, and user consent, 6G network shall support the use of stored sensing data to provide a sensing service and ensure that only authorised entities are able to access the stored sensing data and results. | PR 7.11.6-1 PR 7.12.6-1 | Sensing Data Storage Usage & Security |
| Y.1.10-1-11 | The 6G system shall be able to prioritize communication, sensing and positioning together used in Network Assisted Smart Transportation | PR 7.19.6-2 | Prioritization |
| Y.1.10.1-12 | Subject to operator’s policy, the 6G system shall provide exposure mechanism(s) to activate and deactivate exposing sensing results to a UE (AMR) that are used for prediction in a given sensing area of interest at a particular time of interest to nearby UEs at the request of a trusted third party | PR 7.8.6-1 | Exposure Third party support Delivery synchronization |
| Y..1.10.1-13 | The 6G Network shall provide suitable mechanisms for the exposure of sensing results in a synchronised manner with other types of traffic (e.g. audio, video, haptics) to the sensing service consumer. | PR 7.14.6-2 | Sensing result exposure sync.with other traffic, |
| Y.1.10.1-14 | Subject to operator’s policy, regulation and user consent, the 6G network shall be able to authorise UE and expose sensing results to an application on the UE for a specific service. NOTE: As an example, UE could use the provided sensing results (e.g. environment characteristics around UE) to optimize communication service. | PR 7.20.6-1 PR 7.21.6-1 PR 7.5.6-4 | Exposure to UE |
| Y.1.10.1-15 | Subject to operator’s policy, regulation and user consent, the 6G network shall be able to provide sensing results to a UE for a specific service, where the UE is authorized by mobile network operator providing sensing service. | PR 7.5.6-4 |  |
| Y.1.10.1-16 | Subject to operator’s policy and regulation, the 6G system shall be able to link sensing results with communication service area for communication service. | PR 7.20.6-2 | ISAC Linking |
| Y.1.10.1-17 | Subject to regulatory requirements and user permission, the 6G network shall be able to use the 6G sensing service to monitor and recognize human gestures. | PR 7.24.6-1 | Gesture Recognition |
| Y.1.10.1-18 | Subject to user consent, regulation, and operator’s policy, the 6G network shall be able to provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party, while maintaining the privacy of the sensing target(s). | PR 7.10.6-1 PR 7.10.6-2 | Sensing Data Security and Privacy Protection For Sensing Data, Prediction, Results and Targets |
| Y.1.10.1-19 | The 6G system shall be able to support energy-efficient sensing operations. | PR 7.5.6-2 To be consolidated with 7.8.6-2 | Energy-efficient Sensing Operations |

Y.1.11	Ubiquitous Connectivity

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to):s upport for non-terrestrial networks (NTN) and support for positioning services and capabilities. This includes (but not limited to) PRs from clause 8.

Editor’s Note: PR 8.2.6-1 and PR 8.17.6-1 have Editor’s Notes associated with them, so they are not included for consolidation into Table Y.1.11-1 at this time.

Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Editor’s Note: additional consideration is needed to determine which CPRs are moved from Table Y.1.11-1 (Communications) to Table Y.1.11-3 (Others). Some requirements may currently be shown in both tables, or in different potential consolidation proposal.

Table Y.1.11-1 –Satellite-based communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimize the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 | [Satellite access] LEO Coverage Flex configs |
| Y.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | [Satellite access] Service continuity LEO |
| Y.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 | [Satellite access] PWS Text Messaging |
| Y.1.11-1-4 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | Other non-terrestrial access] UE-HAPS-UE direct communications |
| Y.1.11-1-5 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | Satellite backhaul links selection/switching Should be moved out of this table (about SAT access), e.g. can go in “others” |
| Y.1.11-1-6 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization reference |
| Y.1.11-1-7 | Subject to operator’s policy, the 6G network with satellite access shall support onboard Service Hosting Environment(s) and mechanisms to m Minimise the necessary bandwidth of the inter-satellite and feeder links, and Modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, provide a computing service via a to UEs (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and subject to user consent, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. | PR 8.15.6-1 PR 8.9.6-1 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing It is suggested to seperate which are targeting different aspects. |
| Y.1.11-1-8 | Subject to operator’s policy and user’s consent, the 6G system with satellite access shall be able to support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) , provided by Service Hosting Environment on aboard satellite. | PR 8.9.6-3 |  |
| Y.1.11-1-9 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 |  |
| Y.1.11-1-10 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, | PR 8.18.6-1 | QoS Improvement |

Table Y.1.11-2 – Satellite-based positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-2-1 | The 6G system with satellite access shall be able to provide a mechanism for energy constrained UEs to determine their position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices |
| Y.1.11-2-2 | The 6G system with satellite access shall be able to provide a positioning service supported by the combination of 3GPP satellite and GNSS positioning technologies, including Network-based Assisted GNSS. | PR 8.11.6-1 | Support of NTN & GNSS positioning technologies |
| Y.1.11-2-3 | The 6G system with satellite access shall be able to provide different positioning service to different types of UE (e.g. energy constrained device, UAV, airplane) using satellite access, supported by single satellite based positioning method or hybrid 3GPP positioning methods. | PR 8.5.6-1 PR 8.7.6-1 PR 8.10.6-1 | Positioning Service based on satellite-based positioning technology |
| Y.1.11-2-4 | Subject to regulatory requirements, operator policy, and user’s consent, the 6G system with satellite access shall be able to determine UE location based on 3GPP positioning technologies for UE only using satellite access. | PR 8.5.6-2 | Determine UE location using satellite-based positioning |

Table Y.1.11-3 – Others

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-3-1 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization |
| Y.1.11-3-2 | The 6G system using with satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data) of a computing task between S within the Service Hosting Environments on board ) between satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite Data transfer between onboardof computing inforesources |
| Y.1.11-3-3 | Subject to operator’s policy and user’s consent, the 6G system with satellite access shall be able to share the data (e.g. the processed result based on the non-3GPP sensing data from multiple UEs) provided by the Service Hosting Environment on board satellite among multiple UEs (e.g. UAV). | PR 8.9.6-3 | Computing data sharing |
|  | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. |  | Routing modification |
|  | Subject to operator’s policy and agreement with 3rd party, the 6G network with satellite access shall be able to provide a computing service via a suitable support onboard Service Hosting Environment on board (s) satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, or |  | Onboard Computing Marked in yellow because of the controversial definition of SHE |
|  | Subject to operator’s policy, the 6G network with satellite access may support onboard Service Hosting Environment(s) and mechanisms to - Minimise the necessary bandwidth of the inter-satellite and feeder links, - Modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, - provide a computing service via a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and - subject to user consent, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. This implies some need to transfer computing information in/related to Service Hosting Environment (e.g. pre-processed data within the Service Hosting Environment) between satellites over a given area. | PR 8.15.6-1 PR 8.9.6-3 PR 8.9.6-4 PR 8.15.6-2 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing and transfer of computing service related info related to SHE |
|  | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access may be able to provide to logistic companies computing service in a given area via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.9.6-1 |  |
|  | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities | [PR 8.9.6-1] |  |

Y.1.12	Immersive Communication

Editor’s Note: This includes (but not limited to) PRs from clause 9.

Editor’s Note: PR 9.5.6-4, PR 9.8.6-1, PR 9.8.6-2, and PR 9.18.6-1 have Editor’s Notes associated with them, so they will not be consolidated into Table(s) Y.1.12-x at this time.

Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Table Y.1.12-1 – Immersive Communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.12-1-1 |  |  |  |
| Y.1.12-1-2 |  |  |  |
| Y.1.12-1-3 |  |  |  |
| Y.1.12-1-4 |  |  |  |
| Y.1.12-1-5 |  |  |  |
| Y.1.12-1-6 |  |  |  |
| Y.1.12-1-7 |  |  |  |
| Y.1.12-1-8 |  |  |  |
| Y.1.12-1-9 |  |  |  |
| Y.1.12-1-10 |  |  |  |
| Y.1.12-1-11 |  |  |  |
| Y.1.12-1-12 |  |  |  |
| Y.1.12-1-13 |  |  |  |
| Y.1.12-1-14 |  |  |  |
| Y.1.12-1-15 |  |  |  |

Table Y.1.12-1 – Immersive Communication related to IMS

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.12-1-1 |  |  |  |
| Y.1.12-1-2 |  |  |  |
| Y.1.12-1-3 |  |  |  |
| Y.1.12-1-4 |  |  |  |

Y.1.13	Massive Communication

Editor’s Note: This includes (but not limited to) PRs from clause 10.

Table Y.1.13-1 – Massive Communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

Y.1.14	Industry and Verticals

Editor’s Note: This includes (but not limited to) PRs from clause 11.

Editor’s note: The local network related PRs are also presented in “Resilience”, “Enhancements to legacy services and capabilities” groups. It is FFS whether to set up a new group for localized network.

Editor’s note: It is FFS how to consolidate the computing related PRs.

Table Y.1.14-1 – UAV, UAM and aircraft

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-1-1 | 6G system with satellite access shall provide a means to obtain and expose communication service interruption information (e.g. expected interruptions to the communication service) to authorized third party. | PR 11.5.6-2 | QC:[should go under Satellite CPRs] exposure |
| CPR Y.1.14-1-2 | Subject to regulatory requirements and user consent, the 6G network shall provide secure means to expose to an authorized 3rd party application the information related to a UAV or a UAM aircraft from the stored service data. | PR 11.7.6-2 | Exposure |
| Alternative CPR Y.1.14-1-2 | Subject to regulatory requirements and user consent, the 6G network shall provide secure means to store the service data for a UAV or a UAM aircraft based on the request information (e.g. service type, storage duration, time expiry) and expose the information to an authorized 3rd party application. . NOTE: The service data may be the processing data for a specific network service such as sensing data, positioning measurement data, or the exposed data such as sensing results, positioning information. | PR 11.7.6-2 PR 11.7.6-1 | Store and Exposure |
| CPR Y.1.14-1-3 | Subject to regulatory requirements and user consent, the 6G network shall be able to securely store the service data for a UAV or a UAM aircraft based on the request information (e.g. service type, storage duration, time expiry). NOTE: The service data may be the processing data for a specific network service such as sensing data, positioning measurement data, or the exposed data such as sensing results, positioning information. | PR 11.7.6-1 | Data storage |

Table Y.1.14-2 – Localized network

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-2-1 | Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize UEs owned by a trusted authorized 3rd party (e.g. vertical industry) to access services provided by a localized network (deployed by the PLMN operator for the authorized 3rd party). | PR 11.10.6-1 | Authorization |
| CPR Y.1.14-2-2 | Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize and authenticate UEs owned by a trusted authorized 3rd party to access services from the PLMN operator. NOTE: The realization or deployment of localized network may be an enhancement of NPN, PALS or other network topology options. | PR 11.10.6-2 | Authorization |
| CPR Y.1.14-2-3 | Subject to regulatory requirements and the operators’ policy, the 6G system shall provide the possibility of local authorization for a new device or UE joining the local network. | PR 11.11.6-3 | Authorization |
| CPR Y.1.14-2-4 | The 6G network shall support a mechanism to allow the operator or authorized 3rd party to perform local control of the localized network when the localized network loses connection with the PLMN network. | PR 11.10.6-3 | Local control |
| Alternative CPR Y.1.14-2-4 | The 6G network shall support a mechanism to allow the operator or authorized 3rd party to provide connectivity to a UE in a local network when the local network loses connection with the UE’s home PLMN network. | PR 11.10.6-3 |  |
| CPR Y.1.14-2-5 | Subject to regulatory requirements and the operators’ policy, 6G system shall be able to support (partial-)autonomous local networks for the communication between UEs and devices. NOTE: (Partial-)autonomous means that the management of local network can be done locally even in case of loss of connectivity to the mobile network. | PR 11.11.6-1 | Local control |
| Alternative CPR Y.1.14-2-5 | Subject to regulatory requirements and the operators’ policy, 6G network shall be able to support communication between 3GPP UEs in a local network. NOTE: In the requirements above, management of local network can be done locally in case of the local network loses connectivity with the PLMN network, e.g. based on PLMN configuration. | PR 11.11.6-1 |  |
| CPR Y.1.14-2-6 | Subject to regulatory requirements and operator policy, the 6G network shall allow the operator to provide the required control information including configurations to the local network that enables it to provide necessary configurations for the local communication and to operate also out of coverage of the mobile network. | PR 11.11.6-2 | Local control |
| Alternative CPR Y.1.14-2-5 | Subject to regulatory requirements and the operators’ policy, 6G network shall allow the operator to provide the required control and configuration information to support (partial-)autonomous local networks for the communication between UEs and devices also out of coverage of the 6G network. NOTE: (Partial-)autonomous means that the management of local network can be done locally even in case of loss of connectivity to the mobile network. | PR 11.11.6-1 PR 11.11.6-2 | Local control |
| CPR Y.1.14-2-7 | Subject to operator’s policy, the 6G network shall support means to enable communication between UEs and a service hosting environment in a local 6G network, managed by an authorized 3rd party, when the connection with the remote (non-local) network fails. NOTE 1: The local 6G network is composed of at least 6G RAN and core network of 6G which are deployed in the local area e.g. factory. NOTE 2: The above assumes agreement with the 3rd party and control by the core network of 6G system | PR 11.23.6-1 | Local control |
| Alternative CPR Y.1.14-2-7 | Subject to operator’s policy, the 6G network shall support means to enable communication between UEs and a service hosting environment in a local network, managed by an authorized 3rd party, when the connection with the UE’s PLMN network fails. NOTE 1: The local network is composed of at least RAN and core network which are deployed in the local area e.g. factory. NOTE 2: The above assumes agreement with the 3rd party and control by the PLMN core network | PR 11.23.6-1 |  |
| CPR Y.1.14-2-8 | The 6G system shall support a mechanism to allow the UE to discover and select the services provided by a localized network. | PR 11.10.6-4 | Service discover and select |

Table Y.1.14-3 – Mission critical services

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-3-1 | Subject to regulatory requirements and operator policy, the 6G system shall support MCX service continuity during UE mobility across inter-PLMN, as well as between PLMN and PSBN, (e.g. based on agreed QoS, priority and/orpre-emption settings). NOTE 1: PSBN is a network generally owned and operated by a public safety network operator and specially designed for public safety users (e.g. police, ambulance, fire service), having their own PLMN identity. |  |  |
| CPR Y.1.14-3-2 | The 6G system shall minimize interruption to ongoing MCX service during UE mobility across different access (e.g. terrestrial access and satellite access), and/or when MCX service switch between on-network and off-network operations with or without UE relay through UAVs, HAPS, and Satellites access. NOTE 2: The requirement above may include also ProSe-enabled access, where relay UEs are equipped with co-located routing functionality, capable of maintaining connectivity in dynamic and infrastructure-less environments. | PR 11.14.6-2 |  |
| Alternative CPR Y.1.14-3-2 | The 6G system shall minimize interruption to ongoing MCX service during UE mobility across different access (e.g. terrestrial access and satellite access), and/or when MCX service switches between on-network and off-network operations, with or without UE relay. NOTE 2: The requirement above may include also ProSe-enabled access, where relay UEs are capable of maintaining connectivity in infrastructure-less environments. |  | Mobility should be a separate requirement |
| CPR Y.1.14-3-3 | The 6G system shall enable an MCX UE to determine its position using multiple positioning methods, including network-assisted GNSS via satellite access, and to convey its position information to the MCX server, triggered by an event, or periodically. | PR 11.14.6-3 |  |
| CPR Y.1.14-3-4 | The 6G system shall enable the MCPTT Service to interwork with non-3GPP PTT Systems that are compliant with the PDT standards, in alignment with P25 and TETRA. |  |  |
| CPR Y.1.14-3-5 | The 6G system shall support interworking between the MCData SDS and PDT Short Data Service, in alignment with TETRA. | PR 11.14.6-5 |  |

Table Y.1.14-4 – Robotics

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-4-1 | Subject to operator policy and regulatory requirements, the 6G system shall be able to support UE initiated data sharing among multiple UEs (e.g. AMRs) initiated by a UE (e.g. AMR) considering the dynamics of the communication, e.g. frequent change of the involved UEs and the varied duration of the group communication to match the lifecycle of the task, etc. NOTE 3: The shared data can be fore example collaborative awareness data that a AMR shares with other AMRs in a factory, including real time control/manoeuvre plans. |  |  |
| CPR Y.1.14-4-2 | The 6G system shall be able to provide a means to expose its ability to meet the accuracy level of clock synchronization for a group of UEs (service robots) requested by trusted third party. | PR 11.12.6-1 | exposure |
| Alternative CPR Y.1.14-4-2 | The 6G system shall be able to provide a means to expose UE’s ability to meet the accuracy level of clock synchronization for a group of UEs (service robots) requested by trusted third party. | PR 11.12.6-1 | Exposure |
| Alternative CPR Y.1.14-4-2 | The 6G system shall be able to provide a means to expose, to a trusted 3rd party, clock synchronization information for a group of UEs (e.g., service robots), e.g. to determine if accuracy level requirements from the 3rd party can be met. |  |  |
| CPR Y.1.14-4-3 | The 6G system shall be able to provide a means to check its ability to meet the recovery time from direct network connection interruption, requested by trusted third party. | PR 11.12.6-2 | Exposure |
| Alternative CPR Y.1.14-4-3 | The 6G system shall be able to provide a means to check UE’s ability to meet the recovery time from direct network connection interruption and expose it to other UEs, requested by trusted third party. | PR 11.12.6-2 PR 11.12.6-3 | Check and exposure |
| CPR Y.1.14-4-4 | The 6G system shall be able to provide a means to expose its ability to meet the recovery time from direct network connection interruption, requested by trusted third party | PR 11.12.6-3 | Exposure |
| Alternative CPR Y.1.14-4-4 | The 6G network shall be able to provide a means to expose, to a trusted third party, information related to expected network recovery time (from direct network connection interruption/failure), , e.g. to determine if 3rd party QoS requirements can be met. | PR 11.12.6-3 | Exposure |
| CPR Y.1.14-4-5 | Subject to the operator’s policy and service agreement, the 6G system shall provide a suitable means that enables remote UE to operate under extreme conditions via indirect network connection. NOTE: UEs operating under extreme conditions include UEs under difficult networking conditions where more advanced mechanisms are needed, such as mining robots, carts and collaborative robots in an underground mining site. | PR 11.13.6-1 | indirect network connection |
| CPR Y.1.14-4-6 | While the remote UE is using multiple indirect network connection paths for a single traffic flow, the 6G system shall be able to minimize service interruption when one communication path to the network changes, e.g. an existing path is released and a new path is added. |  |  |

Editor’s Note: SA1 #112 discussions on Consolidation of Y.1.14-5 stopped here. The rapporteur’s initial consolidation proposals for the remaining CPR Tables have been removed and will be resubmitted in a separate TDOC. The CPR numbering will be adjusted as consolidation progresses.

Table Y.1.14-5 – Other

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-5-1 | The 6G network with AI capabilities should be able to collaborate with the AI capabilities in the authorized third-party (e.g.: split inference) to assist Real Time Digital Twin in meeting the required service performance. | PR 11.4.6-2 | Digital Twin |
| CPR Y.1.14-5-2 |  |  |  |
| CPR Y.1.14-5-3 |  |  |  |
| CPR Y.1.14-5-4 |  |  |  |
| CPR Y.1.14-5-5 |  |  |  |
| CPR Y.1.14-5-6 |  |  |  |
| CPR Y.1.14-5-7 |  |  |  |
| CPR Y.1.14-5-8 |  |  |  |
| CPR Y.1.14-5-9 |  |  |  |
| CPR Y.1.14-5-10 |  |  |  |
| CPR Y.1.14-5-11 |  |  |  |
| CPR Y.1.14-5-12 |  |  |  |