3GPP TSG-SA WG1 Meeting #112	S1-254501

17-21 November 2025, DALLAS, USA	was S1-254318

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0853 | rev | 2 | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the end-to-end latency via satellite in clause 7.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI 17, 5GSAT |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
|  | T |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | With the approved SP-241760,CR 0818 (S1-244784 ), the value of propagation delay via satellite in table 7.4.1-1 has been updated since version 17.13.0. But the content which has dependency on changed values in 7.4.2 was not change accordingly. Details in below: In Version 17.12.0, - The yellow highlighted parts in clause 7.4.2 align with table 7.4.1-1. - "One-way delay" used in the note aligns with the colon name in the table 7.4.1-1. Since version 17.13.0, - The yellow highlighted parts do not align with green highlighted part. - The colon name in the table 7.4.1-1 was changed, but the note keeps unchanged. There is no explanation about "One-way delay" in 22.261. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating end-to-end latency values and notes in clause 7.4.2, - Correcting a typo in table 7.4.2-1, - Updating format of clause 7.4.2 title. - Updating the value for GEO from 276 to 272 for alignment with R18, R19 and R20. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The end-to-end latency will not align with propagation delay as explained in the notes. Especially, the MEO case has a wrong value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2, 7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *****************   First Change ***********************

## 7.4	KPIs for a 5G system with satellite access

### 7.4.1	Description

Satellite access networks are based on infrastructures integrated on a minimum of satellites that can be placed in either GEO, MEO or LEO.

The propagation delay associated with these orbit ranges, for the UE to the satellite path, can be summarized in Table 7.4.1-1:

Table 7.4.1-1: UE to satellite propagation delay

|  | UE to serving satellite propagation delay [ms] [NOTE 1] |  | UE to ground Max propagation delay [ms] [NOTE 2] |
| --- | --- | --- | --- |
|  | Min | Max |  |
| LEO | 1 | 13 | 26 |
| MEO | 24 | 99 | 198 |
| GEO | 120 | 136 | 272 |
| NOTE1: The serving satellite provides the satellite radio link to the UE. The delay range for LEO is calculated at elevation angle 90° with 300 km and 10° with 1 500 km. The delay range for MEO is calculated at elevation angle 90° with 7 000 km and 10° with 25 000 km. The delay range for GEO is calculated at elevation angle 90° to 10° with 35 786km. NOTE2: delay between UE and ground station via satellite link; Inter satellite links are not considered |  |  |  |

### 7.4.2	Requirements

A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 277 ms end-to-end latency.

NOTE 1:	 5 ms network latency is assumed and added to UE to ground station via satellite propagation delay as defined in Table 7.4.1-1.

A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 203 ms end-to-end latency.

NOTE 2:	 5 ms network latency is assumed and added to UE to ground station via satellite propagation delay as defined in Table 7.4.1-1.

A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 31 ms end-to-end latency.

NOTE 3:	 5 ms network latency is assumed and added to UE to ground station via satellite propagation delay as defined in Table 7.4.1-1.

A 5G system shall support negotiation on quality of service taking into account latency penalty to optimise the QoE for UE.

The 5G system with satellite access shall support high uplink data rates for 5G satellite UEs.

The 5G system with satellite access shall support high downlink data rates for 5G satellite UEs.

The 5G system with satellite access shall support communication service availabilities of at least 99,99%.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | 1,5 Mbit/s/km2 | 150 kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld |
| Public safety | [3,5] Mbit/s | [3,5] Mbit/s | TBD | TBD | TBD | N/A | 100 km/h | Handheld |
| Vehicular connectivity (note 3) | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Vehicle mounted |
| Airplanes connectivity (note 4) | 360 Mbit/s/ plane | 180 Mbit/s/ plane | TBD | TBD | TBD | N/A | Up to 1000 km/h | Airplane mounted |
| Stationary | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | N/A | Stationary | Building mounted |
| Narrowband IoT connectivity | [2] kbit/s | [10] kbit/s | 8 kbit/s/km2 | 40 kbit/s/km2 | [400]/km2 | [1] % | [Up to 100 km/h] | IoT |
| Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 4: Based on an assumption of 120 users per plane 15/7.5 Mbit/s data rate and 20 % activity factor per user Note 5: All the values in this table are targeted values and not strict requirements. Note 6: Performance requirements for all the values in this table should be analyzed independently for each scenario. |  |  |  |  |  |  |  |  |

## *****************   End of Changes ***********************