3GPP TSG-SA WG1 Meeting #111 	S1-253020r1

Goteborg, Sweden, 25-29th August 2025

Title:	UC - Diversity of UEs for satellite access -UPDATES

Agenda Item:	8.1.2 System and Operation Aspects (FS_6G_REQ)

Doc type:	pCR

Source:	Thales, Novamint

Contact:	nicolas.chuberre@thalesaleniaspace.com

Abstract: This PCR proposes inputs to clause 5.10.2 of the TR 22.870 “Study on 6G Use Cases and Service Requirements”.

* * * First Change * * * *

# 5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services, exposure

Editor’s Note:  The proposed sub-clauses are provided as guidance from the rapporteurs based on the previously agreed EN (above).   If text is not agreed for any clause or sub-clause, these clauses/sub-clauses will be removed from the document.

…

## 5.10	Device support

Editor’s Note:  Text to be provided.  This clause to address potential (new) requirements for device diversity.

…

### 5.10.2	Diversity of UEs for satellite access

#### 5.10.2.1	Description

In the context of the 6G system with satellite access, the following is expected:

•	enhanced connectivity to smart phones and IoT user equipment with enhanced performances compared to 5G (data rate, coverage, throughput,  ..) in frequency bands below 7 GHz

•	enhanced connectivity to vehicle/building mounted user equipment (i.e. with Flat Panel Antenna) in frequency bands above 10 GHz. Vehicles from the automotive, public safety, transport (aeronautic, railways, drone, maritime), utilities, agriculture and media & entertainment sectors are assumed.

In order to meet a variety of consumer, enterprise and vertical use cases, the 6G system with satellite access shall be able to provide connectivity to a wide range of user equipment and their usage conditions.

In TS 22.261 [14], the Table 7.4.2-1: Performance requirements for satellite access in clause 7.4 “KPIs for a 5G system with satellite access” defines performances associated to a set of UEs and usage conditions.

However, these performance requirements need to be updated in the context of 6G.

#### 5.10.2.2	Potential New Requirements needed to support the use case

[PR 5.10.2.2-1]: The 6G system with satellite access shall support UEs with the following performances:

Table 5.10.2.2-1: Performance requirements for 6G satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | [8) kbit/s/km2 | [1.5] kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld | - |  |
| Vehicular connectivity (note 3) | [50] Mbit/s | [25] Mbit/s | TBD | TBD | TBD | [50] % | Up to 250 km/h | Vehicle mounted | - |  |
| High speed train connectivity (note 3a) | TBD Mbit/s per wagon | TBD Mbit/s per wagon | TBD | TBD | TBD | N/A | Up to [500] km/h | Train mounted | - |  |
| Large Vessel connectivity | TBD | TBD | TBD | TBD | TBD | TBD | Up to [50] km/h | Vessel mounted | - |  |
| Drone connectivity (note 3a) | TBD Mbit/s | TBD Mbit/s | TBD | TBD | TBD- | TBD | Up to [250] km/h | Drone mounted | - |  |
| NGSO Satellite connectivity | TBD | TBD | TBD | TBD | TBD | TBD | Up to [7] km/s | NGSO satellite mounted | - |  |
| Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 3a: assuming a UE with max size of 20 cm antenna aperture |  |  |  |  |  |  |  |  |  |  |

Editor’s note: The table above will be filled with new scenarios compared to 5G and with legacy scenarios for which the performance values need to be updated. The Table 7.4.2-1: “Performance requirements for satellite access” in TS 22.261 [14] is considered as a basis. Potential new scenarios include for example flying/surface drone-mounted, railway moving platforms and NGSO satellite-mounted devices.

* * * END of Changes * * * *