SA WG2 Meeting #S2-170	S2-2506119

25 - 29 August, 2025, Goteborg, Sweden

3GPP TSG-CT WG1 Meeting #155	C1-253724

# Bratislava, Slovakia, 19-23 May 2025

# Title:	LS on the path information in 5G ProSe multi-hop discovery procedures

# Response to:	-

# Release:	Release 19

# Work Item:	5G_ProSe_Ph3

Source:	CT1

To:	SA2

Cc:	-

Contact Person:

Name:	Zhou Xingyue (Joy)

E-mail Address:	zhou.xingyue@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

1. Overall Description:

For 5G ProSe multi-hop UE-to-UE relay discovery procedure with reference to TS 23.304 clause 5.8.6.3 (excerpted below), the following two questions to SA2 are raised:

Question 1: For both model A and model B discovery, given that in the path to a 5G ProSe multi-hop layer-3 end UE, the 5G ProSe multi-hop layer-3 end UE's first 5G ProSe layer-3 multi-hop UE-to-UE relay inherently stores the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE, why does the path information need to include the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE?

Question 2: In the 5G ProSe UE-to-UE relay discovery announcement message (for model A discovery), it specifies the path information includes the "source layer-2 ID of the discoverer ProSe end UE (yellow highlighted texts as below)". However, the term "discoverer ProSe end UE" pertains to model B discovery. Why does the announcing relay UE indicate this source layer-2 ID of the discoverer ProSe end UE in the path information, given that model A does not involve a "discoverer" end UE?

"

#### 5.8.6.3	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP PDU type

The 5G ProSe UE-to-UE Relay Discovery Announcement message (Model A) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and contains the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

-	For each user info in the Direct Discovery set:

…

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

… …

The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC and includes the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

…

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer UE.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

…

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the target 5G ProSe End UE matching the RSC includes the following the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

…

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

"

For 5G ProSe multi-hop UE-to-network relay discovery with reference to step 4a in clause 6.3.2.5.3 of TS 23.304 (excerpted below), the following third question to SA2 is raised.

Question 3: Why are the Layer-2 IDs included in the Multi-hop path info IE, together with the list of the User info IDs?

"

4a.	A 5G ProSe Intermediate UE-to-Network Relay sends the Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID and Layer-2 ID of Relays in the path that has relayed the Solicitation message. … …

"

2. Actions:

To SA WG2 group.

ACTION: 	CT1 kindly asks SA2 to provide answers to the three questions above.

3. Date of Next CT1 Meetings:

CT1#156                           25th August- 29th August 2025.....................Goteborg, Sweden

CT1#157                           13th October- 17th October 2025..................Sophia Antipolis, France

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2025..................India