SA WG2 Meeting #S2-170	S2-2506198

25 - 29 August, 2025, Goteborg, Sweden

Title:	[Draft] Reply LS on parameters for ProSe additional parameters announcement request message

Response to:	LS on parameters for ProSe additional parameters announcement request message (C1-254010/S2-2506121)

Release:	Release 19

Work Item:	5G_ProSe_Ph3

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	LaeYoung Kim

Tel. Number:	-

E-mail Address:	laeyoung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.304 CR#0555

# 1	Overall description

SA2 thanks CT1 for their LS on parameters for ProSe additional parameters announcement request message.

Regarding the questions from CT1, SA2 would like to provide the following answers.

Question 1: CT1 would like to get clarification from SA2 regarding what the usage of both relay service code and root relay info for ProSe additional parameter announcement request procedure is.

Answer 1 from SA2: All the Layer-2 links between Remote UE and Intermediate UE-to-Network Relay and between Intermediate UE-to-Network Relay and UE-to-Network Relay are established per RSC. Therefore, the Additional Parameters Announcement Request sent by the Remote UE can be routed up to the UE-to-Network Relay through each Layer-2 link associated with the RSC without the RSC and Root Relay Info.

Question 2: CT1 would like to ask SA2 if both relay service code and root relay info can be excluded from the message for protocol efficiency.

Answer 2 from SA2: Based on the SA2's understanding explained in Answer 1 above, both relay service code and root relay info can be removed from the Additional Parameters Announcement Request message.

As a result, SA2 decided to remove the RSC and Root Relay Info from the Additional Parameters Announcement Request message sent by the Remote UE and agreed the attached CR.

# 2	Actions

To CT1:

ACTION: 	SA2 respectfully asks CT1 to take above information into account.

# 3	Dates of next TSG-SA2 meetings

SA WG2#171	13 – 17, October 2025	Wuhan, China

SA WG2#172			17 – 21, November 2025		Dallas, USA