SA WG2 Meeting #170	S2-2506223

Goteborg, Sweden, 25 – 29 August 2025

Source:	Nokia

Title:	KI#1 Solution #9 updates

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes updates to solution #9 from TR 23.700-04, to address editor’s note for step 0 and to add impacts to entities.

# 1. Discussion

This contribution proposes updates to solution #9 from TR 23.700-04, to

- address editor’s note for step 0

- capture more description for step 1

- address editor’s note in step 2

- capture a new editor’s note related to UE mobility handling during data collection

- list impacts to the different entities

---------- Start of change ----------

## 6.9	Solution #9: High-level UE data collection procedure

### 6.9.0	High-level solution Principles

This solution is based on the following high-level principles:

-	UE model training (OTT) server acting as an AF contacts (via NEF) a "UE data collection NF in 5G Core" to request data collection from specific UEs (identified by Type Allocation Codes), for specific measurement Id(s) and possibly in specific area(s).

-	Configuration of the data collection in the UEs is done via Control Plane.

-	gNBs are responsible for selecting the UEs for data collection based on Type Allocation Codes provided by Training entity OTT server, taking user consent and local gNB information into account.

-	UEs are configured by gNBs for data collection, together with UP information, which then allows UEs to transfer collected data to "UE data collection NF in 5G Core" via User Plane.

### 6.9.1	Description

This solution addresses Key Issue #1.

A "UE data collection NF in 5G Core" is responsible for handling requests from UE model training servers. This NF could be mapped to existing NF, e.g. DCCF, or could be a new NF.

The request from UE model training server (acting as an AF) for data collection may contain specific target UE(s), specific geographical area(s) and specific measurement Id(s). The identification of target UE(s) is made via Type Allocation Code (TAC).

The "UE data collection NF in 5G Core" transfers the request to AMFs together with UP information (allowing UE(s) to address the "UE data collection NF in 5G Core"), The AMF then forward the request to appropriate gNBs.

The gNBs are responsible for selecting the UEs for data collection, based e.g. on other procedures running at gNB for the UEs, e.g. avoiding conflict with MDT or taking radio conditions into account. Once the UEs are selected, the gNBs provide the request to the UEs, together with the UP information.

The UEs then start data collection and data transfer to the UP entity provided in UP information.

### 6.9.2	Procedures

#### 6.9.2.1	Initiating UE data collection

The procedure is shown below:

Figure 6.9.2-1: High-level UE data collection initiation procedure

0.	During UE context setup, AMF provides RAN with Type Allocation Code (in the PEI) of the UE (obtained either from UDM or from the UE itself) and “user consent for data collection” information for the UE as retrieved from UDM.

Editor's note:	Impacts to RAN would need to be agreed by RAN WG2 and RAN WG3.

1.	UE model training server request for UE data collection is provided to "UE data collection NF in 5G Core" via NEF. The request contains Type Allocation Code(s) for the UEs to collect data from, geographical area(s) or cell(s) where to collect data, a time window for data collection and measurement Id(s) indicating the type of measurements expected. The NEF checks whether the UE model training server AF is authorized for this request. If the AF is authorized, the NEF maps the geographical area(s) into a list of TAIs and forwards the request to the “UE data collection NF in 5G Core".

Editor's note:	Whether measurement IDs or AIML feature IDs are provided is FFS.

2.	The "UE data collection NF in 5G Core", per operator configuration, checks whether the request from the UE model training server can be fulfilled, e.g. whether measurement Ids can be provided, and transfers the request to the appropriate AMFs, together with UP information indicating the intended destination of the collected data.

3.	The AMF forwards the request to appropriate gNBs. This uses non-UE associated NGAP signaling. The AMF provides the UP information to the gNB. The UP information provides enough information to the UE to be able to establish the PDU session in step 6 and they are transparently forwarded by gNB to the selected UEs in step 5.

4.	The gNBs are responsible for selecting the UEs based on information received in step 3 and information received in step 0, also taking into account local information at gNB. For example, the gNBs may take into account potential conflict with on-going MDT procedures or radio conditions associated with the specific measurement Id.

NOTE:	Whether and how gNBs take UE capabilities for UE data collection into account if up to RAN2.

5.	The gNB provides radio measurement configuration to the UE, together with the UP information. UP information provide enough information to the UE to be able to establish the PDU session in step 6 and they are transparently forwarded by gNB to the selected UEs.

Editor's note:	Impacts to RAN and UE for step 4 and step 5 would need to be checked with RAN WG2 and CT WG1.

6.	(if such PDU session is not already established) The UE establishes a PDU session for data collection taking the received UP information into account . This may use URSP.

7.	The UE starts collecting data and provides data to the "UE data collection NF in 5G Core" over user plane.

Editor's note:	How to handle UE mobility during data collection is FFS.

8.	The "UE data collection NF in 5G Core" can, if configured by the operator, check collected data, e.g. check that collected data is according to measurement Ids requested at step 2.

9.	The "UE data collection NF in 5G Core" transfers the collected data to the UE model training server.

### 6.9.3	Impacts on services, entities and interface

UE data collection NF in 5G Core:

-	new or existing 5GC NF supporting UE data collection

-	checking whether the request from the UE model training server can be fulfilled

-	transferring the request to appropriate AMFs, together with UP information indicating that the UE needs to send the collected data to the UE data collection NF in 5G Core

-	receiving the collected data and transferring it to the UE model training server

AMF:

- forwarding the data collection request received from the UE data collection NF in 5G Core to appropriate gNBs

- providing the UP information to the gNBs

gNB:

-	selecting the UEs for data collection based on Type Allocation Codes provided by Training entity OTT server, taking user consent and local gNB information into account

-	configuring UEs for data collection, together with UP information provided by the UE data collection NF in 5G Core

UE

-	proceed with data collection as configured by gNBs

-	transfer collected data to "UE data collection NF in 5G Core" via User Plane

---------- End of changes ----------