3GPP TSG|WG-SA2 Meeting #170 	S2-2506277

# Goteborg, Sweden, August 25-29th, 2025					       		          (revision of S2-2504766)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6290 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of Editor’s Note as CT3 is working on UDP options kind value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | clause 5.37.9.4 has the following editor’s note: Editor's note: The details for documenting and assigning a UDP option kind value for carrying media related information in IETF are pending collaboration with IETF. The kind value is being defined in CT3 working along with IETF as defined in TS 29.561 clause 22.3.3. 1) one octet Kind field with the value "TBD" from the set of the UNASSIGNED values defined in draft-ietf-tsvwg-udp-options [67], indicating the UDP option is for the Media Related Information; |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Useless EN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.37.9.4	Usage of UDP Option transferring media related information over N6

The mechanism described in this clause uses connect-udp in tunnel mode for On-path N6 signalling to establish a UDP tunnel between the UPF as HTTP/3 client and the AS as HTTP/3 proxy according to clause 5.37.9.2 to securely transfer media related information. This clause defines the differences with the mechanisms defined in clause 5.37.9.2 when media related information is carried over UDP Option.

The media related information is encoded into UDP-Option as defined in IETF draft-ietf-tsvwg-udp-options [202] and transferred through the UDP tunnel over N6, in which the inner UDP datagram contains end-to-end encrypted XRM media packet between AS proxy and UE and the outer UDP datagram between AS proxy and UPF contains UDP-Option carrying media related information (see Annex W).

NOTE:	The relevant UDP option kind value and the format of the media related information carried by the specific UDP-Option and exchanged between the UPF and the AS proxy is defined in TS 29.561 [132]).

The usage of UDP-Option transferring media related information over N6 is supported as defined in clause 5.37.9.2 with following differences:

-	When the AF provides On-path N6 signalling information for connect-udp, it may also provide an indication of UDP-Option in Protocol Description for transferring media related information. This is to instruct the PSA UPF that the media related information is carried in UDP-Option in UDP datagram over N6.

-	The inner UDP datagrams contain the unmodified payload for both UL and DL traffic and the media related information is included in the UDP option of the DL outer UDP datagram.

-	The UPF supports the HTTP/3 Client functionality for negotiation of security keys used for encrypting media related information carried in the UDP-Option. The UPF shall remove the UDP-Option carrying media related information before forwarding the XRM media packets over right PDU Session towards the (R)AN and the UE.

Editor's note:	Security keys for UDP-Option can be negotiated using connect-udp upgrade token in tunnelled mode between the UPF and AS proxy. The details are up to SA WG3.

NEXT CHANGE (2)

END OF CHANGES