3GPP TSG-WG SA2 Meeting #170	S2-2506284

Goteborg, SE, Aug 25 - 29, 2025	(revison of S2-250xxxx)

Source:	OPPO

Title:	Proposal on Interim agreements for KI#4

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#4 for the FS_Sensing_ARC TR 23.700-14.

# 1	Discussion

Solution#1, #3, #5, #18 are to address KI#4 based on TR 23.700-14 V0.2.0.

Here is a summary on the data collection related aspects of the above solutions:

- Solution #1: Direct On-Demand Collection, establishes direct connections to Sensing Entities only when needed. Uses direct links to request/receive raw sensing data + associated info (e.g., metadata).

- Solution #3: Framework with Functional Split, only high-level procedure defined, details deferred to specific implementations.

- Solution #5: AMF-Centric with Enhanced Interfaces, Requires enhancements to N1/N2 interfaces for sensing data transmission

- Solution #18: Control-Plane Orchestrated Data Connection, dedicated to resolve KI#4, with the following high-level principles.

1) The sensing data and associated information are transmitted between the Sensing Entity (i.e. gNB) and the Sensing Function via data connection. 2) The data connection establishment procedure is carried out on control plane signalling interaction. 3) The Sensing Function provides its connection information (e.g. IP address) for data connection and binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) to the Sensing Entity (i.e. gNB). 4) The Sensing Entity sends binding information via established data connection to sensing function for connection binding. 5) gNB is configured by the sensing function for sensing data and associated information collection over the data connection 6) For any matching sensing service request, sensing data and associated information can be directly collected from the data collection entity instead of Sensing Entity (i.e. gNB).

Apart from Solution#5, all other solutions assume sensing data and the associated information delivered over direct data connections between SF and gNB. Solution#18 also proposes sensing data collection from the data collection entity in addition to gNB. Additionally, Solution#18 proposed details on how data connection is established and configured.

More solutions dedicated for KI#4 are expected to be discussed and accepted on SA2#170; hence, contents of the interim agreements for KI#4 will be updated online.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.2.0.

* * * * Start of Changes * * * *

### 7.1.Y	Agreed Principles for KI#4

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The sensing data and the associated information are directly transmitted between gNB and the Sensing Function for result generation over the data connection.

If the sensing service request matches with any existing data collection service request, sensing data and the associated information can also be collected from the data collection entity to the Sensing Function for result generation over the data connection.

gNB is configured by the sensing function for sensing data and the associated information collection over the data connection.

If there’s no existing data connection between gNB and the Sensing Function, a data connection needs to be established and the sensing operations corresponding to a sensing service request needs to be bound to the data connection:

- 	The Sensing Function provides its connection information (e.g. IP address) for data connection and binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) to the gNB.

-	The gNB sends binding information over the data connection to the Sensing Function.

* * * * End of Changes * * * *