3GPP TSG-WG2 Meeting #170	S2-2506345

August 25 – 29, 2025, Göteborg, Sweden		(revision of S2-2504945)

Source:	Ericsson

Title:	KI#5, a new solution: Baseline for exposing sensing results to an AF

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution:

This contribution proposes a new solution for KI#5 that aims at providing a baseline for exposing sensing results to an AF. This solution is a revision of S2-2504945 that has not been handled at SA2#169.

# 1. Discussion

This document proposes a new solution for KI#5 that aims at providing a baseline for exposing sensing results to an AF.

# 2. Text Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | x |  | x | x | x |  |
| #2 |  | x |  |  | x |  |
| #3 | x | x | x | x | x | x |
| #4 | x |  |  |  |  |  |
| #5 | x |  |  | x |  |  |
| #6 | x | x | x |  |  |  |
| #7 | x |  |  |  |  |  |
| #8 |  | x |  |  |  |  |
| #9 |  | x |  |  |  |  |
| #10 |  | x |  |  |  |  |
| #11 |  | x |  |  |  |  |
| #12 |  | x |  |  |  |  |
| #13 |  |  | x |  |  |  |
| #14 |  |  | x |  |  |  |
| #15 |  |  | x |  |  | x |
| #16 |  |  | x |  |  |  |
| #17 |  |  | x |  |  |  |
| #18 |  |  |  | x |  |  |
| #19 | x | x |  |  | x |  |
| #20 |  |  |  |  |  | x |
| X | x |  |  |  | x |  |

>>>> Next Change (all new text)<<<<

## 6.X	Solution #X: Exposing Sensing Results to an AF

### 6.X.1	High-level solution principles

The solution is based on following principles:

-	AF invokes a sensing service request to an NEF. The NEF authorizes the AF, and, if authorized, selects a Sensing Core Function (SnCF). The SnCF is a new CN NF that handles the sensing service request in the core network.

-	The SnCF selects NG-RAN node(s) for the relevant Target Sensing Area of the authorized sensing service. The SnCF requests the selected NG-RAN node(s) to obtain the sensing data.

-	The NG-RAN node(s) provide sensing data to the SnCF for sensing result determination, if applicable, and/or for further exposure towards the AF.

### 6.X.2	Impacts on the Key Issues

This solution addresses KI#5 aspect related to how the core network exposes sensing data and/or sensing results to the sensing service consumer. However, the solution impacts also other key issue, and the impact could be the following:

-	KI#1: New core network functions are being considered – Sensing Core Function (SnCF) that handles sensing request in the core network, and NEF is used to expose the obtained sensing data and/or sensing results.

-	KI#2: AF authorization is performed by the NEF.

-	KI#3: NEF selects an SnCF, the SnCF selects NG-RAN node(s) to perform sensing data collection.

-	KI#4: NG-RAN node(s) determine the sensing result or transfer the sensing data to the SnCF for sensing result determination.

-	KI#6: SnCF includes the sensing operation specific parameters as inputs inside a sensing request to the selected NG-RAN node(s).

### 6.X.3.	Functional Description

The following scenario is considered for this solution. An application function (AF) invokes a sensing service to request the network to perform (i) object detection or (ii) object detection and tracking. The AF also may request a sensing result detail level to be exposed, i.e. sensing outcome (e.g., indication whether the object has been detected or not) or a more detailed description of what has been sensed by the 5GS (e.g. sensing data is a certain form). In this case, the AF acts as a sensing service consumer and a sensing service requester. The AF request is sent to the NEF.

The NEF selects a Sensing Core Function (SnCF) that will handle the sensing service request. The NEF invokes the corresponding sensing service operation to the selected SnCF. The SnCF selects a NG-RAN node(s) for the core network-requested sensing service and invokes the corresponding sensing service request(s).

The NG-RAN node(s) collects sensing data, determines the sensing result, if applicable, or provides the obtained sensing data to the SnCF for further sensing result determination and/or sensing data exposure.

NOTE X1:	Roles and functionalities of the involved NFs (e.g. NEF, SnCF and NG-RAN) can change depending on the resolution of the ENs in the procedure, see clause 6.X.4.

### 6.X.4	Procedures

Procedure for sensing result exposure in cases when a sensing service consumer (e.g., AF) invokes the sensing service towards a core network is shown in Figure 6.X.4-1.

NEF

NG-RAN

Sensing service consumer (e.g. AF)

SnCF

1. Sensing service request (e.g. Nnef_SensingEvent_Subscribe)

2. AF authorization and, if authorized, SnCF selection

3. Sensing service request (e.g. Nsncf_SensingEvent_Subscribe)

4. Selection of NG-RAN node(s) to handle the authorized sensing service

5. Sensing service request to the selected NG-RAN node(s) with the relevant parameters

6. Determining the resources required for the sensing and obtaining sensing data

7. Determining a sensing result

8. Providing the obtained sensing data and/or sensing result

9a. Sensing result determination

10a. Notifying the NEF (e.g., Nsncf_SensingEvent_Notify) about the sensing result

11a. Notifying the AF (e.g., Nnef_SensingEvent_Notify) about the sensing result

9b. Preparing sensing data for transferring to the NEF

10b. Providing the sensing data to the NEF

11b. Processing of the provided sensing data for caching and exposure

12b. Exposing the sensing data to the AF

Figure 6.X.4-1: Procedure of sensing result exposure to an AF.

1.	The Sensing service consumer (e.g., AF) invokes a sensing service request (e.g., Nnef_SensingEvent_Subscribe) to the NEF. Inside the request the AF includes the following mandatory and optional parameters:

-	mandatory: AF Identifier, External Target Sensing Area information (e.g., geographical area information and, optionally, altitude information), Type of sensing to perform (e.g., SensingEventType = “Object Detection”, “Object Tracking”), sensing result detail level (e.g., an indication about the detected object, parameter determined, and/or time and/or spatial resolution of the sensing data);

-	optional: event reporting mode (e.g., one-time, event-triggered or periodic reporting), sensing data exposure parameters (e.g., type of streaming and transport mechanism to be applied in case the streaming of the obtained sensing data is being requested).

NOTE X3:	Other parameters that AF can include in a sensing service request can be added during the later phases.

NOTE X4:	The exact names of service operations and semantics can be determined in the later phase.

2.	The NEF performs the authorization of the AF for the sensing service.

NOTE X5:	Data used for the AF-requested sensing service authorization and other details on how the NEF authorizes the AF request is outcome of KI#2.

Editor's note:	Whether the NEF performs the consent check per AF identity is FFS and needs coordination with SA WG3.

If the AF request for sensing service is authorized, the NEF maps the External Target Sensing Area information in the AF request to a 3GPP-internal Target Sensing Area information and then discovers and selects an SnCF to handle the sensing service request.

NOTE X6:	How the NEF selects an SnCF is outcome of KI#3.

Editor's note:	 How Target Sensing Area information needs to be described is FFS and requires coordination with RAN WGs.

3.	The NEF invokes the corresponding sensing service request (e.g. Nsncf_SensingEvent_Subscribe) to the selected SnCF and provides the relevant sensing service parameters from the AF request in Step 1.

4.	The SnCF selects NG-RAN node(s) for the relevant Target Sensing Area of the authorized sensing service.

NOTE X7:	How the SnCF selects NG-RAN node(s) to handle the AF-requested sensing service is covered by solutions for KI#3 of this study.

NOTE X8:	How the SnCF authorizes the sensing service operation, if applicable, is the outcome of KI#2 of this study.

5.	The SnCF sends the sensing service request to the selected NG-RAN node(s). Inside the request, the SnCF includes, for instance, Target Sensing Area information, type of sensing to perform, relevant parameters from the AF request in Step 1, and other parameters derived by the SnCF for the authorized sensing service.

6.	Based on the received sensing service request from the SnCF, the NG-RAN node(s) starts obtaining sensing data.

NOTE X9:	How sensing measurements are performed and how the NG-RAN node(s) obtain the sensing data are RAN WGs decisions.

7.	The NG-RAN node(s) may perform determination of a sensing result.

Editor’s note:	Whether NG-RAN nodes can perform determination of a sensing result is FFS and needs to be coordinated with RAN WGs.

8.	The NG-RAN node(s) provides the sensing result/ data to the SnCF.

NOTE X10:	How the NG-RAN node(s) provides the obtained sensing result/data to the SnCF is covered by KI#4 of this study.

9a.	The SnCF may determine the sensing result considering the sensing result detail level included in the AF request in Step 1.

NOTE X11:	How the SnCF uses the sensing data to determine a sensing result is up to implementation.

10a.	The SnCF may notify the NEF using the Nsncf_SensingEvent_Notify service operation about the determined sensing result.

11a.	The NEF forwards the received notification from the SnCF to the AF using the Nnef_SensingEvent_Notify service operation.

9b.	The SnCF may process the received sensing data considering the sensing result detail level from the AF request in Step 1,

10b.	The SnCF may provide the processed sensing data to the NEF for the exposure.

NOTE X12:	How the SnCF processes the sensing data considering the sensing result detail level is up to implementation.

NOTE X13:	How the SnCF provides the sensing data to the NEF is covered by KI#4 of this study.

11b.	The NEF may cache the provided sensing data and prepare the sensing data for exposure using the parameters for sensing data exposure from the AF request in Step 1.

12b.	The NEF may expose the sensing data to the AF according to the sensing result detail level and sensing data exposure parameters authorized for the AF request.

### 6.X.5	Impacts on existing services, entities and interfaces

AF:

-	acts as a Sensing Service Consumer by sending a sensing service request to the core network and consuming the sensing result and sensing data (if applicable).

Network Exposure Function (NEF):

-	performs sensing service consumer (e.g., AF) authorization;

-	selects a Sensing Core Function (SnCF) for handling the sensing service in the core network;

-	maps an External Target Sensing area information to an internal Target Sensing Area information;

-	may cache and expose the determined sensing result and/or sensing data, if applicable, to a sensing service consumer (e.g., AF).

Sensing Core Function (SnCF):

-	selects NG-RAN node(s) capable of sensing for a specific service;

-	may collect the obtained sensing data from the NG-RAN node(s) for determination of a sensing result;

-	may determine a sensing result based on the collected sensing data;

-	may process and provide the received sensing data to the NEF for exposure to the AF;

-	may notify (via NEF) the sensing service consumer (e.g., AF) about the sensing result.

NG-RAN:

-	may obtain sensing data based on the sensing service request from the SnCF;

-	may provide the collected sensing data to the SnCF;

-	may determine a sensing result.

>>>> End of Changes <<<<