3GPP SA WG2 Meeting #170	S2-2506366

# Gothernburg, Sweden, 25-29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0556 | rev |  | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of path information for Multi-hop U2U discovery for Non-IP type and for Multi-hop U2N Model B discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LS from CT1 S2-2506121 raised several questions related to the parameters present in the Multi-Hop U2U/U2N relay discovery messages. For the Multi-hop U2U discovery for Non-IP type, source-layer-2 ID of the discovoer UE is required (as was discussed in SA2#166 CR : S2-2412612), which should be further clarified in specifications; while for Model A this parameter was mistakenly included. For the Multi-hop U2N discovery with Model B, the path information only requires the User Info ID of the Relays and not their layer-2 IDs (as the Relays can just maintain a mapping of the Relays User Infos to their corresponding layer-2 IDs) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed Source layer-2 ID of the Prose End UE in the Multi-Hop Modela A discovery for Non-IP type - Clarified the use of Source layer-2 ID of the Prose End UE in the Multi-Hop Model B discovery for Non-IP type - Removed incorrectly added Layer-2 IDs from the Path infromation in In clause 6.3.2.5.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose messages may contain unneccessary parameters and usage of other parameters will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.6.3; 6.3.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

#### 5.8.6.3	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP PDU type

The 5G ProSe UE-to-UE Relay Discovery Announcement message (Model A) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and contains the following parameters for Multi-hop relaying in addition to as described in clause 5.8.4:

-	For each user info in the Direct Discovery set:

-	Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	(Optional) Hop-Limit: value that indicates the limit of the number of hops of the message.

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the source 5G ProSe End UE and includes the following parameters for Multi-hop relaying in addition to as described in clause 5.8.4:

-	Hop count: indicates the number of hops that the message is already relayed. It is set to 0 initially by the Discoverer End UE and is increased by 1 per hop.

-	(Optional) Hop-Limit: value that indicates the limit of the number of hops of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than or equal to the (pre)configured maximum number of hops.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC and includes the following parameters for Multi-hop relaying in addition to as described in clause 5.8.4:

-	Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	(Optional) Hop-Limit: value that indicates the limit of the number of hops of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than or equal to the (pre)configured maximum number of hops.

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer UE.

NOTE 1:	Source layer-2 ID of the discoverer UE (i.e. source 5G Prose End UE) is included so that first Multi-hop UE-to-UE relay can use it to send the future Response message to the correct destination (i.e. to the discoverer end UE).

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following parameters for Multi-hop relaying in addition to as described in clause 5.8.4:

-	Hop count: indicates the number of hops that the message is already relayed.

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the target 5G ProSe End UE matching the RSC includes the following the following parameters for Multi-hop relaying in addition to as described in clause 5.8.4:

-	Hop count: indicates the number of hops that the message is already relayed.

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

CHANGE 2

##### 6.3.2.5.3	Procedure for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B

Figure 6.3.2.5.3-1: General Procedures for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B

1.	The 5G ProSe Remote UE determines the Hop-Limit for discovery based on policy configuration (i.e. a mapping between maximum number of hops and RSC) or QoS requirements.

If the Hop-Limit is determined based on configuration associated with the RSC, the 5G ProSe Remote UE does not include the Hop-Limit in the Solicitation message. Otherwise, the 5G ProSe Remote UE includes the Hop-Limit in the Solicitation message.

2a.	The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe UE-to-Network Relay Discovery Solicitation message additionally contains following IEs compared with that in clause 6.3.2.3.3: an indication that Multi-hop relay is supported, Hop-Count and Hop-Limit (optional).

The Target Info may contain the User Info ID of the UE-to-Network Relay and Intermediate UE-to-Network Relay(s).

3a.	If an indication that Multi-hop relay is supported is contained in the received Solicitation message, the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1a, of a 5G ProSe Intermediate UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe Intermediate UE-to-Network Relay (if any), the 5G ProSe Intermediate UE-to-Network Relay may decide to send a 5G ProSe UE-to-Network Relay Discovery Solicitation message.

The 5G ProSe Intermediate UE-to-Network Relay shall drop the received Solicitation message when its own User Info ID is contained in the Path information of received Solicitation message, or if the hop count (corresponding to the number of Relays included in the message) has reached the Hop-Limit (if included in the received Solicitation message) or the (pre)configured maximum number of hops associated with the RSC (if no Hop-Limit is included in the received Solicitation message).

The 5G ProSe Intermediate UE-to-Network Relay may send a Response message when it has already found or established PC5 link with 5G ProSe UE-to-Network Relay(s), e.g. based on the unicast link profiles, without sending Solicitation message. i.e. steps 4a-7a are skipped and step 8a is performed directly. The response message additionally contains the User Info ID of UE-to-Network Relay, path information to the UE-to-Network Relay which is an (ordered) list of User Info ID of intermediate UE-to-Network Relay(s), optionally hop count and optionally Accumulated QoS for PC5 link, as described in clause 5.8.3.1.

If the same information on User Info IDs of Remote UE and UE-to-Network Relay is received from different ProSe UEs, the 5G ProSe Intermediate UE-to-Network Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g. hop count, delay, channel quality of received messages, etc.).

4a.	A 5G ProSe Intermediate UE-to-Network Relay sends the Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID of Relays in the path that has relayed the Solicitation message. The hop count is increased by 1. The solicitation message may include Accumulated QoS for PC5 link (e.g. sum of delay of each hop).

The Accumulated QoS for PC5 link, if present in the received solicitation message, will be updated to include the QoS of the PC5 link between the 5G ProSe Intermediate Relay and its child UE (Intermediate Relay or Remote UE).

2b.-5b.	The Solicitation message from the same Remote UE goes through a different ordered list of 5G ProSe Intermediate UE-to-Network Relays.

6-7. If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1, of the 5G ProSe UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe UE-to-Network Relay (if any), then the 5G ProSe UE-to-Network Relay responds to the 5G ProSe Intermediate UE-to-Network Relay with a 5G ProSe UE-to-Network Relay Discovery Response message. The 5G ProSe UE-to- Network Relay Discovery Response message additionally contains the path information compared with that in clause 6.3.2.3.3. The response message may include the received Accumulated QoS for PC5 link.

The 5G ProSe UE-to-Network Relay may choose the path based on e.g. the PC5 signal strength of each message received, hops to the Remote UE, the path information, Accumulated QoS for PC5 link, etc.

NOTE 1:	It is up to implementation how the 5G ProSe UE-to-Network Relay decides when to send the Relay Discovery Response message.

8-9.	When a 5G ProSe UE-to-Network Relay Discovery Response message is received, if the User Info ID of the receiving 5G ProSe Intermediate UE-to-Network Relay is included in the path information, the 5G ProSe Intermediate UE-to-Network Relay forwards the 5G ProSe UE-to-Network Relay Discovery Response message. The Response message additionally contains the path information which is a list of User Info IDs of 5G ProSe Intermediate UE-to-Network Relay(s) and the remote UE in the order (starting with the remote UE and ending with the last 5G ProSe Intermediate UE-to-Network Relay) of position in the path. Based on the path information, the Relay Discovery Response message is forwarded along the path, where each 5G ProSe Intermediate UE-to-Network Relay determines the next hop information (e.g. User Info ID and Layer-2 ID) to forward the Relay Discovery Response message. The response message may include the received Accumulated QoS for PC5 link.

10.	If the Remote UE receives multiple Discovery Response messages within a pre-configured time window, it may perform relay path selection based on e.g. path information, the PC5 signal strength between Remote UE and its neighbour Intermediate UE-to-Network Relay, Accumulated QoS for PC5 link and the number of hops to the 5G ProSe UE-to-Network Relay.

NOTE 2:	If the 5G ProSe Remote UE does not receive any response after a pre-configured time, based on application requirement, it may increase the Hop-Limit and send the discovery message again.

END of Changes