SA WG2 Meeting #170	S2-2506390

25-29 August 2025, Goteborg, Sweden	(Revision of 4652)

Source:	OPPO, MediaTek Inc., vivo

Title:	New solution: IMS call setup procedure without pre-condition for voice over GEO satellite

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution o IMS call setup procedure without pre-condition for voice over GEO satellite

1	Discussion

SA1 has concluded the following requirements and KPIs, the solution is to be further studied by Stage-2.

The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [xa].

The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| …… |  |  |  |  |  |  |  |  |  |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| …… Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [xb]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [xc]; |  |  |  |  |  |  |  |  |  |

In TS23.228 clause 5.7a.2, the procedure for the session establishment without pre-condition is introduced. These clauses present the general end-to-end session flow procedures without preconditions. The flow in clause 5.7a.2 is applicable to services without real-time QoS requirements before session becomes active, and thus do not need to set-up dedicated IP-CAN bearers during IMS call setup but can use existing (pre-established) IP-CAN bearers (e.g., default bearer), and to services which do not require that the terminating endpoint obtains a SIP-level notification when the originating endpoint's IP-CAN bearer becomes available.

Thus, procedure in this clause (no pre-condition) is applied to GEO voice scenario, with some further clarification/additions as below.

2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.1.0.

* * * * Start of Changes (All text new) * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| 1 |  | X |  |  |

6.x	Solution #X: IMS call setup procedure without pre-condition for voice over GEO satellite, AF as B2BUA

6.X.1	High level principle

This solution is to address Key Issue #2: IMS enhancement for GEO NB-IoT NTN access.

This solution is to realize IMS call setup without pre-condition for MO/MT UE connected via GEO, so as to avoid some SIP messages exchanged via GEO access and reduce the call setup time.

6.X.2	Description

Procedure in this solution is basically using IMS Call Setup without pre-condition for MO/MT UE via GEO, with the following clarification/change:

1)  Capability of GEO voice is negotiated during IMS REGISTER. For a UE supporting GEO voice, it needs to support Codec for GEO voice (e.g., Codec-2) and processing SIP INVITE with no pre-condition. For the IMS network allowing GEO voice to UE, it needs to support transcoding and B2BUA service when either MO or MT UE is connecting via GEO.

2) SigComp capability negotiation during IMS REGISTER.

3)  For “no pre-condition” during IMS Call Setup, the “Authorizing QoS Resource” is skipped and UE uses an existing IP-CAN bearer (e.g., default or pre-established) for media based on the media parameters received in the SDP offer/answer. The Establishment of EPS bearer for IMS voice is skipped.

NOTE 1: Since the NB-IoT device has at most 2 EPS bearers over UP, and the GBR bearer is not supported over GEO Satellite access, it is proposed to transmit both IMS signalling and Media via one or two non-GBR EPS bearer.

Editor’s Note: It depends on KI#1 to decide how to transmit the IMS signalling and media flow via EPS Bearers without Authorizing QoS resource during IMS Call Setup.

4)  B2BUA (B2B User Agent) can be used to support the interaction with normal MO/MT UEs. HSS provisions iFC to S-CSCF based on RAT type (GEO access) and UE’s GEO voice capability for involving AF acting as B2BUA in the UE’s IMS call setup signalling path.

5)  Estimation of call setup time:

-  SIP message size:

According to example in 3GPP TS 24.228 clause 7.2.3, the message size would be about

SIP INVITE: 1.4Kbyte; 200 OK: 0.8KByte.

NOTE: Given the codec is very simple for GEO voice, the SDP offer will be a basic one, the 1.4KB for SIP INVITE, and 0.8KB for 200 OK should be enough (No 183 in no pre-condition case).

Additionally, considering “SigComp” for compressed SIP used for Call setup, the compression efficiency of SIP message is about 2:1 (basic value), thus 0.5*SIP message size is the estimated SIP size after compression.

-  PDB (AN+CN)

Referring the 5QI=10 for satellite and the PDB (include AN and CN) is 1100 ms.

-  Data rate over GEO access

Referring the [1-3K] for UL and DL, the data rate for GEO in SA1 requirements

Given the solution assumes to associate IMS signalling and media data into existing bearer, i.e. no establishment during call setup, there is no time consumed for this part. Therefore, the estimation of total latency over GEO is

SIP INVITE (SDP Offer): latency = 0.5*1400*8/(1~3)Kbps+1.1s=2.97~6.7s

SIP 200 OK (Offer Response): 0.5*800*8/(1~3)Kbps+1.1s=2.17~4.3s

NOTE: The Ring can be sent in parallel with 200 OK which is not increase the total call setup time.

Then roughly estimate the transmission in IMS network side is about 1 second. And the transmission time for TN UE side is 2 second (with pre-condition).

The total E2E latency for call setup time is:

GEO_UE-to-TN_UE = (2.97~6.7s) + (2.17~4.3s) + 1s + 2s = 8.14 ~16s

GEO_UE-to-GEO_UE = 16.28 ~ 32s

Editor’s Note: the estimation of call setup time over GEO can be further evaluated

6.X.3	Procedure

6.X.3.1 IMS Register

Figure 6.X.3.1 IMS REGISTER for GEO voice capability negotiation

The purpose of capability negotiation is to determine the GEO voice service is allowed for the UE. Then accordingly, IMS NFs may perform transcoding configuration and B2BUA during the Call Setup/.

Step 1-2

When UE is connecting via GEO, the UE sends SIP Register to P-CSCF in which the SIP Header includes “Capability of GEO voice”. By indicating “Capability of GEO voice”, it means the UE supports codec(s) for GEO voice (e.g. Codec-2). The PANI header indicates “GEO access”.

Meanwhile, the UE may send capability of “SigComp” in SIP header to P-CSCF for Compressed SIP usage during call setup procedure.

When UE is no longer connecting via GEO, the UE performs (re-)RSGITER update to P-CSCF.

NOTE 1: the steps of S-CSCF discovery is not described, but following existing procedure.

Step 3-7

The existing procedure for authorization/authentication for IMS layer.

Step 8

S-CSCF sends Diameter message to HSS with the “GEO access” and “Capability of GEO voice”. The HSS downloads subscription data incl. service parameter and iFC to S-CSCF.

Step 9

Based on the subscription data retrieved from HSS. The S-CSCF may perform Service Control. Specifically, S-CSCF determines whether the UE is allowed for GEO voice service, and install the iFC for the B2BUA service in Call Setup procedure.

Then S-CSCF sends 200 OK to P-CSCF with “confirm of GEO voice”.

Step 10

The P-CSCF stored the “Confirm of GEO voice”, based on which the P-CSCF may instruct AGW to perform transcoding actions described in the Call Setup procedure. Meanwhile, if the UE sends SigComp capability in step-1, the P-CSCF also needs to responds to indicate applying SigComp for Compressed SIP message for Call Setup procedure.

The P-CSCF sends 200 OK to UE indicating the “Confirm of GEO Voice”.

Step 11-12

If the iFC is installed in S-CSCF, the S-CSCF sends REGISTER to AF to check the availability of B2BUA.

6.x.3.2 IMS Call Setup-MO

Figure 6.x.3.2 IMS Call Setup-MO (with B2BUA)

Step 4-13 is described as below for B2BUA. These steps can be skipped if no B2BUA is needed.

Step 1

MO UE connecting via GEO initiates SIP INVIATE with SDP Offer including GEO voice codec and no pre-condition setting parameters (e.g. Support/Require header does not include “precondition”).

Step 2-3

P-CSCF modifies the codec list in SDP Offer by adding legacy codec such as AMR/EVS, and send the INVITE* to S-CSCF

Step 4

By checking iFC (Filter Criteria), the SIP INVITE with no pre-condition are routed to AF supporting B2B User Agent.

Step 5

The AF, as B2B User Agent, terminates the original INVITE and initiate a new INVITE with pre-condition setting, e.g. “Support: precondition” in SIP header, and/or “a=curr:”, “a=des:”, “a=conf:” in SDP offer.

NOTE: The modification of codec list and transcoding configuration to MRF may also be performed by B2B User Agent. If so, the P-CSCF does not necessarily do the transcoding actions in step-2

Step 6

The AF sends the new INVITE with pre condition setting to S-CSCF, and S-CSCF sends the new INVITE to the MT side.

Step 8-11

Acting as B2BUA, the AF process the steps for media negotiation and resource reservation with the MT side. All the messages are transmitted via S-CSCF based on iFC.

Step 13

When receiving final 200 OK, the AF initiate a new SIP: 200 OK with no pre-condition setting.

Step 14-17

The 200 OK (SDP Answer) sends to UE, during which the P-CSCF configures the transcoding to Access GW.

Step 18

The UE uses an existing (e.g., default or pre-established) IP-CAN bearer for media based on the media parameters received in the SDP answer. The P-CSCF does not instruct PCRF to establish a new dedicated EPS bearer for voice.

6.x.3.3 IMS Call Setup-MT

Figure 6.x.3.2 IMS Call Setup-MT (with B2BUA)

Step 2-4, 8-11, 17-18 described below are for B2BUA. Those steps can be skipped if no B2BUA is needed.

Step 1-2

When receiving SIP INVITE (Legacy codecs in SDP offer, pre-condition setting) from MO, the S-CSCF forwards the INVITE to AF (B2BUA) based on iFC.

Step 3

The AF terminates the original SIP INVITE and initiate a new SIP INVITE (no pre-condition setting) for the MT UE.

NOTE: The modification of codec list and transcoding configuration to MRF may also be performed by B2B User Agent. If so, the P-CSCF does not necessarily do the transcoding actions in step-6.

Step 4-5

The new SIP INVITE is sent to P-CSCF via S-CSCF.

Step 6

The P-CSCF determines to modify the codec list by adding codec for GEO voice (e.g. Codec-2) in SDP Offer. Then it sends the INVITE with updated SDP* to the UE connecting via GEO.

Step 7

The UE uses an existing (e.g., default, or pre-established) bearer for media based on the media parameters received in SDP offer. The P-CSCF does not instruct PCRF to establish a new dedicated EPS bearer for voice.

Step 8-11

In parallel to step 4-7, the AF, as B2B User Agent interact, with MO side for media negotiation and resource reservation. All the messages are transmitted via S-CSCF.

Step 13

UE sends 180 Ringing when alerting user.

Step 14-

When user pick up the phone, the MT UE sends final 200 OK to S-CSCF/P-CSCF. The P-CSCF configures transcoding to Access GW.

Step 15-19

The AF initiates a new SIP 200 OK and sends to the MT side via the S-CSCF.

6.X.4	Impacts on services, entities and interfaces

MO/MT UE:

-  Support capability negotiation and indicating the GEO access in IMS REGISTER

-  Support Codec for GEO voice,

-  Support SIP INVITE with no pre-condition (reuse)

-  Support capability of compressed SIP (SigComp) negotiation (reuse)

P-CSCF

-  support capability negotiation in IMS REGISTER

-  Support Codec transcoding for GEO voice (reuse)

-  Support processing SIP INVITE with no pre-condition, such as skipping “Authorize QoS resource” during IMS call setup (reuse)

-  Support capability of compressed SIP (SigComp) negotiation (reuse)

S-CSCF

-  Support service control for GEO voice (reuse)

AF

-  Support B2BUA for interacting with normal MO/MT UE (reuse)

IMS HSS

-  Support new subscription parameter for GEO voice.

* * * * End of Changes * * * *