SA WG2 Meeting #170	S2-2506494

25 – 29 Aug, 2025, Goteborg, SE

Source:	Nokia, NEC

Title:	[WT#1.x] On energy-related aspects

Document for:	Agreement

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: proposes that an extra WT/KI exists for energy related specific features not covered by other WT/KI.

# 1. Justifications

WT#1 of the approved 6G SID in SP-250806 includes the need to assess what 5GS features not mentioned already in the WT#1 itself should be supported in 6G (“In addition to the work tasks above, this study will identify other 5G features that will be supported and/or adapted in 6G.”). This paper proposes that energy-related aspects ought to be considered.

Architectural Motivation: The 6G system should be designed in all aspects with Energy efficiency and sustainability criteria as a basis of the specification work. However, there are also some features, not already covered by other work tasks, that specifically are aimed at addressing energy saving, energy-related information exposure and other energy-related features (e.g. covered by stage 1 requirements), and this may require some ad hoc procedures, dedicated services and impacting or requiring some new NF(s). Since rel-21 will provide an opportunity for feature level alignment among UEs, it is important to identify e.g. any UE impacting aspects of featured that are aimed at e.g. exposing information or saving energy in the network.

Gap Analysis: The KI will discuss whether any change needs to be made to the way the existing EnergySys features are specified. The existing features related to exposure e.g. lack some more precise indication on energy consumption, or precise indication of which the UEs that cause consumption are. Ther may also be dependencies on whether any applicable APIs or procedures currently used by dedicated features underwent any changes in 6G which at present is unknown. Tighter UE and network coordination for the purpose of reducing Network Energy Consumption end to end (e.g. taking into account user preference) could also be considered and 6G study offers a unique opportunity for this.

Technical Impact:

- Dedicated NF(s) could be required or 5GS EIF evolved or alternative functional allocation to NFs used also for other reasons can be determined.

- RAN/CN interaction or UE-network interaction may be impacted to e.g provide indication of high energy consumption for a UE, or to collect user input.

- Interactions between Network Functions (including the PCF)

- Interactions between NFs (including the PCF) and OAM

- If the existing 5GS Energy-related solutions are evolved and if new energy-related solutions are defined for 6G, how these interwork with existing 5GS features (if applicable) needs to be documented.

**** First Change (all new text) ****

# Annex A.X. WT#1.x Energy-related aspects

This WT will study:

- Whether and how to support energy-related features and services, including e.g. identification of the causes of energy consumption and potential actions to address these causes. This may include identifying any new energy-related features and/or evolving or reusing existing 5GS energy-related features.

- Whether and how to enhance UE and network interactions and coordination for the purpose of reducing end-to-end Energy Consumption (e.g. taking user preference into account).

- Whether and how to address the mobility between 5G and 6G for any new energy features supported in 6G.

NOTE: it is expected this work will start after the baseline architecture assumptions have been agreed.

**** Second Change (all new text) ****

# 5.X. Key Issue #X: Energy-related aspects

This KI will study:

- Whether and how to support energy-related features and services, including e.g. identification of the causes of energy consumption and potential actions to address these causes. This may include identifying any new energy-related features and/or evolving or reusing existing 5GS energy-related features.

- Whether and how to enhance UE and network interactions and coordination for the purpose of reducing end-to-end Energy Consumption (e.g. taking user preference into account).

- Whether and how to address the mobility between 5G and 6G for any new energy features supported in 6G.

NOTE: it is expected this work will start after the baseline architecture assumptions have been agreed.

**** End of Changes ****