SA WG2 Meeting #170	S2-2506503

25 – 29 Aug, 2025, Goteborg, SE

Source:	Nokia, T-Mobile US

Title:	WT#1.2, QoS Framework - Study proposal for QoS aspects

Document for:	Agreement

Agenda Item:	WT#1.2, QoS Framework

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the study considerations relating to QoS aspects in 6G under WT1.2 defined in SP-250806.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

SP-250806 indicates WT#1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

In relation to the “QoS framework”, this paper proposes to consider the following inputs:

Architectural Motivation: Quality of Service (QoS) is a critical element to be handled when delivering services to the end user; this involves considering metrics such as latency, bandwidth, packet loss, jitter, etc. 3GPP has worked through generations on the topic and has provided tools/procedures helping the operator to provide corresponding SLAs (for e.g., GBR, non-GBR flows). The QoS framework is constantly evolving to cater new requirements like the QoS impact/implications introduced by Rel-19, Rel-20 features such as XR services, energy consumption decisions.

Gap Analysis: The change in generation is expected to lead to substantial traffic growth while new services are expected to be added to the system. The architecture has to support increased traffic without compromising on the quality of service and the user experience. System resources need to be handled with care and leverage any adaptiveness capabilities of services (e.g. immersive communications). Existing non-GBR resource type may not be sufficient for demanding services while GBR, DC-GBR resource type is not scalable for wide area deployments. 5G has already introduced notification control, alternative QoS framework as part of the GBR architecture to cater for the resource intensiveness of GBR, DC-GBR flows. However, these mechanisms imply additional signalling and are reactive besides not widely adopted. At the same time do not fully serve the purpose of leveraging the adaptiveness of services. For example, alternative QoS profiles are discrete and rigid; changing due to resource limitation to the best alternative QoS profile may imply important changes to the KPIs while from the resourcing point of view it could be possible to offer slightly lighter change to KPIs compared to the initial QoS profile. The study should focus on improved Quality of Service mechanisms, utilizing the capacity optimally and better leveraging the adaptiveness of applications and real time communication services (existing and/or new).

Technical Impact: How would this WT/KI influence:

-	Improvements to the 5G QoS framework are needed to ensure service differentiation while offering better quality of service by better leveraging the adaptiveness of applications.

-	Improvements on the RAN-6G Core interaction related to the 5G QoS architecture should be studied for adaptive applications.

**** First Change ****

# Annex A.X. WT#1.2.X Scope

WT 1.2.X - Enhancements of QoS framework in 6G

Study whether and how 6G shall support improved quality of service including:

-	Study whether and how to introduce a new resource type with better guarantee of QoS than the best effort approach of existing non-GBR and less resource intensive than the existing GBR, DC-GBR resource types to improve capacity utilization in the network while offering better QoS to adaptive applications.

-	Study whether and how to further improve QoS parameters by leveraging PLR instead of PER.

-	Study whether and how to support leveraging information exchanged by the AF/UE and the network on the observed QoE to improve capacity utilization, service offerings with better QoE, and adapt the QoS offered by the network.

**** Second Change ****

# 5.X. Key Issue #X (1.2.X): Support of enhancements of QoS framework in 6G

KI #X (1.2.X) - Enhancements of QoS framework in 6G

Study whether and how 6G shall support improved quality of service including:

-	Study whether and how to introduce a new resource type with better guarantee of QoS than the best effort approach of existing non-GBR and less resource intensive than the existing GBR, DC-GBR resource types to improve capacity utilization in the network while offering better QoS to adaptive applications.

-	Study whether and how to further improve QoS parameters by leveraging PLR instead of PER.

-	Study whether and how to support leveraging information exchanged by the AF/UE and the network on the observed QoE to improve capacity utilization, service offerings with better QoE, and adapt the QoS offered by the network.

**** End of Changes ****