3GPP TSG-SA WG2 Meeting #170	S2-2506613

Goteborg, Sweden, Aug 25 – 29, 2025                                                  (was S2-2505324)

Source:	Nokia

Title:	KI#2 new solution – Dynamic service adjustments based on Energy Behaviour Analytics

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#2 considering the energy consumption inputs from NWDAF.

1. Discussion

The proposed solution aims to implement dynamic service adjustments based on the learning outcomes from Energy Behaviour Analytics of NWDAF.

2. Proposal

It is proposed to agree to have the following text in 3GPP TR 23.700-67.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution# | 1 | 2 | 3 |
| --- | --- | --- | --- |
| X |  | x |  |

## 6.X	Solution #X: Dynamic service adjustments based on Energy Behaviour Analytics

### 6.X.0	High-level solution principles

PCF considers the following factors to determine the UEs and PDU Sessions for carrying out service adjustments.

- New Energy Behaviour Analytics of UE at NWDAF considers inputs related to EC from EIF, and existing UE related analytics described in TS 23.288. These outputs are consumed by the PCF for policy decisions for implementing service adjustments.

### 6.X.1	Description

Energy Behaviour Analytics involves NWDAF collecting inputs from UE communication and mobility analytics along with EC information from EIF and flagging the UEs into corresponding Energy Behaviour categories. Classifying the UEs into different Energy Behaviour categories enables to understand how the UEs are behaving in context to Energy Consumption and accordingly implement policy and charging decisions.

PCF receives Energy Behaviour category of the UE from NWDAF using Energy Behaviour Analytics. Operator configures differentiated polices at the PCF for each of the Energy Behaviour category; and accordingly, the PCF carries out the service adjustments which may result in changes in the AM, UE or SM policies for the UE and/or PDU Sessions.

Editor's note:	Whether and how to consider Energy information in communication with CHF pertaining to Service Adjustments is FFS.

### 6.X.2	Procedures

#### 6.X.2.0	General

Existing procedures are re-used to implement the service adjustments. However, PCF uses the Energy Behaviour Analysis for policy decisions.

#### 6.X.2.1	NWDAF -  Energy Behaviour Analytics

NWDAF gets the inputs from 5GC NF(s) including EIF, existing analytics output of UE Communication, mobility analytics and provides the Energy Behaviour Analytics as a new type of UE related analytics.

| Information | Description |
| --- | --- |
| EIF output | Energy exposure information from EIF |
| UE communication analytics | As per 23.288 |
| UE mobility analytics | As per 23.288 |

Energy Behaviour Inputs

Based on the inputs received, the NWDAF maps the UE to Energy Behaviour category (for e.g., Green, Orange, Red or High, Low, etc.).

The analytics outcome provided by the NWDAF include the Energy Behaviour statistics/predictions as defined in the below tables.

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies the UE(s) for which the statistic applies by a list of SUPIs, or a group of UEs by a list of Internal-Group-Ids defined in clause 5.9.7 of TS 23.501 [2]. |
| Energy Behaviour (1..max) | Energy Behaviour categories (e.g., Green, Orange, High, Low, etc.) |
| > Time period | Indicates the period for which the Energy Behaviour category applies to. |

Energy Behaviour Statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies the UE(s) for which the statistic applies by a list of SUPIs, or a group of UEs by a list of Internal-Group-Ids defined in clause 5.9.7 of TS 23.501 [2]. |
| Energy Behaviour predictions (1..max) | Energy Behaviour category (e.g., Green, Orange, High, Low, etc.) |
| > Confidence | Confidence of the prediction. |

Energy Behaviour Predictions

### 6.X.3	Impacts on services, entities and interfaces

PCF:

-	Performs policy control considering Energy Behaviour Analytics inputs from NWDAF.

NWDAF:

-	NWDAF implements Energy Behaviour Analytics and classifies the UEs into Energy Behaviour categories.

# * * * End of changes * * * *