3GPP TSG-WG SA2 Meeting #170	            S2-2506659

Göteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250XXXX)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6369 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Terminology alignment on local offloading information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The terminology of “local offloading management information” is not aligned in different clauses (e.g. local offloading control information, local offloading management information, local offloading information). 2. “local offloading management policy” is sent from PCF to SMF, and the “local offloading management information” is sent from SMF to I-SMF for authorize the services that can be offloaded by I-SMF. But in some sentences, the IE sent from SMF to I-SMF is named as “local offloading management policy”, which is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Align the terminology of “local offloading management information”. 2. Correct the use of “local offloading management policy” and “local offloading management information”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The terminology is not aligned and some terminologies are used in wrong way. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.6.1, 5.34.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.548 CR 0315 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS 23.502 CR 5538 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.34.6.1	General

This clause applies only in the case of non-roaming or LBO roaming as control of UL CL/Branching Point in VPLMN is not supported in HR case. It applies for the architectures described in clauses 5.34.4 and 5.34.5

If the local offloading management is applied for the PDU session, see clause 6.10 of TS 23.548 [130] and clause 5.34.11.

When the I-SMF is inserted into a PDU Session, e.g. during PDU Session establishment or due to UE mobility, the I-SMF may provide the DNAI list it supports to the SMF. Based on the DNAI list information received from I-SMF, the SMF may provide the DNAI(s) of interest for this PDU Session for local traffic steering and Local Offloading Management Information generated based on Local Offloading Management Policy received from PCF that indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of a DN to the I-SMF e.g. immediately or when a new or updated or removed PCC rule(s) is/are received. The DNAI(s) of interest is derived from PCC rules.

The I-SMF is responsible for the insertion, modification and removal of UPF(s) to ensure local traffic steering. The SMF does not need to have access to local configuration or NRF output related with UPF(s) controlled by I-SMF. Based on the DNAI(s) of interest for this PDU Session for local traffic steering and UE location the I-SMF determines which DNAI(s) are to be selected, selects UPF(s) acting as UL CL/BP and/or PDU Session Anchor based on selected DNAI and insert these UPF(s) into the data path of the PDU Session. If the Indication of considering N6 delay is received the I-SMF triggers N6 delay measurement by considering the N6 delay measurement assistance information as part of EAS Deployment Information as described in clause 6.2.3.4 of TS 23.548 [130].

When a UL CL/BP has been inserted, changed or removed, the I-SMF indicates to the SMF that traffic offload have been inserted, updated or removed for a DNAI, providing also the IPv6 prefix that has been allocated if a new IPv6 prefix has been allocated for the PDU Session.

From now on the SMF and I-SMF interactions entail:

-	Notifying the SMF with the new Prefix (multi-Homing case): the SMF is responsible of issuing Router Advertisement message. The SMF constructs a link-local address as the source IP address. The Router Advertisement message includes the IPv6 multi-homed routing rules provided to the UE to select the source IPv6 prefix among the prefixes related with the PDU Session according to RFC 4191 [8]. The SMF sends the Router Advertisement message to the UE via the PSA UPF controlled by the SMF.

-	N4 interactions related with traffic offloading. The SMF provide N4 information to the I-SMF for how the traffic shall be detected, enforced, monitored in UPF(s) controlled by the I-SMF: the SMF issues requests to the I-SMF containing N4 information to be used for creating / updating /removing PDR, FAR, QER, URR, etc. The N4 information for local traffic offload provided by the SMF to the I-SMF are described in clause 5.34.6.2.

-	Receiving N4 notifications related with traffic usage reporting: the I-SMF forwards to the SMF N4 information corresponding to UPF notifications related with traffic usage reporting; the SMF aggregates and constructs usage reports towards PCF/CHF.

NOTE:	How the SMF decides what traffic steering and enforcement actions are enforced in UPF(s) controlled by I-SMF is left for implementation.

The I-SMF is responsible of the N4 interface towards the local UPF(s) including:

-	the usage of AN Tunnel Info received from the 5G AN via the AMF in order to build PDR and FAR;

-	requesting the allocation of the CN Tunnel Info between local UPFs (if needed);

-	to control UPF actions when the UP of the PDU Session becomes INACTIVE.

-	provide Trace Requirements on the N4 interface towards the UPF(s) it is controlling, using Trace Requirements received from AMF.

SECOND CHANGE

5.34.11	Support for Local Offloading Management

When the Local Offloading Management is allowed based on SMF selection subscription data and the UE is within the local offloading management service area, an I-SMF supporting the Local Offloading Management is inserted between the SMF and the AMF, even if the UE is within the SMF service area. The local offloading management service area may be locally configured in AMF per S-NSSAI and DNN, or generated in the AMF based on the received I-SMF profile received from the NRF.

NOTE:	The I-SMF profile includes the local offloading management service area supported for the I-SMF.

When an I-SMF is inserted to support Local Offloading Management to manage edge computing information locally, EAS deployment information management, EASDF selection, configuration and DNS message handling are performed by I-SMF as described in TS 23.548 [130].

The Local Offloading Management Policy indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of a DN as specified in clause 6.4 of TS 23.503 [45].

The detailed procedures on I-SMF insertion during and after PDU session establishment and on EAS discovery are described in clause 6.10 of TS 23.548 [130]. The UL CL and UPF(s) may be inserted, changed, or removed by the I-SMF for the traffic offloaded for Local Offloading Management. For more detail handling (e.g. the interaction between I-SMF and SMF), refer to clause 6.10 of TS 23.548 [130].

End of CHANGES