3GPP TSG-WG SA2 Meeting #170	S2-2506707

Gothenburg, Sweden, Aug 25 - 29, 2025

Source:	Apple

Title:	KI#2, New Solution: AF notification and preference of energy related policy changes

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_ph2/Rel-20

Abstract of the contribution: New solution for KI#2 for AF to be notified of energy saving policy changes and for AF to provide energy saving preferences/restrictions to 5GC.

# 1. Introduction/Discussion

The Key Issue #2 (Support of service adjustments for the UE), explores whether and how to support policy control based on interaction with EIF. The PCF may make policy decisions related with energy efficiency and energy saving. Those policy decisions can result in service degradation to the UE e.g., by degrading the QoS, restricting the usage of certain RATs. However, the energy saving action for a particular UE related to an AF cannot be done in isolation by the network. If the network makes policy changes for applying energy saving actions, awareness of such policy changes at UE/user/Application level is required for the energy saving action to be effective. It is important that UE/user/application is also aware of the network intention such that actions can be taken that are complementing the network decision to reduce energy consumption.

The solution proposes ways by which an Application Function (AF) can interact with 5GC to provide energy saving preferences/notification and to be notified of any energy saving actions that network takes.

# 2. Text Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-67 v.0.2.0

First change

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| 8 | X |  |  |
| 9 | X |  |  |
| 10 | X |  |  |
| 11 | X |  |  |
| 12 | X |  |  |
| 13 |  | X |  |
| 14 |  | X |  |
| 15 |  | X |  |
| 16 |  | X |  |
| 17 |  | X |  |
| 18 |  | X |  |
| 19 |  |  | X |
| 20 |  |  | X |
| 21 |  |  | X |
| 22 | X |  | X |
| 23 |  |  | X |
| X |  | X |  |

Second change (all new)

## 6.X	Solution #XX: Preference and notification for energy saving policy changes

## 6.X.0	High-level solution principles

## The following are the key principles of this solution

- AF can subscribe to NEF/PCF event exposure to be notified on policy changes due to energy saving action. AF can also subscribe to get an early notification from PCF. This case occurs when the energy consumption has crossed an early threshold of energy consumption for a UE, but not the value for triggering an energy saving policy change by the network. Such notification thresholds can be negotiated between the AF and 5GC, thus giving the opportunity for AF to negotiate for a window of adjustment of the application/UE traffic before service degradation due to energy saving policies from the network kicks in. The notification from PCF may also indicate the time period for which energy saving policy may apply

- For a UE that has Energy saving indicator included in the subscription data, AF can provide an indication requesting  temporary exclusion from one or more service degradation decision (QoS degradation, restriction on RATs etc.) that 5GC may take as part of energy saving action and a time window for such preferences. AF can also inform if the UE is involved in background data transfer activity and hence appropriate energy saving actions may be applied for a certain time duration. PCF may consider this input before deciding on a policy change that affects the service

- Exclusion from service degradation is also allowed by PCF considering whether UE is involved in emergency call, relayed emergency call, whether UE has active MPS/MCS session, whether UE has attached for disaster roaming services even if there is no explicit AF request.

### 6.X.1	Functional Description

This solution addresses KI#2.

PCF may subscribe to EIF for energy consumption information for a UE, for a QoS Flow, a PDU Session or a network slice etc. When the energy consumption crosses a threshold, PCF may apply policy control decisions based on operator configured energy saving actions. This may result in service adjustment for the UE (e.g., by impacting QoS). However,  the energy saving action for the network cannot be done in isolation by the network. If the network makes policy changes for applying  energy saving actions, awareness of such policy changes at UE/user/Application level is required for the energy saving action to be effective. It is important that UE/user/application is also aware of the network intention such that actions can be taken that are complementing the network decision to reduce energy consumption.

For example, if the QoS is degraded for network energy saving and the application is not made aware, it may still try to operate in the same mode. This may result in increased energy consumption on video coding to compensate for the lower QoS it is now subject to.

An AF may subscribe to notification from PCF about a policy decision resulting in service degradation for a UE or an early notification from PCF about a potential future policy change. The AF here refers to an application function that is managing the UE, or the network slice or the application that is impacted by the service degradation decision by PCF. The AF may subscribe to event exposure notifications from PCF if it is a trusted AF or through NEF.

A service adjustment action may be the result of a Session Management policy as described above, but it could also be a result of an AM policy e.g., RAT de-prioritisation decision or UE policy (e.g., URSP changes). The key aspect is that AF can subscribe to be notified of a policy change or a potential future policy change considered for energy saving.

The subscription data for a UE may include an Energy Saving Indicator (Rel-19 definition) that the UE is subject to network energy saving operation.  The values of the Energy Saving Indicator point to energy saving behaviour configured in the NG-RAN and/or AM-PCFs of the PLMN. Such energy saving behaviours for a PLMN and the mapping of the Energy Saving Indicator values to the respective behaviour are operator-defined.

Subscription data in UDM is configured by operator through OAM provisioning. This is a static data field and may be provisioned during the time user signed up for the contract. Even though this subscription field provides an opportunity for the subscriber to get involved in energy saving initiative of the MNO in a meaningful way, there may be times when the subscriber wishes not to be impacted by a QoS adjustment or a RAT de-prioritisation configured by the MNO as part of energy saving action. This may be the case where AF has awareness of the user or applications’ intent and would want to bring that to PCF consideration while applying policy control. This can include the case when the AF can inform if the UE is involved in background or foreground data transfer activity and accordingly appropriate energy saving actions may be applied by PCF based on energy profiles.

AF may request a temporary relief for the UE being considered for energy saving action by 5GC. PCF considers AF request where it is possible to do so. In case the AF request cannot be accepted, PCF informs AF about that situation.

### 6.X.2	Procedures

6.X.2.1 AF subscribes to be notified of energy saving policy changes

Step 1: AF, using NEF EventExposure service,  subscribes to the Event Id for notification on EnergySaving Policy applied, and optionally an early notification threshold (at what % of energy consumption level for policy changes should the AF be notified)

AF also indicates whether notification on EnergySaving policy is required at granularity of UE, PDU Session or SDF.

Step 2:  If the AF is authorized , NEF invokes the PCF event exposure to subscribe to the event as described above

NOTE: NEF discovers the PCF responsible for UE AM and SM policies from UDM (for the given UE ID using Nudm_UECM_Get service)

NOTE 2: A trusted AF may directly invoke PCF even exposure service.

Step 3: PCF stores the subscription information.

Step 4: PCF subscribes to the EIF for energy consumption of the required granularity as per operator policy and AF request.

Step 5: EIF reports energy consumption information.

Step 6: PCF detects that Energy consumption has hit the early notification threshold requested by the AF.

Step 7: PCF generates the early notification event to NEF.

Step 8:  NEF forwards early notification event to AF.

NOTE: AF may, based on early notification, take actions that are out of scope of 3GPP to influence UE behaviour.

Step 9: Later on, EIF generates another event notification with energy consumption information is sent to PCF.

Step 10. PCF determines that it has to apply an energy saving policy and creates appropriate PCC rules.

Step 11: PCF generates the event notification to NEF to inform the policy change. Notification may also include the predicated time period for which the policy is likely to be applied. This may be locally configured in PCF, or according to operator policy or this may be a value indicating how often PCF is re-evaluating energy saving policies

Step 12: NEF forwards the notification to AF.

NOTE 2: AF may take further actions based on notification of policy change due to energy consumption.

6.x.2.2 AF request for temporary exclusion from service degradation

Step 0: This step is out of 3GPP scope and happens over application layer

UE informs the AF about its energy saving preferences/restrictions. This can be derived based on an application layer implementation and shared based on a protocol between UE and AF.

The energy saving preferences/restrictions can include information related to

- Whether UE does not want to be disrupted (either for the entire UE, or a PDU session, or a SDF corresponding to one or more applications).

- Whether UE is performing activity for which energy saving actions can be applied (at a UE level, PDU Session level or a SDF corresponding to one or more application) . This can also refer to whether an activity is a background data transfer that can be delayed or be subject to energy saving action

- The list of energy saving actions that UE can tolerate at the given granularity. The list can include e.g., QoS degradation, RAT de-prioritisation

Step 1:  AF invokes the NEF service Nnef_EnergySaving_Create providing the energy saving preferences/restrictions as inputs.

Step 2: NEF invokes Npcf_PolicyAuthorization service with the energy saving preferences/restrictions information.

Step 3: If the UE identified in the policy authorisation has a UDM Subscription indicating that subscription is subject to energy saving action, PCF stores the AF requested policy parameters.

Step 4: Later on, PCF is triggered to perform an energy saving action.

Step 5: If the intended policy is excluded by AF input on energy saving preferences/restrictions, PCF excludes applying the policy for the UE at the requested granularity.

NOTE: PCF excludes UEs performing emergency call, relayed emergency calls, MPS/MCS session or disaster roaming registration from energy saving actions , even if not requested by the AF.

Step 6: if the network conditions prevents any AF indicated UE preference to be taken into account, PCF decides to generate the notification to AF that policy change for energy saving operation is applied.

Step 7. PCF sends the notification to NEF.

Step 8: NEF forwards the event notification to the AF

### 6.X.3	Impacts on existing services, entities and interfaces

PCF:

- New event for Event Exposure service to notify subscribers about application of energy saving actions or provide early notifications.

- New input for policy authorization service to inform about energy saving preferences/restrictions for a UE , PDU session or Service Data Flow.

NEF:

- New event for Event Exposure service to notify subscribers about application of energy saving actions or provide early notifications.

- New service for AF to inform about energy saving preferences/restrictions for a UE , PDU session or Service Data Flow.

End of changes