SA WG2 Meeting #170	S2-2506802

25 – 29 Aug, 2025, Goteborg , SE

Source:	CEWiT, IITB

Title:	[WT#1.2, Network Exposure] Scope and KIs definition for Network Exposure in 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides detailed scope for WT #1.2 for Network Exposure Framework and proposes corresponding KI description.

# 1. Justifications

The 6G SID approved in TSG SA #108 has the following text in WT #1.2 corresponding to Network Exposure Framework to be supported for 6G

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

The 6G system is assumed to support and/or enhance most of the existing 5GS services and add some new services. Network Exposure is one of the key features of 5G that shall be supported in 6G with enhancements.

The objectives for the study on network exposure Framework can be as follows:

-	Optimization of existing exposure Framework of 5G. Eg: splitting complicated services and merging simpler small services to make them application friendly.

-	Exposure aspects for features and requirements introduced in 6G.

-	Enhancements in security aspects of exposure framework of 5G.

Requirement for optimizing existing exposure Framework is defined in SA1 TR 22.870 v0.3.1 as follows:

[PR 6.7.6-2] Based on user consent, operator policy and regulatory requirements, the 6G system shall provide an efficient way to expose information (e.g. change of QoS) to the application on the UE.

[PR 6.31.6-1] The 6G network shall support mechanism to provide 3GPP services to subscribers requested by user’s intent.

Few requirements for use cases introduced in 6G are defined in SA1 TR 22.870 v0.3.1 as follows:

[PR 5.5.5.2-1] Subject to regulation, operator(s) policy and user consent, the 6G network shall support procedure(s) to expose aggregated (non-sensitive/anonymized/non-personally identifiable) information related to UEs, served by the network, e.g. number of UEs in a geographical location, their mobility pattern, application usage trends, from the network to authorized 3rd parties without exposing UE identities.

[PR 7.9.6-2] Subject to user consent, regulation, and operator’s policy, the 6G network shall be able to provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party.

[PR 6.1.6-1] Based on operator policy, the 6G network shall provide suitable means to allow authorized third parties and/or UEs to retrieve availability information about computational resources (e.g. storage, AI processing units, xPUs information etc.) inside the Service Hosting Environment and to utilize these computational resources for running workloads on demand.

Requirement for security aspects of exposure services in 6G is defined in SA1 TR 22.870 v0.3.1 as follows:

[PR 5.5.7.3-1]    Subject to operator policy and regulatory requirements, the 6G system shall support privacy protection for any information exposure to a 3rd party.

This contribution proposes updates to WT #1.2 description of the approved SID to clarify the scope related to Network Exposure Framework and provides the corresponding Key Issues to be documented in the study TR 23.801.

Gap Analysis:

-	Few of the existing exposure services require higher 3GPP specification understanding by the application, which makes it much tougher, leading to reduced usage of exposure services of 5GS. Eg: The traffic influence service can be invoked for various purposes like traffic routing, subscription to get notified about events related to PDU sessions, request comprising AF relocation information, etc. This multi facet usage of this service makes it’s definition and usage much complex.[x3]

If this can be resolved in 6GS, there will be huge improvement in usage of exposure services. By then, there will be scope for monetization of exposure services.

-	The privacy protection and service authorization mechanism for exposure framework in 5GS can be enhanced in 6GS..

**** First Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x1]	3GPP TS 22.261: "Service requirements for the 5G system".

[x2]	3GPP TR 22.870: "Study on 6G Use Cases and Service Requirements".

[x3]	3GPP TS 23.501: "System architecture for the 5G System".

**** Next Change (all text are new)****

# Annex A.X. WT#1.2 Scope for network exposure in 6G

WT#1.2	Study how to support the network exposure in 6GS, which include:

-	Study how to optimize 5GS exposure services and make the services user friendly for applications using intent based networking in 6GS as specified in TS 22.261 [x1], TR 22.870 [x2].

-	Study how to support exposure services for the new feature and requirements in 6GS as per the service requirements as specified in TR 22.870 [x2].

-	Study how to support enhanced privacy protection and service authorization mechanism in exposure services of 6GS as per the service requirements as specified in TS 22.261 [x1], TR 22.870 [x2].

**** Next Change (all text are new)****

# 5.X1. Key Issue #1.2.X1: How to optimize 5GS exposure services and make the services user friendly for applications using intent based networking in 6GS

This Key Issue aims to study How to optimize 5GS exposure services and make the services user friendly for applications using intent based networking in 6GS as specified TS 22.261 [x1], TR 22.870 [x2].This study shall include

-	Optimizing the exposure services to reduce the complexity.

-	Making the exposure services user friendly for applications using intent based networking. Intent based networking is a AI agent based approach to understand the application’s interest and orchestrate the respective 6G services to cater the need of application.

# 5.X2. Key Issue #1.2.X2: How to support exposure services for the new feature and requirements in 6GS

This Key Issue aims to study how to support exposure requirements of 6G features, few as listed below:

-	Anonymized exposure

-	computational resources exposure

-	Sensing information exposure

-	other requirements in 6GS as per service requirements specified in TR 22.870 [x2].

# 5.X3. Key Issue #1.2.X3: How to support enhanced privacy protection and service authorization mechanism in exposure services of 6GS

This Key Issue aims to study how to support enhanced privacy protection and service authorization mechanism in exposure services of 6GS as per service requirements specified in TS 22.261[x1], TR 22.870 [x2].

**** End of Changes ****