SA WG2 Meeting #170	S2-2506840

25 – 29 Aug, 2025, Goteborg , SE

Source:	Samsung

Title:	[WT#1.1, all topics] Support of control signaling for connectivity and beyond connectivity services

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Scoping and justifications for Work task 1.1 to study the independent evolutions and deployment for connectivity services and beyond connectivity services

1. Justifications

WT#1.1 of the approved 6G SID in SP-250806 includes control signalling for connectivity services and/or beyond connectivity services as below.

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

In 5G systems, the control plane (e.g., NAS) is used to support not only the signalling related to connectivity services (e.g., mobility management, session management, authentication) but also used to support signalling related to other services (e.g., location service, UE Policy, UE configuration, etc.). As a result, the complexity of the control plane have increased along with the evolution and introduction of additional operator services (see Table 1).

Table 1 A list of container types supported by 5G NAS

Type of containers

SMS

LPP message container

SOR transparent container

UE policy container

UE parameters update transparent container

Location services message container

CIoT user data container

UPP-CMI container

SLPP message container

Service-level-AA container

Event notification

…

On the other hand, emerging operator services, such as AI/ML or sensing, have different characteristics (e.g., exchange of large size of data, long-lived connection) and the 5G control plane framework is limited to support them.

In 6G, when accommodating all operator services (including sensing, AI/ML, etc.) in control plane, the re-use of the 5G control plane design for 6G would be limited since the significant change is inevitable to support the emerging operator services with different characteristics.

Consequently, to simplify the control plane, the minimum set of services/functionalities to be supported over the control plane (e.g., mobility management, session management) can be identified first. For the identified minimum set of services/functionalities (e.g., mobility management, session management), the corresponding 5G control plane can be easily re-used and evolved.

Architectural Motivation: Compact control plane for 6G

- Easy to re-use 5G control plane: When the 6G control plane is designed to accommodate the minimum set of NAS functionalities (e.g., mobility management, session management), the 5G control plane can be easily re-used without significant changes.

- Focus on connectivity services in 6G: When the 6G control plane is designed to accommodate only the minimum set of NAS functionalities, the evolutions of the control plane can be focused on them (e.g., optimizations between RAN-CN).

On the other hand, beyond connectivity services, such as Sensing, Agentic AI, Data Collection, are expected to bring new business opportunities in 6G. To facilitate successful commercialization, beyond connectivity services should be able to be evolved and deployed independently from the connectivity services without compromise on operator controllability/visibility.

Architectural Motivation: A framework for operator services including beyond communication operator services

- Independent evolutions/deployment of operator services: Based on a new framework independent from control plane and user plane frameworks, beyond connectivity operator services can be evolved/deployed independently from the existing control plane and/or user plane.

- Customized transport mechanism: A dedicated transport mechanisms between UE and RAN (e.g., new radio bearers), RAN and CN (e.g., new RAN-CN interface), and UE and CN (e.g., dedicated logical connection between UE-CN) can be customized for beyond connectivity operator services (e.g., low protocol overhead, differentiated traffic handling, security aspect) and to enable independent evolutions.

- Common functionality: A set of common functionalities for operator service can be defined to avoid duplication across operator services and to facilitate evolution of operator services (e.g., connection control for dedicated logical connection between UE-CN, security, authorization, user consent, NF discovery, etc.)

Gap Analysis 1: Limitations of 5G control plane framework to support beyond communications operator services

- Large data over NAS: Negative impact on control plane signaling for essential services (e.g., MM) due to the exchange of large data for new operator services (e.g., CSI dataset for AI/ML, sensing measurements for sensing). Additional complexity on control plane to support large data transfer (e.g., segmentation).

- NAS Dependency: AMF needs to be updated to enable a new operator service (long time-to-market)

- Duplication: Since there is no common framework, every operator service defines its own functionalities including similar functionalities required across the services (e.g., control signaling and data transport)

Gap Analysis 2: Limitations of 5G user plane framework to support beyond communications operator services

- Data flow differences: data flows of operator’s services may not need to be transmitted to outside of the 3GPP network via UPF, which is different from the data flows for user communication service.

- Data interpretability: the data field in user plane is not interpretable therefore it’s difficult to be used in the RAN and core network e.g. AI/Sensing for communication.

- Complexity of user plane: User plane complexity can be increased to support beyond communications operator services

Consequently, 6G system shall support decoupling of operator services from the minimum set of services/functionalities in the control plane and the user plane. 6G system shall be able to support a new framework for operator services including beyond connectivity operator services.

Technical Impact: Compact control plane and a new framework for operator services including beyond communications operator services

- Compact 6G control plane: The minimum set of services/functionalities to be supported over the control plane (e.g., mobility management, session management) are identified first to enable re-use of 5G control plane as much as possible. Other services/functionalities than the identified minimum set of services/functionalities can be decoupled from the control plane.

- A new framework for operator services: A set of common functionalities for operator services (e.g., connection control for dedicated logical connection between UE-CN, security, authorization, user consent, NF discovery, etc.) are defined to avoid duplication across operator services and to facilitate evolution of operator services. Independent form the control plane and user plane, a dedicated transport mechanisms between UE and RAN (e.g., radio bearers), RAN and CN, and UE and CN (e.g., dedicated logical connection between UE-CN) can be designed and supported for beyond connectivity operator services (e.g., low protocol overhead, differentiated traffic handling, security aspect).

**** First Change ****

Annex A.X. WT#1.1 Scope

WT#1.1 Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including the following:

- Whether and how to identify a minimal set of non-access stratum functionalities for the control plane that does not get impacted by additional operator services.

- Whether and how to enable the introduction of a beyond connectivity operator service without impacting a minimal set of non-access stratum functionalities.

- Whether and how to develop a common framework (e.g., authorization, discovery, signalling) for UE-Core Network interaction to support operator services including beyond connectivity operator services

**** Second Change ****

5.X. Key Issue #X: Support of Control Signaling for Connectivity Services and Beyond Connectivity Services

The key issue will study support for control signaling between UE and Core for 6G for connectivity services and/or beyond connectivity services, including the following:

- Identify a minimal set of NAS functionalities (e.g., mobility management, session management) that does not get impacted by a beyond connectivity operator services.

- How to mitigate the impact on existing NAS functionalities when introducing a new NAS functionality

- How to support a common framework to support operator services including beyond connectivity services (e.g., Sensing, AI/ML) without impacting the minimal set of NAS functionalities

  - Investigate an architecture to support a logical connection between the UE and Core that is used to transfer control/data signaling for operator services

  - How to support control (e.g., establish, update or terminate) of the logical connection

  - How to transfer and route the control/data signaling between the UE and the service NF (e.g., NF for sensing) over the established logical connection

  - How to support mobility management for operator service

  - How to support authentication to support operator services

**** End of Changes ****