3GPP TSG-SA WG2#170	S2-2506876

25 – 29 Aug, 2025, Goteborg , SE	(was S2-250xxxx)

Source:	Qualcomm Incorporated, Deutsche Telekom, Intel

Title:	TR 23.700-14: FS_Sensing_ARC new solutions on SBI based sensing service operations

Document for:	Discussion/Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposed a new solution on the sensing service operations.

# 1.	Discussion

TR 23.700-14v0.2.0 has documented a solution #1 (clause 6.1) that supports sensing operations with a direct interface between the Sensing Function in the core network and the Sensing Entity(ies). For the sensing service operations over the direct interface, there were multiple options identified in clause 6.1.2.3:

-	define a point-to-point protocol over the direct connection for sensing.

-	define a new interface for the Sensing Entity, so that the Sensing Function can access over the direct connection;

-	allow the Sensing Entity to access the new interface offered by the Sensing Function over the direct connection.

Based on the approved RAN Rel-20 SID on ISAC (RP-251861), the focus would be on "gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case".

In this proposal, a solution with the "new interface" is presented, focusing on the case of a gNB based sensing entity(ies).

# 2.	Text proposal

It is proposed to agree the following changes to TS 23.700-14:

# >>>>BEGINNING OF CHANGES<<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #x |  |  | x | x |  | x |

# >>>>NEXT CHANGE<<<<

## 6.X	Solution #X: Support of Sensing service operations with SBI based direct interfaces

### 6.X.0	High-level solution Principles

The solution is based on the framework presented in Solution#1 (clause 6.1). The focus of the solution is on the sensing service operation over the direct interface for the RAN node based Sensing Entity(ies), with the following options:

-	a new service based interface for the Sensing Entity over the direct interface for the Sensing Function to access;

-	a new interface offered by the Sensing Function over the direct connection for the Sensing Entities to access.

It addresses the KI#3, KI#4, and KI#6.

### 6.X.1	Description

#### 6.X.1.1 General

As shown in clause 6.1.2.1, the Sensing Function performs the Sensing Entity discovery and performs the sensing service operation over the direct interface. Overall, the following procedures may be involved:

- 	registration of the Sensing Entity(ies) to the core network function to allow the discovery by the Sensing Function when needed;

-	discover the appropriate Sensing Entity(ies) by the Sensing Function for a specific sensing service request;

-	configuring the selected Sensing Entity(ies) for the sensing service operation;

-	acquiring the sensing data/results from the Sensing Entity by the Sensing Function.

When the Sensing Entity is a RAN node, there may be different options to support the sensing service operations over the direct interface, as described in clause 6.1.2.3. This solution focus on two of the options, wherein the "new interfaces" are service based:

-	define a new interface for the Sensing Entity, so that the Sensing Function can access over the direct connection, with the procedures defined in clause 6.x.2.1;

-	allow the Sensing Entity to access the new interface offered by the Sensing Function over the direct connection, with the procedures defined in clause 6.x.2.2.

Both options would achieve the same features for the sensing operation support. The differences are on whether Sensing Entity is acting as a service producer (as in the case of clause 6.x.2.1) or service consumer (as in the case of clause 6.x.2.2).

### 6.X.2	Procedures

##### 6.X.2.1 Sensing service operations based on SBI of the Sensing Entity

In this option, the RAN node based Sensing Entity offers a set of services-based interface, i.e. the new interface: Nse_SensingService_Request/Response/Notify.

The operation example is illustrated in Figure 6.X.2.1-1.

6.X.2.1-1. Sensing operations using SBI of the Sensing Entity

1-2. The Sensing Entity registers itself to the NRF after establishing a direct connection to the core network that allows the use of Nnrf_NFManagement_NFRegister service. The establishment of the direct connection can be based on configurations from the OAM.

Alternatively, the Sensing Entity may register itself to the Sensing Function over the direct interface established for the sensing service operation as described in clause 6.1.2.1. The Sensing Function may in turn register the Sensing Entity to the NRF.

The registration can include the information about the Sensing Entity's capability, served areas, supported sensing method, etc.

3.	Sensing Function receives a sensing service request. This corresponds to the step 0 of Figure 6.1.2.1-1.

4 -5. The Sensing Function discovers the corresponding Sensing Entity(ies) required for a sensing service by querying the NRF, e.g. using Nnrf_NFDiscovery_Request service. The query can be based on the target sensing area, type of sensing service request, etc.

6. Sensing Function invokes the Sensing Entity service, e.g. Nse_SensingService_Request, via the direct interface. This sensing service request also contains the corresponding configuration information for the Sensing Entity(ies) to carry out the sensing operations, e.g. type of sensing operation, periodicity, type of sensing data/results requested, level of QoS for the sensing operations, etc.

7.	Sensing Entity(ies) performs the sensing operations. This corresponds to step 5 of  Figure 6.1.2.1-1.

8. Sensing Entity provides sensing data/results via a response: e.g. Nse_SensingService_Response, according to the configurations received in step 6.

9. (optional) If an event-based or periodic sensing data/result report was configured in step 6, the Sensing Entity may performs additional reporting to the Sensing Function over the direct interface when the criteria are met, using Nse_SensingService_Notify.

##### 6.X.2.2 Sensing service operations based on SBI of the Sensing Function

In this option, the Sensing Entity access the services of the Sensing Function over by the SBI of the Sensing Function via the direct connection.

The operation example is illustrated in Figure 6.X.2.2-1.

Figure 6.X.2.2-1. Sensing operations using SBI of the Sensing Function

1.	The Sensing Entity establishes a direct connection to the Sensing Function, i.e. not going through the AMF. The Sensing Entity may establish this connection based on configurations from OAM, or based on a triggered from the Sensing Function as in step 2 of Figure 6.1.2.1-1.

The Sensing Entity may also use the NRF to locate a Sensing Function to establish a connection with.

2 -3. 	The Sensing Entity uses Nsnf_SensingEntity_Register service to register itself and its capabilities, e.g. sensing service operation supported, sensing areas supported, etc., to the Sensing Function.

4.	The Sensing Entity subscribe to the Sensing Function, according to the sensing services it would provide, e.g. Nsnf_SensingEntity_ServiceEvent_Subscribe. Each of the Service Events may represent a target sensing area, a sensing method, or sensing result type combination. The Sensing Function may use the capability information or network configurations to determine if such subscribe request can be accepted.

5.	The Sensing Function received a sensing service request, as step 0 and 1 of Figure 6.1.2.1-1.

6.	Based on the sensing service request, the Sensing Function generates corresponding service event, e.g. corresponds to a target sensing area, sensing result type, etc. The Sensing Function uses Nsnf_SensingEntity_ServiceEvent_Notify to trigger and configure sensing entity for a sensing service, to the Sensing Entities that have subscribed to the corresponding event, as in step 4. This is an implicit Sensing Entity selection process. In this step, the Sensing Function can also include corresponding configurations for the Sensing Entity operations.

7. 	Sensing Entity(ies) performs the sensing operations. This corresponds to step 5 of  Figure 6.1.2.1-1.

8-9. The Sensing Entity(ies) invokes the Nsnf_SensingEntity_ServiceReport to report sensing data/result to the Sensing Function.

### 6.X.3	Impacts on Services, Entities and Interfaces

Depends on the solution options, RAN node based Sensing Entities needs to support:

- 	in case of option in clause 6.x.2.1, a service based interface for the RAN node based Sensing Entity to offer sensing service; or

- 	in case of option in clause 6.x.2.2, allowing the RAN node based Sensing Entity to directly access the SBI offered by the Sensing Functions over the direct interface without going via AMF.

Therefore, the applicability of the solution has dependency on the availability of the SBI support for RAN node.

Editor's Note: The impacts to the system architecture and interfaces need to be reviewed by RAN WGs.

# >>>>END OF CHANGES<<<<